3GPP TSG-RAN WG2 Meeting #120		R2-2211320

Toulouse, France, 14th Nov– 18th Nov, 2022

Agenda Item:	8.15.2

Source:	vivo

Title:	Further discussion on RAN2 impact due to SL LBT

Document for:	Discussion and Decision

Introduction

In RAN2#119bis-e meeting, the following agreements were made regarding consistent SL LBT failure [1]:

Agreement on consistent LBT failure:

1: 	SL-specific LBT failure indication from PHY is needed for SL-specific consistent LBT failure detection in the MAC. How/whether it is used for other purposes can be further discussed.

2:	Support SL-specific consistent LBT failure detection and recovery procedure in the MAC for SL-U. Details of recovery to be further worked on granularity of (consistent) LBT failure.

3:	Send LS to RAN1 asking “When an SL-specific LBT failure indication is notified for an SL transmission by the PHY, in which resource granularity the SL-specific LBT failure can be considered as being detected (e.g. per Resource Pool, per RB set, per SL BWP, etc.)?

- Detailed wording can be discussed during the email discussion. Some background information (e.g. why/what we (actually) ask) can be also provided.

4:	As the general principle, reuse the consistent LBT failure detection procedure in NR-U as the baseline for SL-specific consistent LBT failure detection in SL-U.

5:	As in NR-U, introduce the following parameters and variables for the SL-specific consistent LBT failure detection in SL-U as the baseline:

- An SL-specific LBT failure indication counter (e.g. SL_LBT_COUNTER);

- An SL-specific maximum LBT failure instance count threshold (e.g. sl-LBT-FailureInstanceMaxCount);

- An SL-specific LBT failure detection timer (e.g. sl-LBT-FailureDetectionTimer).

6:	Reuse the following MAC behaviors on TIMER/COUNTER handling in NR-U for SL-specific consistent LBT failure detection procedure in SL-U as the baseline:

- As in NR-U, if an SL-specific LBT failure indication is received from the lower layer, the SL-specific LBT failure indication counter (e.g. SL_LBT_COUNTER) is incremented by one.

- As in NR-U, if an SL-specific LBT failure indication is received from the lower layer, start or restart the SL-specific LBT failure detection timer (e.g. sl-LBT-FailureDetectionTimer)

- As in NR-U, if the SL-specific LBT failure indication counter value is equal to or larger than the SL-specific maximum LBT failure instance count threshold (e.g. sl-LBT-FailureInstanceMaxCount), consistent LBT failure is triggered/declared by the MAC entity.

- As in NR-U, if the SL-specific LBT failure detection timer (e.g. sl-LBT-FailureDetectionTimer) expires, the SL-specific LBT failure indication counter (e.g. SL_LBT_COUNTER) is reset to 0.

- As in NR-U, if the maximum LBT failure instance count threshold (e.g. sl-LBT-FailureInstanceMaxCount) or SL-specific LBT failure detection timer (e.g. sl-LBT-FailureDetectionTimer) is reconfigured, SL-specific LBT failure indication counter (e.g. SL_LBT_COUNTER) is reset to 0.

7:	Support the mechanism that a mode-1 UE can indicate the SL-specific consistent LBT failure to the gNB. FFS on a mode-2 UE in RRC_CONNECTED.

In this contribution, we will address the above leftover issues and discuss some further RAN2 impacts due to sidelink LBT.

Discussion

Consistent LBT failure handling

As per the agreements from last meeting, there is an FFS point on whether to support a mode-2 UE in RRC_CONNECTED indicating consistent SL LBT failure to the gNB. Considering that an RRC_CONNECTED UE in mode-2 is also provided with dedicated SL resource configurations by the gNB which may reconfigure/adjust the resources according to the actual radio/channel condition experienced by the UE, we think it could be helpful for an RRC_CONNECTED UE in mode-2 to indicate consistent SL LBT failure (if triggered and not cancelled) to the gNB as well, in order to facilitate the gNB’s resource reconfiguration for the UE’s recovery. Therefore, we propose to support the indication of consistent LBT failure to the gNB also for a mode-2 UE in RRC_CONNECTED.

- A mode-2 UE in RRC_CONNECTED can indicate consistent SL LBT failure to the gNB.

In the offline discussion in [2] last meeting, some companies proposed that consistent SL LBT failure detection can be at a per DST/per unicast link/per cast type level to realize finer granularity of SL LBT failure management. Although the granularity of the consistent SL LBT failure detection and recovery procedure that are related to the resource granularity is pending RAN1 feedback, whether a per DST/per unicast link/per cast type consistent LBT failure handling is a RAN2 level discussion. On the one hand, it is noted that the resource configuration in NR SL is common to all cast types/DSTs/unicast links since Rel-16 (where this issue was intentionally discussed in Rel-16 NR V2X), which is a principle highly likely to be inherited to SL-U. From this perspective, there is no clear motivation/logic to take consistent SL LBT failure detection as a cast-type/DST/unicast link specific operation, while still keeping the resource configuration common to all of them. On the other hand, such per cast-type/per DST/per unicast consistent LBT failure detection may require the MAC entity to determine for which DST/unicast link/cast type each SL LBT failure instance is detected, and may further require the MAC entity to maintain multiple sets of COUNTERs and TIMERs, with each set corresponding to a given DSTs/unicast links/cast types respectively. This would significantly increase complexity for UE implementation.

In addition, since all DSTs/unicast links/cast types are sharing the same resource configuration, there is the possibility that such a per DST/per unicast link/per cast type consistent LBT handling may defer the detection of consistent SL LBT failure on the intended resources, resulting in the consistent SL LBT failure recovery procedure unable to be carried out timely (e.g. when none of the cast type/DST/unicast link reaches its own maximum count number, but the total accumulated number of SL LBT failure instances is already big on the related resource set).

Considering unclear benefit being observed as well as the extra complication brought to UE implementation, we do not support to introduce such a per DST/per unicast link/per cast type consistent LBT failure detection and recovery procedure for SL-U. Therefore, we propose that the per DST/per unicast link/per cast type consistent SL LBT failure detection or recovery is not supported for SL-U.

- The per DST/per unicast link/per cast type level consistent SL LBT failure detection or recovery is not supported for SL-U.

In NR-U, when consistent LBT failure is triggered, the UE may trigger consistent LBT failure reporting and/or attempt to perform accessing to other resources, e.g. autonomous switching to another UL BWP (for SpCell only). Specifically, if the LBT MAC CE is reported w/o LBT failure indicated by L1, the UE shall cancel all the triggered consistent LBT failure(s) in SCell(s) for which consistent LBT failure was indicated in the transmitted LBT failure MAC CE. If random access attempt is considered to be successful after BWP switching on the SpCell, the UE shall cancel all the triggered consistent LBT failure on the SpCell.

Specifically, for SCell(s) in NR-U, the basic logic is that the UE informs the gNB of the SCell(s) that are experiencing consistent LBT failure via another cell (SCell or SpCell) where consistent LBT failure does not happen (thus enabling a transmission of the LBT failure MAC CE to be really performed). Then, it is up to the gNB how to recover the consistent LBT failure on the SCell where consistent LBT failure is happening (e.g. signaling a BWP switching, reconfigure/activate other SCell(s), etc.), and thus the triggered consistent LBT failures on the SCells can all be cancelled accordingly.

This logic for SCell(s) in NR-U can be applied as a baseline for an RRC_CONNECTED in SL-U, because according to the current WID description [4], an RRC_CONNECTED UE operating on SL-U carrier is scheduled/configured by the gNB in licensed band, meaning that the UE will use Uu carrier to report the consistent SL LBT failure detected on the SL-U carrier. Thanks to this licensed band assumption for Uu, when consistent SL LBT failure is triggered, an RRC_CONNECTED UE can easily report it to the gNB without performing LBT procedure on Uu, which is also agreed by all companies in [1]. Therefore, we propose that similar cancellation condition as to the SCell in NR-U can be applied as a baseline to consistent SL LBT failure handling for an RRC_CONNECTED UE in SL-U.

- As in NR-U, RAN2 agree the following cancellation condition for consistent SL LBT failure as a baseline for an RRC_CONNECTED UE: if the UE reports the consistent SL LBT failure (if triggered and not cancelled) to the gNB, the UE cancels all the triggered consistent SL LBT failure(s) which have already been reflected in the reporting to the gNB.

As another cancellation condition in NR-U, when lbt-FailureRecoveryConfig (including LBT-FailureInstanceMaxCount and LBT-FailureDetectionTimer) is reconfigured by upper layers for a Serving Cell in NR-U, the UE cancels all the triggered consistent LBT failure(s) in this Serving Cell. Since it was agreed by all companies in [1] that similar parameters are introduced for SL-U (e.g. sl-LBT-FailureInstanceMaxCount and sl-LBT-FailureDetectionTimer), it is straightforward to apply a similar cancellation condition also for SL-U. However, an aspect specific to SL-U is how these parameters for consistent SL LBT failure should be configured. Whereas the lbt-FailureRecoveryConfig in NR-U is configured per serving cell, the configuration for consistent SL LBT failure recovery may be provided at a different level (e.g. per resource pool, per RB set, per SL BWP, etc.). This SL-U specific aspect should be further looked into in more detail.

- As in NR-U, RAN2 agree the following cancellation condition as a baseline for consistent SL LBT failure: if the parameters in a given configuration for consistent SL LBT failure recovery (e.g. sl-LBT-FailureRecoveryConfig) are reconfigured, the UE cancels all the triggered consistent SL LBT failure(s) corresponding to this configuration. FFS how the consistent SL LBT failure recovery parameters are configured.

As for whether to apply other cancellation conditions similar to NR-U (e.g. upon BWP switching, upon MAC Reset, upon Cell activation/deactivation, etc.), we think they are more or less related to whether/how the UE autonomous recovery procedure is supported which in the last meeting was regarded by companies as much depending on RAN1’s input for the granularity of SLLBT failure indication. Therefore, we suggest to discuss them later.

Impact to SL DRX in SL-U

In SL-U, TX UE & RX UE should be aligned on the active/inactive time considering LBT failure. This may lead to potential impact to SL DRX design in the SL-U environment. For example, if an sl-drx-HARQ-RTT-Timer expires and if the HARQ feedback is not transmitted for unicast due to UL/SL prioritization, sl-drx-RetransmissionTimer for the corresponding Sidelink process is started as in Rel-17 NR SL. In SL-U, we think a similar operation should also be applied to the case if the HARQ feedback is not transmitted due to LBT failure. Therefore, we suggest RAN2 to discuss whether sl-drx-HARQ-RTT-Timer should be started when the SL HARQ feedback is not transmitted due to LBT failure.

If yes, RAN2 can further discuss whether to remove the ‘due to UL/SL prioritization’ description regarding sl-drx-HARQ-RTT-Timer in [3, 5.28.2] (as cited in the Annex).

- RAN2 to discuss whether to remove the ‘due to UL/SL prioritization’ description regarding sl-drx-HARQ-RTT-Timer operation in 5.8.2 in the MAC Spec, encompassing also the case that SL HARQ feedback fails to be transmitted due to LBT failure.

SL CG retransmission

In Rel-15, using CG transmission opportunities to perform HARQ retransmission is not supported. When the initial transmission of the CG is completed, the UE will start configuredGrantTimer. If the UE has not received a dynamic scheduling from the gNB for a retransmission of a given TB before the configuredGrantTimer expires, the UE will consider that the TB previously transmitted by the configured grant was successfully received by the gNB after the configuredGrantTimer expiry. Then, the UE will use further configured grant for new transmissions.

In Rel-16 NR-U, CG retransmission mechanism was introduced. Specifically, this mechanism intends to cope with two use cases. One of the use cases is that due to the gNB’s LBT failure, the UE cannot receive the DL feedback or CS-RNTI scheduling from the gNB for an earlier new transmission performed on CG resources. To cope with this use cases, RAN2 introduced cg-RetransmissionTimer (cg-RT) co-working with the configuredGrantTimer (CGT) to enable the UE to perform retransmission for that new transmission on future CG transmission opportunities. The other use case is that the HARQ process instructs the lower layer to perform a new transmission on a CG transmission opportunity, but due to LBT failure, the TB was not really transmitted in L1. To deal with this case, RAN2 introduced HARQ process pending status, based on which retransmission via future CG transmission opportunities is also enabled.

In SL-U, we may consider to apply such NR-U design as well, since this design stems from the occurrence of LBT failure on the unlicensed carrier. Since there is also LBT failure happing in SL, the two UEs communicating with each other may also suffer from the lack of PSFCH transmission due to LBT failure, as well as the LBT failure that prevents the transmission of a TB from really being performed in L1. However, there would be some differences between SL design and Uu design when we decide the feasibility to apply this NR-U mechanism into SL. For example, in legacy SL, there is no SL CGT, but there was sl-MaxTransNum introduced to realize similar function as CGT. For new transmission on SL CG resource, when the total number of (re)transmissions of the related MAC PDU has reached sl-MaxTransNum, this MAC PDU is assumed to be successfully received. the UE will then flush the HARQ buffer of the associated Sidelink process. But whether this sl-MaxTransNum can completely take the role of CGT, if SL CG retransmission is introduced, may needs further investigation. As another example, SL has both mode-1 SL CG and mode-2 resource reservation, which share similar natures. Whether to apply this CG retransmission mechinism to both cases should also be considered by RAN2.

In a word, we suggest RAN2 to first confirm the benefit and necessity to support an NR-U-like CG retransmission mechanism for SL-U. If RAN2 finally decide to support such a feature for SL-U, RAN2 may further decide whether to support it for mode-1 CG and/or mode-2 resource reservation (i.e. applicability), and whether both use cases due to LBT failure supported in NR-U are supported in SL-U as well.

- RAN2 to confirm whether to support an NR-U-like CG retransmission mechanism for SL-U.

- If the need is confirmed in P6, RAN2 to further discuss whether to support such CG retransmission mechanism for mode-1 SL CG and/or mode-2 resource reservation, and the applicable use cases such CG retransmission aims to address in SL-U.

Conclusion

Based on the discussion, we have the following proposals:

- A mode-2 UE in RRC_CONNECTED can indicate consistent SL LBT failure to the gNB.

- The per DST/per unicast link/per cast type level consistent SL LBT failure detection or recovery is not supported for SL-U.

- As in NR-U, RAN2 agree the following cancellation condition for consistent SL LBT failure as a baseline for an RRC_CONNECTED UE: if the UE reports the consistent SL LBT failure (if triggered and not cancelled) to the gNB, the UE cancels all the triggered consistent SL LBT failure(s) which have already been reflected in the reporting to the gNB.

- As in NR-U, RAN2 agree the following cancellation condition as a baseline for consistent SL LBT failure: if the parameters in a given configuration for consistent SL LBT failure recovery (e.g. sl-LBT-FailureRecoveryConfig) are reconfigured, the UE cancels all the triggered consistent SL LBT failure(s) corresponding to this configuration. FFS how the consistent SL LBT failure recovery parameters are configured.

- RAN2 to discuss whether to remove the ‘due to UL/SL prioritization’ description regarding sl-drx-HARQ-RTT-Timer operation in 5.8.2 in the MAC Spec, encompassing also the case that SL HARQ feedback fails to be transmitted due to LBT failure.

- RAN2 to confirm whether to support an NR-U-like CG retransmission mechanism for SL-U.

- If the need is confirmed in P6, RAN2 to further discuss whether to support such CG retransmission mechanism for mode-1 SL CG and/or mode-2 resource reservation, and the applicable use cases such CG retransmission aims to address in SL-U.

References

- R2-2210805	Report from session on NR SL	Session Chair (Samsung)

- R2-2210935	Summary of [AT119bis-e][504][V2XSL] Consistent SL LBT failure (vivo)

- TS 38.321, v17.2.0

- RP-221798 Revised WID: NR sidelink evolution

- Annex: Citation from [3]

### 5.28.2	Behaviour of UE receiving SL-SCH Data

When SL DRX is configured, the Active Time includes the time while:

-	sl-drx-onDurationTimer/sl-DRX-GC-BC-OndurationTimer or sl-drx-InactivityTimer/sl-DRX-GC-InactivityTimer is running; or

-	sl-drx-RetransmissionTimer/sl-DRX-GC-RetransmissionTimer is running; or

-	period of sl-LatencyBoundCSI-Report configured by RRC in case SL-CSI reporting MAC CE is not received; or

-	the time between the transmission of the request of SL-CSI reporting and the reception of the SL-CSI reporting MAC CE in case SL-CSI reporting MAC CE is received; or

-	Slot(s) associated with the announced periodic transmission(s) by the UE transmitting SL-SCH Data; or

-	the time between transmission/reception of Direct Link Establishment Request message (TS 24.587 [28]) or ProSe Direct Link Establishment Request message (TS 24.554 [29]) and reception of RRCReconfigurationSidelink message including initial DRX configuration or the link establishment procedure being aborted by upper layer; or

-	the time between transmission of RRCReconfigurationSidelink message including initial DRX configuration and reception of corresponding RRCReconfigurationCompleteSidelink or RRCReconfigurationFailureSidelink message.

When one or multiple SL DRX is configured, the MAC entity shall:

1>	if a single sl-DRX-GC-BC-Cycle that is mapped with one or multiple SL-QoS-Profile is configured to a Destination and interested cast type is associated to groupcast or broadcast:

2>	select the sl-DRX-GC-BC-Cycle that is mapped with one or multiple SL-QoS-Profile associated with the Destination:

2>	select the sl-DRX-GC-BC-OndurationTimer that is mapped with one or multiple SL-QoS-Profile associated with the Destination.

1>	else if multiple sl-DRX-GC-BC-Cycle that are mapped with multiple SL-QoS-Profile are associated to a Destination Layer-2 ID and interested cast type is associated to groupcast or broadcast:

2>	select the sl-DRX-GC-BC-Cycle whose length is the shortest one among multiple sl-DRX-GC-BC-Cycle that are mapped with multiple SL-QoS-Profile associated with the Destination Layer-2 ID:

2>	select the sl-DRX-GC-BC-OndurationTimer whose length is the longest one among multiple sl-DRX-GC-BC-OndurationTimer that are mapped with multiple SL-QoS-Profile associated with the Destination Layer-2 ID.

1>	if a single sl-DRX-GC-InactivityTimer that is mapped with one or multiple SL-QoS-Profile is configured to a Destination and interested cast type is associated to groupcast:

2>	select the sl-DRX-GC-InactivityTimer that is mapped with one or multiple SL-QoS-Profile associated with the Destination:

1>	else if multiple sl-DRX-GC-InactivityTimer that are mapped with multiple SL-QoS-Profile of a Destination Layer-2 ID and interested cast type is associated to groupcast:

2>	select sl-DRX-GC-InactivityTimer whose length is the longest one among multiple sl-DRX-GC-InactivityTimer that are mapped with multiple SL-QoS-Profile associated with the Destination Layer-2 ID.

1>	if an sl-drx-HARQ-RTT-Timer/sl-drx-HARQ-RTT-Timer1/sl-drx-HARQ-RTT-Timer2 expires:

2>	if the data of the corresponding Sidelink process was not successfully decoded or if the HARQ feedback (i.e., negative acknowledgement) is not transmitted for unicast due to UL/SL prioritization:

3>	start the sl-drx-RetransmissionTimer/sl-DRX-GC-RetransmissionTimer for the corresponding Sidelink process in the first slot after the expiry of sl-drx-HARQ-RTT-Timer1 or sl-drx-HARQ-RTT-Timer2.

When the cast type is groupcast or broadcast as indicated by upper layer, the sl-drx-StartOffset and sl-drx-SlotOffset are derived from the following equations:

sl-drx-StartOffset (ms) = Destination Layer-2 ID modulo sl-DRX-GC-BC-Cycle (ms).

sl-drx-SlotOffset (ms) = Destination Layer-2 ID modulo the number of slots in one subframe (ms).

1>	if the SL DRX cycle is used, and [(DFN × 10) + subframe number] modulo (sl-drx-Cycle or sl-DRX-GC-BC-Cycle) = sl-drx-StartOffset:

2>	start sl-drx-onDurationTimer/sl-DRX-GC-BC-OndurationTimer after sl-drx-SlotOffset from the beginning of the subframe.

1>	if an SL DRX is in Active Time:

2>	monitor the SCI (i.e., 1st stage SCI and 2nd stage SCI) in this SL DRX.

2>	if the SCI indicates a new SL transmission:

3>	if Source Layer-1 ID of the SCI is equal to the 8 LSB of the intended Destination Layer-2 ID and Destination Layer-1 ID of the SCI is equal to the 16 LSB of the intended Source Layer-2 ID and the cast type indicator in the SCI is set to unicast:

4>	start or restart sl-drx-InactivityTimer for the corresponding Source Layer-2 ID and Destination Layer-2 ID pair in the first slot after SCI reception.

3>	if Destination Layer-1 ID of the SCI (i.e., 2nd stage SCI) is equal to the 16 LSB of the intended Destination Layer-1 ID and the cast type indicator in the SCI is set to groupcast:

4>	start or restart sl-drx-InactivityTimer for the corresponding Destination Layer-2 ID in the first slot after SCI reception.

2>	if the SCI indicates an SL transmission:

3>	if a next retransmission opportunity is indicated in the SCI:

4>	derive the sl-drx-HARQ-RTT-Timer from the retransmission resource timing of the next retransmission resource in the SCI.

3>	else if PSFCH resource is configured for the SL grant associated to the SCI:

4>	set the sl-drx-HARQ-RTT-Timer based on sl-drx-HARQ-RTT-Timer1 configured by upper layer when HARQ feedback is enabled, or based on sl-drx-HARQ-RTT-Timer2 configured by upper layer when HARQ feedback is disabled, for resource pool configured with PSFCH.

3>	else (i.e., if PSFCH resource is not configured for the SL grant associated to the SCI):

4>	set the sl-drx-HARQ-RTT-Timer as 0 slots.

3>	if PSFCH resource is not configured for the SL grant associated to the SCI:

4>	start the sl-drx-HARQ-RTT-Timer2 for the corresponding Sidelink process in the slot following the end of PSSCH transmission (i.e., currently received PSSCH).

3>	if PSFCH resource is configured for the SL grant associated to the SCI:

4>	if HARQ feedback is enabled by the SCI and the cast type indicator in the SCI is set to unicast; or

4>	if HARQ feedback is enabled by the SCI and the cast type indicator in the SCI is set to groupcast and positive-negative acknowledgement is selected;

5>	start the sl-drx-HARQ-RTT-Timer1 for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH transmission carrying the SL HARQ feedback; or

5>	start the sl-drx-HARQ-RTT-Timer1 for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH resource for the SL HARQ feedback when the SL HARQ feedback is not transmitted due to UL/SL prioritization;

4>	if HARQ feedback is enabled by the SCI and the cast type indicator in the SCI is set to groupcast and negative-only acknowledgement is selected;

5>	start the sl-drx-HARQ-RTT-Timer1 for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH transmission carrying the SL HARQ feedback; or

5>	start the sl-drx-HARQ-RTT-Timer1 for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH resource for the SL HARQ feedback when the SL HARQ feedback is not transmitted due to UL/SL prioritization; or

5>	start the sl-drx-HARQ-RTT-Timer1 for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH resource for the SL HARQ feedback when the SL HARQ feedback is a positive acknowledgement.

4>	if HARQ feedback is disabled by the SCI and the resource(s) for one or more retransmission opportunities is not scheduled in the SCI:

5>	start the sl-drx-HARQ-RTT-Timer2 for the corresponding Sidelink process in the slot following the end of PSFCH resource.

4>	if HARQ feedback is disabled by the SCI and the resource(s) for one or more retransmission opportunities is scheduled in the SCI:

5>	start the sl-drx-HARQ-RTT-Timer2 for the corresponding Sidelink process in the slot following the end of PSSCH transmission (i.e., currently received PSSCH).

NOTE:	Void.

3>	stop the sl-drx-RetransmissionTimer/sl-DRX-GC-RetransmissionTimer for the corresponding Sidelink process.

1>	if the SCI indicates a new transmission where the cast type is set to groupcast is transmitted:

2>	start or restart sl-DRX-GC-InactivityTimer for the corresponding Destination Layer-2 ID in the first slot after SCI transmission.

1>	if an SL DRX Command MAC CE is received for the Source Layer-2 ID and Destination Layer-2 ID pair of a unicast:

2>	stop sl-drx-onDurationTimer for the Source Layer-2 ID and Destination Layer-2 ID pair of a unicast;

2>	stop sl-drx-InactivityTimer for the Source Layer-2 ID and Destination Layer-2 ID pair of a unicast.