3GPP TSG RAN WG2 Meeting #120	R2-2211330

Toulouse, France

Nov. 14th – Nov. 18th

Agenda item:	8.11.3

Source:	Chengdu TD Tech, TD Tech

Title:	MBS reception interruption problem in LTE and NR

Document for:	Discussion and Decision

- Introduction

NR MBS was introduced in 3GPP R17 by using multicast mode and broadcast mode to provide multicast sessions and broadcast sessions respectively. Broadcast mode of NR MBS is based on SC-PTM in LTE. SC-PTM in LTE is used to transmit an MBS session. SC-PTM suffers the MBS reception interruption problem in the LTE network as illustrated below. As the solution evolved from SC-PTM in LTE, broadcast mode of NR MBS suffers the same problem.

- A RRC_IDLE UE will undergo a long MBS reception interruption time during its mobility from the source cell to the target cell.

- A RRC_CONNECTED UE will undergo a long MBS reception interruption time during handover.

The quantitative analysis of the MBS reception interruption time during the UE mobility in the LTE network is given in section 2. The solution to the MBS reception interruption problem in NR is discussed in section 3. In section 4, the proposals for solving the MBS reception interruption problem in NR are suggested for the 3GPP R18 WI “NR MBS enhancement”[1].

- MBS reception interruption problem during UE mobility in LTE

In LTE, SC-PTM introduces SC-MCCH and SIB20. SC-MCCH carries the SC-PTM configuration information which consists of the configuration informations of all MBS sessions. SIB 20 carries the SC-MCCH configuration information.

When a RRC_IDLE UE moves from the source cell to the target cell which is transmitting the MBS session received by the UE, it stops receiving the MBS session in the source cell. It acquires the configuration information of the MBS session in the target cell and then continue to receive the MBS session. The specific procedure for a RRC_IDLE UE to restore the MBS reception in the target cell is listed as below.

- Receive PBCH, SIB1 and SIB2 (SIB2 carries the cell barring information)

- Receive SIB20

- Receive SC-MCCH

- Receive the MBS session

Therefore, a RRC_IDLE UE will undergo the MBS reception interruption in the target cell. Similarly a RRC_CONNECTED UE receiving an MBS session in the source cell will undergo the MBS reception interruption in the target cell during handover. The quantitative analysis of the MBS reception interruption time for a RRC_IDLE UE is illustrated as below.

PBCH is transmitted one times every 10ms. SIB1 is transmitted one times every 20ms. Use X (ms) to denote the MBS reception interruption time. X can be calculated as below:

X=10+20+T1+T2+P+D

where the period of SIB2/SIB20 is denoted by T1/T2 (ms), the repetition period of SC-MCCH is denoted as P (ms) and the period of the DRX mode of an MBS session is denoted by D (ms).

The MBS reception interruption time of X (ms) is a worse scenario for acquiring the configuration information of the MBS session in the target cell. But it’s not the worst scenario because a RRC_IDLE UE may not decode SIB2/SIB20/SC-MCCH correctly for the first time. Certainly, there exists a better scenario where the MBS reception interruption time is about Y=X/2 (ms).

For an MBS session, if the source cell and the target cell have the same configuration information, the UE has no need to receive SIB20 and SC-MCCH in the target cell. It can directly receive the MBS session after it receives PBCH, SIB1 and SIB2 in the target cell. In detail, when the source cell and target cell have the same configuration information, the UE restores the MBS reception in the target cell according to the following procedure.

- Receive PBCH, SIB1 and SIB2

- Receive the MBS session according to the configuration information acquired in the source cell

During the above procedure, the MBS reception interruption time is X1=10+20+T1+D (ms) or Y1 =X1/2 (ms).

In the LTE network, some important SIBs have a period of 160ms while the other SIBs have a period of 320ms or 640ms. Usually T1=160ms for SIB2 and T2=640ms for SIB20. The radio resource for the SIBs is about 5%. Usually P=80ms for SC-MCCH. The minimum value of D is D=10ms. The following conclusion can be made:

- If UE needs to acquire SIB20 and SC-MCCH in the target cell, the MBS reception interruption time of an MBS session is about X=920ms or Y=460ms.

- If UE has no need to acquire SIB 20 and SC-MCCH in the target cell, the MBS reception interruption time of an MBS session is about X1=200ms or Y1=100ms.

Based on the above discussion, if the same configuration is applied in the target cell, the MBS reception interruption time has no relationship with SIB20 and SC-MCCH in the target cell and thus can be reduced greatly, which means the QoE of UE will be improved greatly.

- Solutions to MBS reception interruption problem in NR

As the solution evolved from SC-PTM in LTE, broadcast mode of NR MBS suffers the MBS reception interruption problem. In order to solve the MBS reception interruption problem for broadcast mode of NR MBS, the PTM configuration information of a broadcast session can be applied to a certain area. The certain area has three application scenarios: the intra-gNB-DU case, the intra-gNB-CU case and the inter-gNB-CU case. For the intra-gNB-DU/ intra-gNB-CU case, the PTM configuration information of a broadcast session can be applied to the cells of a gNB-DU/gNB-CU. For the inter-gNB-CU case, the PTM configuration information of a broadcast session can be applied to the cells of several gNB-CUs. The intra-gNB-DU/intra-gNB-CU case only involves F1 while the inter-gNB-CU case involves both F1 and Xn. Whether or not the signalling interaction over Xn is acceptable for the inter-gNB-CU case shall be studied. If the PTM configuration information of a broadcast session can be applied to a certain area, indicate the certain area to UE on MCCH. Based on the discussion above, the following proposals are suggested for a broadcast session.

Proposal 1: The PTM configuration information of a broadcast session can be applied to:

- Intra-gNB-DU case

- Intra-gNB-CU case

- FFS: inter-gNB-CU case

Proposal 2: If the PTM configuration information of a broadcast session is applied to a certain area, indicate the certain area on MCCH

- Conclusion

As the solution evolved from SC-PTM in LTE, broadcast mode of NR MBS suffers the MBS reception interruption problem just as SC-PTM does. In order to solve the MBS reception interruption problem for broadcast mode of NR MBS, the following proposals are suggested for the 3GPP R18 WI “NR MBS enhancement”[1].

Proposal 1: The PTM configuration information of a broadcast session can be applied to:

- Intra-gNB-DU case

- Intra-gNB-CU case

- FFS: inter-gNB-CU case

Proposal 2: If the PTM configuration information of a broadcast session is applied to a certain area, indicate the certain area on MCCH

- References

- RP-213568, New WID: Enhancements of NR Multicast and Broadcast Services, RAN Meeting #94-e.