3GPP TSG-RAN WG2 Meeting #120		R2-2211352

Toulouse, France, 14th - 18th November 2022

Source:	CATT

Title:	SON Enhancement for NR-U

Agenda Item:	8.13.5

Document for:	Discussion and Decision

Introduction

In RAN2#119bis-e meeting, preliminary agreements on SON for NR-U were achieved [1]. Based on the agreements, enhancements on existing SON signalling reports are studied with high priority. In this contribution, we further analyse NR-U aspect in SON and our proposals are provided.

Discussion

LBT failure information in RA-InformationCommon

In RAN2#119bis-e meeting, RAN2 discussed what information can be included in RA-InformationCommon, and had below agreements:

Agreements: 1 Introduce a new raPurpose in the RA-Report to indicate that the RA was initiated following a “consistent LBT failures” in the SpCell. 2 RAN2 agree to log kind of “the number of LBT failures” in the RA report. LBT failure is the failure to access the channel before transmission. The definition of “the number of LBT failures” should be clarified. FFS how to log the number of LBT failures in the RA report.

Based on the agreement, firstly, RAN2 needs to clarify the definition of the number of LBT failures for RA report.

Based on below description in specification [2], for failed RA preamble transmission by LBT failure, if lbt-FailureRecoveryConfig is not configured, PREAMBLE_TRANSMISSION_COUNTER will be increased by 1, if lbt-FailureRecoveryConfig is configured, the failed RA preamble transmission will be not counted into PREAMBLE_TRANSMISSION_COUNTER.

### 5.1.3 Random Access Preamble transmission The MAC entity shall, for each Random Access Preamble: /Omitted/ 1> instruct the physical layer to transmit the Random Access Preamble using the selected PRACH occasion, corresponding RA-RNTI (if available), PREAMBLE_INDEX, and PREAMBLE_RECEIVED_TARGET_POWER. 1> if LBT failure indication is received from lower layers for this Random Access Preamble transmission: 2> if lbt-FailureRecoveryConfig is configured: 3> perform the Random Access Resource selection procedure (see clause 5.1.2). 2> else: 3> increment PREAMBLE_TRANSMISSION_COUNTER by 1; 3> if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1: /Omitted/

Here, based on above analysis, we can discuss LBT failures in RA report in the two cases:

Case 1: lbt-FailureRecoveryConfig is NOT configured

In this case, the failed RA preamble transmission will be counted into PREAMBLE_TRANSMISSION_COUNTER. Then, the preamble blocked by LBT failure can be regarded as one RACH attempt since the maximum number of RA attempt in PerRAInfoList is defined as the maximum value of the PREAMBLE_TRANSMISSION_COUNTER. Therefore, LBT failure for failed preamble transmission can be logged in each RACH attempt. Or, optionally, the number of LBT failure per RA procedure could be used to log “the number of LBT failures”. Hence, we suggest that:

Proposal 1: The definition of the number of LBT failures for RA report includes at least the LBT failure for failed RA preamble transmission which is counted into the PREAMBLE_TRANSMISSION_COUNTER (i.e., the case of that lbt-FailureRecoveryConfig is not configured).

Proposal 2: For the case of lbt-FailureRecoveryConfig is not configured, two ways to log the number of LBT failure in RA report could be for down-selection:

- Enhance the current PerRAInfoList  to  log the failed RA preamble transmission caused by LBT failure per RA attempt;

- Include the number of LBT failures (i.e., failed RA preamble transmission) per RA procedure in RA report.

Case 2: lbt-FailureRecoveryConfig is configured

For SON MDT, the legacy RA attempt in RA report is corresponding to the successful RA preamble transmission since no NR-U is supported for SONMDT. Now, RAN2 agreed to log kind of “the number of LBT failures” in the RA report, which means the RA attempt could include the failed RA preamble transmission. In case 2, based on the specification, since the UE will perform the Random Access Resource selection procedure and PREAMBLE_ TRANSMISSION _COUNTER by 1 will not be counted, the mechanism to trigger failure RA procedure may be stopped. The extremely case is that consistent LBT failure happens, and the UE triggers a new RA procedure on another BWP for SpCell or report LBT failure MAC CE for SCell. This kind of number of failed RA preamble transmission in case 2 may be very large and unpredictable. So it is not suitable to record the failed RA preamble transmission in legacy RA attempt. Hence, we suggest define one new IE in RA-InformationCommon to log the failed RA preamble transmission caused by LBT failure when lbt-FailureRecoveryConfig is not configured.

Proposal 3: If RAN2 agree to define the number of LBT failures including the failed RA preamble transmission when lbt-FailureRecoveryConfig is configured (i.e., the failed RA preamble transmission will not counted in PREAMBLE_ TRANSMISSION _COUNTER ),  it is suggested to define one new IE in RA-InformationCommon to log the failed RA preamble transmission caused by LBT failure.

LBT failure information for SHR

In NR-U, it may happen that the handover is performed successfully while UL consistent LBT failures has been triggered and not cancelled. Based on current specification, this information will not be reported in SHR. Some companies propose some enhancements to SHR, e.g. report LBT failure information in SHR.

In current SHR, when SHR is triggered and reported, it indicates that the UE experiences poor channel condition in the source cell before assessing the target cell. From this perspective, the network knows that this is one late handover. Hence, the network can perform corresponding optimizations. For example, the network can update the threshold for measurement report or trigger one earlier handover for the UE.

In NR-U, the gNB and UE may both apply LBT before performing a transmission including handover procedure. Hence, the network may figure out whether the channel is busy or not by LBT detection for large time scale. So the benefits for reporting LBT failure in SHR should be studied carefully.

In current specification, the UE will trigger SHR in the following cases [3]:

- if the ratio between value of the elapsed time of the timer T304 and the configured value of the timer T304,  is greater than thresholdPercentageT304; or

- if the ratio between the value of the elapsed time of the timer T310 and the configured value of the timer T310 is greater than thresholdPercentageT310 included in the successHO-Config; or

- if the T312 associated to the measurement identity of the target cell was running at the time of initiating the execution of the reconfiguration with sync procedure and if the ratio between the value of the elapsed time of the timer T312 and the configured value of the timer T312 is greater than thresholdPercentageT312; or

- if sourceDAPS-FailureReporting is included in the successHO-Config before executing the last reconfiguration with sync and is set to true and if the last executed handover was a DAPS handover and if an RLF occurred at the source PCell during the DAPS handover while T304 was running

Similarly, one condition for SHR for consistent LBT failure should be defined. Some possible solutions for SHR can be:

- Solution 1: the number of LBT failure in RA procedure is larger than a threshold during a duration when UE receives the HO command to the time of HO is successful;

- Solution 2: the ratio between the LBT failure number and the total number of RA preamble transmission exceeds on threshold after the UE initiates handover in successful HO procedure.

Proposal 4: Triggering condition for SHR reporting consistent LBT failure information can be:

- Solution 1: the number of LBT failure in RA procedure is larger than a threshold during a duration when UE receives the HO command to the time of HO is successful;

- Solution 2: the ratio between the LBT failure number and the total number of RA preamble transmission exceeds on threshold after the UE initiates handover in successful HO procedure.

As the content of the SHR, we think we can follow the discussion in section 2.1.

Proposal 5: The content in SHR for LBT failure can follow the content in RA-InformationCommon for LBT failure.

Consistent LBT failure consideration in RLF report

In accordance to the Rel-16 specification, the UE will declare radio link failure and set the rlf-Cause in RLF report as lbtFailure, when the UE detects consistent LBT failure triggered in all UL BWPs on the SpCell. Therefore, the network can figure out the RLF reason by the RLF report and do some corresponding optimizations.

Some companies propose that LBT failure may contribute to the RLF due to reasons, for example, reaching of maximum number of retransmissions from the MCG RLC, random access failure and etc. So the related information can also be reported to the network. We agree that the consistent LBT failure information may be useful for the network to figure out the LBT failure contributes to some extent to the RLF failure. But it should be noted that radio link failure can be triggered by other reasons except for consistent LBT failure. Take RACH procedure as one example, preamble collision, poor channel condition may also lead to RACH failure. But these reasons are not reported in RLF report. From this perspective, it is useful to study what to be included in the RLF report to reflect that the RLF which is caused by LBT failure indirectly.

Proposal 6: RAN2 to further study what to be included in RLF report to reflect the RLF which is caused by consistent LBT failure indirectly.

Conclusion

According to the analysis in section 2, we propose that:

Proposal 1: The definition of the number of LBT failures for RA report includes at least the LBT failure for failed RA preamble transmission which is counted into the PREAMBLE_TRANSMISSION_COUNTER (i.e., the case of that lbt-FailureRecoveryConfig is not configured).

Proposal 2: For the case of lbt-FailureRecoveryConfig is not configured, two ways to log the number of LBT failure in RA report could be for down-selection:

- Enhance the current PerRAInfoList  to  log the failed RA preamble transmission caused by LBT failure per RA attempt;

- Include the number of LBT failures (i.e., failed RA preamble transmission) per RA procedure in RA report.

Proposal 3: If RAN2 agree to define the number of LBT failures including the failed RA preamble transmission when lbt-FailureRecoveryConfig is configured (i.e., the failed RA preamble transmission will not counted in PREAMBLE_ TRANSMISSION _COUNTER ),  it is suggested to define one new IE in RA-InformationCommon to log the failed RA preamble transmission caused by LBT failure.

Proposal 4: Triggering condition for SHR reporting consistent LBT failure information can be:

- Solution 1: the number of LBT failure in RA procedure is larger than a threshold during a duration when UE receives the HO command to the time of HO is successful;

- Solution 2: the ratio between the LBT failure number and the total number of RA preamble transmission exceeds on threshold after the UE initiates handover in successful HO procedure.

Proposal 5: The content in SHR for LBT failure can follow the content in RA-InformationCommon for LBT failure.

Proposal 6: RAN2 to further study what to be included in RLF report to reflect the RLF which is caused by consistent LBT failure indirectly.

Reference

- RAN2#119bis-e meeting report;

- 38.321, 17.2.0, NR; Medium Access Control (MAC) protocol specification;

- 38.331, V17.2.0, NR; Radio Resource Control (RRC);