3GPP TSG-RAN WG2 Meeting #120	R2-2113634

Toulouse, France, 14 – 18 November 2022

Agenda item:	6.21.1

Source:	Nokia, Nokia Shanghai Bell

Title:	More granular per-FR gaps

WID/SID:	TEI17 - Release 17

Document for:	Discussion and Decision

# 1	Introduction

RAN#97e concluded the following:

RP-222590 Moderator's summary for discussion [97e-30-UE-Capabilities] RAN2 chair (MediaTek) handled Tdocs: RP-222068, 2276, 2366, 2473, 2474, 2475 conclusions: Per FR Gaps: - Conclusion F1: The following Root Cause / Justification is applicable: a) the possibility to do gapless measurements is valuable, which is possible by using per-FR-gaps, b) the assumption behind per-FR-gaps that FR1 and FR2 has separate resources in the UE is no longer true, and is the main reason why per-FR gaps need more fine-granular capability, or why other solution like needforgap should be considered. - Conclusion F2: RAN2 to be tasked to progress this issue (also taking into account comments collected at TSG RAN), including solutions - Alt 1.1 (More fine grained capability for Per-FR-Gaps, 1 bit per BC), - Alt 1.3 (more fine grained capability for Per-FR-Gaps - limited by number of carriers), and - Alt 2 (Use similar framework/procedure as for ”NeedForGap”). - Conclusion F3: RAN2 to consider Alt. 1.1, 1.3 and 2, and discuss the signaling overhead and network processing requirements/complexity. RAN2 is tasked to provide results after one Quarter, and leave final decision(s) to TSG RAN.

Based on this, RAN2#119bis-e discussed the topic and conclude on the following:

Exclude Alt 1.1 for now.

On the table: Alt 1.3, Alt 1.3 per BC, Alt 2 (add info, based on current config as today, FFS excl/incl DC)

In this contribution, we propose how to finalize the discussion.

# 2	Per-FR gap capabilities

## 2.1	Signalling options

The following options are on the table:

On the table: Alt 1.3, Alt 1.3 per BC, Alt 2 (add info, based on current config as today, FFS excl/incl DC)

Normal style, normal style, normal style, normal style, normal style, normal style, normal style, normal style, normal

-	Alt.1.3: UE capability indicating maximum supported CCs for per-FR gaps

-	A: UE capability is per-UE (similar to existing per-FR gap capability)

-	B: UE capability is per BC (allowing UE to indicate different capabilities for each BC based on corresponding processing capabilities).

-	Alt.2: Use NeedForGap framework (i.e. network requests per-FR gap support in RRCReconfiguration, corresponding to current UE configuration).

-	UE just indicates whether it supports per-FR gaps (similar to existing per-FR gap capability)

-	Alt.3.3: Alt.1.3 + Alt.2, i.e. Use NeedForGap framework with max CCs for per-FR gaps

-	UE indicates up to which number of CCs it supports per-FR gaps (allowing UE to consider current processing capabilities).

We note that the Alt.3.3 is just a term to denote the “Alt.1.3 + Alt.2A”, and note that the real question is just whether RAN2 extends the NeedForGaps or just uses “plain” UE capabilities. As we have noted earlier in R2-2210237, whether UE can use the per-FR gaps depends entirely on the used configuration, but not on the actually measured frequencies: Gaps are either needed or not needed, and the measured frequencies only impact on how long the measurements may take.

Observation 1: Whether UE can use per-FR gaps depends on the used RRC configuration, not on the measured frequencies.

As already discussed during RAN2#119bis-e, the per-BC capabilities always open up a question on the fallback BC capabilities: UE normally only reports the highest-order CA/DC band combination, where it is most likely that UE may not support the per-FR gaps in case there are processing issues. So any per-BC capability tends to open up the discussion on how the fallback indications work. In contrast, with the NeedForGaps framework, the result is always conditioned on the currently used configuration: If a serving cell is dropped from the configuration, network has to do that with an RRCReconfiguration, which also allows network to ask for UE to provide updated per-FR capabilities.

Observation 2: With per-BC capabilities for per-FR gaps, there is a need to discuss how the capabilities work with fallback BCs.

Observation 3: With NeedForGaps, the per-FR gap capabilities are always related to the current configuration and require no discussion of fallback BC capabilities.

Some questions were raised in RAN2#119bis-e that the current NeedForGap -framework does not support MR-DC, and specifically not NR-DC. This is because during Rel-16 there was no time to discss how to do MN-SN coordination of the queries, which might require additional answers (e.g. Can SN ask for NeedForGaps? If it can, does it need to inform MN? Can there be conflicts and how to resolve the need for signalling the information between the nodes?). However, this was because the NeedForGaps was inherently about the measured frequencies: Does the UE need gaps to measure frequencies f1,…,fN with the current configuration? For the per-FR gaps, the measured frequencies are less important: The question is about whether UE can use per-FR gaps with the current configuration? The need for MN-SN coordination is thus only relevant in case both MN and SN can use per-FR gaps, but already with current specification (see below) this is limited to MCG (in NR-DC or NE-DC) or SCG (in case of EN-DC or NGEN-DC). Thus, since it is anyway always controlled by one CG, there is no real risk of having issues with per-FR gap coordination in NR-DC.

gapFR1 Indicates measurement gap configuration that applies to FR1 only. In (NG)EN-DC, gapFR1 cannot be set up by NR RRC (i.e. only LTE RRC can configure FR1 measurement gap). In NE-DC, gapFR1 can only be set up by NR RRC (i.e. LTE RRC cannot configure FR1 gap). In NR-DC, gapFR1 can only be set up in the measConfig associated with MCG. gapFR1 can not be configured together with gapUE. The applicability of the FR1 measurement gap is according to Table 9.1.2-2 and Table 9.1.2-3 in TS 38.133 [14].

gapFR2 Indicates measurement gap configuration applies to FR2 only. In (NG)EN-DC or NE-DC, gapFR2 can only be set up by NR RRC (i.e. LTE RRC cannot configure FR2 gap). In NR-DC, gapFR2 can only be set up in the measConfig associated with MCG. gapFR2 cannot be configured together with gapUE. The applicability of the FR2 measurement gap is according to Table 9.1.2-2 and Table 9.1.2-3 in TS 38.133 [14].

Observation 4: Current RRC already limits per-FR gaps to MCG (in NR-DC or NE-DC) or SCG (in case of EN-DC or NGEN-DC), without need for the per-FR gap coordination.

There might be some problems with NE-DC, (NG)EN-DC but as long as the request about per-FR NeedForGaps is limited to NR-DC and NR CA, we don’t foresee issues with MN-SN coordination. Thus, limiting the procedure to MCG only seems straightforward and compatible with existing specifications.

Observation 5: Limiting usage of NeedForGaps with per-FR gap information to MCG only resolves all problems with NR-DC usage and is compatible with existing specification.

With this in mind, we propose that RAN2 adopts the NeedForGaps framework for the per-FR gap capability extension.

Proposal 1: Use NeedForGaps-framework for the per-FR gap capability query.

To allow signalling optimization and avoid network from having to always query the information, we also think that UE could still indicate a per-BC capability that tells whether UE supports per-FR gaps when the number of CCs is less than or equal to the indicated UE capability.

Proposal 2: Allow per-BC capability to indicate UE can support per-FR gaps in the band combination when the number of serving cells is less than or equal to indicated number of CCs.

We have provided example CRs for this in R2-2212373 and R2-2212374.

Proposal 3: Agree to the CRs in R2-2212373 and R2-2212374.

# 3	Conclusion

This document has made the following observations:

Observation 1: Whether UE can use per-FR gaps depends on the used RRC configuration, not on the measured frequencies.

Observation 2: With per-BC capabilities for per-FR gaps, there is a need to discuss how the capabilities work with fallback BCs.

Observation 3: With NeedForGaps, the per-FR gap capabilities are always related to the current configuration and require no discussion of fallback BC capabilities.

Observation 4: Current RRC already limits per-FR gaps to MCG (in NR-DC or NE-DC) or SCG (in case of EN-DC or NGEN-DC), without need for the per-FR gap coordination.

Observation 5: Limiting usage of NeedForGaps with per-FR gap information to MCG only resolves all problems with NR-DC usage and is compatible with existing specification.

And proposed the following:

Proposal 1: Use NeedForGaps-framework for the per-FR gap capability query.

Proposal 2: Allow per-BC capability to indicate UE can support per-FR gaps in the band combination when the number of serving cells is less than or equal to indicated number of CCs.

Proposal 3: Agree to the CRs in R2-2212373 and R2-2212374.