3GPP TSG-RAN WG2 Meeting #120	R2-2211371

# Toulouse, France, 14 – 18 Nov, 2022

Agenda item:	6.10.3

Source:	MediaTek Inc.

Title:	Discussion on UE behaviour based on the neighbour cell information between SIB3, SIB4, measObjectNR and SIB19

WID/SID:	NR_NTN_solutions-Core

Document for:	Discussion and Decision

Introduction

In the last meeting, a clarification on the linkage of neighbor cell/frequency information between SIB19 and SIB3/4/measObjectNR was proposed [1] and discussed in offline [2] and online.  The online discussion and conclusion are listed below [3]:

Proposal 1 Discuss online whether CR R2-2210412 (HW proposal) is adopted or no further enhancements are made to neighbor cell SI. - Although it is kept for online discussion, MediaTek would like to say that they do not understand how to link SIB3/4/measobject and SIB19, especially when PCI is not configured. That is what they explained in our CR (R2-2209538) with pretty minor changes and specification impacts. They don’t see this in R2-2210664. - Continue online - MTK thinks what they suggested is a correction, not an enhancement and this needs to be address somehow - QC thinks MTK clarification would be good to have - Ericsson is ok to go for MTK proposal if we don’t have other corrections - Google thinks the MTK proposal is not needed - Samsung thinks we don’t need further clarification - HW thinks that for the frequency part it’s obvious; for the PCI part the UE will use all possible PCIs so they prefer not to mention PCI. Nokia agrees. - RAN2 understands that the NW needs to configure the NTN neighbour cell frequencies in SIB19 if it wants the UE to measure them - Come back in the next meeting to see whether we can converge on some enhancements (e.g as in R2-2210412)

Based on the discussion, in this contribution we would like to further discuss the UE behavior based on the current spec/agreements and to confirm the RAN2 common understanding.

Discussion

In order to simplify the discussion on the PCI usage between SIB3/4/measObjectNR and SIB19, the PCIs on the same NTN carrier frequency are assumed in this contribution.  Note that the carrier frequency for NTN or TN can be differentiated by the FreqBandIndicatorNR in SIB3/4/measObjectNR (e.g., band n256 and n255 are for NTN).  Therefore, only the neighbour cell/frequency information in SIB3/4/measObjectNR for a detected cell on NTN carrier frequency is considered in this contribution.

Information for measuring/re-selection a neighbor cell

Neighbour Cell Information in SIB3/4/measObjectNR

The cell specific parameters for cell reselection in SIB3/4/measObjectNR are only applicable to the PCIs listed in SIB3/4/measObjectNR. The frequency specific parameters for cell reselection are only applicable to the carrier frequencies listed in SIB3/4/measObjectNR.  There is no difference between TN and NTN.

Observation 1: The cell/frequency specific parameters for cell reselection in SIB3/4/measObjectNR are only applicable to the PCIs/carrier frequencies listed in SIB3/4/measObjectNR, respectively.

Neighbour Cell Information in SIB19

In SIB19, NTN-NeighCellConfig provides ntn-Config, carrierFreq and PCI for UE to perform neighbor cell measurement.  If a PCI field is configured in NTN-NeighCellConfig of SIB19, the corresponding ntn-Config is applicable to the cell with the PCI on the carrierFreq of NTN-NeighCellConfig. Thus, if a detected PCI is in the PCI field configured in NTN-NeighCellConfig of SIB19, the corresponding ntn-Config  is applicable to the cell with the detected PCI.

Observation 2: If a PCI field is configured in NTN-NeighCellConfig of SIB19, the corresponding ntn-Config is applicable to the cell with the PCI on the carrierFreq.

In other words, if UE detects a cell with different PCI on the carrierFreq of NTN-NeighCellConfig, the ntn-Config cannot be applied to this cell.  Consequently, it is not required for UE to perform the subsequent measurement on this cell since no ntn-Config is available to predict when the next SSB is coming.

Observation 3: If a detected PCI on the carrierFreq is NOT in the PCI field configured in NTN-NeighCellConfig of SIB19, the ntn-Config of the corresponding carrierFreq may be not applicable to the cell with this detected PCI. Hence, it is not required for UE to perform the subsequent measurement on this cell.

Based on this observations and discussions, we make the following proposal:

Proposal 1: RAN2 confirms that if a detected PCI on the carrierFreq is NOT in the PCI field configured in NTN-NeighCellConfig of SIB19, the ntn-Config of the corresponding carrierFreq may be not applicable to the cell with this detected PCI.

How to Apply Neighbour Cell Information in SIB3/4/measObjectN and in SIB19 Together

When UE detects a PCI on an NTN carrier frequency, how to apply the neighbour cell information in SIB3/4/measObjectNR and in SIB19 together to this cell is still unclear.  For example, if a detected PCI is in the PCI list of SIB3/4/measObjectNR but not in the PCI field of SIB19, UE may be not able to perform the subsequent measurement on this cell.  In this case, should UE read the neighbour cell’s SIB19 to obtain the satellite ephemeris information? Is the serving cell’s satellite ephemeris information utilized? Or is it left to UE implementation?

Furthermore, since the PCI field is optional in NTN-NeighCellConfig of SIB19, there may have two possible configurations.  In the following subsections, we further discuss the UE behaviour under the different configurations.

PCI field is configured in NTN-NeighCellConfig of SIB19

When PCI field is configured in NTN-NeighCellConfig of SIB19, there are four cases for a detected PCI on an NTN carrier frequency.

- Case 1: A detected PCI is in both the PCI field of NTN-NeighCellConfig of SIB19 and the PCI list of SIB3/4/measObjectNR.

The corresponding ntn-Config and the corresponding cell specific parameters in SIB3/4/measObjectNR are applicable to the cell with this PCI.

- Case 2: A detected PCI is in the PCI field of NTN-NeighCellConfig of SIB19 but NOT in the PCI list of SIB3/4/measObjectNR.

NTN-config is applicable to the cell with this PCI but there is no cell specific parameter in SIB3/4/measObjectNR applicable to this cell with this PCI.

- Case 3: A detected PCI is in the PCI list of SIB3/4/measObjectNR but NOT in the PCI field of NTN-NeighCellConfig of SIB19.

There is no corresponding NTN-config applicable to the cell with this PCI but the cell specific parameters in SIB3/4/measObjectNR are still applicable to this cell with this PCI.

- Case 4: A detected PCI is NEITHER in the PCI list of SIB3/4/measObjectNR NOR in the PCI field of NTN-NeighCellConfig of SIB19.

There is neither the corresponding NTN-config nor the cell specific parameters in SIB3/4/measObjectNR applicable to this cell with this PCI.

For case 1 and case 2, the configurations are valid and UE behaviour is clear.

For case 3 and case 4, UE may not perform subsequent measurement on the cell with this PCI without ntn-Config, especially for NGSO cell.  So, UE may assume the cell with this PCI is a GSO/GEO cell or the cell is on the same satellite as the serving cell (utilize the same NTN-config as the serving cell) and try to measure it.  For case 3, network may avoid configuring these configurations.

Proposal 2: For case 3 and case 4, i.e., for a detected PCI NOT in the PCI field configured in NTN-NeighCellConfig of SIB19, how to perform subsequent measurement on this cell with this PCI is left to UE implementation in this release.  If the detected PCI is in the PCI list of SIB3/4/measObjectNR, the corresponding cell specific parameters are still applicable to this cell.

PCI field is NOT configured in NTN-NeighCellConfig of SIB19

When PCI field is NOT configured in NTN-NeighCellConfig of SIB19, i.e., only ntn-Config and carrierFreq are configured, there are two options for a detected PCI on an NTN carrier frequency:

- Option 1: The corresponding ntn-Config is applicable to all possible detected PCIs on this NTN carrier frequency (i.e., carrierFreq in NTN-NeighCellConfig of SIB19).

Whether applying cell specific parameters in SIB3/4/measObjectNR depends on whether the detected PCI is in the PCI list or not.  It follows legacy behaviour.

- Option 2: The corresponding ntn-Config is only applicable to the detected PCI, which is in the PCI list of SIB3/4/measObjectNR with the same carrier frequency as carrierFreq in NTN-NeighCellConfig of SIB19.

The cell specific parameters in SIB3/4/measObjectNR are also applied to the detected PCI which is in the PCI list of SIB3/4/measObjectNR.  For the cell with detected PCI not in PCI list of SIB3/4/measObjectNR, UE may not perform subsequent measurement on it.  It can be left to UE implementation like case 4. If the PCI list is not configured in SIB3/4/measObjectNR, the corresponding ntn-Config and the frequency specific parameters in SIB3/4/measObjectNR are applicable to the cells with all possible detected PCIs.

Observation 4: For option2, if PCI field is NOT configured in NTN-NeighCellConfig of SIB19 and a detected PCI is NOT in the PCI list of SIB3/4measObjectNR on the same NTN carrier frequency as carrierFreq in the NTN-NeighCellConfig of SIB19, it is left to UE implementation whether to perform subsequent measurement on this cell.

Observation 5: For option2, if PCI field is NOT configured in NTN-NeighCellConfig of SIB19 and the PCI list is NOT configured in SIB3/4measObjectNR, the corresponding ntn-Config and the frequency specific parameters in SIB3/4/measObjectNR are applicable to the cells with all possible detected PCIs on the carrierFreq.

Option 1 can reduce the number of PCI fields needed in NTN-NeighCellConfig of SIB19.  If network can guarantee that all detectable PCIs have the same satellite ephemeris information (i.e., the same NTN-config), it can reduce the overhead of PCI fields.  However, if the detectable PCIs have different satellite ephemeris information (i.e., different NTN-config), UE may waste power to continue measuring this cell with wrong satellite ephemeris information.  It also needs further study how to extend the list of PCI field in NTN-NeighCellConfig of SIB19 in order to contain multiple neighbour cells in the same satellite.

In Option 2, PCI field overhead in NTN-NeighCellConfig of SIB19 can be reduced, i.e., the PCI field is implicitly extended according to the PCI list in SIB3/4/measObjectNR and the PCI list is based on network control.

For both the options, if a detected PCI is in the PCI list of SIB3/4measObjectNR on the same NTN carrier frequency as carrierFreq in the NTN-NeighCellConfig of SIB19, both ntn-Config in the NTN-NeighCellConfig of SIB19 and the corresponding cell specific parameters in SIB3/4/measObjectNR are applicable to the cell with the detected PCI.

Observation 6: If PCI field is NOT configured in NTN-NeighCellConfig of SIB19 and a detected PCI is in the PCI list of SIB3/4measObjectNR on the same NTN carrier frequency as carrierFreq in the NTN-NeighCellConfig of SIB19, both ntn-Config in the NTN-NeighCellConfig of SIB19 and the corresponding cell specific parameters in SIB3/4/measObjectNR are applicable to the cell with the detected PCI.

Proposal 3: If PCI field is NOT configured in NTN-NeighCellConfig of SIB19, RAN2 confirms how to apply the corresponding ntn-Config to the cell with the detected PCI based on the following options:

- Option 1: The corresponding ntn-Config is applicable to all possible detected PCIs on this NTN carrier frequency (i.e., carrierFreq in NTN-NeighCellConfig of SIB19).

- Option 2: The corresponding ntn-config is only applicable to the detected PCI which is in the PCI list of SIB3/4/measObjectNR with the same carrier frequency as carrierFreq in NTN-NeighCellConfig of SIB19.

Proposal 4: Adopt the changes based on either Option 1 or Option 2 in Appendix.

Conclusion

Based on the above discussion, we have the following observations and proposals.

Observation 1: The cell/frequency specific parameters for cell reselection in SIB3/4/measObjectNR are only applicable to the PCIs/carrier frequencies listed in SIB3/4/measObjectNR, respectively.

Observation 2: If a PCI field is configured in NTN-NeighCellConfig of SIB19, the corresponding ntn-Config is applicable to the cell with the PCI on the carrierFreq.

Observation 3: If a detected PCI on the carrierFreq is NOT in the PCI field configured in NTN-NeighCellConfig of SIB19, the ntn-Config of the corresponding carrierFreq may be not applicable to the cell with this detected PCI. Hence, it is not required for UE to perform the subsequent measurement on this cell.

Observation 4: For option2, if PCI field is NOT configured in NTN-NeighCellConfig of SIB19 and a detected PCI is NOT in the PCI list of SIB3/4measObjectNR on the same NTN carrier frequency as carrierFreq in the NTN-NeighCellConfig of SIB19, it is left to UE implementation whether to perform subsequent measurement on this cell.

Observation 5: For option2, if PCI field is NOT configured in NTN-NeighCellConfig of SIB19 and the PCI list is NOT configured in SIB3/4measObjectNR, the corresponding ntn-Config and the frequency specific parameters in SIB3/4/measObjectNR are applicable to the cells with all possible detected PCIs on the carrierFreq.

Observation 6: If PCI field is NOT configured in NTN-NeighCellConfig of SIB19 and a detected PCI is in the PCI list of SIB3/4measObjectNR on the same NTN carrier frequency as carrierFreq in the NTN-NeighCellConfig of SIB19, both ntn-Config in the NTN-NeighCellConfig of SIB19 and the corresponding cell specific parameters in SIB3/4/measObjectNR are applicable to the cell with the detected PCI.

Proposal 1: RAN2 confirms that if a detected PCI on the carrierFreq is NOT in the PCI field configured in NTN-NeighCellConfig of SIB19, the ntn-Config of the corresponding carrierFreq may be not applicable to the cell with this detected PCI.

Proposal 2: For case 3 and case 4, i.e., for a detected PCI NOT in the PCI field configured in NTN-NeighCellConfig of SIB19, how to perform subsequent measurement on this cell with this PCI is left to UE implementation in this release.  If the detected PCI is in the PCI list of SIB3/4/measObjectNR, the corresponding cell specific parameters are still applicable to this cell.

Proposal 3: If PCI field is NOT configured in NTN-NeighCellConfig of SIB19, RAN2 confirms how to apply the corresponding ntn-Config to the cell with the detected PCI based on the following options:

Option 1: The corresponding ntn-Config is applicable to all possible detected PCIs on this NTN carrier frequency (i.e., carrierFreq in NTN-NeighCellConfig of SIB19).

Option 2: The corresponding ntn-Config is only applicable to the detected PCI, which is in the PCI list of SIB3/4/measObjectNR with the same carrier frequency as carrierFreq in NTN-NeighCellConfig of SIB19.

Proposal 4: Adopt the changes based on either Option 1 or Option 2 in Appendix.

References

- R2-2209538		Correction on neighbour cells’ satellite ephemeris information (38.331)	MediaTek Inc.

- R2-2210858		[offline-115] RRC corrections		Ericsson

- R2-2210801		Report from Break-Out Session on NTN, IoT NTN, RedCap and CE            Vice Chairman (ZTE)   Report

Appendix

Option 1

- 6.3.1	System information blocks

#### –	SIB19

SIB19 contains satellite assistance information for NTN access.

SIB19 information element

-- ASN1START

-- TAG-SIB19-START

SIB19-r17 ::= SEQUENCE {

ntn-Config-r17                           NTN-Config-r17                                  OPTIONAL,       -- Need R

t-Service-r17                            INTEGER (0..549755813887)                       OPTIONAL,       -- Need R

referenceLocation-r17                    ReferenceLocation-r17                           OPTIONAL,       -- Need R

distanceThresh-r17                       INTEGER(0..65525)                               OPTIONAL,       -- Need R

ntn-NeighCellConfigList-r17              NTN-NeighCellConfigList-r17                     OPTIONAL,       -- Need R

lateNonCriticalExtension                 OCTET STRING                                    OPTIONAL,

...,

[[

ntn-NeighCellConfigListExt-v1720         NTN-NeighCellConfigList-r17                     OPTIONAL        -- Need R

]]

}

NTN-NeighCellConfigList-r17 ::=          SEQUENCE (SIZE(1..maxCellNTN-r17))  OF NTN-NeighCellConfig-r17

NTN-NeighCellConfig-r17 ::=              SEQUENCE {

ntn-Config-r17                           NTN-Config-r17                                  OPTIONAL,       -- Need R

carrierFreq-r17                          ARFCN-ValueNR                                   OPTIONAL,       -- Need R

physCellId-r17                           PhysCellId                                      OPTIONAL        -- Need R

}

-- TAG-SIB19-STOP

-- ASN1STOP

SIB19 field descriptions

distanceThresh Distance from the serving cell reference location and is used in location-based measurement initiation in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20]. Each step represents 50m.

ntn-Config Provides parameters needed for the UE to access NR via NTN access such as Ephemeris data, common TA parameters, k_offset, validity duration for UL sync information and epoch.

ntn-NeighCellConfigList, ntn-NeighCellConfigListExt Provides a list of NTN neighbour cells including their ntn-Config, carrier frequency and PhysCellId. This set includes all elements of ntn-NeighCellConfigList (without suffix) and all elements of ntn-NeighCellConfigListExt-v1720. If ntn-Config is absent for an entry in ntn-NeighCellConfigListExt, the ntn-Config provided in the entry at the same position in ntn-NeighCellConfigList applies. If physCellId is not configured but carrierFreq is configured, the corresponding ntn-Config is applicable to any cell on carrierFreq.

referenceLocation Reference location of the serving cell provided via NTN quasi-Earth fixed system and is used in location-based measurement initiation in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20].

t-Service Indicates the time information on when a cell provided via NTN quasi-Earth fixed system is going to stop serving the area it is currently covering. The field indicates a time in multiples of 10 ms after 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900). The exact stop time is between the time indicated by the value of this field minus 1 and the time indicated by the value of this field.

Option 2

- 6.3.1	System information blocks

#### –	SIB19

SIB19 contains satellite assistance information for NTN access.

SIB19 information element

-- ASN1START

-- TAG-SIB19-START

SIB19-r17 ::= SEQUENCE {

ntn-Config-r17                           NTN-Config-r17                                  OPTIONAL,       -- Need R

t-Service-r17                            INTEGER (0..549755813887)                       OPTIONAL,       -- Need R

referenceLocation-r17                    ReferenceLocation-r17                           OPTIONAL,       -- Need R

distanceThresh-r17                       INTEGER(0..65525)                               OPTIONAL,       -- Need R

ntn-NeighCellConfigList-r17              NTN-NeighCellConfigList-r17                     OPTIONAL,       -- Need R

lateNonCriticalExtension                 OCTET STRING                                    OPTIONAL,

...,

[[

ntn-NeighCellConfigListExt-v1720         NTN-NeighCellConfigList-r17                     OPTIONAL        -- Need R

]]

}

NTN-NeighCellConfigList-r17 ::=          SEQUENCE (SIZE(1..maxCellNTN-r17))  OF NTN-NeighCellConfig-r17

NTN-NeighCellConfig-r17 ::=              SEQUENCE {

ntn-Config-r17                           NTN-Config-r17                                  OPTIONAL,       -- Need R

carrierFreq-r17                          ARFCN-ValueNR                                   OPTIONAL,       -- Need R

physCellId-r17                           PhysCellId                                      OPTIONAL        -- Need R

}

-- TAG-SIB19-STOP

-- ASN1STOP

SIB19 field descriptions

distanceThresh Distance from the serving cell reference location and is used in location-based measurement initiation in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20]. Each step represents 50m.

ntn-Config Provides parameters needed for the UE to access NR via NTN access such as Ephemeris data, common TA parameters, k_offset, validity duration for UL sync information and epoch.

ntn-NeighCellConfigList, ntn-NeighCellConfigListExt Provides a list of NTN neighbour cells including their ntn-Config, carrier frequency and PhysCellId. This set includes all elements of ntn-NeighCellConfigList (without suffix) and all elements of ntn-NeighCellConfigListExt-v1720. If ntn-Config is absent for an entry in ntn-NeighCellConfigListExt, the ntn-Config provided in the entry at the same position in ntn-NeighCellConfigList applies. If physCellId is not configured but carrierFreq is configured, the corresponding ntn-Config is applicable to the neighbour cells listed in SIB3/SIB4/measObjectNR on the same carrier frequency as carrierFreq.

referenceLocation Reference location of the serving cell provided via NTN quasi-Earth fixed system and is used in location-based measurement initiation in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20].

t-Service Indicates the time information on when a cell provided via NTN quasi-Earth fixed system is going to stop serving the area it is currently covering. The field indicates a time in multiples of 10 ms after 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900). The exact stop time is between the time indicated by the value of this field minus 1 and the time indicated by the value of this field.