3GPP TSG-RAN2 Meeting #120	R2-2211392

# Toulouse, France, 14 Nov – 18 Nov, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3593 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous correction to TS 38.331 for eIAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Core |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In the message of ULInformationTransferMRDC, the IABOtherInformation IE can be included in ul-DCCH-MessageNR IE as described in the table of ULInformationTransferMRDC field descriptions, but it is missing in the text corresponding to the ULInformationTransferMRDC message. - Current description of f1c-TransferPathNRDC IE does not restrict the gNB that makes this configuration. There could be the conflicted value provided in f1c-TransferPathNRDC from MCG and from SCG. In NR-DC scenario, if both gNBs (F1-terminating donor and non-(F1-terminating) donor) configure the f1c-TransferPathNRDC, collision may occur. Besides, it is unreasonable to allow the F1-C transfer path configured by the non-(F1-terminating) donor gNB. See the RAN3#111 meeting agreement: “To support CP-UP separation, the node terminating F1 interface for the IAB-node determines the transfer path of F1-C traffic” |  |  |  |  |  |  |  |  |
| Th |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In the text description of ULInformationTransferMRDC message, add that it can be used to transfer the IABOtherInformation. - Clarify the f1c-TransferPathNRDC can be only configured by the CG of F1-terminating donor. Impact analysis Impacted functionality: - The transferring of IABOtherInformation. - The configuration of F1-C traffic transfer path. Inter-operability: - If the network implements according to this CR but the IAB-MT not, there is no inter-operability issue. - If the IAB-MT implements according to this CR but the network not, the transfer path of F1-C is ambiguous. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The text of the ULInformationTransferMRDC description does not match the table. The F1-C transfer path can be configured by both gNBs in NR-DC, which may cause configuration collision. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2, 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

6.2.2	Message definitions

……

–	ULInformationTransferMRDC

The ULInformationTransferMRDC message is used for the uplink transfer of MR-DC dedicated information (e.g. for transferring the NR or E-UTRA RRC MeasurementReport message, the FailureInformation message, the UEAssistanceInformation message, the RRCReconfigurationComplete message, the IABOtherInformation message or the NR or E-UTRA RRC MCGFailureInformation message).

Signalling radio bearer: SRB1, SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

ULInformationTransferMRDC message

-- ASN1START

-- TAG-ULINFORMATIONTRANSFERMRDC-START

ULInformationTransferMRDC ::=               SEQUENCE {

criticalExtensions                          CHOICE {

c1                                          CHOICE {

ulInformationTransferMRDC                   ULInformationTransferMRDC-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

ULInformationTransferMRDC-IEs::=           SEQUENCE {

ul-DCCH-MessageNR                           OCTET STRING                    OPTIONAL,

ul-DCCH-MessageEUTRA                        OCTET STRING                    OPTIONAL,

lateNonCriticalExtension                    OCTET STRING                    OPTIONAL,

nonCriticalExtension                        SEQUENCE {}                     OPTIONAL

}

-- TAG-ULINFORMATIONTRANSFERMRDC-STOP

-- ASN1STOP

ULInformationTransferMRDC field descriptions

ul-DCCH-MessageNR Includes the UL-DCCH-Message. In this version of the specification, the field is only used to transfer the NR RRC MeasurementReport, RRCReconfigurationComplete, UEAssistanceInformation, FailureInformation, and IABOtherInformation messages when sent via SRB1 and to transfer the NR MCGFailureInformation message when sent via SRB3.

ul-DCCH-MessageEUTRA Includes the UL-DCCH-Message. In this version of the specification, the field is only used to transfer the E-UTRA RRC MeasurementReport message when sent via SRB1 and to transfer the E-UTRA MCGFailureInformation message when sent via SRB3.

Next Change

6.3.2	Radio resource control information elements

……

–	CellGroupConfig

The CellGroupConfig IE is used to configure a master cell group (MCG) or secondary cell group (SCG). A cell group comprises of one MAC entity, a set of logical channels with associated RLC entities and of a primary cell (SpCell) and one or more secondary cells (SCells).

CellGroupConfig information element

-- ASN1START

-- TAG-CELLGROUPCONFIG-START

-- Configuration of one Cell-Group:

CellGroupConfig ::=                        SEQUENCE {

cellGroupId                                CellGroupId,

rlc-BearerToAddModList                     SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig                        OPTIONAL,   -- Need N

rlc-BearerToReleaseList                    SEQUENCE (SIZE(1..maxLC-ID)) OF LogicalChannelIdentity                  OPTIONAL,   -- Need N

mac-CellGroupConfig                        MAC-CellGroupConfig                                                     OPTIONAL,   -- Need M

physicalCellGroupConfig                    PhysicalCellGroupConfig                                                 OPTIONAL,   -- Need M

spCellConfig                               SpCellConfig                                                            OPTIONAL,   -- Need M

sCellToAddModList                          SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig                       OPTIONAL,   -- Need N

sCellToReleaseList                         SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex                        OPTIONAL,   -- Need N

...,

[[

reportUplinkTxDirectCurrent                ENUMERATED {true}                                                   OPTIONAL    -- Cond BWP-Reconfig

]],

[[

bap-Address-r16                            BIT STRING (SIZE (10))                                                  OPTIONAL,   -- Need M

bh-RLC-ChannelToAddModList-r16             SEQUENCE (SIZE(1..maxBH-RLC-ChannelID-r16)) OF BH-RLC-ChannelConfig-r16 OPTIONAL,   -- Need N

bh-RLC-ChannelToReleaseList-r16            SEQUENCE (SIZE(1..maxBH-RLC-ChannelID-r16)) OF BH-RLC-ChannelID-r16     OPTIONAL,   -- Need N

f1c-TransferPath-r16                       ENUMERATED {lte, nr, both}                                              OPTIONAL,   -- Need M

simultaneousTCI-UpdateList1-r16            SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousTCI-UpdateList2-r16            SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousSpatial-UpdatedList1-r16       SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousSpatial-UpdatedList2-r16       SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

uplinkTxSwitchingOption-r16                ENUMERATED {switchedUL, dualUL}                                         OPTIONAL,   -- Need R

uplinkTxSwitchingPowerBoosting-r16         ENUMERATED {enabled}                                                    OPTIONAL    -- Need R

]],

[[

reportUplinkTxDirectCurrentTwoCarrier-r16  ENUMERATED {true}                                                       OPTIONAL    -- Need N

]],

[[

f1c-TransferPathNRDC-r17                   ENUMERATED {mcg, scg, both}                                             OPTIONAL,   -- Need M

uplinkTxSwitching-2T-Mode-r17              ENUMERATED {enabled}                                                    OPTIONAL,   -- Cond 2Tx

uplinkTxSwitching-DualUL-TxState-r17       ENUMERATED {oneT, twoT}                                                 OPTIONAL,   -- Cond 2Tx

uu-RelayRLC-ChannelToAddModList-r17        SEQUENCE (SIZE(1..maxUu-RelayRLC-ChannelID-r17)) OF Uu-RelayRLC-ChannelConfig-r17

OPTIONAL,   -- Need N

uu-RelayRLC-ChannelToReleaseList-r17       SEQUENCE (SIZE(1..maxUu-RelayRLC-ChannelID-r17)) OF Uu-RelayRLC-ChannelID-r17

OPTIONAL,   -- Need N

simultaneousU-TCI-UpdateList1-r17          SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousU-TCI-UpdateList2-r17          SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousU-TCI-UpdateList3-r17          SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousU-TCI-UpdateList4-r17          SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

rlc-BearerToReleaseListExt-r17             SEQUENCE (SIZE(1..maxLC-ID)) OF LogicalChannelIdentityExt-r17           OPTIONAL,   -- Need N

iab-ResourceConfigToAddModList-r17  SEQUENCE (SIZE(1..maxNrofIABResourceConfig-r17)) OF IAB-ResourceConfig-r17   OPTIONAL, -- Need N

iab-ResourceConfigToReleaseList-r17 SEQUENCE (SIZE(1..maxNrofIABResourceConfig-r17)) OF IAB-ResourceConfigID-r17 OPTIONAL  -- Need N

]],

[[

reportUplinkTxDirectCurrentMoreCarrier-r17 ReportUplinkTxDirectCurrentMoreCarrier-r17                            OPTIONAL  -- Need N

]]

}

-- Serving cell specific MAC and PHY parameters for a SpCell:

SpCellConfig ::=                        SEQUENCE {

servCellIndex                       ServCellIndex                                               OPTIONAL,   -- Cond SCG

reconfigurationWithSync             ReconfigurationWithSync                                     OPTIONAL,   -- Cond ReconfWithSync

rlf-TimersAndConstants              SetupRelease { RLF-TimersAndConstants }                     OPTIONAL,   -- Need M

rlmInSyncOutOfSyncThreshold         ENUMERATED {n1}                                             OPTIONAL,   -- Need S

spCellConfigDedicated               ServingCellConfig                                           OPTIONAL,   -- Need M

...,

[[

lowMobilityEvaluationConnected-r17  SEQUENCE {

s-SearchDeltaP-Connected-r17        ENUMERATED {dB3, dB6, dB9, dB12, dB15, spare3, spare2, spare1},

t-SearchDeltaP-Connected-r17        ENUMERATED {s5, s10, s20, s30, s60, s120, s180, s240, s300, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

}                                                                                               OPTIONAL,   -- Need R

goodServingCellEvaluationRLM-r17    GoodServingCellEvaluation-r17                               OPTIONAL,   -- Need R

goodServingCellEvaluationBFD-r17    GoodServingCellEvaluation-r17                               OPTIONAL,   -- Need R

deactivatedSCG-Config-r17           SetupRelease { DeactivatedSCG-Config-r17 }                  OPTIONAL    -- Cond SCG-Opt

]]

}

ReconfigurationWithSync ::=         SEQUENCE {

spCellConfigCommon                  ServingCellConfigCommon                                     OPTIONAL,   -- Need M

newUE-Identity                      RNTI-Value,

t304                                ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000},

rach-ConfigDedicated                CHOICE {

uplink                              RACH-ConfigDedicated,

supplementaryUplink                 RACH-ConfigDedicated

}                                                                                               OPTIONAL,   -- Need N

...,

[[

smtc                                SSB-MTC                                                     OPTIONAL    -- Need S

]],

[[

daps-UplinkPowerConfig-r16      DAPS-UplinkPowerConfig-r16                                      OPTIONAL    -- Need N

]],

[[

sl-PathSwitchConfig-r17         SL-PathSwitchConfig-r17                                         OPTIONAL    -- Cond DirectToIndirect-PathSwitch

]]

}

DAPS-UplinkPowerConfig-r16 ::=      SEQUENCE {

p-DAPS-Source-r16                   P-Max,

p-DAPS-Target-r16                   P-Max,

uplinkPowerSharingDAPS-Mode-r16     ENUMERATED {semi-static-mode1, semi-static-mode2, dynamic }

}

SCellConfig ::=                     SEQUENCE {

sCellIndex                          SCellIndex,

sCellConfigCommon                   ServingCellConfigCommon                                     OPTIONAL,   -- Cond SCellAdd

sCellConfigDedicated                ServingCellConfig                                           OPTIONAL,   -- Cond SCellAddMod

...,

[[

smtc                                SSB-MTC                                                     OPTIONAL    -- Need S

]],

[[

sCellState-r16                  ENUMERATED {activated}                                          OPTIONAL,   -- Cond SCellAddSync

secondaryDRX-GroupConfig-r16    ENUMERATED {true}                                               OPTIONAL    -- Cond DRX-Config2

]],

[[

preConfGapStatus-r17             BIT STRING (SIZE (maxNrofGapId-r17))                           OPTIONAL,   -- Cond PreConfigMG

goodServingCellEvaluationBFD-r17 GoodServingCellEvaluation-r17                                  OPTIONAL,   -- Need R

sCellSIB20-r17                   SetupRelease { SCellSIB20-r17 }                                OPTIONAL    -- Need M

]]

}

SCellSIB20-r17 ::= OCTET STRING (CONTAINING SystemInformation)

DeactivatedSCG-Config-r17 ::=       SEQUENCE {

bfd-and-RLM                         BOOLEAN,

...

}

GoodServingCellEvaluation-r17 ::=       SEQUENCE {

offset-r17                              ENUMERATED {db2, db4, db6, db8}                         OPTIONAL   -- Need S

}

SL-PathSwitchConfig-r17 ::=         SEQUENCE {

targetRelayUE-Identity-r17          SL-SourceIdentity-r17,

t420-r17                            ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000},

...

}

IAB-ResourceConfig-r17 ::=          SEQUENCE {

iab-ResourceConfigID-r17            IAB-ResourceConfigID-r17,

slotList-r17                        SEQUENCE (SIZE (1..5120)) OF INTEGER (0..5119)                           OPTIONAL,    -- Need M

periodicitySlotList-r17             ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10, ms20, ms40, ms80, ms160}     OPTIONAL,    -- Need M

slotListSubcarrierSpacing-r17       SubcarrierSpacing                                                        OPTIONAL,    -- Need M

...

}

IAB-ResourceConfigID-r17 ::=        INTEGER(0..maxNrofIABResourceConfig-1-r17)

ReportUplinkTxDirectCurrentMoreCarrier-r17 ::= SEQUENCE (SIZE(1.. maxSimultaneousBands)) OF IntraBandCC-CombinationReqList-r17

IntraBandCC-CombinationReqList-r17::=   SEQUENCE {

servCellIndexList-r17                   SEQUENCE (SIZE(1.. maxNrofServingCells)) OF ServCellIndex,

cc-CombinationList-r17                  SEQUENCE (SIZE(1.. maxNrofReqComDC-Location-r17)) OF IntraBandCC-Combination-r17

}

IntraBandCC-Combination-r17::=      SEQUENCE (SIZE(1.. maxNrofServingCells)) OF CC-State-r17

CC-State-r17::=                     SEQUENCE {

dlCarrier-r17                       CarrierState-r17  OPTIONAL,

ulCarrier-r17                       CarrierState-r17  OPTIONAL

}

CarrierState-r17::=                 CHOICE {

deActivated-r17                     NULL,

activeBWP-r17                       INTEGER (0..maxNrofBWPs)

}

-- TAG-CELLGROUPCONFIG-STOP

-- ASN1STOP

CC-State field descriptions

dlCarrier Indicates DL carrier activation state for this carrier and the related active BWP Index, if activated.

ulCarrier Indicates UL carrier activation state for this carrier and the related active BWP Index, if activated.

CellGroupConfig field descriptions

bap-Address BAP address of the parent node in cell group.

bh-RLC-ChannelToAddModList Configuration of the backhaul RLC entities and the corresponding MAC Logical Channels to be added and modified.

bh-RLC-ChannelToReleaseList List of the backhaul RLC entities and the corresponding MAC Logical Channels to be released.

f1c-TransferPath The F1-C transfer path that an EN-DC IAB-MT should use for transferring F1-C packets to the IAB-donor-CU. If IAB-MT is configured with lte, IAB-MT can only use LTE leg for F1-C transfer. If IAB-MT is configured with nr, IAB-MT can only use NR leg for F1-C transfer. If IAB-MT is configured with both, it is up to IAB-MT to select an LTE leg or a NR leg for F1-C transfer. If the field is not configured, the IAB node uses the NR leg as the default one.

f1c-TransferPathNRDC The F1-C transfer path that an NR-DC IAB-MT should use for transferring F1-C packets to the IAB-donor-CU. If IAB-MT is configured with mcg, IAB-MT can only use the MCG for F1-C transfer. If IAB-MT is configured with scg, IAB-MT can only use the SCG for F1-C transfer. If IAB-MT is configured with both, it is up to IAB-MT to select the MCG or the SCG for F1-C transfer. This field is only present in the CellGroupConfig from the F1-terminating donor.

mac-CellGroupConfig MAC parameters applicable for the entire cell group.

rlc-BearerToAddModList Configuration of the MAC Logical Channel, the corresponding RLC entities and association with radio bearers.

reportUplinkTxDirectCurrent Enables reporting of uplink and supplementary uplink Direct Current location information upon BWP configuration and reconfiguration. This field is only present when the BWP configuration is modified or any serving cell is added or removed. This field is absent in the IE CellGroupConfig when provided as part of RRCSetup message. If UE is configured with SUL carrier, UE reports both UL and SUL Direct Current locations.

reportUplinkTxDirectCurrentMoreCarrier Enables reporting of uplink Direct Current location information when the UE is configured with intra-band CA. This field is absent in the IE CellGroupConfig when provided as part of RRCSetup message. The UE only report the uplink Direct Current location information that are related to the indicated cc-CombinationList. The network does not include carriers which locate in DL only spectrum described in TS 38.101-2 [39] clause 5.3A.4 and defined by Fsd according to Table 5.3A.4-3 in FR2 in the IntraBandCC-CombinationReqList. I.e. DL-only carrier in FR2 frequency spectrum is not used to calculate the default DC location.

reportUplinkTxDirectCurrentTwoCarrier Enables reporting of uplink Direct Current location information when the UE is configured with uplink intra-band CA with two carriers. This field is absent in the IE CellGroupConfig when provided as part of RRCSetup message.

rlc-BearerToReleaseListExt List of the RLC entities and the corresponding MAC Logical Channels to be released for multicast MRBs.

rlmInSyncOutOfSyncThreshold BLER threshold pair index for IS/OOS indication generation, see TS 38.133 [14], table 8.1.1-1. n1 corresponds to the value 1. When the field is absent, the UE applies the value 0. Whenever this is reconfigured, UE resets N310 and N311, and stops T310, if running. Network does not include this field.

sCellSIB20 This field is used to transfer SIB20 of the SCell in order to allow the UE for MBS broadcast reception on SCell. The network configures this field only for a single SCell at a time.

sCellState Indicates whether the SCell shall be considered to be in activated state upon SCell configuration. If the field is included for an SCell configured with TRS for fast activation of the SCell, such TRS is not used for the corresponding SCell.

sCellToAddModList List of secondary serving cells (SCells) to be added or modified.

sCellToReleaseList List of secondary serving cells (SCells) to be released.

secondaryDRX-GroupConfig The field is used to indicate whether the SCell belongs to the secondary DRX group. All serving cells in the secondary DRX group shall belong to one Frequency Range and all serving cells in the legacy DRX group shall belong to another Frequency Range.

simultaneousSpatial-UpdatedList1, simultaneousSpatial-UpdatedList2 List of serving cells which can be updated simultaneously for spatial relation with a MAC CE. The simultaneousSpatial-UpdatedList1 and simultaneousSpatial-UpdatedList2 shall not contain same serving cells. Network should not configure serving cells that are configured with a BWP with two different values for the coresetPoolIndex in these lists.

simultaneousTCI-UpdateList1, simultaneousTCI-UpdateList2 List of serving cells which can be updated simultaneously for TCI relation with a MAC CE. The simultaneousTCI-UpdateList1 and simultaneousTCI-UpdateList2 shall not contain same serving cells. Network should not configure serving cells that are configured with a BWP with two different values for the coresetPoolIndex in these lists.

simultaneousU-TCI-UpdateList1, simultaneousU-TCI-UpdateList2, simultaneousU-TCI-UpdateList3, simultaneousU-TCI-UpdateList4 List of serving cells for which the Unified TCI States Activation/Deactivation MAC CE applies simultaneously, as specified in TS 38.321 [3] clause 6.1.3.47. The different lists shall not contain same serving cells. Network only configures in these lists serving cells that are configured with unifiedTCI-StateType.

spCellConfig Parameters for the SpCell of this cell group (PCell of MCG or PSCell of SCG).

uplinkTxSwitchingOption Indicates which option is configured for dynamic UL Tx switching for inter-band UL CA or (NG)EN-DC. The field is set to switchedUL if network configures option 1 as specified in TS 38.214 [19], or dualUL if network configures option 2 as specified in TS 38.214 [19]. Network always configures UE with a value for this field in inter-band UL CA case and (NG)EN-DC case where UE supports dynamic UL Tx switching.

uplinkTxSwitchingPowerBoosting Indicates whether the UE is allowed to enable 3dB boosting on the maximum output power for transmission on carrier2 under the operation state in which 2-port transmission can be supported on carrier2 for inter-band UL CA case with dynamic UL Tx switching as defined in TS 38.101-1 [15]. Network can only configure this field for dynamic UL Tx switching in inter-band UL CA case with power Class 3 as defined in TS 38.101-1 [15].

uplinkTxSwitching-2T-Mode Indicates 2Tx-2Tx switching mode is configured for inter-band UL CA or SUL, in which the switching gap duration for a triggered uplink switching (as specified in TS 38.214 [19]) is equal to the switching time capability value reported for the switching mode. If this field is absent and uplinkTxSwitching is configured, it is interpreted that 1Tx-2Tx UL Tx switching is configured as specified in TS 38.214 [19]. In this case, there is one uplink (or one uplink band in case of intra-band) configured with uplinkTxSwitching, on which the maximum number of antenna ports among all configured P-SRS/A-SRS and activated SP-SRS resources should be 1 and non-codebook based UL MIMO is not configured.

uplinkTxSwitching-DualUL-TxState Indicates the state of Tx chains if the state of Tx chains after the UL Tx switching is not unique (as specified in TS 38.214 [19]) in case of 2Tx-2Tx switching is configured and uplinkTxSwitchingOption is set to dualUL. Value oneT indicates 1Tx is assumed to be supported on the carriers on each band, value twoT indicates 2Tx is assumed to be supported on that carrier.

uu-RelayRLC-ChannelToAddModList List of the Uu RLC entities and the corresponding MAC Logical Channels to be added or modified.

uu-RelayRLC-ChannelToReleaseList List of the Uu RLC entities and the corresponding MAC Logical Channels to be released.

DeactivatedSCG-Config field descriptions

bfd-and-RLM If the field is set to true, the UE shall perform RLM and BFD on the PSCell when the SCG is deactivated and the network ensures that beamFailure is not configured in the radioLinkMonitoringConfig of the DL BWP of the PSCell in which the UE performs BFD. If set to false, the UE is not required to perform RLM and BFD on the PSCell when the SCG is deactivated.

DAPS-UplinkPowerConfig field descriptions

p-DAPS-Source The maximum total transmit power to be used by the UE in the source cell group during DAPS handover.

p-DAPS-Target The maximum total transmit power to be used by the UE in the target cell group during DAPS handover.

uplinkPowerSharingDAPS-Mode Indicates the uplink power sharing mode that the UE uses in DAPS handover (see TS 38.213 [13]).

GoodServingCellEvaluation field descriptions

offset The parameter "X" (dB) for the good serving cell quality criterion in RRC_CONNECTED, for a cell operating in FR1 and FR2, respectively. If this field is absent, the UE applies the (default) value of 0 dB for "X".

IAB-ResourceConfig field descriptions

iab-ResourceConfigID This ID is used to indicate the specific resource configuration addressed by the MAC CEs specified in TS 38.321 [3].

periodicitySlotList Indicates the periodicity in ms of the list of slot indexes indicated in slotList.

slotList Indicates the list of slot indexes to which the information indicated in the specific MAC CE applies to, as specified in TS 38.321 [3]. The values of the entries in the slotList are strictly less than the value of the periodicitySlotList.

slotListSubcarrierSpacing Subcarrier spacing used as reference for the slotList configuration. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1: 60 or 120 kHz FR2-2: 120 or 480 kHz

ReconfigurationWithSync field descriptions

rach-ConfigDedicated Random access configuration to be used for the reconfiguration with sync (e.g. handover). The UE performs the RA according to these parameters in the firstActiveUplinkBWP (see UplinkConfig).

smtc The SSB periodicity/offset/duration configuration of target cell for NR PSCell change and NR PCell change. The network sets the periodicityAndOffset to indicate the same periodicity as ssb-periodicityServingCell in spCellConfigCommon or sets to the same periodicity as ssb-Periodicity-r17 in nonCellDefiningSSB-r17 if the first active DL BWP included in this RRC message is configured with nonCellDefiningSSB-r17 for RedCap. For case of NR PCell change, the smtc is based on the timing reference of (source) PCell. For case of NR PSCell change, it is based on the timing reference of source PSCell. If both this field and targetCellSMTC-SCG are absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message. For a RedCap UE, if the first active DL BWP included in this RRC message is configured with nonCellDefiningSSB-r17, this field corresponds to the NCD-SSB indicated by nonCellDefiningSSB-r17, otherwise, this field corresponds to the CD-SSB indicated by absoluteFrequencySSB in frequencyInfoDL.

ReportUplinkTxDirectCurrentMoreCarrier field descriptions

IntraBandCC-Combination Indicates carriers states and BWPs indexes in a CC combination, each carrier in this combination correspondes an entry in servCellIndexList with same order. This field shall have same size with servCellIndexList.

IntraBandCC-CombinationReqList Indicates the list of the requested carriers/BWPs combinations for an intra-band CA component.

servCellIndexList indicates the list of cell index for an intra-band CA component.

SCellConfig field descriptions

goodServingCellEvaluationBFD Indicates the criterion for a UE to detect the good serving cell quality for BFD relaxation in an SCell in RRC_CONNECTED. This field is always configured when the network enables BFD relaxation for the UE in this SCell.

preConfGapStatus Indicates whether the pre-configured measurement gaps (i.e. the gaps configured with preConfigInd) are activated or deactivated while this SCell is deactivated. If this field is configured, the UE shall apply network-controlled mechanism for activation and deactivation of the pre-configured measurement gaps, otherwise the UE shall apply the autonomous activation/deactivation mechanism, as specified in TS 38.133 [14]. The first/leftmost bit corresponds to the measurement gap with gap ID 1, the second bit corresponds to measurement gap with gap ID 2, and so on. Value 0 indicates that the corresponding pre-configured measurement gap is deactivated while value 1 indicates that the corresponding pre-configured measurement gap is activated. The UE shall ignore the bit if the corresponding measurement gap is not a pre-configured measurement gap.

smtc The SSB periodicity/offset/duration configuration of target cell for NR SCell addition. The network sets the periodicityAndOffset to indicate the same periodicity as ssb-periodicityServingCell in sCellConfigCommon. The smtc is based on the timing of the SpCell of associated cell group. In case of inter-RAT handover to NR, the timing reference is the NR PCell. In case of intra-NR PCell change (standalone NR) or NR PSCell change (EN-DC), the timing reference is the target SpCell. If the field is absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message.

SpCellConfig field descriptions

deactivated-SCG-Config Configuration applicable when the SCG is deactivated. The network always configures this field before or when indicating that the SCG is deactivated in an RRCReconfiguration, RRCResume, E-UTRA RRCConnectionReconfiguration or E-UTRA RRCConnectionResume message.

goodServingCellEvaluationBFD Indicates the criterion for a UE to detect the good serving cell quality for BFD relaxation in the SpCell in RRC_CONNECTED. The field is always configured when the network enables BFD relaxation for the UE in this SpCell.

goodServingCellEvaluationRLM Indicates the criterion for a UE to detect the good serving cell quality for RLM relaxation in the SpCell in RRC_CONNECTED. The field is always configured when the network enables RLM relaxation for the UE in this SpCell.

lowMobilityEvaluationConnected Indicates the criterion for a UE to detect low mobility in RRC_CONNECTED in an SpCell. The s-SearchDeltaP-Connected is the parameter "SSearchDeltaP-connected". Value dB3 corresponds to 3 dB, dB6 corresponds to 6 dB and so on. The t-SearchDeltaP-Connected is the parameter "TSearchDeltaP-Connected". Value s5 means 5 seconds, value s10 means 10 seconds and so on. Low mobility criterion is configured in NR PCell for the case of NR SA/ NR CA/ NE-DC/NR-DC, and in the NR PSCell for the case of EN-DC.

reconfigurationWithSync Parameters for the synchronous reconfiguration to the target SpCell.

rlf-TimersAndConstants Timers and constants for detecting and triggering cell-level radio link failure. For the SCG, rlf-TimersAndConstants can only be set to setup and is always included at SCG addition.

servCellIndex Serving cell ID of a PSCell. The PCell of the Master Cell Group uses ID = 0.

SL-PathSwitchConfig field descriptions

targetRelayUE-Identity Indicates the L2 source ID of the target L2 U2N Relay UE during path switch.

T420 Indicates the timer value of T420 to be used during path switch.

| Conditional Presence | Explanation |
| --- | --- |
| 2Tx | The field is optionally present, Need R, if uplinkTxSwitching is configured; otherwise it is absent, Need R. |
| BWP-Reconfig | The field is optionally present, Need N, if the BWPs are reconfigured or if serving cells are added or removed. Otherwise it is absent. |
| DirectToIndirect-PathSwitch | The field is mandatory present for the L2 U2N remote UE at path switch to the target L2 U2N Relay UE. It is absent otherwise. |
| DRX-Config2 | The field is optionally present, Need N, if drx-ConfigSecondaryGroup is configured. It is absent otherwise. |
| PreConfigMG | The field is optionally present, Need R, if there is at least one per UE gap configured with preConfigInd or there is at least one per FR gap of the same FR which the SCell belongs to and configured with preConfigInd. It is absent, Need R, otherwise. |
| ReconfWithSync | The field is mandatory present in the RRCReconfiguration message: - in each configured CellGroupConfig for which the SpCell changes, - in the masterCellGroup: - at change of AS security key derived from KgNB, - in an RRCReconfiguration message contained in a DLInformationTransferMRDC message, - path switch of L2 U2N remote UE to the target PCell, - path switch of L2 U2N remote UE to the target L2 U2N Relay UE, - in the secondaryCellGroup at: - PSCell addition, - SCG resume with NR-DC or (NG)EN-DC, - update of required SI for PSCell, - change of AS security key derived from S-KgNB in NR-DC while the UE is configured with at least one radio bearer with keyToUse set to secondary and that is not released by this RRCReconfiguration message, - MN handover in (NG)EN-DC. Otherwise, it is optionally present, need M. The field is absent in the masterCellGroup in RRCResume and RRCSetup messages and is absent in the masterCellGroup in RRCReconfiguration messages if source configuration is not released during DAPS handover. |
| SCellAdd | The field is mandatory present upon SCell addition; otherwise it is absent, Need M. |
| SCellAddMod | The field is mandatory present upon SCell addition; otherwise it is optionally present, need M. |
| SCellAddSync | The field is optionally present, Need N, in the masterCellGroup and, if the SCG is not indicated as deactivated, in the secondaryCellGroup in case of SCell addition, reconfiguration with sync, and resuming an RRC connection. It is absent otherwise. |
| SCG | The field is mandatory present in an SpCellConfig for the PSCell. It is absent otherwise. |
| SCG-Opt | The field is optionally present, Need M, in an SpCellConfig for the PSCell. It is absent otherwise. |

NOTE:	In case of change of AS security key derived from S-KgNB/S-KeNB, if reconfigurationWithSync is not included in the masterCellGroup, the network releases all existing MCG RLC bearers associated with a radio bearer with keyToUse set to secondary. In case of change of AS security key derived from KgNB/KeNB, if reconfigurationWithSync is not included in the secondaryCellGroup, the network releases all existing SCG RLC bearers associated with a radio bearer with keyToUse set to primary.

End of Change