3GPP TSG RAN WG2 #120												    R2-2211414

Toulouse, France, November 14th –18th, 2022

Agenda Item:	8.9.4

Source: 	NEC

Title:	Considerations on Multipath of Sidelink Relay

Document for:	Discussion

- Introduction

In RAN2#119bis-e, the following agreements were made [1].

Alternative proposal 7-1 (modified): FFS CPDU submission; if legacy CPDU submission behaviour is supported, the primary RLC entity of the MP split bearer for DRB can be configured on any of the paths for Scenario 1. Proposal 8-1 (modified): PDCP DRB duplication is supported for the MP split bearer in Scenario 1 based on the existing framework. Proposal 8-2 (modified): PDCP DRB duplication is supported for the MP split bearer in Scenario 2 based on the existing framework. Proposal 3A: RAN2 assumes that in Scenario 2, without the adaptation layer over non-3GPP link, a PDCP PDU can be delivered to an intended PDCP entity or RLC entity for support of more than one RB over UE-to-UE link based on UE implementation. Proposal 4A (modified): RAN2 does not impose a requirement for interoperability between two UEs from different vendors for scenario 2 in this release. Proposal 1B: RAN2 understand that UE identification in L2 PDU over non-3GPP link is not in 3GPP scope in Scenario 2. Proposal 9A (modified): Do not specify adaptation layer over UE-to-UE link for scenario 2 in RAN2. Proposal 1C (modified): UE identification is not needed over Uu link in Scenario 2, if relay UE serve s only one remote UE (as in Proposal 1A) and different Uu RLC channels can be assumed for the remote UE and the relay UE (as in Proposal 5A). Working assumptions: Proposal 3A: Bearer identification except LCID is not needed in L2 PDU over Uu link in Scenario 2. Only 1:1 bearer mapping is supported over Uu link for the indirect path. FFS how t o configure the mapping. Proposal 3B: Without the adaptation layer over Uu link in scenario 2, a PDCP PDU can be delivered to an intended PDCP entity or RLC entity for support of more than one RB over Uu link e.g. by configuring 1:1 bearer mapping and different Uu RLC channels for relay UE local traffic and relay traffic for PDU delivery. Proposal 9B: Do not specify adaptation layer over Uu link for scenario 2 in RAN2. Proposal 12 [21/21] (modified) When UE operating in multi-path Relay, it performs RLM for Uu interface, for Scenario-1 and Scenario-2. For PC5 interface in Scenario-1, it performs sidelink RLF detection based on Rel-16 V2X specification [20/21]. For UE-UE link in Scenario-2, whether/how to have failure detection is out of 3GPP scope. FFS whether there is impact to layers under our control from a failure of the UE-UE link in scenario 2.

This contribution provides our considerations on multi-path relaying.

- Discussion

2.1 Considerations on Scenario 1

As per the Scenario 1 of the multi-path relaying, Remote UE is connected to the same gNB via one direct path and one indirect path with Layer-2 UE-to-Network relay. It is possible that:

- Scenario 1.1: Remote UE and Relay UE of the indirect path are served by different cells of the same gNB.

- Scenario 1.2: Remote UE and Relay UE of the indirect path are served by the same cell, and

2.1.1 RRC model

For Scenario 1.1, different cell group can be configured based on different cells. When going to the detailed model for RRC, the legacy DC design can be applied to Scenario 1.1, in which there are one RRC module at MN and the other RRC module at SN, even though there is a single/combined RRC state maintained by the UE for DC operation.

Two single cell groups based Multipath Relay is allowed for the scenario that Remote UE and Relay UE served by different cells.

For Scenario 1.2, in case of single cell based Multipath Relay or single cell group based Multipath Relay (i.e., different DUs sharing the same CU), there can be one RRC module at the network. From UE perspective, there can be one-to-one mapping between UE RRC and network RRC.

Only a single cell group based Multipath Relay is allowed for the scenario that Remote UE and Relay UE served by the same cell.

2.1.2 MAC model

When going to the detailed model for MAC,

- The PC5 connection is established between Remote UE and Relay UE over the indirect path no matter whether they are served by different cells (i.e. Scenario 1.1). or by the same cell (i.e. Scenario 1.2). Thus, the PC5 MAC function is needed for the indirect path.

- In the meantime, the Uu MAC function is needed for the direct path,

Thus, legacy combined Uu/PC5 operation for sidelink can be adopted for the multipath relaying, and a single MAC entity model for Uu and SL. Moreover, the sidelink can be treated as a separate BWP for NR, even though the sidelink functions are independent from the Uu functions.

Only one MAC entity is assumed for to support Multipath Relay for the UE in case of intra-gNB operation.

2.1.3 Basic Management Procedures for Remote UE and Relay UE served by different cells

For NR-DC, MN and SN can be served by the same gNB [2]. For Scenario 1.2, since Remote UE and Relay UE are served by different cells of the same gNB, it is possible to model these two cells as in MN/MCG and in SN/SCG separately. Therefore, procedures for NR-DC management can be adopted for the management of multi-path of sidelink relay, and some modifications are needed, such as introducing procedures for the relay bearer configuration.

RAN2 to consider procedures for NR-DC management as the baseline to manage intra-gNB multi-path relaying, especially for intra-gNB multi-path relaying with Remote UE and Relay UE served by different cells of the same gNB.

Assuming that the intra-gNB multi-path relaying is modeled as NR-DC, Relay UE can be served by either MN or SN, so that the indirect path can be managed by either MN or SN. And there is no need to restrict that the relay UE or indirect path is always managed by the MN. Therefore, if the first path is the direct path and the second path to be added is the indirect path controlled by SN, the SN addition procedure can be used to add the second path. Additionally, the configuration of relay channel is executed during this procedure. On the other hand, if the first path is the indirect path controlled by the MN, the legacy SN addition procedure can be used to add the direct path as the second path.

RAN2 to use the SN addition procedure to configure the SN together with the relay channel if the indirect path is added as the second path.

2.1.4 Basic Management Procedures for Remote UE and Relay UE served by different cells

For Scenario 1.1, whether these procedures for NR-DC management can be applied to it needs further study since both Relay UE and Remote UE are served by the same cell rather than different cells as in Scenario 1.2. If those procedures are not applicable to scenario 1.1, some new procedures need to be defined, which will introduce further work load for standardization.

For intra-gNB multi-path relaying with Remote UE and Relay UE served by the same cell, RAN2 to further study on reusing NR-DC procedures or define new procedures.

2.1.5 MP split bearer

At the previous meeting, PDCP DRB duplication is agreed to be supported for the MP split bearer based on the existing framework in both Scenario 1 and Scenario 2. The duplicated PDCP PDU discarding is one basic function of the split bearer in NR-DC, and it can be supported by the MP split bearer.

- For Scenario 1, we apply the adaptation layer over both Uu link and PC5 link of the indirect path. Thus, when we discard the duplicated PDCP Data PDU, the influence of SRAP layer over PC5 link should be considered. For example, if the acknowledgement of one PDCP PDU is received from the AM RLC entity of the direct path, whether the PDCP entity should indicate both the SRAP entity and the AM RLC entity of the indirect path to delete the acknowledged PDCP PDU. As another example, if one PDCP Data PDU is confirmed at the AM RLC entity of the indirect path, whether to notify the PDCP entity by the AM RLC entity directly or via the SRAP layer indirectly also needs to be discussed and decided.

- For Scenario 2, as per the agreement of the last meeting, the adaptation layer is not specified over UE-to-UE link of the indirect path. Therefore, the solutions for Scenario 1 are not suitable for Scenario 2. Moreover, the lower layers of UE-to-UE link are not in 3GPP scope, whether and how to enable the duplicated PDU discarding can be left to UE implementation.

For Scenario 1, to enable the duplicated PDCP PDU discarding, the influence of SRAP layer should be considered.

For Scenario 2, whether and how to enable the duplicated PDCP PDU discarding can be left to UE implementation.

2.2 Considerations on Scenario 2

2.1.1 Protocol Stack

One purpose to introducing Scenario 2 is to boost the throughput of Remote UE. Thus we can assume that at least one DRB or split DRB is configured over the indirect path. If SRB or split SRB (if allowed by RAN2 later) is also configured over the indirect path, the following problems arise.

- The UE-to-UE link of the indirect path is assumed to be ideal, which means that no RLC channel is configured between Remote UE and Relay UE. Thus, Relay UE are not able to determine one RB is for SRB or DRB based on the RLC channel transmitting it.

- Moreover, the same problem occurs when MP split bearer is configured for SRB.

So we propose:

RAN2 to discuss whether to configure SRB and DRB on the indirect path simultaneously for Scenario#2.

If SRB and DRB are configured over the indirect path simultaneously, or if the MP split bearer is configured for SRB, an extra specification effort is needed to differentiate SRB from DRB, especially when there is no adaptation layer over UE-to-UE link. So we propose:

RAN2 to discuss how to differentiate SRB from DRB for Scenario 2, if they are configured on the indirect path simultaneously.

2.1.2 RLF over UE-to-UE link

At the previous meeting, for UE-to-UE link in Scenario-2, it was agreed that whether/how to have failure detection is out of 3GPP scope. However, the 3GPP upper layer should be aware of the failure of the UE-to-UE link. Then, the 3GPP upper layer can stop the data transmission over the indirect path and trigger the reselection of Relay UE or waiting for the recovery of the UE-to-UE link.

For Scenario 2, the 3GPP upper layer should be aware of the failure of the UE-to-UE link. And how to notify the 3GPP upper layer can be left to UE implementation.

- Conclusion

Proposal 1:	Two single cell groups based Multipath Relay is allowed for the scenario that Remote UE and Relay UE served by different cells.

Proposal 2:	Only a single cell group based Multipath Relay is allowed for the scenario that Remote UE and Relay UE served by the same cell.

Proposal 3:	Only one MAC entity is assumed for to support Multipath Relay for the UE in case of intra-gNB operation.

Proposal 4:	RAN2 to consider procedures for NR-DC management as the baseline to manage intra-gNB multi-path relaying, especially for intra-gNB multi-path relaying with Remote UE and Relay UE served by different cells of the same gNB.

Proposal 5:	RAN2 to use the SN addition procedure to configure the SN together with the relay channel if the indirect path is added as the second path.

Proposal 6:	For intra-gNB multi-path relaying with Remote UE and Relay UE served by the same cell, RAN2 to further study on reusing NR-DC procedures or define new procedures.

Proposal 7:	For Scenario 1, to enable the duplicated PDCP PDU discarding, the influence of SRAP layer should be considered.

Proposal 8:	For Scenario 2, whether and how to enable the duplicated PDCP PDU discarding can be left to UE implementation.

Proposal 9:	RAN2 to discuss whether to configure SRB and DRB on the indirect path simultaneously for Scenario#2.

Proposal 10:	RAN2 to discuss how to differentiate SRB from DRB for Scenario 2, if they are configured on the indirect path simultaneously.

Proposal 11:	For Scenario 2, the 3GPP upper layer should be aware of the failure of the UE-to-UE link. And how to notify the 3GPP upper layer can be left to UE implementation.

- References

[1] Chairman’s Notes, RAN2#119bis-e.

[2] TS 37.340 V17.2.0