3GPP TSG-RAN WG2 Meeting #120                                                        R2-2211441

Toulouse, France, 14th – 18th Nov, 2022

Source:              CATT

Title:	             Further consideration on BSR

Agenda Item:	8.5.4.1

Document for:	Discussion and Decision

Introduction

In last meeting, RAN2 reached below agreements on BSR table and delay information of buffered data. This paper will discuss further the necessary BSR enhancement and propose BSR trigger and report on PDU set level.

1: introduce new BS table(s) to reduce the quantisation errors (e.g. for high bit rates). FFS how new BSR tables are created and how they impact BSR formats (can be discussed in WI phase). Delay information consists of at least “remaining time”. 2: RAN2 considers a delay information is useful for XR. FFS if dynamic reporting from UE to network (e.g. via BSR) is needed, or whether PSDB is sufficient. If we have delay information, it needs to distinguish how much data is buffered for which delay value. Stage-3 details (e.g. what’s contained, how the triggering is done) can be discussed in the WI phase. If we have delay information reporting, RAN2 aims to define how the UE determines the “remaining time” in the delay information

Discussion

2.1 PDU set level BSR

As we discussed before, PDU set information is important for the scheduling of the XR services. The agreed “delay information” should be related to specific PDU set too. Current BSR is per LCG which contains one or multiple LCHs. Even if 1 LCG contains only 1 LCH, the buffer size of the LCH will reflect the data volume of all buffered PDU sets.

PDU set level BSR can be triggered in two cases:

- Case 1: new PDU set with tight PSDB arrives at UE buffer;

- Case 2: part of PDU set can be included in current UL transmission and gNB is not aware of the remaining data of the PDU set.

An example of case 2 is shown in Figure 1. Part of PDU set1 is assembled in TB1. If there is no BSR is triggered, such as periodic BSR and padding BSR, gNB is not aware of the remaining data of PDU set1 and PSDB of PDU set1 may not be guaranteed. Hence a PDU set level BSR reporting the remaining data of PDU set1 should be triggered and reported in TB1. And then gNB can allocate UL resource for the remaining data of PDU set1.

Another benefit of the PDU set level BSR is that it can report finer buffer size to gNB to improve capacity. Although we agreed new BSR tables, the quantization error cannot be avoided especially for the large buffer size. If UE can report PDU set level BSR for the remaining data, gNB can adjust the resource allocation timely.

Figure 1 Example of timely schedule based on PDU set level BSR

According to above discussion, PDU set level BSR should be considered for XR services and the details can be discussed in stage 3.

Proposal 1: PDU set level BSR should be introduced for XR services.

2.2 Delay information

For delay information, RAN2 leave “FFS if dynamic reporting from UE to network (e.g. via BSR) is needed, or whether PSDB is sufficient.”

Generally speaking, when the gNB receives a BSR containing the BS value belonging to the LCG with high priority for XR service, it should allocate UL resources for the XR services immediately since it knows its low-latency requirements. SA2 agreed to support in TSCAI the periodicity and jitter of XR traffic bursts for both UL and DL traffic. Considering PDU sets are expected to be confined within such bursts, gNB can expect, based on the previously received bursts, when, in average, to expect the next burst, hence PDU sets. Therefore, if gNB knows the PSDB associated with a specific PDU set type (now also formally agreed to support by SA2, at least in DL) and it receives the first PDU of a PDU set of that type, it can derive the remaining time for this PDU set, and should schedule the PDU set within its PSDB. From this aspect, PSDB, combined with XR traffic burst periodicity is sufficient.

Observation 1: PSDB, combined with XR traffic burst periodicity, is sufficient for the gNB to determine the remaining time left for scheduling a PDU set within its PSDB.

Now, whether any finer and more dynamic delay information is required needs careful evaluation. Indeed “remaining time” and buffer status are different in nature, which yields the following observations:

In case of failed transmission, the BSR can arrive late at the gNB, but the buffer status still gives information gNB can make use of in the sense that the data is still in the buffer (since the transmission failed) and so gNB can still schedule UL grants accordingly. More data may have arrived in the meantime, but at least the UL grants won’t be wasted. On the contrary, the remaining time information is useful for gNB to know if it has pad for scheduling other data before this data, thus maximizing capacity. However, if this information arrives late at the gNB e.g. due to HARQ retransmissions, the “remaining time” information may be outdated and so not useful to gNB.

Observation 2: Unlike the buffer status, outdated “remaining time” information due to e.g. HARQ retransmissions may not be useful to gNB.

Then what triggers the delay reporting? A typical candidate could be a threshold on the elapsed time. But would that elapsed time monitored at PDU set level? Or finer granularity is needed, e.g. at PDU level?

Observation 3: If dynamic delay reporting is supported, it should be studied what triggers the delay reporting: a threshold on the elapsed time? Is the threshold at PDU level or PDU set level?

Furthermore, we had the following RAN2 agreement: “If we have delay information, it needs to distinguish how much data is buffered for which delay value”.

From the above agreement, it seems delay and buffer status are tightly coupled, meaning,  a trigger for a delay report also triggers a buffer status report (hopefully in same BSR), and potentially vice-versa. This will significantly increase both the number of reports and the report size. Considering the efforts through generations of BSR designs in minimizing their overhead, we are afraid the resulting BS/delay combo design could end-up complex, and involve a lot of reporting overhead, which then becomes a power consumption and capacity penalty.

Observation 4: If dynamic delay reporting is coupled with the buffer status (per RAN2 agreement) the resulting BS/delay combo design could end-up complex, and involve a lot of reporting overhead, which then becomes a power consumption and capacity penalty?

On the other hand, the capacity benefits of such dynamic delay reporting are to be proven. As we understand from supporting companies, they result from enabling the use of delay aware schedulers, rather than the regular PF scheduler. But we need more evaluation and RAN1 should be involved to assess the actual capacity gain of delay aware scheduler compared with PF scheduler for XR traffic. This, only, can determine the necessity of dynamically reporting delay information in BSR.

RAN1#110-e Agreement Whether/how to enhance BSR to improve capacity performance of XR traffic is within RAN2 scope and is not handled by RAN1. - Note that companies should indicate if and what BSR enhancement is assumed in their RAN1 proposals on CG and DG enhancements. - RAN1 can evaluate BSR enhancement to improve capacity performance

Observation 5: Dynamic delay reporting is justified by the expected capacity gains of delay aware UL scheduler compared with PF UL scheduler for XR traffic. But these remain to be shown which should be assessed in RAN1.

Proposal 2: Considering the design complexity and potential high overhead of the dynamic delay reporting, the expected associated capacity gains for XR traffic should be evaluated in RAN1 first.

Conclusion

According to the analysis in section 2, we have the following observations and proposals::

Proposal 1: PDU set level BSR should be introduced for XR services.

Observation 1: PSDB, combined with XR traffic burst periodicity, is sufficient for the gNB to determine the remaining time left for scheduling a PDU set within its PSDB.

Observation 2: Unlike the buffer status, outdated “remaining time” information due to e.g. HARQ retransmissions may not be useful to gNB.

Observation 3: If dynamic delay reporting is supported, it should be studied what triggers the delay reporting: a threshold on the elapsed time? Is the threshold at PDU level or PDU set level?

Observation 4: If dynamic delay reporting is coupled with the buffer status (per RAN2 agreement) the resulting BS/delay combo design could end-up complex, and involve a lot of reporting overhead, which then becomes a power consumption and capacity penalty?

Observation 5: Dynamic delay reporting is justified by the expected capacity gains of delay aware UL scheduler compared with PF UL scheduler for XR traffic. But these remain to be shown which should be assessed in RAN1.

Proposal 2: Considering the design complexity and potential high overhead of the dynamic delay reporting, the expected associated capacity gains for XR traffic should be evaluated in RAN1 first.

The corresponding TP on Proposal 1 is attached in the Annex.

References

- R2-2210814, TR 38.835, v030

Annex: TP to TR 38.835 (R2-2210814)

5.3		Capacity Improvements Techniques

5.3.1	Physical Layer Enhancements

5.3.2	Layer 2 Enhancements

In order to enhance the scheduling of uplink resources for XR, the following improvements are envisioned:

-	New BS table(s) to reduce the quantisation errors in BSR reporting (e.g. for high bit rates);

-	Delay knowledge of buffered data, consisting of at least remaining time, and distinguishing how much data is buffered for which delay.

Editor's Note: FFS if dynamic reporting of the uplink delay (e.g. via BSR) is needed, or whether PSDB is sufficient.

-	BSR trigger and report on PDU set level.

Furthermore, when, for a PDU set, the number of PDUs known to either be lost or associated to discarded SDUs, exceeds a threshold (see subclause 5.1.1), all remaining PDUs of that PDU set could be discarded at the transmitter to free up radio resources.

NOTE: this means that the granularity of the discard operation at PDCP in the transmitter should be the PDU set.

Editor's Note: the discard operation could also be captured in 5.2.2.