3GPP TSG-RAN WG2 Meeting #120	R2-2211508

# Toulouse, France, 14 – 18 November, 2022

Agenda Item:	8.15.2

Source:	Ericsson

Title:	CAPC table and MAC multiplex rules

Document for:	Discussion, Decision

# 1	Introduction

In the WID [1], Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation (SL-U). In NR-U, channel access priority class (CAPC) has been defined for Type 1 channel access procedures, based on which the initiating device applies different channel access parameters (e.g., congestion window (CW) or maximum channel occupancy time (MCOT)) for different services. In this paper, we discuss how to reuse NR-U CAPC table for SL-U.

# 2	Discussion

Based on the RAN2 agreements in RAN2#119bis-e, the following aspects need to be further discussed

Working assumption: PQI is used to determine the CAPC mapping as in NR-U. FFS whether the same principle is also applied to the UE side.

If PQI-based CAPC mapping is agreed, as in NR-U, to determine the CAPC of the SL TB when the CAPC is not indicated in the DCI:

- If only SL MAC CE(s) are included in the SL TB, the highest priority SL CAPC is used; FFS whether this rule can be extended to the case when SL MAC CE(s) multiplexed with STCH.

- If SCCH SDU(s) are included in the SL TB, the highest priority SL CAPC is used;

- FFS how to select SL CAPC when SL CAPC of the SL logical channel(s) with MAC SDU multiplexed in the SL TB is used otherwise.

For the working assumption, we see no reasons to not confirm it. In the previous RAN2 meetings, some companies have suggested to consider other priority indicator e.g., L1 priority in the mapping table. However, the L1 priority is set according to LCH priorities configured by the gNB, which are UE specific configuration. While, the CAPC mapping table needs to be defined as a common table to all UEs and gNBs, so that all UEs or gNBs can apply common rules to treat the CAPC for each transmissions. It is necessary to consider global QoS identifiers (e.g., PQI) in the table rather than UE specific QoS identifiers (e.g., L1 priority in the SCI). In this way, fairness between SL-U UEs when accessing unlicensed channels can be better ensured.

It is necessary to consider global QoS identifiers (e.g., PQI) in the table rather than UE specific QoS identifiers (e.g., L1 priority in the SCI).

Therefore, we suggest RAN2 to confirm the above work assumption and close the issue.

To confirm that PQI is used to determine the CAPC mapping as in NR-U.

For the FFS whether the same principle is also applied to the UE side, based on the same arguments, i.e., channel access to SL-U channels need to be ensure to be fair between SL-UEs, it is straightforward for RAN2 to agree that the common CAPC table needs to be applied by both UEs and gNBs.

The same CAPC table is applied to both the gNB (i.e., SL transmissions using Mode 1 RA) and the UE (i.e., SL transmissions using Mode 2 RA).

In NR-U, the following multiplex rules have been defined.

When performing Type 1 LBT for the transmission of an uplink TB (see TS 37.213 [37], clause 4.2.1.1) and when the CAPC is not indicated in the DCI, the UE shall select the CAPC as follows:

- If only MAC CE(s) are included in the TB, the highest priority CAPC of those MAC CE(s) is used; or

- If CCCH SDU(s) are included in the TB, the highest priority CAPC is used; or

- If DCCH SDU(s) are included in the TB, the highest priority CAPC of the DCCH(s) is used; or

- The lowest priority CAPC of the logical channel(s) with MAC SDU multiplexed in the TB is used otherwise.

RAN2 has discussed whether highest priority CAPC is used if the MAC PDU contains both SL MAC CE and SL LCHs. The intention is understandable, however, the rules of CAPC selection were defined for NR-U mainly from ensuring fairness between nodes and between UEs and Wifi devices perspective rather than QoS improvement. The same rules are also applicable to SL-U operation. Therefore, it is sufficient for SL-U to adopt same rules as for NR-U without any special treatment.

The rules of CAPC selection were defined for NR-U mainly from ensuring fairness between nodes and between UEs and Wifi devices perspective rather than QoS improvement.

If PQI-based CAPC mapping is agreed, as in NR-U, the lowest priority CAPC of the logical channel(s) with MAC SDU multiplexed in the TB is used regardless of whether the TB also contains SL MAC CEs in addition to MAC SDUs.

# 3 Conclusion

In the previous sections we made the following observations:

Observation 1	It is necessary to consider global QoS identifiers (e.g., PQI) in the table rather than UE specific QoS identifiers (e.g., L1 priority in the SCI).

Observation 2	The rules of CAPC selection were defined for NR-U mainly from ensuring fairness between nodes and between UEs and Wifi devices perspective rather than QoS improvement.

Based on the discussion in the previous sections we propose the following:

Proposal 1	To confirm that PQI is used to determine the CAPC mapping as in NR-U.

Proposal 2	The same CAPC table is applied to both the gNB (i.e., SL transmissions using Mode 1 RA) and the UE (i.e., SL transmissions using Mode 2 RA).

Proposal 3	If PQI-based CAPC mapping is agreed, as in NR-U, the lowest priority CAPC of the logical channel(s) with MAC SDU multiplexed in the TB is used regardless of whether the TB also contains SL MAC CEs in addition to MAC SDUs.

# 4 References

- RP- 221798, WID revision: NR sidelink evolution, RAN #96, Budapest, Hungary, June 6-9, 2022.