3GPP TSG-RAN WG2 Meeting #120	R2-2211544

# Toulouse, France, 14th - 18th November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0390 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous corrections to LPP capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2022-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - NR-DL-PRS-ProcessingCapability field descriptions: in the description of prs-ProcessingCapabilityOutsideMGinPPW a NOTE is defined with references to the fields ppw-maxNumOfDL-BandwidthFR1 and ppw-maxNumOfDL-BandwidthFR2. However, both fields do not exist in ASN.1. Instead, they should be replaced by ppw-maxNumOfDL-Bandwidth. This issue is originated from the fact that the field ppw-maxNumOfDL-Bandwidth was originally defined as separate field for FR1 and FR2, see below. But this was then later changed to a single field with CHOICE structure. [[ ppw-maxNumOfDL-BandwidthFR1-r17 ENUMERATED {mhz5, mhz10, mhz20, mhz40, mhz50, mhz80, mhz100} OPTIONAL, ppw-maxNumOfDL-BandwidthFR2-r17 ENUMERATED {mhz50, mhz100, mhz200, mhz400} OPTIONAL ]] Furthermore, in the NOTE the part “defined in TS 38.331 [35]” related to the pre-requisite support of the PRS processing window capabilities can be removed. The reason is that prs-ProcessingCapabilityOutsideMGinPPW-r17 and the PRS processing window capabilities (prs-ProcessingWindowType1A, prs-ProcessingWindowType1B, prs-ProcessingWindowType2) are specified in both NR RRC and LPP. Thus, there is no need to specify such cross-layer dependency. Instead, it is recommended to specify the pre-requisite support of the PRS processing window capabilities separately in each layer. - NR-UL-SRS-Capability: referring to the latest RAN1 features list R1-2207923 the FG 27-15 and 27-15a are defined with regards to positioning SRS transmission in RRC_INACTIVE state for initial BWP, see table below. In NR RRC the FGs have been specified in separate capabilities, i.e. FG 27-15 in srs-PosResourcesRRC-Inactive-r17 and FG 27-15a in srs-SemiPersistent-PosResourcesRRC-Inactive-r17. However, in LPP both FGs have been merged into the same capability posSRS-RRC-Inactive-InInitialUL-BWP-r17. Since FG 27-15 is pre-requisite for FG 27-15a this results in a very confusing condition for the presence of the fields related to FG 27-15a (maxNumOfSemiPersistentSRSposResources-r17 and maxNumOfSemiPersistentSRSposResourcesPerSlot-r17) saying: “The UE can include this field only if the UE supports posSRS-RRC-Inactive-InInitialUL-BWP. Otherwise, the UE does not include this field.” To avoid the confusing presence condition and considering the fact that FG 27-15 is pre-requisite for other FGs (27-15b, 27-15c, 27-16a, 27-19a) but FG 27-15a is not pre-requisite for other FGs, it is recommended to specifiy FG 27-15a as separate capability as it was done in NR RRC. FG; Description; Components; Pre-requisite. 27-15; Positioning SRS transmission in RRC_INACTIVE state for initial UL BWP (->ASN.1: posSRS-RRC-Inactive-InInitialUL-BWP-r17); 1. Max number of SRS Resource Sets for positioning supported by UE (->ASN.1: maxNumOfSRSposResourceSets-r17) 2. Max number of P/SPSRS Resources for positioning (->ASN.1: maxNumOfPeriodicAndSemiPeristentSRSposResources-r17) 3. Max number of P/SPSRS Resources for positioning per slot (->ASN.1: maxNumOfPeriodicAndSemiPeristentSRSposResourcesPerSlot-r17) 4. Max number of periodic SRS Resources for positioning (->ASN.1: maxNumOfPeriodicSRSposResources-r17) 5. Max number of periodic SRS Resources for positioning per slot (->ASN.1: maxNumOfPeriodicSRSposResourcesPerSlot-r17). 27-15a; Support of positioning SRS transmission in RRC_INACTIVE state for initial BWP with semi-persistent SRS (->ASN.1: posSRS-RRC-Inactive-InInitialUL-BWP-r17); 1. Max number of semi-persistent SRS Resources for positioning (->ASN.1: maxNumOfSemiPeristentSRSposResources-r17) 2. Max number of semi-persistent SRS Resources for positioning per slot (->ASN.1: maxNumOfSemiPersistentSRSposResourcesPerSlot-r17); 27-15. 27-15b; Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP (->ASN.1: posSRS-RRC-Inactive-OutsideInitialUL-BWP-r17); 1. Maximum SRS bandwidth supported for each SCS that UE supports within a single CC 2. Max number of SRS Resource Sets for positioning supported by UE 3. Max number of periodic SRS Resources for positioning 4. Max number of periodic SRS Resources for positioning per slot 5. Support of ifferent numerology between the SRS and the initial UL BWP 6. Support of SRS operation without restriction on the BW: BW of the SRS may not include BW of the CORESET#0 and SSB 7. Max number of P/SP SRS Resources for positioning 8. Max number of P/SP SRS Resources for positioning per slot 9. Support a different center frequenecy between the SRS for positioning and the initial UL BWP 10. Switching time between SRS Tx and other Tx in initial UL BWP or Rx in initial DL BWP; 27-15. 27-15c; Support of positioning SRS transmission in RRC_INACTIVE state outside initial BWP with semi-persistent SRS (->ASN.1: posSRS-RRC-Inactive-OutsideInitialUL-BWP-r17); 1. Max number of semi-persistent SRS Resources for positioning (->ASN.1: maxNumOfSemiPersistentSRSposResources-r17) 2. Max number of semi-persistent SRS Resources for positioning per slot (->ASN.1: maxNumOfSemiPersistentSRSposResourcesPerSlot-r17); 27-15. 27-16a; OLPC for positioning SRS in RRC_INACTIVE state – location server (->ASN.1: olpc-SRS-PosRRC-Inactive-r17); Same as LPP OLPC-SRS-Pos-r16; 27-15. 27-19a; Spatial relation for positioning SRS in RRC_INACTIVE state – location server (->ASN.1: spatialRelationsSRS-PosRRC-Inactive-r17); Same as LPP SpatialRelationsSRS-Pos-r16; 27-15. - NR-DL-AoD-MeasurementCapability: With regards to FG 27-2-2 new Rel-17 fields maxDL-PRS-RSRP-MeasurementFR1-r17, maxDL-PRS-RSRP-MeasurementFR2-r17 were introduced to extend the Rel-16 value range to {16, 24}. FG; Description; Components; Pre-requisite. 27-2-2; DL PRS RSRP reporting for more than 8 measurements for UE-assisted DL-AoD positioning; Support reporting K> 8 DL PRS RSRP measurements per TRP.; 13-5. But since the corresponding Rel-16 fields (FG 13-5) are pre-requisite for FG 27-2-2 a confusing condition for the presence of the Rel-17 fields has been defined in the respective field descriptions, e.g. for maxDL-PRS-RSRP-MeasurementFR1: “The UE can include maxDL-PRS-RSRP-MeasurementFR1 only if the UE supports one of maxDL-PRS-RSRP-MeasurementFR1 and maxDL-PRS-RSRP-MeasurementFR2. Otherwise, the UE does not include this field.” To avoid the confusing condition and considering the fact that the Rel-17 fields merely represent an extension of the value range for the existing Rel-16 fields, it is recommended to define the new Rel-17 fields as Rel-17 non-critical extensions of the Rel-16 fields maxDL-PRS-RSRP-MeasurementFR1-r16, maxDL-PRS-RSRP-MeasurementFR2-r16. - There are editorial issues in ASN.1 and field descriptions which need to be fixed (typos etc.). |
| Summary of change: |  | - NR-DL-PRS-ProcessingCapability field descriptions: in the description of prs-ProcessingCapabilityOutsideMGinPPW the fields ppw-maxNumOfDL-BandwidthFR1 and ppw-maxNumOfDL-BandwidthFR2 have been replaced by ppw-maxNumOfDL-Bandwidth. Furthermore, the part “defined in TS 38.331 [35]” has been removed. - NR-UL-SRS-Capability: - FG 27-15a has been specified as separate capability posSRS-SP-RRC-Inactive-InInitialUL-BWP-r17. - In IE PosSRS-RRC-Inactive-InInitialUL-BWP-r17 the fields maxNumOfSemiPersistentSRSposResources-r17, maxNumOfSemiPersistentSRSposResourcesPerSlot-r17) have been dummified. - NR-DL-AoD-MeasurementCapability: - In ASN.1 the suffices of the Rel-17 fields maxDL-PRS-RSRP-MeasurementFR1 and maxDL-PRS-RSRP-MeasurementFR2 have been corrected to “-v1730”. - In the respective field descriptions the confusing condition for the presence of the Rel-17 fields have been removed. - Editorial issues in ASN.1 and field descriptions have been fixed. Impact analysis Impacted functionality: LPP capability signalling Inter-operability: To change 1), 3) and 4): There are no interoperability issues. To change 2): - If the UE is implemented according to this CR while the network is not, the network will not comprehend the capability signalling of maxNumOfSemiPersistentSRSposResources-r17 and maxNumOfSemiPersistentSRSposResourcesPerSlot-r17 in posSRS-SP-RRC-Inactive-InInitialUL-BWP-r17 and will not configure the UE with semi-persistent SRS in initial UL BWP for positioning in RRC_INACTIVE. - If the network is implemented according to this CR while the UE is not, the network will ignore the capability signalling of maxNumOfSemiPersistentSRSposResources-r17 and maxNumOfSemiPersistentSRSposResourcesPerSlot-r17 in posSRS-RRC-Inactive-InInitialUL-BWP-r17 and will not configure the UE with semi-persistent SRS in initial UL BWP for positioning in RRC_INACTIVE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | To change 1), 3) and 4): Inconsistencies in the specification of LPP capabilities remain. To change 2): The specification of the FGs 27-15, 27-15a, 27-15b, 27-15c, 27-16a, 27-19a remains misaligned with the RAN1 features list. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3, 6.5.11.6a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

### 6.4.3	Common NR Positioning Information Elements

<Text omitted>

#### –	NR-DL-PRS-ProcessingCapability

The IE NR-DL-PRS-ProcessingCapability defines the common DL-PRS Processing capability. In the case of capabilities for multiple NR positioning methods are provided, the IE NR-DL-PRS-ProcessingCapability applies across the NR positioning methods and the target device shall indicate the same values for the capabilities in IEs NR-DL-TDOA-ProvideCapabilities, NR-DL-AoD-ProvideCapabilities, and NR-Multi-RTT-ProvideCapabilities.

The PRS-ProcessingCapabilityPerBand is defined for a single positioning frequency layer on a certain band (i.e., a target device supporting multiple positioning frequency layers is expected to process one frequency layer at a time).

-- ASN1START

NR-DL-PRS-ProcessingCapability-r16 ::= SEQUENCE {

prs-ProcessingCapabilityBandList-r16	SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

PRS-ProcessingCapabilityPerBand-r16,

maxSupportedFreqLayers-r16				INTEGER (1..4),

simulLTE-NR-PRS-r16						ENUMERATED { supported }		OPTIONAL,

...,

[[

dummy									ENUMERATED { m1, m2, ... }		OPTIONAL

]]

}

PRS-ProcessingCapabilityPerBand-r16 ::= SEQUENCE {

freqBandIndicatorNR-r16				FreqBandIndicatorNR-r16,

supportedBandwidthPRS-r16			CHOICE {

fr1										ENUMERATED {mhz5, mhz10, mhz20, mhz40,

mhz50, mhz80, mhz100},

fr2										ENUMERATED {mhz50, mhz100, mhz200, mhz400},

...

},

dl-PRS-BufferType-r16		 		ENUMERATED {type1, type2, ...},

durationOfPRS-Processing-r16		SEQUENCE {

durationOfPRS-ProcessingSymbols-r16	ENUMERATED {nDot125, nDot25, nDot5, n1,

n2, n4, n6, n8, n12, n16, n20, n25,

n30, n32, n35, n40, n45, n50},

durationOfPRS-ProcessingSymbolsInEveryTms-r16

ENUMERATED {n8, n16, n20, n30, n40, n80,

n160,n320, n640, n1280},

...

},

maxNumOfDL-PRS-ResProcessedPerSlot-r16	SEQUENCE {

scs15-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

scs30-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

scs60-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

scs120-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

...

},

...,

[[

supportedDL-PRS-ProcessingSamples-RRC-CONNECTED-r17	ENUMERATED { supported }		OPTIONAL,

prs-ProcessingWindowType1A-r17			ENUMERATED { option1, option2, option3}		OPTIONAL,

prs-ProcessingWindowType1B-r17			ENUMERATED { option1, option2, option3}		OPTIONAL,

prs-ProcessingWindowType2-r17			ENUMERATED { option1, option2, option3}		OPTIONAL,

prs-ProcessingCapabilityOutsideMGinPPW-r17

SEQUENCE (SIZE(1..3)) OF

PRS-ProcessingCapabilityOutsideMGinPPWperType-r17

OPTIONAL,

dl-PRS-BufferType-RRC-Inactive-r17		ENUMERATED { type1, type2, ... }			OPTIONAL,

durationOfPRS-Processing-RRC-Inactive-r17	SEQUENCE {

durationOfPRS-ProcessingSymbols-r17			ENUMERATED {nDot125, nDot25, nDot5, n1,

n2, n4, n6, n8, n12, n16, n20, n25,

n30, n32, n35, n40, n45, n50},

durationOfPRS-ProcessingSymbolsInEveryTms-r17

ENUMERATED {n8, n16, n20, n30, n40, n80,

n160,n320, n640, n1280},

...

}																					OPTIONAL,

maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive-r17	SEQUENCE {

scs15-r17								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs30-r17								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs60-r17								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs120-r17								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

...

}																					OPTIONAL,

supportedLowerRxBeamSweepingFactor-FR2-r17	ENUMERATED { n1, n2, n4, n6 }			OPTIONAL

]],

[[

supportedDL-PRS-ProcessingSamples-RRC-Inactive-r17	ENUMERATED { supported }		OPTIONAL

]]

}

PRS-ProcessingCapabilityOutsideMGinPPWperType-r17 ::= SEQUENCE {

prsProcessingType-r17						ENUMERATED { type1A, type1B, type2 },

ppw-dl-PRS-BufferType-r17		 			ENUMERATED { type1, type2, ... },

ppw-durationOfPRS-Processing1-r17			SEQUENCE {

ppw-durationOfPRS-ProcessingSymbolsN-r17

ENUMERATED { msDot125, msDot25, msDot5, ms1, ms2, ms4,

ms6, ms8, ms12, ms16, ms20, ms25, ms30, ms32, ms35,

ms40, ms45, ms50 },

ppw-durationOfPRS-ProcessingSymbolsT-r17

ENUMERATED { ms1, ms2, ms4, ms8, ms16, ms20, ms30, ms40, ms80,

ms160, ms320, ms640, ms1280 }

}																					OPTIONAL,

ppw-durationOfPRS-Processing2-r17			SEQUENCE {

ppw-durationOfPRS-ProcessingSymbolsN2-r17

ENUMERATED { msDot125, msDot25, msDot5, ms1, ms2, ms3, ms4, ms5,

ms6, ms8, ms12 },

ppw-durationOfPRS-ProcessingSymbolsT2-r17

ENUMERATED { ms4, ms5, ms6, ms8 }

}																					OPTIONAL,

ppw-maxNumOfDL-PRS-ResProcessedPerSlot-r17	SEQUENCE {

scs15-r17									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

scs30-r17									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

scs60-r17									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

scs120-r17									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

...

},

...,

[[

ppw-maxNumOfDL-Bandwidth-r17		CHOICE {

fr1										ENUMERATED {mhz5, mhz10, mhz20, mhz40,

mhz50, mhz80, mhz100},

fr2										ENUMERATED {mhz50, mhz100, mhz200, mhz400}

}																					OPTIONAL

]]

}

-- ASN1STOP

NR-DL-PRS-ProcessingCapability field descriptions

maxSupportedFreqLayers Indicates the maximum number of positioning frequency layers supported by UE.

simulLTE-NR-PRS Indicates whether the UE supports parallel processing of LTE PRS and NR PRS.

dummy This field is not used in the specification. If received it shall be ignored by the receiver.

supportedBandwidthPRS Indicates the maximum number of DL-PRS bandwidth in MHz, which is supported and reported by UE.

dl-PRS-BufferType Indicates DL-PRS buffering capability. Value type1 indicates sub-slot/symbol level buffering and value type2 indicates slot level buffering.

durationOfPRS-Processing Indicates the duration N of DL-PRS symbols in units of ms a UE can process every T ms assuming maximum DL-PRS bandwidth provided in supportedBandwidthPRS and comprises the following subfields: - durationOfPRS-ProcessingSymbols: This field specifies the values for N. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 8, 12, 16, 20, 25, 30, 35, 40, 45, 50 ms. - durationOfPRS-ProcessingSymbolsInEveryTms: This field specifies the values for T. Enumerated values indicate 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280 ms. See NOTE.

maxNumOfDL-PRS-ResProcessedPerSlot Indicates the maximum number of DL-PRS resources that UE can process in a slot. SCS: 15 kHz, 30 kHz, 60 kHz are applicable for FR1 bands. SCS: 60 kHz, 120 kHz are applicable for FR2 bands.

supportedDL-PRS-ProcessingSamples-RRC-CONNECTED Indicates the UE capability for support of measurements based on measuring M=1 or M=2 (instances) of a DL-PRS Resource Set. The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList. Otherwise, the UE does not include this field. NOTE: This feature is supported for both UE-assisted and UE based positioning.

prs-ProcessingWindowType1A Indicates the supported DL-PRS processing types subject to the UE determining that DL-PRS to be higher priority for DL-PRS measurement outside MG and in a DL-PRS Processing Window. Type 1A refers to the determination of prioritization between DL-PRS and other DL signals/channels in all OFDM symbols within the PRS Processing Window. The DL signals/channels from all DL CCs (per UE) are affected across LTE and NR. Enumerated value indicates supported priority handing options of DL-PRS: - option1: UE indicates support of two priority states. - State 1: DL-PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: DL-PRS is lower priority than all PDCCH/PDSCH/CSI-RS - option2: UE indicates support of three priority states - State 1: DL-PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: DL-PRS is lower priority than PDCCH and URLLC PDSCH and higher priority than other PDSCH/CSI-RS Note: The URLLC channel corresponds a dynamically scheduled PDSCH whose PUCCH resource for carrying ACK/NAK is marked as high-priority. - State 3: DL-PRS is lower priority than all PDCCH/PDSCH/CSI-RS - option3: UE indicates support of single priority state - State 1: DL-PRS is higher priority than all PDCCH/PDSCH/CSI-RS The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList. Otherwise, the UE does not include this field. NOTE: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP.

prs-ProcessingWindowType1B Indicates the supported DL-PRS processing types subject to the UE determining that DL-PRS to be higher priority for DL-PRS measurement outside MG and in a DL-PRS Processing Window. Type 1B refers to the determination of prioritization between DL-PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from a certain band are affected. Enumerated value indicates supported priority handing options of DL-PRS (see prs-ProcessingWindowType1A). The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList. Otherwise, the UE does not include this field. NOTE: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP.

prs-ProcessingWindowType2 Indicates the supported DL-PRS processing types subject to the UE determining that DL-PRS to be higher priority for DL-PRS measurement outside MG and in a DL-PRS Processing Window. Type 2 refers to the determination of prioritization between DL-PRS and other DL signals/channels only in DL-PRS symbols within the PRS processing window. Enumerated value indicates supported priority handing options of DL-PRS (see prs-ProcessingWindowType1A). The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList. Otherwise, the UE does not include this field. NOTE: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP.

prs-ProcessingCapabilityOutsideMGinPPW Indicates the DL-PRS Processing Capability outside MG and comprises the following subfields: - prsProcessingType: Indicates the DL-PRS Processing Window Type for which the prs-ProcessingCapabilityOutsideMGinPPW are provided. - ppw-dl-PRS-BufferType: Indicates DL-PRS buffering capability. Value 'type1' indicates sub-slot/symbol level buffering and value 'type2' indicates slot level buffering. - ppw-durationOfPRS-Processing1: Indicates the duration of DL-PRS symbols N in units of ms a UE can process every T ms assuming maximum DL-PRS bandwidth provided in ppw-maxNumOfDL-Bandwidth and comprises the following subfields: - ppw-durationOfPRS-ProcessingSymbolsN: This field specifies the values for N. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms. - ppw-durationOfPRS-ProcessingSymbolsT: This field specifies the values for T. Enumerated values indicate 1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280 ms. - ppw-durationOfPRS-Processing2: Indicates the duration of DL-PRS symbols N2 in units of ms a UE can process inT2 ms assuming maximum DL-PRS bandwidth provided in ppw-maxNumOfDL-Bandwidth and comprises the following subfields: - ppw-durationOfPRS-ProcessingSymbolsN2: This field specifies the values for N2. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 3, 4, 5, 6, 8, 12 ms. - ppw-durationOfPRS-ProcessingSymbolsT2: This field specifies the values for T2. Enumerated values indicate 4, 5, 6, 8 ms. - ppw-maxNumOfDL-PRS-ResProcessedPerSlot: Indicates the maximum number of DL-PRS resources that UE can process in a slot. SCS: 15 kHz, 30 kHz, 60 kHz are applicable for FR1 bands. SCS: 60 kHz, 120 kHz are applicable for FR2 bands. - ppw-maxNumOfDL-Bandwidth: Indicates the maximum number of DL PRS bandwidth in MHz, which is supported and reported by UE for PRS measurement outside MG within the PPW. The UE can include this field only if the UE supports one of prs-ProcessingWindowType1A, prs-ProcessingWindowType1B and prs-ProcessingWindowType2. Otherwise, the UE does not include this field. NOTE: A UE that supports one of prs-ProcessingWindowType1A, prs-ProcessingWindowType1B or prs-ProcessingWindowType2 shall always support ppw-dl-PRS-BufferType, ppw-durationOfPRS-Processing1, ppw-durationOfPRS-Processing2, ppw-maxNumOfDL-PRS-ResProcessedPerSlot, and ppw-maxNumOfDL-Bandwidth.

dl-PRS-BufferType-RRC-Inactive Indicates DL-PRS buffering capability in RRC_INACTIVE state. Value 'type1' indicates sub-slot/symbol level buffering and value 'type2' indicates slot level buffering.

durationOfPRS-Processing-RRC-Inactive Indicates the duration N of DL-PRS symbols in units of ms a UE can process every T ms in RRC_INACTIVE state assuming maximum DL-PRS bandwidth provided in supportedBandwidthPRS and comprises the following subfields: - durationOfPRS-ProcessingSymbols: This field specifies the values for N. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms. - durationOfPRS-ProcessingSymbolsInEveryTms: This field specifies the values for T. Enumerated values indicate 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280 ms. See NOTE.

maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive Indicates the maximum number of DL-PRS resources a UE can process in a slot in RRC_INACTIVE state. SCS: 15 kHz, 30 kHz, 60 kHz are applicable for FR1 bands. SCS: 60 kHz, 120 kHz are applicable for FR2 bands.

supportedLowerRxBeamSweepingFactor-FR2 Indicates support of the lower Rx beam sweeping factor than 8 for FR2. Enumerated value indicates the number of Rx beam sweeping factors supported.

supportedDL-PRS-ProcessingSamples-RRC-Inactive Indicates the UE capability for support of measurements based on measuring M=1 or M=2 samples (instances) of a DL-PRS Resource Set in RRC_INACTIVE state. The UE can include this field only if the UE supports prs-ProcessingRRC-Inactive defined in TS 38.331 [35]. Otherwise, the UE does not include this field.

NOTE:	When the target device provides the durationOfPRS-Processing capability (N, T) for any P(≥T) time window defined in TS 38. 214 [45] clause 5.1.6.5, the target device should be capable of processing all DL-PRS resources within P, if

-	N≥K where K is defined in the TS 38.214 [45] clause 5.1.6.5, and

-	the number of DL-PRS Resources in each slot does not exceed the maxNumOfDL-PRS-ResProcessedPerSlot, and

-	the configured measurement gap and a maximum ratio of measurement gap length (MGL) / measurement gap repetition period (MGRP) is as specified in TS 38.133 [46].

<Text omitted>

#### –	NR-UL-SRS-Capability

The IE NR-UL-SRS-Capability defines the UE uplink SRS capability.

-- ASN1START

NR-UL-SRS-Capability-r16 ::= SEQUENCE {

srs-CapabilityBandList-r16					SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

SRS-CapabilityPerBand-r16,

srs-PosResourceConfigCA-BandList-r16		SEQUENCE (SIZE (1..nrMaxConfiguredBands-r16)) OF

SRS-PosResourcesPerBand-r16			OPTIONAL,

maxNumberSRS-PosPathLossEstimateAllServingCells-r16

ENUMERATED {n1, n4, n8, n16}			OPTIONAL,

maxNumberSRS-PosSpatialRelationsAllServingCells-r16

ENUMERATED {n0, n1, n2, n4, n8, n16}	OPTIONAL,

...

}

SRS-CapabilityPerBand-r16 ::= SEQUENCE {

freqBandIndicatorNR-r16			FreqBandIndicatorNR-r16,

olpc-SRS-Pos-r16				OLPC-SRS-Pos-r16									OPTIONAL,

spatialRelationsSRS-Pos-r16		SpatialRelationsSRS-Pos-r16							OPTIONAL,

...,

[[

posSRS-RRC-Inactive-InInitialUL-BWP-r17		PosSRS-RRC-Inactive-InInitialUL-BWP-r17	OPTIONAL,

posSRS-RRC-Inactive-OutsideInitialUL-BWP-r17

PosSRS-RRC-Inactive-OutsideInitialUL-BWP-r17

OPTIONAL,

olpc-SRS-PosRRC-Inactive-r17				OLPC-SRS-Pos-r16						OPTIONAL,

spatialRelationsSRS-PosRRC-Inactive-r17		SpatialRelationsSRS-Pos-r16				OPTIONAL

]],

[[

posSRS-SP-RRC-Inactive-InInitialUL-BWP-r17	PosSRS-SP-RRC-Inactive-InInitialUL-BWP-r17	OPTIONAL

]]

}

OLPC-SRS-Pos-r16 ::= SEQUENCE {

olpc-SRS-PosBasedOnPRS-Serving-r16		ENUMERATED {supported}						OPTIONAL,

olpc-SRS-PosBasedOnSSB-Neigh-r16 		ENUMERATED {supported}						OPTIONAL,

olpc-SRS-PosBasedOnPRS-Neigh-r16		ENUMERATED {supported}						OPTIONAL,

maxNumberPathLossEstimatePerServing-r16	ENUMERATED {n1, n4, n8, n16}				OPTIONAL,

...

}

SpatialRelationsSRS-Pos-r16 ::=	SEQUENCE {

spatialRelation-SRS-PosBasedOnSSB-Serving-r16		ENUMERATED {supported}			OPTIONAL,

spatialRelation-SRS-PosBasedOnCSI-RS-Serving-r16	ENUMERATED {supported}			OPTIONAL,

spatialRelation-SRS-PosBasedOnPRS-Serving-r16		ENUMERATED {supported}			OPTIONAL,

spatialRelation-SRS-PosBasedOnSRS-r16				ENUMERATED {supported}			OPTIONAL,

spatialRelation-SRS-PosBasedOnSSB-Neigh-r16			ENUMERATED {supported}			OPTIONAL,

spatialRelation-SRS-PosBasedOnPRS-Neigh-r16			ENUMERATED {supported}			OPTIONAL,

...

}

SRS-PosResourcesPerBand-r16 ::= SEQUENCE {

freqBandIndicatorNR-r16							FreqBandIndicatorNR-r16,

maxNumberSRS-PosResourceSetsPerBWP-r16			ENUMERATED {n1, n2, n4, n8, n12, n16},

maxNumberSRS-PosResourcesPerBWP-r16				ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumberPeriodicSRS-PosResourcesPerBWP-r16		ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumberAP-SRS-PosResourcesPerBWP-r16			ENUMERATED {n1, n2, n4, n8, n16, n32, n64}

OPTIONAL,

maxNumberSP-SRS-PosResourcesPerBWP-r16			ENUMERATED {n1, n2, n4, n8, n16, n32, n64}

OPTIONAL,

...

}

PosSRS-RRC-Inactive-InInitialUL-BWP-r17 ::= SEQUENCE {

maxNumOfSRSposResourceSets-r17			ENUMERATED {n1, n2, n4, n8, n12, n16 }		OPTIONAL,

maxNumOfPeriodicAndSemiPersistentSRSposResources-r17

ENUMERATED {n1, n2, n4, n8, n16, n32, n64 }

OPTIONAL,

maxNumOfPeriodicAndSemiPersistentSRSposResourcesPerSlot-r17

ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14}

OPTIONAL,

maxNumOfPeriodicSRSposResources-r17

ENUMERATED {n1, n2, n4, n8, n16, n32, n64 }

OPTIONAL,

maxNumOfPeriodicSRSposResourcesPerSlot-r17

ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14}

OPTIONAL,

dummy1

ENUMERATED {n1, n2, n4, n8, n16, n32, n64}	OPTIONAL,

dummy2

ENUMERATED { n1, n2, n3, n4, n5, n6, n8, n10, n12, n14 }

OPTIONAL,

...

}

PosSRS-RRC-Inactive-OutsideInitialUL-BWP-r17 ::= SEQUENCE {

maxSRSposBandwidthForEachSCS-withinCC-FR1-r17

ENUMERATED { bw5, bw10, bw15, bw20, bw25, bw30, bw35,

bw40, bw45, bw50, bw60, bw70, bw80,

bw90, bw100 }					OPTIONAL,

maxSRSposBandwidthForEachSCS-withinCC-FR2-r17

ENUMERATED { bw50, bw100, bw200, bw400 }	OPTIONAL,

maxNumOfSRSposResourceSets-r17			ENUMERATED { n1, n2, n4, n8, n12, n16 }		OPTIONAL,

maxNumOfPeriodicSRSposResources-r17		ENUMERATED { n1, n2, n4, n8, n16, n32, n64 }

OPTIONAL,

maxNumOfPeriodicSRSposResourcesPerSlot-r17

ENUMERATED { n1, n2, n3, n4, n5, n6, n8, n10, n12, n14 }

OPTIONAL,

differentNumerologyBetweenSRSposAndInitialBWP-r17

ENUMERATED { supported }					OPTIONAL,

srsPosWithoutRestrictionOnBWP-r17

ENUMERATED { supported }					OPTIONAL,

maxNumOfPeriodicAndSemiPersistentSRSposResources-r17

ENUMERATED {n1, n2, n4, n8, n16, n32, n64}	OPTIONAL,

maxNumOfPeriodicAndSemiPersistentSRSposResourcesPerSlot-r17

ENUMERATED { n1, n2, n3, n4, n5, n6, n8, n10,

n12, n14 }						OPTIONAL,

differentCenterFreqBetweenSRSposAndInitialBWP-r17

ENUMERATED { supported }					OPTIONAL,

maxNumOfSemiPersistentSRSposResources-r17

ENUMERATED { n1, n2, n4, n8, n16, n32, n64 }

OPTIONAL,

maxNumOfSemiPersistentSRSposResourcesPerSlot-r17

ENUMERATED { n1, n2, n3, n4, n5, n6, n8, n10,

n12, n14 }						OPTIONAL,

switchingTimeSRS-TX-OtherTX-r17			ENUMERATED { us100, us140, us200, us300, us500 }

OPTIONAL,

...

}

PosSRS-SP-RRC-Inactive-InInitialUL-BWP-r17 ::= SEQUENCE {

maxNumOfSemiPersistentSRSposResources-r17

ENUMERATED {n1, n2, n4, n8, n16, n32, n64}	OPTIONAL,

maxNumOfSemiPersistentSRSposResourcesPerSlot-r17

ENUMERATED { n1, n2, n3, n4, n5, n6, n8, n10, n12, n14 }

OPTIONAL,

...

}

-- ASN1STOP

NR-UL-SRS-Capability field descriptions

srs-PosResourceConfigCA-BandList This field indicates the number of SRS for positioning resources supported by the target device. The target device includes this field for each band for the current configured CA band combination. The capability signalling comprises the following parameters: - freqBandIndicatorNR indicates the current configured NR band of the target device. - maxNumberSRS-PosResourceSetsPerBWP indicates the maximum number of SRS Resource Sets for positioning supported by the target device per BWP. Enumerated values n1, n2, n4, n8, n12, n16 correspond to 1, 2, 4, 8, 12, 16 SRS Resource Sets for positioning, respectively. - maxNumberSRS-PosResourcesPerBWP indicates the maximum number of periodic, semi-persistent, and aperiodic SRS Resources for positioning supported by the target device per BWP. Enumerated values n1, n2, n4, n8, n16, n32, n64 correspond to 1, 2, 4, 8, 16, 32, 64 SRS Resources for positioning, respectively. - maxNumberPeriodicSRS-PosResourcesPerBWP indicates the maximum number of periodic SRS Resources for positioning supported by the target device per BWP. Enumerated values n1, n2, n4, n8, n16, n32, n64 correspond to 1, 2, 4, 8, 16, 32, 64 periodic SRS Resources for positioning, respectively. - maxNumberAP-SRS-PosResourcesPerBWP indicates the maximum number of aperiodic SRS Resources for positioning supported by the target device per BWP. Enumerated values n1, n2, n4, n8, n16, n32, n64 correspond to 1, 2, 4, 8, 16, 32, 64 aperiodic SRS Resources for positioning, respectively. - maxNumberSP-SRS-PosResourcesPerBWP indicates the maximum number of semi-persistent SRS Resources for positioning supported by the target device per BWP. Enumerated values n1, n2, n4, n8, n16, n32, n64 correspond to 1, 2, 4, 8, 16, 32, 64 semi-persistent SRS Resources for positioning, respectively.

maxNumberSRS-PosPathLossEstimateAllServingCells Indicates the maximum number of pathloss estimates that the UE can simultaneously maintain for all the SRS resource sets for positioning across all cells in addition to the up to four pathloss estimates that the UE maintains per serving cell for the PUSCH/PUCCH/SRS transmissions. The UE shall include this field if the UE supports any of olpc-SRS-PosBasedOnPRS-Serving, olpc-SRS-PosBasedOnSSB-Neigh and olpc-SRS-PosBasedOnPRS-Neigh. Otherwise, the UE does not include this field.

maxNumberSRS-PosSpatialRelationsAllServingCells indicates the maximum number of maintained spatial relations for all the SRS resource sets for positioning across all serving cells in addition to the spatial relations maintained spatial relations per serving cell for the PUSCH/PUCCH/SRS transmissions. It is only applied for FR2. The UE can include this field only if the UE supports any of spatialRelation-SRS-PosBasedOnSSB-Serving, spatialRelation-SRS-PosBasedOnCSI-RS-Serving, spatialRelation-SRS-PosBasedOnPRS-Serving, spatialRelation-SRS-PosBasedOnSSB-Neigh or spatialRelation-SRS-PosBasedOnPRS-Neigh. Otherwise, the UE does not include this field.

olpc-SRS-Pos Indicates whether the UE supports open-loop power control for SRS for positioning. The capability signalling comprises the following parameters: - olpc-SRS-PosBasedOnPRS-Serving indicates whether the UE supports OLPC for SRS for positioning based on PRS from the serving cell in the same band. The UE can include this field only if the UE supports NR-DL-PRS-ProcessingCapability and srs-PosResources TS38.331 [35] Otherwise, the UE does not include this field. - olpc-SRS-PosBasedOnSSB-Neigh indicates whether the UE supports OLPC for SRS for positioning based on SSB from the neighbouring cell in the same band. The UE can include this field only if the UE supports srs-PosResources TS 38.331 [35]. Otherwise, the UE does not include this field. - olpc-SRS-PosBasedOnPRS-Neigh indicates whether the UE supports OLPC for SRS for positioning based on PRS from the neighbouring cell in the same band. The UE can include this field only if the UE supports olpc-SRS-PosBasedOnPRS-Serving. Otherwise, the UE does not include this field. Note: A PRS from a PRS-only TP is treated as PRS from a non-serving cell. - maxNumberPathLossEstimatePerServing indicates the maximum number of pathloss estimates that the UE can simultaneously maintain for all the SRS resource sets for positioning per serving cell in addition to the up to four pathloss estimates that the UE maintains per serving cell for the PUSCH/PUCCH/SRS transmissions. The UE shall include this field if the UE supports any of olpc-SRS-PosBasedOnPRS-Serving, olpc-SRS-PosBasedOnSSB-Neigh and olpc-SRS-PosBasedOnPRS-Neigh. Otherwise, the UE does not include this field.

spatialRelationsSRS-Pos Indicates whether the UE supports spatial relations for SRS for positioning. It is only applicable for FR2. The capability signalling comprises the following parameters: - spatialRelation-SRS-PosBasedOnSSB-Serving indicates whether the UE supports spatial relation for SRS for positioning based on SSB from the serving cell in the same band. The UE can include this field only if the UE supports srs-PosResources TS 38.331 [35]. Otherwise, the UE does not include this field. - spatialRelation-SRS-PosBasedOnCSI-RS-Serving indicates whether the UE supports spatial relation for SRS for positioning based on CSI-RS from the serving cell in the same band. The UE can include this field only if the UE supports spatialRelation-SRS-PosBasedOnSSB-Serving. Otherwise, the UE does not include this field. - spatialRelation-SRS-PosBasedOnPRS-Serving indicates whether the UE supports spatial relation for SRS for positioning based on PRS from the serving cell in the same band. The UE can include this field only if the UE supports any of DL-PRS Resources for DL-AoD, DL-PRS Resources for DL-TDOA or DL-PRS Resources for Multi-RTT, or srs-PosResources TS 38.331 [35]. Otherwise, the UE does not include this field. - spatialRelation-SRS-PosBasedOnSRS indicates whether the UE supports spatial relation for SRS for positioning based on SRS in the same band. The UE can include this field only if the UE supports srs-PosResources TS 38.331 [35]. Otherwise, the UE does not include this field. - spatialRelation-SRS-PosBasedOnSSB-Neigh indicates whether the UE supports spatial relation for SRS for positioning based on SSB from the neighbouring cell in the same band. The UE can include this field only if the UE supports spatialRelation-SRS-PosBasedOnSSB-Serving. Otherwise, the UE does not include this field. - spatialRelation-SRS-PosBasedOnPRS-Neigh indicates whether the UE supports spatial relation for SRS for positioning based on PRS from the neighbouring cell in the same band. The UE can include this field only if the UE supports spatialRelation-SRS-PosBasedOnPRS-Serving. Otherwise, the UE does not include this field. Note: A PRS from a PRS-only TP is treated as PRS from a non-serving cell.

posSRS-RRC-Inactive-InInitialUL-BWP Indicates whether the UE supports positioning SRS transmission in RRC_INACTIVE state for initial UL BWP. The capability signalling comprises the following parameters: - maxNumOfSRSposResourceSets indicates the maximum number of SRS Resource Sets for positioning supported by the UE. - maxNumOfPeriodicAndSemiPersistentSRSposResources indicates the maximum number of periodic and semi-persistent SRS Resources for positioning supported by the UE. - maxNumOfPeriodicAndSemiPersistentSRSposResourcesPerSlot indicates the maximum number of periodic and semi-persistent SRS Resources for positioning per slot supported by the UE. - maxNumOfPeriodicSRSposResources indicates the maximum number of periodic SRS Resources for positioning supported by the UE. - maxNumOfPeriodicSRSposResourcesPerSlot indicates the maximum number of periodic SRS Resources for positioning per slot supported by the UE. - dummy1, dummy2 are not used in the specification. If received they shall be ignored by the receiver.

posSRS-RRC-Inactive-OutsideInitialUL-BWP Indicates whether the UE supports positioning SRS transmission in RRC_INACTIVE state outside initial UL BWP. The UE can include this field only if the UE supports posSRS-RRC-Inactive-InInitialUL-BWP. Otherwise, the UE does not include this field. The capability signalling comprises the following parameters: - maxSRSposBandwidthForEachSCS-withinCC-FR1 indicates the maximum SRS bandwidth supported for each SCS that UE supports within a single CC for FR1. - maxSRSposBandwidthForEachSCS-withinCC-FR2 indicates the maximum SRS bandwidth supported for each SCS that UE supports within a single CC for FR2. - maxNumOfSRSposResourceSets indicates the maximum number of SRS Resource Sets for positioning supported by the UE. - maxNumOfPeriodicSRSposResources indicates the maximum number of periodic SRS Resources for positioning supported by the UE. - maxNumOfPeriodicSRSposResourcesPerSlot indicates the maximum number of periodic SRS Resources for positioning per slot supported by the UE. - differentNumerologyBetweenSRSposAndInitialBWP indicates whether different numerology between the SRS and the initial UL BWP is supported by the UE. If the field is absent, the UE only supports same numerology between the SRS and the initial UL BWP. - srsPosWithoutRestrictionOnBWP indicates whether SRS operation without restriction on the BW is supported by the UE; BW of the SRS may not include BW of the CORESET#0 and SSB. If the field is absent, the UE supports only SRS BW that includes the BW of the CORESET #0 and SSB. - maxNumOfPeriodicAndSemiPersistentSRSposResources indicates the maximum number of periodic and semi-persistent SRS Resources for positioning supported by the UE. - maxNumOfPeriodicAndSemiPersistentSRSposResourcesPerSlot indicates the maximum number of periodic and semi-persistent SRS Resources for positioning per slot supported by the UE. - differentCenterFreqBetweenSRSposAndInitialBWP indicates whether different center frequency between the SRS for positioning and the initial UL BWP is supported by the UE. If the field is absent, the UE only supports same center frequency between the SRS for positioning and initial UL BWP. - maxNumOfSemiPersistentSRSposResources indicates the maximum number of semi-persistent SRS Resources for positioning supported by the UE. The UE can include this field only if the UE supports posSRS-RRC-Inactive-InInitialUL-BWP. Otherwise, the UE does not include this field. - maxNumOfSemiPersistentSRSposResourcesPerSlot indicates the maximum number of semi-persistent SRS Resources for positioning per slot supported by the UE. The UE can include this field only if the UE supports posSRS-RRC-Inactive-InInitialUL-BWP. Otherwise, the UE does not include this field. - switchingTimeSRS-TX-OtherTX indicates the switching time between SRS Tx and other Tx in initial UL BWP or Rx in initial DL BWP.

olpc-SRS-PosRRC-Inactive Indicates whether the UE supports open-loop power control for SRS for positioning in RRC_INACTIVE state. The UE can include this field only if the UE supports posSRS-RRC-Inactive-InInitialUL-BWP. Otherwise, the UE does not include this field.

spatialRelationsSRS-PosRRC-Inactive Indicates whether the UE supports spatial relations for SRS for positioning in RRC_INACTIVE state on FR2. The UE can include this field only if the UE supports posSRS-RRC-Inactive-InInitialUL-BWP. Otherwise, the UE does not include this field.

posSRS-SP-RRC-Inactive-InInitialUL-BWP Indicates whether the UE supports positioning SRS transmission in RRC_INACTIVE state for initial UL BWP with semi-persistent SRS. The UE can include this field only if the UE supports posSRS-RRC-Inactive-InInitialUL-BWP. Otherwise, the UE does not include this field. The capability signalling comprises the following parameters: - maxNumOfSemiPersistentSRSposResources indicates the maximum number of semi-persistent SRS Resources for positioning supported by the UE. - maxNumOfSemiPersistentSRSposResourcesPerSlot indicates the maximum number of semi-persistent SRS Resources for positioning per slot supported by the UE.

Next change

#### 6.5.11.6a	NR DL-AoD Capability Information Elements

#### –	NR-DL-AoD-MeasurementCapability

The IE NR-DL-AoD-MeasurementCapability defines the DL-AoD measurement capability. The UE can include this IE only if the UE supports NR-DL-PRS-ResourcesCapability for DL-AoD. Otherwise, the UE does not include this IE;

-- ASN1START

NR-DL-AoD-MeasurementCapability-r16 ::= SEQUENCE {

maxDL-PRS-RSRP-MeasurementFR1-r16		INTEGER (1..8),

maxDL-PRS-RSRP-MeasurementFR2-r16		INTEGER (1..8),

dl-AoD-MeasCapabilityBandList-r16		SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

DL-AoD-MeasCapabilityPerBand-r16,

...,

[[

maxDL-PRS-RSRP-MeasurementFR1-v1730		ENUMERATED { n16, n24 }					OPTIONAL,

maxDL-PRS-RSRP-MeasurementFR2-v1730		ENUMERATED { n16, n24 }					OPTIONAL

]]

}

DL-AoD-MeasCapabilityPerBand-r16 ::= SEQUENCE {

freqBandIndicatorNR-r16					FreqBandIndicatorNR-r16,

simul-NR-DL-AoD-DL-TDOA-r16				ENUMERATED { supported}					OPTIONAL,

simul-NR-DL-AoD-Multi-RTT-r16			ENUMERATED { supported}					OPTIONAL,

...,

[[

maxDL-PRS-FirstPathRSRP-MeasPerTRP-r17	ENUMERATED { n1, n2, n4, n8, n16, n24 }	OPTIONAL,

dl-PRS-MeasRRC-Inactive-r17				ENUMERATED { supported }				OPTIONAL

]]

}

-- ASN1STOP

NR-DL-AoD-MeasurementCapability field descriptions

maxDL-PRS-RSRP-MeasurementFR1 Indicates the maximum number of DL-PRS RSRP measurements on different PRS resources from the same TRP supported by the UE on FR1. If this field with suffix -v1730 is present, the target device should set the field with suffix -r16 to value '8'.

maxDL-PRS-RSRP-MeasurementFR2 Indicates the maximum number of DL-PRS RSRP measurements on different PRS resources from the same TRP supported by the UE on FR2. If this field with suffix -v1730 is present, the target device should set the field with suffix -r16 to value '8'.

simul-NR-DL-AoD-DL-TDOA Indicates whether the UE supports simultaneous processing for DL-AoD and DL-TDOA measurements. The UE can include this field only if the UE supports DL-TDOA and DL-AoD. Otherwise, the UE does not include this field.

simul-NR-DL-AoD-Multi-RTT Indicates whether the UE supports simultaneous processing for DL-AoD and UE Multi-RTT measurements. The UE can include this field only if the UE supports Multi-RTT and DL-AoD. Otherwise, the UE does not include this field.

maxDL-PRS-FirstPathRSRP-MeasPerTRP This field, if present, indicates that the target device supports measuring and reporting the PRS RSRPP of the first path. The enumerated value indicates the maximum number of RSRPP of first path per TRP supported. The UE can include this field only if the UE supports one of maxDL-PRS-RSRP-MeasurementFR1 and maxDL-PRS-RSRP-MeasurementFR2. Otherwise, the UE does not include this field. NOTE 1: The maximum number of first path PRS RSRP per TRP should be less than or equal to the maximum number of PRS RSRP defined in maxDL-PRS-RSRP-MeasurementFR1 and maxDL-PRS-RSRP-MeasurementFR2.

dl-PRS-MeasRRC-Inactive This field, if present, indicates that the target device supports DL-PRS measurement in RRC_INACTIVE state. The UE can include this field only if the UE supports maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer, maxNrOfTRP-AcrossFreqs, maxNrOfPosLayer and dl-PRS-BufferType-RRC-Inactive. Otherwise, the UE does not include this field. NOTE 1: This capability is applicable to both, UE-assisted and UE-based DL-TDOA. NOTE 2: The capabilities NR-DL-PRS-ResourcesCapability, simul-NR-DL-AoD-DL-TDOA are the same in RRC_INACTIVE state.

End of changes