3GPP TSG-RAN WG2 Meeting #120	R2-2211593

Toulouse, France, 14 – 18 November 2022

Agenda item:	6.1.4

Source:	Nokia, Ericsson, Nokia Shanghai Bell, Qualcomm Incorporated

Title:	MBS DRX Corrections

WID/SID:	NR_MBS-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

This contribution details an alternative to the DRX corrections discussed for MBS at the last meeting.

# 2	DRX Corrections

User plane corrections for MBS were discussed at the last meeting in AT email discussion #603 and POST email discussion #606. Amongst the MAC corrections that were debated, one was related to DRX: how to stop drx-RetransmissionTimerDL-PTM when either multicast DRX, or unicast DRX is not configured.

In our opinion, the earlier agreement to split unicast and broadcast/multicast DRX was made in order to keep things modular and as isolated as possible. Unfortunately, the changes suggested in R2-2211060 as an outcome to POST email discussion #606 breaks that principle: a UE has to execute most of 5.7 even when unicast DRX is not configured, just to stop a retransmission timer on the multicast side.

In order to avoid that, we believe it is beneficial to extract the corresponding behaviour from the main loop, even if that means having more text since what should matter the most when writing specification is clarity, not pseudo-code optimisation.

An example of such an approach is given below.

Text Proposal

## 5.7	Discontinuous Reception (DRX)

[…]

The following MAC timers are used for DRX operation in a non-terrestrial network:

-	HARQ-RTT-TimerDL-NTN (per DL HARQ process configured with HARQ feedback enabled): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity;

-	HARQ-RTT-TimerUL-NTN (per UL HARQ process configured with HARQModeA): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity.

When multicast DRX is configured and DRX is not configured, the MAC entity shall:

1>	if a MAC PDU is received in a configured downlink assignment for unicast; or

1>	if the PDCCH indicates a DL unicast transmission:

2>	stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process.

When DRX is configured, the MAC entity shall:

1>	if a MAC PDU is received in a configured downlink assignment for unicast:

2>	if this Serving Cell is configured with downlinkHARQ-FeedbackDisabled:

3>	if the corresponding HARQ process is configured with HARQ feedback enabled:

4>	set HARQ-RTT-TimerDL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerDL plus the latest available UE-gNB RTT value;

4>	start the HARQ-RTT-TimerDL-NTN for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

2>	else:

3>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

[…]

## 5.7b	Discontinuous Reception (DRX) for MBS Multicast

[…]

When multicast DRX is configured for a G-RNTI or G-CS-RNTI, the Active Time includes the time while:

-	drx-onDurationTimerPTM or drx-InactivityTimerPTM or drx-RetransmissionTimerDL-PTM for this G-RNTI or G-CS-RNTI is running.

When unicast DRX is configured and multicast DRX is not configured for a G-RNTI or G-CS-RNTI, the MAC entity shall for this G-RNTI or G-CS-RNTI:

1>	monitor the PDCCH as specified in TS 38.213 [6];

1>	if the PDCCH indicates a DL multicast transmission; or

1>	if a MAC PDU is received in a configured downlink multicast assignment:

2>	if HARQ feedback is enabled:

3>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

2>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

When multicast DRX is configured for a G-RNTI or G-CS-RNTI, the MAC entity shall for this G-RNTI or G-CS-RNTI:

1>	if a MAC PDU is received in a configured downlink multicast assignment:

2>	if HARQ feedback is enabled:

3>	start the drx-HARQ-RTT-TimerDL-PTM for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback;

[…]