3GPP TSG-RAN WG2 Meeting #120	 R2-2211611

Toulouse France, 14 Nov – 18 Nov, 2022

Agenda Item:	8.11.2

Source: 	NEC

Title:  	Discussion on multicast reception in RRC_INACTIVE

Document for:	Discussion and Decision

- 1 Introduction

In RAN2#119b meeting, lots of agreements are made [1] and to avoid redundancy, we list each related agreement in the corresponding discussion part.

In this paper, we continue to discuss how to enhance multicast reception in RRC_INACTIVE based on the available agreements.

- 2 Discussion

- 2.1 State transition for multicast reception in RRC_INACTIVE

2.1.1 From RRC_CONNECTED to RRC_INACTIVE

Dedicated RRC signalling (i.e. RRC release message with suspendConfig) is used for switching a multicast receiving UE from RRC_CONNECTED to RRC_INACTIVE and continue multicast reception (details FFS).

RAN2 agreed that using RRCRelease with suspendConfig to switch UE from RRC_CONNECTED to RRC_INACTIVE no, no matter which option (opt.1: dedicated RRC / opt.2: SIB/MCCH-like) will be selected for PTM configuration delivery. Furthermore, when UE is transferred to RRC_INACTIVE, how UE gets PTM configuration still need to be discussed.

In Rel-17 Mulicast, the mulicast configuration is configured via RRCReconfiguration for a dedicated set of UEs (i.e., Not all UEs in the multicast service area are authorized to receive the data). If we go for option 2, i.e., gNB provides the configuration by a kind of broadcast-like way (SIB/MCCH), it means gNB is not able to be aware of which UE is receiving the multicast session in RRC_INACTIVE (i.e., lacking of control on the (un)authorized UE). It seems like all UEs can receive multicast session, then what is the defference between multicast and broadcast? It turns multicast into broadcast to some extent. By the way, continuously monitoring MCCH (e.g., MCCH information change notification) during RRC_INACTIVE is also a drawback for broadcast-like way.

On the other hand, we had agreement saying the same one multicast session is assumed between RRC_CONNECTED and RRC_INACTIVE [2], it basically means the configuration from RRCReconfiguration can be reused for multicast reception during RRC_INACTIVE. Thus usging dedicated RRC signalging (e.g., RRCRelease, pre-RRCReconfiguration) to provide PTM configtuation is more propriate. For instance, since UE anyway will receive RRCRelease as a transfer notification, RRCRelease can also be used to provice the PTM configuration or just indicate UE that the configuration from RRCReconfiguration for multicast is still valid during RRC_INACTIVE. And more details can be further discussed (e.g., the coexist between RRCRelease and RRCReconfiguration).

Observation 1 : By means of broadcast-like method (SIB/MCCH), gNB is not able to be aware of which UE is receving the multicast session in RRC_INACTIVE, i.e., lacking of control on the (un)authorized UE. Furthermore continuously monitoring MCCH during RRC_INACTIVE is also a drawback for broadcast-like way.

Proposal 1 : Using dedicated RRC signaling (e.g., RRCRelease, RRCReconfiguration) to provide multicast configuration to UE when its RRC state is switched from RRC_CONNECTED to RRC_INACTIVE and details FFS.

2.1.2 From RRC_INACTIVE to RRC_CONNECTED

For both option 1 and option 2, as a baseline, group paging can be used to switch UEs receiving multicast from RRC_INACTIVE to RRC_CONNECTED, and UEs continue the multicast reception in CONNECTED. FFS if there is any potential issue if Rel-17 group paging is reused. FFS if there are other cases when UE triggers resume. FFS if MCCH can also be used in case of option 2.

Firstly we need to figure out why network switch UE from RRC_INACTIVE to RRC_CONNECTED for multicast reception. When look at the objective of this WID, it has two reasons : 1. Power saving; 2. Network congestion.

For power saving, if there is temporarily no unicast data for a RRC_CONNECTED UE that is currently monitoring/receiving mulicast session, network may let UE enter RRC_INACTIVE and continue monitoring/receiving multicast session. But if unicast data is coming, UE will be paged to re-connect with network. In this situation, the current RAN paging mechanism is sufficient. No further enhancement is required.

For network congestion, if network is experiencing congestion, UEs may be released to RRC_INACTIVE but can continue monitoring/receiving multicast session such as Mission Critical Service, Public Safty. Once the congestion is alleviated, it is up to network that whether let all UEs or a set of UEs re-connect with network. For a set of UEs, the current RAN paging(i.e., individual notification) is proper to be used. For all UEs within one session (TMGI), group paging which indicates a specific TMGI is more proper to be used. However it is noted that in Rel-17, group paging only means session is about to start, how does UE differetiate what group paging means (i.e., whether directly start session in RRC_INACTIVE or fallback), thus group paging needs to be enhanced in this case and we analyze this issue in next section.

Observation 2: When transfer UE from RRC_INACTIVE to RRC_CONNECTED, legacy RAN paging is used for a set of UEs for either power saving or network congestion, and group paging is used for all UEs within one multicast session (TMGI), which is correspond to network congestion case.

Proposal 2: RAN2 is suggested to consider the following method when switching UE from RRC_INACTIVE to RRC_CONNECTED when receiving multicast session:

- Option 1: Using legacy RAN paging to individually page UE (i.e., no enhancement)

- Option 2 : Enhance group paging to page all UEs within one session (i.e., TMGI)

- 2.2 Multicast session state notification

2.2.1 Paging mechanism

As a baseline, group paging can be used to inform Rel-18 UE(s) about the session activation (Details FFS, e.g., UE behavior when receiving such group notification).

FFS how UE determines whether it can receive the multicast session in RRC_INACTIVE or not when the session is activated.

If a UE is in RRC_INACTIVE and is configured to receive a multicast session in RRC_INACTIVE, the UE may be notified when the multicast session is deactivated. FFS how (e.g., informed via group paging, MCCH, or other ways).

If option 1 is supported for PTM configuration

- As a baseline, group paging may be used to inform the UE when network changes the PTM configurations, and UE upon reception triggers RRC connection resume procedure to obtain the updated configurations (details of group paging can be FFS).

- FFS whether and how to solve the issue in signalling/system load when a large number of UEs in the cell need PTM configuration update.

Rel-17 mechanism (NAS-based indication) is applicable for multicast session release. FFS if any enhancement is needed.

In RAN2#119b meeting, we have agreements that group paging basically can be reused for multicast session activation, deactivation, and modification. In Rel-17 multicast, the session configuration is controlled by RRCReconfiguration from RAN perspective, if the session is activated, network will configure the related multicast configuration including MRB, PDCP/RLC/MAC/PHY/ServingCellConfig for UEs. If the session is modified or deactivated, network still use RRCReconfiguration to modify/release the related configuration. However in Rel-18, UE can even receive multicast session in RRC_INACTIVE, but there is no dedicated signalling reaching to a RRC_INACTIVE UE.

In our understanding, paging is a straightforward way to provide such indication about multicast session state (i.e., session activation/deactivation/modification/release). But comparing with Rel-17 group paging, there is only one TMGI indication within pagingGroupList indicating whether the multicast session (i.e., TMGI) is started or not, it is not sufficient for Rel-18 session state notification. Therefore, a paging cause list for pagingGroupList can be introduced to provide session state which is correspond to each entry (i.e., TMGI) within pagingGroupList.

Given an example shown in fig.1, apart from TMGI indication, network can also send paging cause – session activation with connected, session activation with inactive, session deactivation, session modification, corresponding to each entry in group paging, so that UE in RRC_INACTIVE can differentiate each session state. For instance, UE 1 has joined the multicast session 1 (correspond to TMGI-1), once the TMGI indication within pagingGroupList is indicated by paging cause that the session is deactivated, UE 1 may stop monitoring multicast session in RRC_INACTIVE, furthermore UE 1 may store the related configuration in UE Inactive AS Context for future use. For UE 2 and UE 3, by means of this way, not only the session activation is informed but also indicate which RRC state UE should stay for session reception, which solves the issue mentioned in above section. For UE 3 is currently receiving multicast session during RRC_INACTIVE, such a UE can use Random Access to update its configuration, and more details will be analysed in next section.

Fig.1 Introduce paging cause for session state notification in paging message

Observation 3: Different from Rel-17 group paging (i.e., only TMGI indicating session is activated), more multicast session state need to be introduced in Rel-18 group paging, e.g., session activation with connected, session activation with inactive, session deactivation, session modification.

Proposal 3: Introduce paging cause – session activation with connected, session activation with inactive, session deactivation, session modification, which is correspond to each entry (i.e., TMGI) within pagingGroupList for multicast session state notification.

2.2.2 Random Access

If option 1 is supported for PTM configuration

- As a baseline, group paging may be used to inform the UE when network changes the PTM configurations, and UE upon reception triggers RRC connection resume procedure to obtain the updated configurations (details of group paging can be FFS).

- FFS whether and how to solve the issue in signalling/system load when a large number of UEs in the cell need PTM configuration update.

First, we analyze whether signaling/system load should be solved when a large number of UEs need Random Access. Indeed, when there is one TMGI indicating UEs that the configuration need to be updated, all UEs interesting in this TMGI will initiate RA for configuration update. However, in our understanding, there is no need to solve this problem, maybe it exists sometime, but based on paging determination formula, we can see that UEs are already divided into different PO which means different time zone, so UEs can receive paging message including session modification at different time. Note that in one paing DRX cycle (e.g., 320ms, 640ms, …), there could be multiple SSB periods (e.g., 20ms) to be selected for random access. Therefore, the problem can be alleviated.

Observation 4: The issue of large number of UEs initiating Random Access can be alleviated by paging determination formula, i.e., all UEs interesting one TMGI can initiate Random Access at different time zone.

Proposal 4: No need to solve the issue on large number of UEs initiate Random Access for configuration update.

Furthermore, for configuration update by Random Access, a RNA-like method can be considered for requesting new multicast configuration without entering RRC_CONNECTED. If we agree that multicast configuration can be carried in RRCRelease message, there is a chance that network may send RRCRelease to UE as a response to such a Random Access. By this way, UE can update/acquire the multicast configuration without entering RRC_CONNECTED.

More details, we can reuse RRCResumeRequest by adding a new resume cause for multicast request to achieve this function, or introduce a new UL-CCCH(1) message like RRCMulticstRequest to request the multicast configuration, shown in Fig.2.

Fig.2 Enhanced Random Access for configuration request without entering RRC_CONNECTED

Proposal 5: RAN2 is suggested to consider following methods to enhance Random Access Msg.3 for multicast configuration request:

- Option 1: Introduce a new resumeCause in RRCResumeRequest for multicast request

- Option 2: Introduce a new UL-CCCH message for multicast request

Proposal 6: Using RRCRelease carrying multicast configuration as a response to multicast request during random access procedure.

- 2.3 Mobility for multicast reception in RRC_INACTIVE

FFS whether to introduce PTM configuration applicable area, i.e., the mechanism that the PTM configurations, once acquired by a UE, may apply to a certain area (i.e., a set of cells instead of a single cell).

Whether the configuration can be applicable for an area (e.g., RNA/a set of cells) need RAN3 discussion, final decision can be made after RAN3 make some agreements on this part.

If the multicast configuration of the new cell is available for the UE, then UE still can continue receiving multicast session after cell reselection. The key point is that how UE can acquire the multicast configuration of the new selected cell. There are several ways to achieve this.

One way is that the dedicated RRC signaling (e.g., pre-RRCReconfiguration, RRCRelease) may provide a multicast configuration list for multiple cells when UE is receiving multicast session in RRC_INACTIVE, When cell reselection happens, UE will apply the multicast configuration which is correspond to the new cell. Furthermore, considering there could be some neighbor cells which does not provide multicast session, a multicast specific frequency priority can be introduced to let UE optimally reselect a mulicast cell.

Another way is that an area-specific multicast configuration can be provided to UE, e.g., part or the whole of RNA. In this area, UE can still use the same configuration to receive multicast session. Furthermore, an indication is required to notify UE of area information, for example, dedicated RRC signaling need to carry a cell list indicating which cell the UE can use the same configuration, or each cell may provide such an area information via system information.

Proposal 7: RAN2 is suggested to consider the following methods for the mobility of multicast reception in RRC_INACTIVE:

- Option 1: Use dedicated RRC signaling to provide multicast configuration list for multiple cells

- Option 2: Introduce area-specific multicast configuration

Proposal 8: Multicast-specific frequency priority can be further considered.

If the multicast configuration of the new cell is not available for the UE, we agreed that UE is required to resume RRC connection to get the multicast configuration. In our understanding, if proposal 5 and 6 in this paper can be agreed. Then UE who supports multicast reception in RRC_INACTIVE can use RNA updated-like method to get the multicast configuration without entering RRC_CONNECTED, and it is more efficient.

Proposal 9: Proposal 5 and 6 are applicable for the mobility case where UE is required to get the multicast configuration in a new cell.

- 2.4 Other issues

For multicast, the same service and the same specific content data are provided simultaneously to a dedicated set of UEs. It can be seen that the multicast scheduling is not for an individual UE, so the scheduling scheme is uniform. In legacy mechanism, gNB can determine the scheduling based on CSI feedback from large number of RRC_CONNECTED UEs. However, RRC_INACTIVE UE can not report its channel information to network, once such UEs experiencing poor channel environment, the multicast reception performance will be degraded but network is still not aware of the situation.

In our understanding, if for power saving reason, UE is in RRC_INACTIVE but can not receive multicast very well (e.g., channel quality is not good enough), it should fall back RRC_CONNECTED to let network know this channel quality and may make some modifications such as modifying scheduling, using PTP mode or unicast bearer, and so on.

Since UE still performs measurements in RRC_INACTIVE, some measurement criteria can be introduced to determine whether UE should fall back RRC_CONNECTED for multicast reception or not. In a word, multicast scheduling problem due to channel quality for RRC_INACTIVE UE should be considered.

Proposal 10: RAN2 is suggested to consider the multicast scheduling problem due to channel quality for RRC_INACTIVE UE.

- 3 Conclusions

Observation 1 : By means of broadcast-like method (SIB/MCCH), gNB is not able to be aware of which UE is receving the multicast session in RRC_INACTIVE, i.e., lacking of control on the (un)authorized UE. Furthermore continuously monitoring MCCH during RRC_INACTIVE is also a drawback for broadcast-like way.

Observation 2: When transfer UE from RRC_INACTIVE to RRC_CONNECTED, legacy RAN paging is used for a set of UEs for either power saving or network congestion, and group paging is used for all UEs within one multicast session (TMGI), which is correspond to network congestion case.

Observation 3: Different from Rel-17 group paging (i.e., only TMGI indicating session is activated), more multicast session state need to be introduced in Rel-18 group paging, e.g., session activation with connected, session activation with inactive, session deactivation, session modification.

Observation 4: The issue of large number of UEs initiating Random Access can be alleviated by paging determination formula, i.e., all UEs interesting one TMGI can initiate Random Access at different time zone.

Proposal 1 : Using dedicated RRC signaling (e.g., RRCRelease, RRCReconfiguration) to provide multicast configuration to UE when its RRC state is switched from RRC_CONNECTED to RRC_INACTIVE and details FFS.

Proposal 2: RAN2 is suggested to consider the following method when switching UE from RRC_INACTIVE to RRC_CONNECTED when receiving multicast session:

- Option 1: Using legacy RAN paging to individually page UE (i.e., no enhancement)

- Option 2 : Enhance group paging to page all UEs within one session (i.e., TMGI)

Proposal 3: Introduce paging cause – session activation with connected, session activation with inactive, session deactivation, session modification, which is correspond to each entry (i.e., TMGI) within pagingGroupList for multicast session state notification.

Proposal 4: No need to solve the issue on large number of UEs initiate Random Access for configuration update.

Proposal 5: RAN2 is suggested to consider following methods to enhance Random Access Msg.3 for multicast configuration request:

- Option 1: Introduce a new resumeCause in RRCResumeRequest for multicast request

- Option 2: Introduce a new UL-CCCH message for multicast request

Proposal 6: Using RRCRelease carrying multicast configuration as a response to multicast request during random access procedure.

Proposal 7: RAN2 is suggested to consider the following methods for the mobility of multicast reception in RRC_INACTIVE:

- Option 1: Use dedicated RRC signaling to provide multicast configuration list for multiple cells

- Option 2: Introduce area-specific multicast configuration

Proposal 8: Multicast-specific frequency priority can be further considered.

Proposal 9: Proposal 5 and 6 are applicable for the mobility case where UE is required to get the multicast configuration in a new cell.

Proposal 10: RAN2 is suggested to consider the multicast scheduling problem due to channel quality for RRC_INACTIVE UE.

- 4 References

[1] RAN2#119b MBS session notes (Dawid)

[2] RAN2#119 MBS session notes (Dawid)