3GPP TSG-RAN WG2 Meeting #120		R2-2211664

Toulouse, France, Nov. 14th- Nov. 18th, 2022

Agenda Item:	8.3.2

Source:	vivo

Title:	Discussion on cell DTX/DRX

Document for:	Discussion and Decision

Introduction

According to RAN2#119bis discussion, cell DTX/DRX mechanism for network energy saving was further discussed, with the below agreements [1]:

=>	Let’s start with understanding solution in the context of connected

•	Example 1: gNB is expected to turn off all transmission and reception for data traffic and reference signal during Cell DTX / DRX OFF duration.

•	Example 2: gNB is expected to turn off its transmission / reception only for data traffic during Cell DTX / DRX OFF duration (i.e. gNB will still transmit / receive reference signals).

•	Example 3: gNB is expected to turn off its dynamic transmission / reception during Cell DTX / DRX OFF duration (i.e. gNB is expected to still perform periodic transmission / reception, including SPS, CG-PUSCH, SR, RACH, and SRS).

•	Example 4: gNB is expected to only transmit reference signals (e.g. CSI-RS for measurement).

=>	RAN2 assumes that the options above are possible for gNB DTX/DRX behavior and discuss UE RAN2 behavior/impact during the DTX/DRX.

=>	For the purpose of our discussion we will focus on a single UE behavior at any point in time.  FFS if we allow multiple configuration of NW DRX/DTX behaviors.

=>	Periodic DTX is assumed as a baseline.  The gNB provides indication to UE about NW DTX mode/configuration via dedicated dynamic L1/L2 signaling.

=>	Dynamic L1/L2 group signalling from NW to provide NW DTX mode/configuration is also considered in RAN2

=>	It is beneficial to align UE DRX with network DTX and DRX alignment among multiple UEs.  Details are FFS, including UE transmission/reception behavior during DTX.  RAN2 to study the alignment.

In this contribution, we further provide our consideration on the technique of cell DTX/DRX for network energy saving.

Discussion

According to the TP on cell DTX/DRX provided in post-RAN2#119bis email discussion [2], some possible examples on cell DTX/DRX behavior during network non-active periods are provided. It can be briefly summarized as whether gNB is expected to turn off transmission/reception for dynamic data traffic (i.e., PDCCH/PUCCH scheduling, PUSCH/PDSCH data), for periodic resources (i.e., SPS, CG-PUSCH, SR, RACH) or for reference signal (i.e., CSI-RS, SRS).

- Reference signal and periodic resources

The following four cases need to be discussed if gNB turns off transmission/reception for reference signal during cell DTX/DRX non-active periods:

- Case 1: periodic reference signal, such as CSI-RS, SRS, SSB;

- Case 2: semi-persistent reference signal, such as CSI-RS, SRS;

- Case 3: aperiodic reference signal, such as CSI-RS, SRS;

- Case 4: other reference signal, such as PDCCH/PUCCH/PDSCH/PUSCH DMRS, PDSCH/PUSCH PTRS;

For case 1 and 2, periodic and semi-persistent reference signals are both periodic and configured by RRC. The difference is that periodic reference signals are activated upon RRC configuration, while the configured semi-persistent reference signal needs to be activated by MAC CE. The resources of periodic and semi-persistent reference signals can be configured within cell DTX/DRX active periods by current NW implementation.

For the aperiodic reference signal, the network triggers it through DCI, and the resource depends on the offset between DCI and reference signals which are also configurable by the network. The network itself can avoid sending DCI to trigger aperiodic reference signal.

For case 4, the specification defines the time/frequency resources of these reference signals, which are fixed. For example, for PDSCH DMRS, the first DMRS symbol is fixedly mapped to the first OFDM symbol of PDSCH in time domain of mapping type B. Two consecutive DMRS REs are in one group, and each group is separated by four REs in frequency domain of DMRS type 2. In this case, only when UE receives the PDSCH, the UEs measure the reference signal.

- The resource of reference signal can be configured or went into effect during cell DTX/DRX active periods by current NW implementation without spec impact.

Similarly, concerning the periodic resources, such as SPS, CG-PUSCH, SR, RACH, when network configures the corresponding resources and enters the cell DTX/DRX non-active periods, the network does not transmit/receive corresponding data, but the UEs continue to monitor/transmit data. To reduce UEs power consumption, the above solutions for case 2 can be reused.

- The periodic transmission/reception, including SPS, USCH, SR, RACH etc., can be configured or went into effect during cell DTX/DRX active periods by current NW implementation without spec impact.

- PDCCH scheduling

For RRC connected mode UEs, network can configure the same C-DRX parameters for all UEs, aiming at aligning with UE C-DRX. The network could achieve energy saving by not transmitting or scheduling during the overlapped UE inactive periods, which is transparent for UEs.

- The C-DRX configuration of all UE can be aligned by current NW implementation without spec impact. The cell DTX/DRX operation of network in the existing mechanism can be transparent to UEs.

- Leave to current NW implementation to achieve cell DTX/DRX without spec impact.

The TP on cell DTX/DRX provided in post-RAN2#119bis mentioned that “It is beneficial to align UE DRX with Cell DTX and DRX alignment among multiple UEs”. This description is based on the assumption that there is a cell DTX/DRX pattern. According to our above analysis, the network can align C-DRX configuration of all UEs. The operation of network in the existing mechanism is transparent to the UE and cell DTX has been implemented without spec impact. Based on this and we propose the following:

- Change ‘It is beneficial to align UE DRX with Cell DTX and DRX alignment among multiple UEs’ to ‘It is beneficial to align UE DRX among multiple UEs’.

Conclusion

Based on the discussion, we have the following observation and proposals:

- The resource of reference signal can be configured or went into effect during cell DTX/DRX active periods by current NW implementation without spec impact.

- The periodic transmission/reception, including SPS, USCH, SR, RACH etc., can be configured or went into effect during cell DTX/DRX active periods by current NW implementation without spec impact.

- The C-DRX configuration of all UE can be aligned by current NW implementation without spec impact. The cell DTX/DRX operation of network in the existing mechanism can be transparent to UEs.

- Leave to current NW implementation to achieve cell DTX/DRX without spec impact.

- Change ‘It is beneficial to align UE DRX with Cell DTX and DRX alignment among multiple UEs’ to ‘It is beneficial to align UE DRX among multiple UEs’.

References

[1] R2-2210803 Report of Session on NES and UAV.

[2] R2-2211067, TP on Cell DTX DRX to TR 38.864