3GPP TSG RAN WG2#120	R2-2211709

Toulouse, France, November 14-18, 2022

Agenda item:		8.4.2.3

Source:		Apple Inc

Title:		DU aspects for LTM and MAC CE contents

Document for:		Discussion and Decision

Work Item Code: 		NR_Mob_enh2-Core

# Introduction

RAN1 triggered an LS to RAN2 and RAN3 on how the source gNB-DU can facilitate the handling for configuration of target gN-DU (handling of ref signals of target DU, handling of (L1) measurement reports from the UE of the target DUs etc..).

Agreement - Send an LS to RAN2/RAN3 asking the clarification on intra-/inter-DU scenario: - RAN1 has started the discussion on the configuration for L1 measurement and TCI states for candidate cells. Regarding the following RAN2 agreements captured in RAN2 LS (R1-2208331/R2-2209257), it is not clear for RAN1 which kind of information/configuration for candidate cell(s) are available at a serving cell for inter-DU case for Rel-18 L1/L2 mobility. Thus, companies have different understanding on the implication of the sentence “as much commonality as reasonable” in the LS. - The design for intra-DU and inter-DU L1/L2-based mobility should share as much commonality as reasonable. FFS which aspects need to be different. - RAN1 respectfully asks RAN2 and RAN3 if the serving DU knows the measurement RS configuration and TCI state configuration of cells served by another DU

In RAN2-119bis-e, RAN2 the below agreement that MAC CE is the method for cell switch in LTM while not precluding DCI based.

RAN2 assumes L1/2 mobility trigger information is conveyed in a MAC CE, FFS if the MAC CE or a DCI is used for the actual triggering. RAN2 assumes the MAC CE for L1/2 mobility trigger contains at least a candidate configuration index. FFS if it should be possible to perform SCell activation/deactivation (amongst SCells associated with the candidate configuration) simultaneously with L1 L2 mobility trigger MAC CE (if so, FFS how this is determined).

With this paper we try to provide our views on this and also formulate the contents of the MAC CE.

# Target DU Handling by Source DU

We start off with the view that this is primarily RAN3 domain, and RAN3 is the WG that can decide if a new interface is needed between DUs, or if CU is to mediate. However, we think we can provide our views on this topic to RAN3 for their reference. We have listed below our views on this:

Observation 1: While RAN2 has agreed that L1 based measurement reporting (beam meas for eg) of the target cell is to be delivered via L1 signalling, without any changes, it is still CU that decides a cell switch is needed or not.

Observation 2: If the target DU information is needed at the source DU, then (using current method), it’s the CU that needs to relay this information to the source CU from the target DU.

Observation 3: For the CU to decide on a cell switch, CU needs to be aware of the L1 measurements that source DU gets, and either the L1 measurements need to be “up-converted” to Xn inter-node message or UE needs to report using L3 in inter-DU LTM.

We also made the below agreement in RAN2-119e regarding keeping inter-DU as similar to intra-DU as possible

- RAN2-119e The design for intra-DU and inter-DU L1/L2-based mobility should share as much commonality as reasonable. FFS which aspects need to be different.

Observation 4: Limiting LTM enhancements to intra-DU alone for L1 based LTM is not an efficient design, as many practical deployments have gNB-CU driving several gNB-DUs, and mobility between these DUs is expected.

Based on the above observations, it can be inferred that for LTM to be effective, advantages associated with LTM using L1 based communication, should also be usable when using inter-DU LTM. So we propose to RAN3 that RAN2 views that RAN3 takes into consideration that it is beneficial to have same/or almost same latency reduction advantages with inter-DU as in intra-DU.

If RAN2 feels that an LS is not needed, we think atleast an agreement in RAN2 on the above can be useful.

Proposal 1: RAN2 views that inter-DU LTM using L1 communication should have the same (or very similar) advantages as in intra-DU LTM.

Proposal 2: Inform RAN3 of the above RAN2 view.

# MAC CE information at cell switch

RAN2 already made the below agreement on the RRC configuration related to LTM, with the direction of MAC CE containing a configuration index.

RAN2 assumes L1/2 mobility trigger information is conveyed in a MAC CE, FFS if the MAC CE or a DCI is used for the actual triggering. RAN2 assumes the MAC CE for L1/2 mobility trigger contains at least a candidate configuration index. FFS if it should be possible to perform SCell activation/deactivation (amongst SCells associated with the candidate configuration) simultaneously with L1 L2 mobility trigger MAC CE (if so, FFS how this is determined).

We think that the RRC LTM configuration for a candidate LTM cell should be able to provide more than one configurations such that the MAC CE can “select” one config to be applied by the UE.

Proposal 3: Multiple configurations for each candidate LTM can be configured with RRC, and MAC CE points to one index that the UE should apply.

We also think that the MAC CE should be flexible enough for the target gNB-DU to provide a BWP the UE should switch to.

Proposal 4: MAC CE for LTM cell switch can contain a BWP index.

In addition, to facilitate faster handover (and in cases also RACH less), we think the MAC CE should contain target gNB-DU C-RNTI. Also, we think the below information is useful:

- Whether RACH is needed or not

- TA info: In case of RACH less, or for TA adjustment when the UE can Tx on the target DU

- TCI state: The TCI confir to use in the target DU beam

- Beam info : A refinement of a beam (in case TCI is not enough) – pending RAN1 approval

Proposal 5: MAC CE for LTM cell switch can contain target C-RNTI, an indication if RACH is needed, TA to be applied, TCI state, additional beam related info (based on RAN1 input).

We acknowledge that further input in MAC CE, and even for the above ones, the RAN3 progress on inter-node signalling and other enhancements are needed, but atleast from RAN2 perspective, we think that the above information is likely to be present in the MAC CE.

# Conclusions

Observation 1: While RAN2 has agreed that L1 based measurement reporting (beam meas for eg) of the target cell is to be delivered via L1 signalling, without any changes, it is still CU that decides a cell switch is needed or not.

Observation 2: If the target DU information is needed at the source DU, then (using current method), it’s the CU that needs to relay this information to the source CU from the target DU.

Observation 3: For the CU to decide on a cell switch, CU needs to be aware of the L1 measurements that source DU gets, and either the L1 measurements need to be “up-converted” to Xn inter-node message or UE needs to report using L3 in inter-DU LTM.

Observation 4: Limiting LTM enhancements to intra-DU alone for L1 based LTM is not an efficient design, as many practical deployments have gNB-CU driving several gNB-DUs, and mobility between these DUs is expected.

Proposal 1: RAN2 views that inter-DU LTM using L1 communication should have the same (or very similar) advantages as in intra-DU LTM.

Proposal 2: Inform RAN3 of the above RAN2 view.

Proposal 3: Multiple configurations for each candidate LTM can be configured with RRC, and MAC CE points to one index that the UE should apply.

Proposal 4: MAC CE for LTM cell switch can contain a BWP index.

Proposal 5: MAC CE for LTM cell switch can contain target C-RNTI, an indication if RACH is needed, TA to be applied, TCI state, additional beam related info (based on RAN1 input).

# References