3GPP TSG-RAN WG2 Meeting #120	R2-2211718

Toulouse, France, 14 – 18 November 2022

Agenda item:	8.5.2.1

Source:	Apple

Title:	PDU Set based QoS

WID/SID:	FS_NR_XR_enh

Document for:	Discussion / Decision

# Introduction

In continuation of the 3GPP work on XR in RAN1 and SA4 in Rel-17, RAN has approved a RAN2-led study item on XR enhancements for NR in Rel-18 [1]. According to the study item description, RAN2 should study how XR awareness can help aid XR-specific traffic handling.

The study is to be based on Release 17 TR 38.838, on corresponding Release 17 work from SA4 (as per SP-210043) and on Release 18 work from SA2 (as per SP-211166). 1. Objectives on XR-awareness in RAN (RAN2): - Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of. - Study how the above information aids XR-specific traffic handling.

During RAN2#119e, initial agreements have been reached to identify the main directions of potential enhancements for XR-awareness improvement in this SI. In RAN2#119bis-e, RAN2 agreed to capture possible candidate models for how the mapping of PDU sets onto QoS flows is done in the NAS/CN and how QoS flows are mapped onto DRBs in the AS. It was left FFS how LCH options work in each case.

- RAN2#119bis-e Agreements From RAN2 viewpoint, the following information would be useful for PDU set handling in UL and DL: Semi-static information (from CN to RAN): At least PSER and PSDB. Dynamic information: At least identifying which PDU belongs to which data burst/PDU set is also needed, including means to determine at least PDU set boundaries. Discuss whether to send LS on AS re-ordering once we discuss bearer handling. Capture the models 1a/b, 2a/b in TR and indicate what is possible in current specifications and how. FFS how LCH options work in each case SDAP maps each data packet in a PDU set to a single PDCP SDU, as in legacy (i.e. each PDU is only mapped to a single SDU). HARQ and RLC re-/transmissions for XR traffic are done as in legacy (i.e. they are not based on XR PDU sets).

This paper aims to discuss some of our views on how RAN2 can make use of PDU sets and/or data bursts in UL or DL direction. In continuation of the last meeting, we provide considerations on the mapping of PDU Sets to QoS flows and DRBs, including the impact on the mapping of DRBs to logical channels for different variants. The last section contains a perspective on XR traffic flows not based on PDU Sets.

# Discussion

## QoS flows and DRBs

SA2 agreed in clause 8.4.3 of [5] that RAN performs PDU Set based QoS handling based on a) PDU Set QoS Parameters received via control plane signalling and b) based on PDU Set Information received via user plane. SA2 assumes that the details of RAN behaviors are defined in RAN WGs. In particular, SA2 has been studying CP/UP enhancements for PDU Set based QoS handling to define the treatment of PDU Sets in the DL direction, and it will be up to RAN groups to define the treatment in the UL direction.

SA2 agreed that the UPF identifies PDU Set Information and delivers it to the RAN as in-band signalling via the GTP-U header of a user plane packet. A decision on potential extensions to the 5G QoS model for the treatment of XR traffic is still open in SA2. The current status is reflected in the following paragraph in the SA2 TR [8].

23.700-60 v1.2.0 8.4.2.3 Delivering PDU Set Information to RAN PDU Set Information (listed in clause 8.4.2.1) are informed by UPF to RAN via GTP-U header of user plane packet. Editor's note: Whether PDU Set importance is used for mapping different QoS Flows, sub-QoS Flows, or included in GTP-U header is FFS. (Potential SoH).

As there is no SA2 decision at the time of writing, the analysis and views provided in our contribution to the last meeting still hold true for the QoS mapping [4]. If XR traffic requires mapping of PDUs and PDU Sets to streams with different QoS characteristics, then enhancements to PDCP or SDAP are preferred over lower layer enhancements.

SA2 has agreed in the conclusions for Key Issue #8 to use at least the periodicity of the QoS flow as a baseline for normative work [8] by re-using/extending the TSCAI/TSCAC definitions in clause 5.27.2.1 of TS 23.501. Also, PDU Sets with non-integer periodicity are meant to be transmitted in separate QoS flows. In addition, a QoS Flow level explicit indication may be provided to PSA UPF to enable the ECN marking for the purpose of L4S. In other words, ECN marking for L4S will also be configured per QoS flow.

These are already three cases where a treatment of XR packets has to be done at the granularity of a QoS flow (not PDU Sets). One could go a step further and say that model 1 (Alternative 111/NN1), where there is a 1:1 mapping between PDU Sets and QoS flows, nicely matches with these assumptions.

Proposal 1: RAN2 should rely on the existing QoS model for as much as possible. A one to one mapping of PDU Sets to QoS flows to DRBs is the most preferred approach.

## Mapping of PDU Sets to DRBs and LCHs

The current 5G system models the QoS mapping in a 2-step approach. In the first step, traffic flows are classified at the UPF and the UE’s NAS layer through PDRs and QoS rules to perform IP flow to QoS flow mapping. This step classifies packets as part of a traffic flow detection process for QoS enforcement in the 5G system. In the second step, the SDAP layer performs QoS flow to DRB mapping, where different QoS Flows can be mapped to the same or different DRBs (n:1 or 1:1). While multiple QoS flows belonging to the same PDU session can be mapped to the same DRB, QoS flows belonging to different PDU session cannot be mapped to the same DRB.

The current 5G architecture also supports an option where one DRB can be linked with multiple RLC entities, for example, in the case of a split bearer or for PDCP duplication.

For XR, it is expected that packets associated with different types of PDU Sets may require different packet forwarding treatments or at least some form of discriminative prioritization. Such treatment can be achieved without changes to the existing NR design when there is a 1:1 mapping of PDU Sets to QoS flows and a 1:1 mapping of QoS flows to DRBs as in Model 1a / Alternative 111. In order to fulfill the same requirement for other alternatives listed in section 5.1.2 of [5] the existing 5G design would require some changes.

For options where multiple QoS flows or multiple PDU Sets are mapped to the same DRB (i.e., as in model 1b/2a or alternative NN1/N11), AS layers need to provide a differentiated treatment to the QoS flows or PDU Sets in a single DRB. One relatively straightforward option is to use different RLC entities for transmission of different QoS flows or PDU Sets, where the existing NR radio architecture with multiple RLC entities connected to the same DRB can be reused. Doing so would probably make the QoS mapping a 3-step approach for XR, with the steps being the following:

- IP flow to QoS flow mapping (in 5G NAS, as already done today)

- QoS flow to DRB mapping (in SDAP, as already done today)

- DRB to RLC entity mapping (e.g., in PDCP)

The third step could be done either based on QoS flows (e.g., for alternative NN1) or based on PDU Sets (possible in both alternative NN1 and N11). Another option could be to associate different RLC entities in a defined order for a given DRB, or by enumerating them. The mapping could be provided, for example, in the pdcp-config IE with a new bitmap to provide an association to RLC entities. Details can be defined in work item phase or after SA2 decision on the QoS mapping option.

Although the use of the PDU Set Importance is still inconclusive in SA2, RAN2 could map different PDU Set Importance levels to different LCH priority levels in RAN even under the same DRB, for example, in SA2 option 2.2 (where a single QoS Flow is used for different PDU Set importance levels).

Proposal 2: When PDU Sets are mapped to the same DRB, PDU Set integrated packet handling and differentiated QoS treatment of PDU Sets can be achieved by mapping PDU Sets with different QoS characteristics to different logical channels / RLC entities.

For Model 2b / Alternative N1N the mapping options would reside in the SDAP layer, and different PDU Sets could be mapped to different DRBs. So, there is not necessarily an impact to LCH mapping here.

Finally, another aspect is about how to ensure in-order delivery between flows, which was also discussed earlier [4]. One may generally assume that the application layer ensures to process packets in the desired order for most XR cases (SA2/SA4 may need to confirm).

Proposal 3: Types of PDU Sets associated with different QoS characteristics may be mapped to different DRBs. In-ordering delivery can be maintained in higher layers. When XR traffic flows require in-order delivery in AS, different types of PDU Sets may be mapped to the same DRB.

## Packet Header Aspects

According to the above assumptions when multiple QoS flows or multiple PDU Sets are mapped to the same DRB, the PDCP layer could map packets contained in QoS flows (or PDU Sets) to RLC entities associated with different logical channels, thus allowing for additional prioritization options, or even differentiated QoS. Whether QoS flows or PDU Sets are used will be based on the mapping decision by SA2. In the current NR design the PDCP layer does not have a notion of the QFI, and PDU Sets are anyhow new in Rel-18. However, for mapping a PDCP PDU to the correct RLC entity the PDCP layer would require means to identify this information on a per-packet basis, e.g., at the transmitter.

Looking at the PDU Set related information concluded by SA2 in [8], new parameters can be added to user plane packets in packet headers, and the information contained therein can be used for the LCH mapping. As far as the AS is concerned, the exact layer where new packet headers are needed may depend on the direction SA2 is going to take for the QoS mapping.

One approach is to introduce new packet headers. In order to avoid a repetition of the same information at different layers, the design may involve having a lower layer (e.g., at the UE or at the gNB) identify PDU set related information from within a packet header of a higher layer. This is not exactly following the layering principle, however, 5G NR already uses a similar principle for RoHC where PDCP needs to figure out information from within part of the payload of a PDCP SDU. Another option is to let gNB/UE implementation handle this process, for example, based on meta-data associated with each packet.

We show an initial perspective on packet header information in our contribution in [6].

Proposal 4: The exact location (layer) of new packet headers can be defined based on SA2 progress.

## Traffic flows not based on PDU Sets

In the last meeting, RAN2 had a brief discussion on traffic flows not based on PDU Sets. The topic was initially discussed in email discussion [Post-119][261][XR] leading up to [7] and could not be concluded in RAN2#119bis-e.

We assume XR traffic flows consist of multiple flows where some of them are based on groups of packets (i.e., PDU Sets) and some are not based on PDU Sets (such as pose or control information, or traditional audio). The traffic flows not based on PDU Sets literally consist of single packets. However, such traffic flows may need to be transmitted in a manner synchronized with other XR traffics. For example, pose control information relates to other XR traffic flows and influences the way information is represented to the user. Therefore, it needs to be synchronized within bounds just like audio has a close timing relation with video (lip-sync).

As far as power saving is concerned (which was the original discussion) we think that it is useful for to provide the RAN with their periodicity, start offset and (where applicable) with a range of jitters for UE power savings.

In our understanding, traffic flows not based on PDU Sets can be handled by two ways: a) in traditional way based on single packet QoS, or b) based on PDU Set QoS. For case b), traffic flow can be considered to consist of only one packet per PDU Set. Their treatment according to a) or b) may differ depending on whether there is a close relation to other traffic flows within the multiple flows that encompass an XR service.

For example, SA2 assumes in their conclusions for Key Issue #1 that streams that are closely related and require strong application coordination are transmitted in a single PDU session by a single UE. However, those data streams that contribute to the immersive experience, but may still be valid stand-alone, may be transmitted over separate PDU sessions from multiple UEs [8].

Proposal 5: “Traffic flows not based on PDU Sets” should be characterized based on their contextual relation to other XR traffic flows and PDU Sets to be treated.

Proposal 6: “Traffic flows not based on PDU Sets” can be treated in two ways on a QoS flow, DRB or LCH:

- In traditional per-packet fashion (when its PDUs are independent of other XR traffic flows); or

- As PDU Set with “number of packets = 1” (when its PDUs are closely related to other XR traffic flows) e.g. to keep them in the framework for XR traffic.

# Conclusions

This contribution provides a view on study areas around XR awareness as part of the RAN2 study for NR enhancements for XR. We have following observations and proposals:

Proposal 1: RAN2 should rely on the existing QoS model for as much as possible. A one to one mapping of PDU Sets to QoS flows to DRBs is the most preferred approach.

Proposal 2: When PDU Sets are mapped to the same DRB, PDU Set integrated packet handling and differentiated QoS treatment of PDU Sets can be achieved by mapping PDU Sets with different QoS characteristics to different logical channels / RLC entities.

Proposal 3: Types of PDU Sets associated with different QoS characteristics may be mapped to different DRBs. In-ordering delivery can be maintained in higher layers. When XR traffic flows require in-order delivery in AS, different types of PDU Sets may be mapped to the same DRB.

Proposal 4: The exact location (layer) of new packet headers can be defined based on SA2 progress.

Proposal 5: “Traffic flows not based on PDU Sets” should be characterized based on their contextual relation to other XR traffic flows and PDU Sets to be treated.

Proposal 6: “Traffic flows not based on PDU Sets” can be treated in two ways on a QoS flow, DRB or LCH:

- In traditional per-packet fashion (when its PDUs are independent of other XR traffic flows); or

- As PDU Set with “number of packets = 1” (when its PDUs are closely related to other XR traffic flows) e.g. to keep them in the framework for XR traffic.

# References

- RP-220285, Revised SID, Study on XR Enhancements for NR, Nokia, RAN#95e

- XRM_KI45_view_collection_summary_v1.docx, “KI#4 and KI#5, Summary of company views”, Huawei, SA2 reflector

- R2-2207429, Considerations on XR-awareness, QoS-metrics, and XR-specific traffic handling, Apple, RAN2#119e

- R2-2209777, PDU Sets and Mapping of QoS flows and DRBs for XR, Apple, RAN2#119bis-e

- R2-2210814, TR 38.835 v030 Nokia (Rapporteur), draft TR, RAN2#119bis-e

- R2-2211721, PDU Set Parameters and Descriptors, Apple, RAN2#120

- R2-2209455, Information to RAN for UE power savings, Qualcomm Incorporated, RAN2#119bis-e

- TR 23.700-60, v1.2.0, Study on XR (Extended Reality) and media services, Rel-18