3GPP TSG-RAN WG2 Meeting #120	R2-2211721

Toulouse, France, 14 – 18 November 2022

Agenda item:	8.5.3.2

Source:	Apple

Title:	PDU Set Parameters and Descriptors

WID/SID:	FS_NR_XR_enh

Document for:	Discussion / Decision

# Introduction

In continuation of the 3GPP work on XR in RAN1 and SA4 in Rel-17, RAN has approved a RAN2-led study item on XR enhancements for NR in Rel-18 [1]. According to the study item description, the study encompasses three areas, namely XR-awareness, XR-specific power saving, and XR-specific capacity improvements.

RAN2#119e had a post email discussion [Post-119][261][XR] on how to make use of PDU Sets and/or Data Bursts in UL or DL direction. The discussion was aimed at identifying QoS parameters that would be useful to RAN for UE power savings. During the discussion, companies touched upon also the more general set of QoS parameters that describes a PDU Set. The outcome of the email discussion [3][2] was revisited online in RAN2#119bis-e, and by that time SA2 had already reached a number of conclusions (summarized in observations 2-6 below).

Consequently, RAN2#119bis-e made following observations and agreements:

- RAN2#119bis-e Observations Observation 1. There are strong support among companies that the following set of information on DL/UL traffic is useful to RAN for UE power savings: • Traffic parameters (e.g. periodicity, start offset, etc) are useful to RAN in configuring DRX and can be semi-statically signalled to RAN. • Jitter statistics (e.g. range) are useful to RAN, e.g. in configuring DRX on duration, and can be semi-statically signalled to RAN. • Boundary indication (e.g. start and/or end of a PDU Set or a Data Burst) is useful to RAN, e.g. in timely termination of DRX active time. It can be dynamically signaled to RAN. • Information for identifying a PDU Set (e.g. sequence number) is useful to RAN and can be dynamically signalled to RAN. • Explicit indications and/or conditions for RAN to decide whether to deliver or discard a media unit is useful to RAN, e.g. to avoid unnecessary re-/transmissions and thus save UE power. • For traffic flows not based on PDU Sets, their periodicity, start offset and range of jitters are useful information to RAN, e.g. in DRX configuration. Observation 2. SA2 have agreed that periodicity of DL/UL QoS flows and jitter information associated with each periodicity are provided to NG-RAN at PDU Session Establishment/Modification. Observation 3. SA2 have agreed that for downlink traffic, end of data burst can be optionally signalled in the header of the last PDU of the Data Burst. Observation 4. SA2 have agreed that for downlink traffic, PDU Set related information such as PDU Set Identifier, start/end indication of a PDU Set or number of PDUs within a PDU Set are signalled via user plane for PDU Set based handling. Observation 5. SA2 have agreed that on downlink, whether all PDUs are needed for the usage of PDU Set by application layer is part of PDU Set QoS parameters provided to RAN. Observation 6. SA2 have agreed that uplink PDU Set handling should be studied and led by RAN WGs. - RAN2#119bis-e Agreements RAN2 discuss whether additional traffic or QoS related information on downlink traffic beyond what has been agreed by SA2 needs to be provided to RAN for UE power savings. RAN2 study what traffic and QoS related information on uplink traffic (e.g. counterpart of what has been agreed by SA2) should be provided to RAN for UE power savings and how the information may be provided to RAN. From RAN2 viewpoint, the following information would be useful for PDU set handling in UL and DL: Semi-static information (from CN to RAN): At least PSER and PSDB. Dynamic information: At least identifying which PDU belongs to which data burst/PDU set is also needed, including means to determine at least PDU set boundaries.

This contribution aims to discuss some of our views on how RAN2 can make use of PDU Sets and/or Data Bursts in UL and DL direction. We identify remaining parameters and open items, indicate our view on QoS parameters required to be defined by RAN2 in relation to recent agreements made by SA2 indicated in the LS in [7]. Finally, the paper captures some general points about PDU Set based parameters and packet headers.

# Discussion

## SA2 Parameters for supporting PDU Sets

SA2 has defined PDU Set QoS Parameters and PDU Set Information for supporting PDU Set in downlink [4]. The further PDU Set handling for uplink was left to RAN WGs.

We first analyze which parameters are already there in the SA2 TR and whether additional aspects are recommended to be considered from RAN2 perspective. Tables 1 and 2 summarize the SA2 parameters per the current SA2 conclusions in [4], along with a view on what that could mean from a RAN2 perspective including which parameters are needed in uplink. The two tables also indicate which information is per PDU Set and which information is per Data Burst. Parameters in red font indicate potential additional parameters and/or aspects for further discussion.

Table 1 shows PDU Set QoS Parameters that are assumed to rely on configuration mainly via the control plane, to support PDU Set based QoS treatment. Table 2 shows PDU Set related Information that is assumed to rely on per-packet indication in the user plane.

Table 1: Control Plane Parameters

| SA2 PDU Set QoS Parameters | RAN2 Parameter Assumptions |
| --- | --- |
| DL PSDB: PDU Set Delay Budget (per PDU Set) - using dynamic or non-dynamic PCC - AF information provisioning to NEF/PCF (for dynamic PCC), or PCF local config | Ok to follow SA2 - N2/N4 signalling - PDCP discard config based on PSDB |
|  | UL PSDB: PDU Set Delay Budget (per PDU Set) - using dynamic or non-dynamic PCC - AF information provisioning to NEF/PCF (for dynamic PCC), or PCF local config - N2/N4 signalling - PDCP discard config based on PSDB |
| DL PSER: PDU Set Error Rate (per PDU Set) - using dynamic or non-dynamic PCC - AF information provisioning to NEF/PCF (for dynamic PCC), or PCF local config | Ok to follow SA2 - N2/N4 signalling |
|  | UL PSER: PDU Set Error Rate (per PDU Set) - using dynamic or non-dynamic PCC - AF information provisioning to NEF/PCF (for dynamic PCC), or PCF local config - N2/N4 signalling |
| DL PDU Set Integrated Indication (per PDU Set) - Whether all PDUs are needed for the usage of the PDU Set - AF information provisioning to NEF/PCF (for dynamic PCC), or PCF local config | Generally ok to follow SA2 FFS: Parameter details and parameter range - Can this parameter be indicated as a percentage or minimum required number of packets or a statistical distribution (max range, mean), or will it be a Boolean? - Also, where do we expect this parameter - in NAS signalling or AS signalling? - Assumption: The parameter affects the PDU discard policy, hence AS needs to be aware. It could be part of a PDU Set Descriptor or sent as control signalling. |
|  | UL PDU Set Integrated Indication (per PDU Set) - Generally ok to follow SA2 - Same opens as in DL |
| DL burst periodicity (per PDU Set) | Generally ok to follow SA2 - The granularity of this information is not crystal clear from the SA2 conclusions, although assistance information provisioning in dynamic PCC is per PDU Set. We assume it is per PDU Set (preferred). |
|  | UL burst periodicity (per PDU Set) - Generally ok to follow SA2 - Same opens as in DL |
| Traffic/assistance information to NG-RAN during PDU session establishment/modification (per QoS flow) - Periodicity for UL and DL traffic of the QoS flow, where non-integer periodicities are associated with separate QoS flows - Traffic jitter information associated with each periodicity | Generally ok to follow SA2 - Open: Details as to how jitter information is to be signaled, e.g., what is the range and whether a probability distribution should be part of the definition. - FFS: It would be beneficial to signal at least periodicity at the granularity of a PDU Set (if not already included in ‘burst periodicity’ above), as well as jitter. |
|  | UE assistance information over RAN signalling, applicable to UL and DL - Provides an option to allow for fast parameter adjustments when needed (e.g., periodicity, delay, pattern offsets, jitter, etc.) |

Table 2: User Plane Parameters (per packet)

| SA2 PDU Set Information | RAN2 Parameter Assumptions |
| --- | --- |
| DL PDU Set Identifier | Ok to follow SA2 |
|  | UL PDU Set Identifier - Same as in DL |
| Optional DL Start PDU and End PDU of the PDU Set | Ok to follow SA2 - FFS: Start and End PDU may be represented in different ways, for example, using a “Delimiter Info” to indicate whether a packet is the first, middle or last packet in the PDU Set (e.g., 2 bits), or by simply adding a “Continuation Flag” (1 bit), a Start/End Marker, etc., depending on the layer where it is signaled. |
|  | Optional UL Start PDU and End PDU of the PDU Set - Same assumptions as in DL |
| DL PDU Set SN within the PDU Set | Ok to follow SA2 |
|  | UL PDU Set SN within the PDU Set - Same as in DL |
| Optional: DL PDU Set Size (or number of PDUs within a PDU Set) | Ok to follow SA2 |
|  | Optional: UL PDU Set Size (or number of PDUs within a PDU Set) - Same as in DL |
| DL PDU Set Importance - Note 1: The inclusion of PDU Set Importance in the GTP-U header is still FFS in SA2 (e.g., SA2 option 2.2). - Note 2: If PDU Set Importance is used for mapping PDU Sets to different sub-QoS Flows at the UPF side (as in SA2 option 2.1), we assume the sub-QoS flow needs to be communicated e.g. for the tunnel. - Note 3: If PDU Set Importance is used for mapping PDU Sets to different QoS Flows at the UPF (as in SA2 option 1), a notion of the QFI is already there in the current design. | Ok to follow SA2 - Presence of this field is conceptionally connected with the mapping option of PDU Sets to QoS flows that SA2 will decide - FFS: Implications to RAN/CN design |
|  | UL PDU Set Importance - Same assumptions as in DL |
| Optional End of Data Burst indication in the header of the last DL PDU of the Data Burst | Ok to follow SA2 - FFS: How to ensure the last DL PDU is not lost and received/processed in-sequence |
|  | Optional End of Data Burst indication in the header of the last UL PDU of the Data Burst - FFS: How to ensure the last UL PDU is not lost and received/processed in-sequence |

The observations in table 1 and table 2 above can be summarized as follows.

Proposal 1: RAN2 assumes to consider SA2’s conclusions for PDU Set QoS Parameters and PDU Set Information for parameters in the downlink direction. Details can be discussed in WI phase.

Proposal 2: RAN2 assumes PDU Set QoS Parameters and PDU Set Information for control/user plane enhancements in downlink (as concluded by SA2 in TR 23.700-60v1.2.0) as a baseline for RAN and UE enhancements for supporting PDU Set in the uplink direction.

Proposal 3: FFS on parameter range and RAN2 details associated with PDU Set Integrated Indication, Burst Periodicity, Assistance Information to NG-RAN. FFS how to ensure the last PDU is not lost and received/processed in-sequence when indicating the End of Data Burst in the header of the last PDU of the Data Burst.

## Additional QoS Parameters

Secondly, we identify any additional QoS parameters that can be useful for the RAN to be aware of. Table 3 lists a number of further parameters not currently considered by SA2. These parameters can enable additional UE power savings and provide capacity benefits in the cell as well.

Table 3: Additional Parameters for XR

| Additional Parameters considered useful to recommend to SA2 for PDU Sets | RAN2 Parameter Assumptions |
| --- | --- |
| DL burst timing (per PDU Set) - Including an estimated arrival time for the first and the last packet (burst timing) - Control plane / semi-static signalling | This parameter is not currently present in the SA2 parameter set. We consider it useful for more accurate resource allocation. |
|  | UL burst timing (per PDU Set) - Including an estimated arrival time for the first and the last packet (burst timing) - Control plane / semi-static signalling |
| Inter PDU Set Dependency (DL, UL) | This parameter is not currently present in the SA2 parameter set. We consider it useful for power saving and capacity since it can prevent the transmission of packets that are not useful to the application, and there can be many such packets. However, based on the latest SA4 assumptions the feasibility needs to be clarified. |
| Intra PDU Set dependency (DL, UL) | This parameter is not currently present in the SA2 parameter set; it might be an explicit indication. - Dynamic indication of critical packets in a PDU set - For example, a higher reliability may be needed for certain PDUs or portion of a PDU set depending on the situation, the remaining delay budget etc., to avoid a loss of the whole PDU set, thus, to avoid a waste of transmission power already spent. |
| Synchronization Preference (DL, UL) - Per current SA2 assumptions, streams that are closely related and require strong coordination are transmitted in a single PDU session by a single UE (but they can still be in different DRBs) | This parameter (or possibly a structure) is considered useful for more accurate resource allocation when multiple traffic flows are related. - Synchronization delay tolerance between flows with a synchronization requirement - It can be part of UE assistance information |
| Congestion information (DL, UL) | Congestion information, refer to our contribution in [5] |
|  | RAN2/RAN3 specific Discard Indication to minimize reordering delay, refer to our contribution in [6] |
|  | Dynamic indication of control information, e.g., for updates of traffic parameters (only if necessary) or to request an adjustment. |

We assume the above set of parameters can help facilitate UE power saving. At the same time, the parameters encompass information useful for XR-Awareness in the RAN and at the UE, which is a prerequisite for capacity enhancements as well.

Proposal 4: RAN2 can study additional parameters for PDU Set based QoS (such as parameters to describe the Burst Timing of a PDU Set, Inter/Intra PDU Set Dependency, Synchronization Preference, Congestion Information) to understand further functions/parameters and potentially recommend them to SA2.

## Packet Headers and Descriptors

RAN2 had a first discussion on how to handle PDU Sets efficiently in both UL and DL. It was assumed that information that facilitates the handling of PDU Sets could be structured as semi-static and dynamic information. Following agreements were made:

- RAN2#119bis-e Agreements From RAN2 viewpoint, the following information would be useful for PDU set handling in UL and DL: Semi-static information (from CN to RAN): At least PSER and PSDB. Dynamic information: At least identifying which PDU belongs to which data burst/PDU set is also needed, including means to determine at least PDU set boundaries.

In our understanding, a portion of PDU Set information will be common to all packets in a PDU Set. Such information could be communicated in a PDU Set Descriptor, in a special header, or as a separate message. The information might be considered primarily as semi-static, but it can be understood as somewhat dynamic as well.

We assume that common information could be signaled in two ways.

- As control plane signalling over RRC/NAS

- As user plane signalling

  - Using a Control PDU with dedicated contents

  - Using a Data PDU for certain packets. Other Data PDUs may carry additional packet-specific PDU Set Information.

Examples of common PDU Set Information useful to include through the User Plane signalling include:

- PDU Set Importance

- PDU Set Size or number of packets in the PDU Set

- PDU Set Identifier

- PDU Set Integrated Indication (in case the packet dropping criteria changes on a per PDU Set basis)

Figure 1 shows a high level approach to accumulate common information in a PDU Set Descriptor.

Figure 1: Association between PDU Sets and Packets using a PDU Set Descriptor

To account for information common to all packets in a PDU Set, one user plane packet (typically the first packet) could carry a larger header with a PDU Set descriptor. The descriptor needs to be present only once per PDU Set. However, a drawback of this approach is that the PDU Set descriptor may have to be protected against packet loss to avoid impacting other PDUs in the PDU Set. Hence, such option might be restricted to acknowledged mode or applied in an environment where packets are not likely to be lost. Alternatively, the PDU Set descriptor can be placed in a packet (SDU) of high importance and the receiver buffers earlier PDU Set packets (SDUs) until it has received the PDU Set descriptor, and only full interpretation of PDU Set packets starts only then. The latter seems to imply some delay although it may reduce signalling overhead.

Alternatively, instead of using a PDU Set descriptor in, e.g., the first user plane packet of a PDU Set, a descriptor could be provided as semi-static information over AS or NAS signalling. In this case, the descriptor may also include typical PDU Set parameter values associated with a QoS or traffic flow, DRB, or PDU session. Obviously, such approach is useful for PDU Sets consisting of a very large number of individual packets. At the same time, a user plane packet could additionally contain a PDU Set Descriptor to carry a delta to the common parameters signaled via AS or NAS, if needed. In the latter case, the PDU Set descriptor is valid for a) the current PDU Set only or b) all PDU Sets onwards.

Proposal 5: PDU Set Information common to all packets in a PDU Set may be transmitted in a PDU Set Descriptor.

The considerations above lead to a second approach where user plane packets might carry per-packet header contents only and a PDU Set Descriptor is not used. An example is shown in Figure 2 (parameter contents will have to be adjusted once finalized).

Figure 2: PDU Set header in every user plane packet without using a PDU Set Descriptor

This approach has the advantage that PDU Set signalling does not depend on some central information a single message, and so out-of-sequence scenarios are easier to support and flexibility increases. The per-packet information would have to carry a minimum set of parameters required to describe a PDU Set. In our understanding, what has been assumed by SA2 so far is already close to that set. However, some repeated information (e.g., PDU Set Id) would be needed in every packet header.

PDU Set Information is eventually going to be signaled in new packets headers. Which AS layers are extended with new packet headers and what interfaces need to be considered will depend on the choice SA2 will be making on the extension of the QoS model and the mapping of PDU Sets to DRBs. Based on what is known to date at least the GTP-U interface will be extended with new header information. If SA2 decides to use Model 1a / Alternative 111, only a limited number of RAN2 packet headers (if at all) will be needed. On the other hand, all other mapping alternatives may need new packet headers, e.g., in PDCP or SDAP.

Proposal 6: When multiple types of PDU Sets are carried in the same DRB or QoS flow, new packet headers indicate PDU Set Information over RAN2 user plane. Details can be defined in work items phase.

# Conclusions

This contribution provides a view on PDU Set Parameters and Packet Headers. It touches upon both power saving and XR-awareness aspects. We have the following proposals:

Proposal 1: RAN2 assumes to consider SA2’s conclusions for PDU Set QoS Parameters and PDU Set Information for parameters in the downlink direction. Details can be discussed in WI phase.

Proposal 2: RAN2 assumes PDU Set QoS Parameters and PDU Set Information for control/user plane enhancements in downlink (as concluded by SA2 in TR 23.700-60v1.2.0) as a baseline for RAN and UE enhancements for supporting PDU Set in the uplink direction.

Proposal 3: FFS on parameter range and RAN2 details associated with PDU Set Integrated Indication, Burst Periodicity, Assistance Information to NG-RAN. FFS how to ensure the last PDU is not lost and received/processed in-sequence when indicating the End of Data Burst in the header of the last PDU of the Data Burst.

Proposal 4: RAN2 can study additional parameters for PDU Set based QoS (such as parameters to describe the Burst Timing of a PDU Set, Inter/Intra PDU Set Dependency, Synchronization Preference, Congestion Information) to understand further functions/parameters and potentially recommend them to SA2.

Proposal 5: PDU Set Information common to all packets in a PDU Set may be transmitted in a PDU Set Descriptor.

Proposal 6: When multiple types of PDU Sets are carried in the same DRB or QoS flow, new packet headers indicate PDU Set Information over RAN2 user plane. Details can be defined in work items phase.

# References

- RP-220285, Revised SID, Study on XR Enhancements for NR, Nokia, RAN#95e

- R2-2210825, Information to RAN for UE power savings based on SA2 progress, Qualcomm Incorporated, RAN2#119bis-e

- R2-2209455, Information to RAN for UE power savings, Qualcomm Incorporated, RAN2#119bis-e

- TR 23.700-60, v1.2.0, Study on XR (Extended Reality) and media services, Rel-18

- R2-2211719, Enhancements for Traffic Prioritization in XR, Apple, RAN2#120

- R2-2211720, Packet Discarding and Reordering Enhancements for XR, Apple, RAN2#120

- S2-2209979, LS on XR and Media Services, SA2	 LS in, Rel-18 FS_XRM, To: RAN1, RAN2, RAN3