3GPP TSG-RAN WG2 Meeting #120	R2-2211783

# Toulouse, France, November 14 – 18, 2022

Agenda item:	8.9.4

Source: 	China Telecom

Title:	Discussion on multi-path relaying

WID/SID:	NR_SL_relay_enh-Core

Document for:	Discussion and Decision

- Introduction

In the last RAN2 meeting, fruitful agreements were reached on supporting multi-path in both Scenario 1 and 2, including the supported path management cases and other CP aspects for multi-path. However, there are still some remaining issues on path management cases for multi-path that may need further discussion.

Agreements:

Proposal 1-1A (modified): The following cases are to be supported for Scenario 1.

A.	The remote UE operating only on the direct path adds the indirect path under the same gNB;

B.	The remote UE operating only on the indirect path adds the direct path under the same gNB;

C.	The remote UE operating in multi-path releases the indirect path;

D.	The remote UE operating in multi-path releases the direct path;

G.	The remote UE operating in multi-path changes to a new relay UE for the indirect path while keeping the direct path under the same gNB.  FFS if this case would be supported via separate release-and-add (A+C in separate reconfigurations) or a single switch procedure (e.g. similar to i2i service continuity).

The following case can be supported via separate release-and-add for scenario 1 (B+D in separate reconfigurations):

E.	The remote UE operating in multi-path changes the direct path to a different cell of the same gNB while using the serving relay UE for the indirect path under the same gNB.

FFS if a single procedure for this case would be supported.

Proposal 1-2A: The following cases are proposed to be supported for Scenario 2.

A.	The remote UE configured only on the direct path adds the indirect path under the same gNB;

C.	The remote UE configured with multi-path releases the indirect path;

Proposal 1-2C: Whether to support the following case can be further discussed for Scenario 2.

B.	The remote UE configured only on the indirect path adds the direct path under the same gNB;

D.	The remote UE configured with multi-path releases the direct path;

E.	The remote UE configured with multi-path changes the serving cell of the remote UE for the direct path while keeping the serving relay UE for the indirect path under the same gNB;

G.	The remote UE configured with multi-path changes to a new relay UE for the indirect path while keeping the direct path under the same gNB.

In this contribution, we would like to share our opinions on the remaining issues on path management cases and also RLF handling for multi-path relaying.

- Discussion

Path management

To support multi-path, the path management function shall be supported, including the addition/ modification/ release of the secondary path. Based on the agreements reached in the last RAN2 meeting, there are a few FFS left for the supported path management cases in Scenario 1 and 2.

- Path management cases in Scenario 1

Regarding the path management cases supported in Scenario 1, the path modification cases E and G are supported. For case G, the remote UE operating in multi-path changes to a new relay UE for the indirect path while keeping the direct path under the same gNB, which is quite similar to intra-gNB i2i service continuity scenario. Although this case can be supported via separate release-and-add procedures, the signalling overhead and user experience shall be taken into account. Two separate reconfiguration procedures may lead to the reduction of data rate and unnecessary signalling overhead compared to a single switch procedure. Considering the intra-gNB i2i service continuity scenario will be supported in Rel-18, we suggest supporting a single switch procedure and taking the intra-gNB i2i path switching procedure as a baseline for case G.

Proposal 1: For indirect path change case in Scenario 1(i.e. Case G), a single switch procedure would be supported. The intra-gNB i2i path switch procedure for service continuity could be taken as a baseline.

For case E in Scenario 1, it has been agreed that separate release-and-add can be used to support the direct path change while keeping the indirect path unchanged. Considering the tight timeline for the study phase of multi-path, we suggest further discussing whether to support a single procedure in the normative work phase if time allowed.

Proposal 2: For direct path change case in Scenario 1(i.e. Case E), leave the discussion on whether to support a single procedure for this case to the normative work phase.

- Path management cases in Scenario 2

Regarding the path management cases supported in Scenario 2, the indirect path addition and release cases A and C are supported. For case B and D in Scenario 2, there exists the scenario that the remote UE accesses to the NW by the relay UE through non-3GPP interface, which seems not yet supported in the current spec. For the support of case E and G, there may be some dependency on the support of case B and D at least when separate release-and-add procedures are considered. In this sense, these cases in Scenario 2 may need more discussion and extra spec efforts compared to those in Scenario 1. Besides, whether these cases are useful or valid in Scenario 2 is still unclear. Considering the time limitation in study phase, we suggest focusing on the basic path management cases in Scenario 2 and we can further discuss the other cases in later release.

Proposal 3: For Scenario 2, the following path management cases (i.e. case B, D, E and G) are not supported in this release.

- B.	The remote UE configured only on the indirect path adds the direct path under the same gNB;

- D.	The remote UE configured with multi-path releases the direct path;

- E.	The remote UE configured with multi-path changes the serving cell of the remote UE for the direct path while keeping the serving relay UE for the indirect path under the same gNB;

- G.	The remote UE configured with multi-path changes to a new relay UE for the indirect path while keeping the direct path under the same gNB.

- Measurement report for path management in multi-path relaying

For Scenario 1, similar to legacy cell group management, the NW might initiate the path management for multi-path mainly based on the service requirements and the channel quality of the Uu link and the PC5 link. Therefore, the measurement configuration and reporting function may need further enhancement to cover the supported path management cases for multi-path relaying. As listed in the table below, we can analyse the potential spec impact, such as whether new event is needed case by case.

Table 1 Path management cases and potential spec impact in Scenario 1

| Function | Path management case | Criteria | Potential spec impact |
| --- | --- | --- | --- |
| Path addition | directdirect + indirect (Case A) | Candidate relay RSRP > threshold | Event Y2 can be reused. No spec impact. |
|  | indirectdirect + indirect (Case B) | Neighbour (NR cell) RSRP> threshold | Event A4 can be reused. No spec impact. |
| Path modification | Change indirect path (Case G, assuming a single procedure is used) | Serving relay RSRP < threshold1 and candidate relay RSRP > threshold2; or Candidate relay RSRP > serving relay RSRP | The new Event Z1 introduced for i2i path switch can be reused. The new Event Z2 which is FFS for i2i path switch could be used. |
|  | Change direct path (Case E, assuming a single procedure is used) | Neighbour RSRP> SpCell RSRP; or SpCell RSRP< threshold1 and neighbour RSRP> threshold2 | Event A3 and/ or A5 can be reused. No spec impact. |
| Path release | Release indirect path (Case C) | Serving Relay RSRP < threshold | Event X2 can be reused. No spec impact. |
|  | Release direct path (Case D) | Serving (NR cell)RSRP < threshold | Event A2 can be reused. No spec impact. |

Based on the above analysis, most of the new path management cases could be covered by the current events, except for the case of indirect path change. The key point is to introduce new events that can trigger the UE to provide necessary measurement results to help the NW evaluate the serving relay UE as well as the candidate relay UEs, so that the NW can initiate the change of relay UE accordingly. Therefore, the potential new events for indirect-to-indirect path switching scenarios could also be used for the change of indirect path in multi-path scenario. Actually, the new Event Z1 has been agreed in the last RAN2 meeting while Event Z2 is still under discussion in the i2i path switch topic. To support the indirect path change cases in Scenario 1, at least the newly introduced Event Z1 can be reused; for Event Z2, we can wait for the progress on i2i path switch topic.

Proposal 4: To support indirect path change cases in Scenario 1, the new Event Z1 introduced for i2i indirect path switch can be used. Wait for the progress on i2i path switch topic to see whether Event Z2 is needed or not.

For Scenario 2, we can focus on the basic path management cases including Case A and C. For the overall procedure, we can take Scenario 1 as a baseline. For the measurement aspects, the measurement between the remote UE and relay UE is out of 3GPP scope, so the legacy Event X2 and Y2 might not be reused directly. The candidate relay selection can be up to remote UE implementation and the details can be discussed in the normative work phase.

Proposal 5: For Scenario 2, the path management procedure of Scenario 1 can be taken as baseline and the details can be further discussed in the normative work phase.

RLF handling in multi-path relaying

In the last RAN2 meeting, the following agreement is reached on RLF detection related issues, while RLF handling is still under discussion.

Agreements:

Proposal 12	[21/21] (modified) When UE operating in multi-path Relay, it performs RLM for Uu interface, for Scenario-1 and Scenario-2. For PC5 interface in Scenario-1, it performs sidelink RLF detection based on Rel-16 V2X specification [20/21]. For UE-UE link in Scenario-2, whether/how to have failure detection is out of 3GPP scope.

FFS whether there is impact to layers under our control from a failure of the UE-UE link in scenario 2.

For multi-path relaying in both Scenario 1 and 2, when the RLF is detected on one path, it is very likely that the other path is still available. Considering the main motivation of introducing multi-path is to further improve the throughput and the reliability, it is better for the remote UE to inform the gNB about the path failure as soon as possible so that the NW can help the remote UE recovery the path. Similar to DC scenario, the path failure information or/and fast path recovery procedure could be introduced in multi-path scenarios. Upon detection of RLF or path failure on one path, the remote UE can report the path failure information and may also report available measurement results on the other path if available. Based on the failure information, the NW can decide whether to release or recover the failed path.

Moreover, upon the RLF of the PCell (or the primary path if introduced), the remote UE may trigger RRC re-establishment. Considering the latency and service interruption caused by RRC re-establishment procedure, it is reasonable to avoid unnecessary RRC re-establishment if the other path is still available even when the primary path is failed. Similar to the fast MCG failure recovery mechanism, introducing the path failure information or/and fast path recovery procedure in multi-path scenarios could also help reduce service interruption and improve user experience.

Proposal 6: Upon detection of RLF or path failure on one path, the remote UE in both Scenario 1 and 2 can report the path failure information on the other path if available.

- Conclusion

In this contribution, we discuss path management cases and RLF handling related issues in multi-path relaying.  We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: For indirect path change case in Scenario 1(i.e. Case G), a single switch procedure would be supported. The intra-gNB i2i path switch procedure for service continuity could be taken as a baseline.

Proposal 2: For direct path change case in Scenario 1(i.e. Case E), leave the discussion on whether to support a single procedure for this case to the normative work phase.

Proposal 3: For Scenario 2, the following path management cases (i.e. case B, D, E and G) are not supported in this release.

- B.	The remote UE configured only on the indirect path adds the direct path under the same gNB;

- D.	The remote UE configured with multi-path releases the direct path;

- E.	The remote UE configured with multi-path changes the serving cell of the remote UE for the direct path while keeping the serving relay UE for the indirect path under the same gNB;

- G.	The remote UE configured with multi-path changes to a new relay UE for the indirect path while keeping the direct path under the same gNB.

Proposal 4: To support indirect path change cases in Scenario 1, the new Event Z1 introduced for i2i indirect path switch can be used. Wait for the progress on i2i path switch topic to see whether Event Z2 is needed or not.

Proposal 5: For Scenario 2, the path management procedure of Scenario 1 can be taken as baseline and the details can be further discussed in the normative work phase.

Proposal 6: Upon detection of RLF or path failure on one path, the remote UE in both Scenario 1 and 2 can report the path failure information on the other path if available.

- Reference

RP-221262	Revised WID on NR sidelink relay enhancements	LG Electronics

3GPP TS 38.331 17.0.0: "NR; Radio Resource Control (RRC) protocol specification".

Report from session on positioning and sidelink relay