3GPP TSG-RAN WG2 Meeting #120                                                   R2-2211788

Toulouse, 14th - 18th November, 2022

Source:	Qualcomm Incorporated

Title:	Further discussion on multi-path relay for Scenario 1 and Scenario 2

Agenda Item:	8.9 Enhanced NR Sidelink Relay

Document for:	Discussion and Decision

# Introduction & Background

According to WID [1] and RAN2 agenda for REl-18 Relay enhancement, and his contribution discusses the open issues for scenario 1 and scenario 2.

- Study the benefit and potential solutions for multi-path support to enhance reliability and throughput (e.g., by switching among or utilizing the multiple paths simultaneously) in the following scenarios [RAN2, RAN3]:

  - A UE is connected to the same gNB using one direct path and one indirect path via 1) Layer-2 UE-to-Network relay, or 2) via another UE (where the UE-UE inter-connection is assumed to be ideal), where the solutions for 1) are to be reused for 2) without precluding the possibility of excluding a part of the solutions which is unnecessary for the operation for 2).

Note 3A: Study on the benefit and potential solutions are to be completed in RAN#98 which will decide whether/how to start the normative work.

Note 3B: UE-to-Network relay in scenario 1 reuses the Rel-17 solution as the baseline.

Note 3C: Support of Layer-3 UE-to-Network relay in multi-path scenario is assumed to have no RAN impact and the work and solutions are subject to SA2 to progress.

# Discussion

## 2.1 Basic control plane issues for scenario 1

##### 2.1.1 The path of PCell

It was also agreed in the last RAN2 meeting that the Remote UE can connect with gNB over indirect path with direct path addition and removal as following case B and case D.

A.	The remote UE operating only on the direct path adds the indirect path under the same gNB;

B.	The remote UE operating only on the indirect path adds the direct path under the same gNB;

C.	The remote UE operating in multi-path releases the indirect path;

D.	The remote UE operating in multi-path releases the direct path;

On top of the above agreement, it was further discussed which path should be the path of PCell considering all above four cases. Generally, there are two options:

Option 1): PCell can be on either direct path or indirect path

Option 2): PCell is always on direct path.

As current definition of PCell “Primary Cell: The MCG cell, operating on the primary frequency, in which the UE either performs the initial connection establishment procedure or initiates the connection re-establishment procedure.”, when the Remote UE initiates RRC connection establishment procedure on direct path or indirect path, it is natural the cell with which the UE is connected is PCell. Therefore, Option 1) is aligned with the current PCell definition.

Observation 1:The option that PCell can be on either direct path or indirect path is aligned with current PCell definition.

During the last RAN2 meeting discussion, some companies raised the comment that there could be issue which is there is only SCells on direct path without PCell if PCell is on indirect path. Definitely, there should be Special Cell configured on direct path to provide random access functions. If PCell is on indirect path, the Special Cell on direct path can be configured as PSCell on direct path and then SCG can be provided on direct path.

Some companies also commented it is unclear whether it is feasible that only SCG is configured to the Remote UE on direct path. According to current specification, it is already supported that MCG is optionally configured and in EN-DC, only SCG is configured in NR path. And we don’t see any issue if providing only SCG configuration to the Remote UE on direct path. Therefore, it should be feasible to provide only SCG configuration to the Remote UE.

Observation 2: It is already supported in current specification that only SCG configuration is provided to the UE on NR path, e.g. in case of EN-DC.

For option 2) i.e. the PCell can only be on direct path, it requires PCell and SCell role change each time the direct path is added or removed. Currently, PCell and SCell role change is achieved by intra-gNB HO with SCell removal and addition, that means the original SCell is removed and then configured to the PCell, or the original PCell is removed and configured as SCell. Along with the SCell removal and addition, the correspond configuration (e.g., radio bearer, RLC entity, MAC entity) on the indirect path has to be released and established again. And during PCell change, the security key has to be changed. All of  these will bring interruption time and data loss for ongoing services and increase additional signaling overhead and implementation complexity.

Observation 3: The option that PCell is always on direct path requires PCell and SCell role change at each time the direct path is added or removed, which brings interruption time and data loss for ongoing services and increase additional signaling overhead and implementation complexity.

Based on the above observations, the option that the PCell can be on either direct or indirect path has more benefits than the option that the PCell is always on direct path from ongoing service performance, signaling overhead, implementation complexity point of view.

Proposal 1: Stick to current PCell definition and PCell can be either on direct or indirect path.

Proposal 2: PCell and SCell role change is not required at each time direct/indirect path is added or removed.

In case that PCell is located on indirect path, PSCell should be configured on direct path as Special Cell to provide random access functions.

Proposal 3: If PCell is located on indirect path, the special cell on direct path is PSCell.

During the last RAN2 meeting, whether to introduce primary path was discussed but not agreed. Since the two paths may support different functions in control plane procedure based on which path the PCell is located , and in order to make our discussion convenient, it is beneficial to introduce a path terminology to distinguish the two paths, the path terminology could be e.g. primary path or anchor path. And the primary path or anchor path is the path PCell is located on.

Observation 4: It is beneficial to introduce a path terminology to make RAN2 discussion more convenient on distinguishing the different path functions in control plane procedure.

Please note that such path terminology does not mean it needs to be reflected in stage 3 specification.

Proposal 4: Introduce a path terminology (e.g. primary path or anchor path) to distinguish  the different path functions in control plane procedure during RAN2 discussion.

Proposal 5: The primary path or anchor path is the path PCell is located on.

Proposal 6: The path terminology used during RAN2 discussion does not has to be reflected in stage 3 specification.

##### 2.1.2 Basic control plane procedures

This section discusses basic control plane procedures as shown in the following Table-1.

Table-1: Control plane procudures

| Procedures | Current mechanism | Clarification or Enhancement |
| --- | --- | --- |
| RRC inactive state | For camped cell in Inactive state - PCell or indicated cell in RRCRelease message in response to an RRCResumeRequest or an RRCResumeRequest1 | Following current mechanism, no further clarification or enhancement. |
| SI acquisition in CONNECTED state | For PCell SI, - The SI can be transmitted to the UE using RRCReconfiguration or UE acquires the SIs which depends on whether an active BWP with common search space configured to the UE. For SCell or PSCell SI, - In DCCA, the SIs of SCells and PSCell are transmitted to the UE using RRCReconfiguration message. - In Rel-17, The Remote UE can but is not required to obtain SI over Uu interface. | For the cells on non-anchor path, since the Remote UE is not required to obtain SI over Uu interface, then it should reuse DCCA-like mechanism, i.e. the SIs of the cells on non-anchor path are transmitted to the UE using RRCReconfiguration message in case of SI(s) update or the SI(s) are not valid in the UE. |
| RLF recovery | - UE can report SCG/MCG failure over MCG/SCG path if MCG/SCG is not suspended; otherwise, initiate RRC Re-establishment procedure. | Similar with existing DC mechanism, Remote UE can report indirect path/direct path failure over direct path/indirect path if direct path/indirect path is not suspended. Otherwise, initiated RRC Re-establishment procedure. |
| RRC re-establishment | UE performs RRC re-establishment on PCell according to clause 5.3.7 in TS 38.331 | Following current specification that UE perform RRC re-eatablishment procedure on PCell according to clause 5.3.7 in TS 38.331 with MP relay context released. |
| SRB configuration | SRB1 and SRB2 or the primary RLC entity for split bearers are configured on MCG link which PCell is located on. | Following existing configuration that SRB1 and SRB2 are configured on the anchor path (if introduced) i.e., the path PCell is located on. |

Based on the analyses in the Table-1, it is proposed to clarify the control plane procedures.

Proposal 7: For MP relay, the control plane procedures are clarified as following.

- The SIs of the cells on non-anchor path are transmitted to the UE using RRCReconfiguration message in case of SIs update or the SI(s) is not valid in the UE.

- Remote UE can report indirect path/direct path failure over direct path/indirect path if direct path/indirect path is not suspended. Otherwise, initiated RRC Re-establishment procedure.

- UE performs RRC re-establishment procedure on PCell according to clause 5.3.7 in TS 38.331 with MP relay context released.

- In Inactive state, Remote UE camps on PCell or indicated cell in RRCRelease message in response to an RRCResumeRequest or an RRCResumeRequest1

Proposal 8: SRB1 and SRB2 are configured on the anchor path (if introduced) i.e. the path PCell is located on.

2.2 User plane issues for scenario 1

##### 2.2.1 MAC entity

For user plane, there is one open issue is how to configure MAC entity for MP relay. There are two potential options to be considered.

Option 1: Two separate MAC entities configured for direct path and indirect path.

This option borrows existing DC framework that there separate MAC entities configured for MCG and SCG. For MP relay, one Uu MAC entity is configured for direct path, and one SL MAC entity is configured for indirect path.

Option 2: One MAC entity is configured for both direct path and indirect path

This option is similar with existing Rel-16 mechanism that the SL MAC functions and Uu MAC functions share the same MAC entity.

Both options can work base do existing mechanism and whether one MAC entity or two entities are used can be left to UE implementation.

Proposal 9: Whether one MAC entity or two entities are used for MP Relay can be left to UE implementation.

2.3 Mobility-related procedure for scenario 1

##### 2.3.1 MP path addition

In the last RAN2 meeting, it was agreed to support both of following A and B.

A.	The remote UE operating only on the direct path adds the indirect path under the same gNB;

B.	The remote UE operating only on the indirect path adds the direct path under the same gNB;

Therefore, to support the agreed scenarios, some issues need to be discussed for MP path addition, e.g,

- How to ensure the direct path and the indirect path on the same gNB.

- Based on what information the gNB determines to configure MP relay.

- How to trigger RRC_IDLE/RRC_INACTIVE target relay UE to setup or resume RRC connection.

Figure 2: Indirect MP path addition-Relay UE in Connected state

Figure 3: Direct MP path addition

As show in the call flow, the current Event Y2 can be reused to trigger indirect path addition. When the Remote UE discovers a Relay UE and the radio quality on sidelink satisfies Event Y2, the Remote UE can send measurement to the gNB, including the candidate Relays and serving cells. The gNB will select the target Relay which serving cell is in the same gNB as Remote UE Uu cell. The gNB will configure the indirect path as secondary path using existing relay configuration mechanism.

Similarly, the existing Event A4 can be reused for direct path addition and reuse existing Uu reconfiguration procedure; and it should be gNB to select the target cell which is in the same gNB as the Relay UE’s serving cell.

Proposal 10: gNB can add the indirect/direct path based on the measurement report from the Remote UE triggered by existing event, e.g. Event Y2 for indirect path addition or Event A4 for direct path addition.

Proposal 11: It is up to gNB to select the target Relay UE whose serving cell is in the same gNB in case of indirect path addition; gNB selects the target cell which is in the same gNB as the Relay UE’s serving cell.

As existing service continuity for intra-gNB, the target Relay can be in any RRC state as agreed in the last RAN2 meeting.

Proposal 10	[21/21] For multi-path Relay, support RRC_IDLE/RRC_INACTIVE target relay UE, for the path switching scenario where there is an addition of indirect path or a change of indirect path.

For this scenario, one issue is how to trigger the IDLE/Inactive state Relay UE to setup/resume RRC connection. In existing path switching to indirect path, it relies on SRB1 (i.e. RRCReconfigurationComplete) message to trigger Relay UE to setup/resume RRC connection. However, in MP relay case, when the Remote UE has already established direct path and SRB1 is configured on direct path, RRCReconfigurationComplete message cannot be sent over the target indirect path. This issue was discussed in the last meeting by email discussion but no conclusion. There are two options to address this issue.

Option 1: Configure split SRB1 to Remote UE and allow Remote UE to send SRB1 over indirect path when adding indirect path. SL-RLC1 message can trigger Relay UE to setup/resume RRC connection

Option 2: Using PC5 message to trigger Relay UE to setup/resume RRC connection

Option 1 require SRB1 is always configured as split bearer; and option 2 relies on SA2 discussion on PC5 message.

Since this is study phase, it is proposed to postpone the issue to be discussed in normative work.

Proposal 12: Use PC5 message to trigger the IDLE/Inactive target Relay UE to setup/resume RRC connection.

##### 2.3.2 MP path removal

Figure 4: Indirect path removal

Similar with MP path addition, the path removal can be based on the measurement report from the Remote UE and the Relay UE as shown in Figure 4. The existing Event X2 (for indirect path removal) and Event A2 (for direct path removal) evaluated by Remote UE and be reused to indicate the current sidelink on the indirect path or the Uu link on the direct is getting worse, and the gNB can determine to remove the indirect path or the direct path. Similarly, the indirect path can be removed based on the measurement report from Relay UE triggered by existing events, e.g. Event A2 in case that all of the serving cells are getting worse.

Proposal 13: gNB can remove the indirect or direct path based on the measurement report from the Remote UE triggered by existing events, e.g. Event X2 for indirect path removal or Event A2 for direct path removal.

Proposal 14: gNB can remove the indirect path based on the measurement report from the Relay UE triggered by existing events, e.g Event A2 in case that all of the serving cells are getting worse.

##### 2.3.3 MP path change

Case 1: Indirect path change

Indirect path change is targeting to the scenario that the serving Relay UE changes but the Remote UE’s PCell on direct path is not changed. RAN2 discussed this scenario in last RAN2 meeting and achieve the following agreement, but it still FFS if it can be supported via separate release+add or a single path switch procedure.

G.	The remote UE operating in multi-path changes to a new relay UE for the indirect path while keeping the direct path under the same gNB.  FFS if this case would be supported via separate release-and-add (A+C in separate reconfigurations) or a single switch procedure (e.g. similar to i2i service continuity).

The indirect-to-indirect path service continuity is being discussed, and the indirect path change procedure for MP realy should be similar with indirect-to-indirect path switching. The differenct issue between the two scenarios is how to trigger the target IDLE/Inactive Relay UE to setup/resume RRC connection, which is same as the issue in indirect path addition procedure.

Proposal 15: Service continuity on indirect path to indirect path for intra-gNB can be used as baseline for indirect path changes during MP relay operation with the same issue addressed in indirect path addition on how to trigger IDLE/Inactive Relay UE connection setup or resume.

Case 2: Direct path change

Indirect path change is targeting to the scenario that Remote UE’s Special Cell changes on direct path but the Remote UE’s serving Relay UE and corresponding PCell is not changed, and RAN2 agreed to support this scenario. Still it is FFS if a single procedure would be supported.

E.	The remote UE operating in multi-path changes the direct path to a different cell of the same gNB while using the serving relay UE for the indirect path under the same gNB.

FFS if a single procedure for this case would be supported.

The existing PCell or PSCell change within the same gNB can be supported using a single procedure, i.e. intra-gNB handover with existing SCell removal and new SCell addition in one RRCReconfiguration message.

For the direct path change procedure in MP relay operation, it should be similar with the PCell or PSCell change procedure and can be achieved by intra-gNB handover with SCell removal and addition in one RRCReonfiguration message.

Proposal 16: Exiting PCell/PSCell change procedure on direct path for intra-gNB can be reused as baseline for direct path change in MP relay operation, i.e. achieved by intra-gNB handover with SCell removal and addition in one RRCReconfiguration message.

Case 3: Both of direct path and indirect path change

Direct path and indirect path change at the very same time is rare case, and it should be deprioritized.

Proposal 17: Deprioritize direct path and indirect path change simultaneously.

2.4 Discussion for Scenario 2

##### 2.4.1 Supported mobility cases

In the last RAN2 meeting, the supported mobility scenarios were discussed and the following scenarios are still FFS.

Proposal 1-2C: Whether to support the following case can be further discussed for Scenario 2.

B.	The remote UE configured only on the indirect path adds the direct path under the same gNB;

D.	The remote UE configured with multi-path releases the direct path;

As explained in the WID, multi-path relay solution can also be utilized for UE aggregation where a UE (called Remote UE) is connected to the network via direct path and via another UE (called Relay UE) using a non-standardized UE-UE interconnection. Such multi-path operation aims to provide two benefits: one is to improve bitrates in case that normal UEs transmission power is limited; another is to improve the reliability, stability and reduce delay of services in case that the Uu radio condition of the Remote UE is deteriorating, the Relay UE can be used to make up for the traffic performance unsteadiness caused by channel condition variation. This could happen in many use cases, e.g. in factory, the two IoT devices are connected using wireline and can be aggregated to offer reliable service performance for the Remote device.

From the above-mentioned motivations, it can be seem it is beneficial to allow Remote UE to access to gNB via ideal connection indirect path only, and when the Remote UE is out of coverage or the Uu radio condition is deteriorating, the ideal connection towards Relay UE can be used to guarantee service performance.

Observation 5: It is beneficial to allow Remote UE accessing to gNB via ideal connection indirect path only that the Remote UE is out of coverage or the Uu radio condition is deteriorating, the ideal connection towards Relay UE can be used to guarantee service performance.

For the ideal connection between the two UEs, it is assumed a non-standardized UE-UE interconnection is applied. During last RAN2 discussion, it is assumed everything on the ideal link can be addressed by UE implementation, then no SRAP layer is needed on ideal link.

Similar to SARP function discussion on UE-UE ideal link, it can also assumed the procedure of relay discovery, relay (re-)selection, Paging and SI forwarding, the transmission of SRB0 and SRB1 transmission during RRC establishment or resume on ideal link should be left to UE implementation. And this does not require additional enhancements in Rel-18.

Observation 6: It is feasible that Remote UE access to gNB via ideal connection indirect path only by reusing existing defined L2 based U2N relay mechanism with the follow restrictions: the procedure of relay discovery, relay (re-)selection, Paging and SI forwarding, SRB0 and SRB1 transmission during RRC establishment or resume on ideal connection should be left to UE implementation.

Based on observation 7 and 8, the cases B, D can be supported.

Proposal 18: For Scenario 2, the following cases B, D should be supported.

B.	The remote UE configured only on the indirect path adds the direct path under the same gNB;

D.	The remote UE configured with multi-path releases the direct path;

##### 2.4.2 Multi-path operation for Scenario 2

For PCell location

Similar with Scenario 1, if cases B, D are supported for Scenario 2, PCell can be on either direct path or indirect path.

Proposal 19: if cases B, D are supported for Scenario 2, PCell can be on either direct path or indirect path.

Bearer management

In the last RAN2 meeting, if is FFS whether there are restrictions on the configuration and whether SRB1 and SRB2 can be configured on both paths.

For scenario 2, SRB1 and SRB2 can be configured at least on the direct path.  FFS if there are restrictions on the configuration and if they can be configured on both paths.

Similar with Scenario 1, if PCell is on indirect path, then SRB1 and SRB2 should be on indirect path. In the last RAN2 meeting, it was agreed the Remote UE bearer and Uu logical channel is 1:1 mapped, then there is no confusion on how the E2E bearers to be transmitted over indirect path in case no SRAP configured. Then it should be possible to configure SRB1 and SRB2 on both paths.

Proposal 20 : For scenario 2, SRB1 and SRB2 can be configured either the direct path or indirect path or both path.

For path addition/release

The proposals for scenario 1 can be taken as baseline for scenario 2. The only difference for scenario 2 is the measurement and candidate Relay discovery on ideal connection should not be specified and can be left to UE implementation. And currently how and when to trigger measurement report is well specified. But for scenario 2, we cannot specify the trigger for measurement report. Then it is better to reuse SUI message to informs the gNB the candidate Relay UE information.

Proposal 21: gNB can add or release the indirect path based on the candidate Relay UE information received from the Remote on e.g. SidelinkUEInformationNR. And it is left to Remote UE implementation to determine the candidate Relay UE.

For path change

In the last RAN2 meeting, the following path change cases were discussed but agreed. The proposals for scenario 1 can be taken as baseline for scenario 2. The only difference for scenario 2 is the measurement and candidate Relay discovery on ideal connection should not be specified and can be left to UE implementation. Then the path change proposals for scenario 1 can be taken as baseline for scenario 2.

E.	The remote UE configured with multi-path changes the serving cell of the remote UE for the direct path while keeping the serving relay UE for the indirect path under the same gNB;

G.	The remote UE configured with multi-path changes to a new relay UE for the indirect path while keeping the direct path under the same gNB.

Proposal 22: The following cases E and G can be supported for Scenario 2, and the mechanism defined for Scenario 1 can be taken baseline. For indirect path change, the service continuity on indirect path to indirect path for intra-gNB can be used as baseline with Remote UE including candidate Relay UE in  SidelinkUEInformationNR. And it is up to Remote UE implementation to determine the candidate Relay UE.

E.	The remote UE configured with multi-path changes the serving cell of the remote UE for the direct path while keeping the serving relay UE for the indirect path under the same gNB;

G.	The remote UE configured with multi-path changes to a new relay UE for the indirect path while keeping the direct path under the same gNB.

# Conclusion

This contribution discusses the aspects of architecture, bearer management, path management for multi-path operation, and provides the proposals.

For the path of PCell,

Observation 1: The option that PCell can be on either direct path or indirect path is aligned with current PCell definition.

Observation 2: It is already supported in current specification that only SCG configuration is provided to the UE on NR path, e.g. in case of EN-DC.

Observation 3: The option that PCell is always on direct path requires PCell and SCell role change at each time the direct path is added or removed, which brings interruption time and data loss for ongoing services and increase additional signaling overhead and implementation complexity.

Proposal 1: Stick to current PCell definition and PCell can be either on direct or indirect path.

Proposal 2: PCell and SCell role change is not required at each time direct/indirect path is added or removed.

Proposal 3: If PCell is located on indirect path, the special cell on direct path is PSCell.

Observation 4: It is beneficial to introduce a path terminology to make RAN2 discussion more convenient on distinguishing the different path functions in control plane procedure.

Proposal 4: Introduce a path terminology (e.g. primary path or anchor path) to distinguish  the different path functions in control plane procedure during RAN2 discussion.

Proposal 5: The primary path or anchor path is the path PCell is located on.

Proposal 6: The path terminology used during RAN2 discussion does not has to be reflected in stage 3 specification.

For basic control plane procedures,

Proposal 7: For MP relay, the control plane procedures are clarified as following.

- The SIs of the cells on non-anchor path are transmitted to the UE using RRCReconfiguration message.

- Remote UE can report indirect path/direct path failure over direct path/indirect path if direct path/indirect path is not suspended. Otherwise, initiate RRC re-establishment procedure.

- UE performs RRC re-establishment procedure on PCell according to clause 5.3.7 in TS 38.331 with MP relay context released.

- In Inactive state, Remote UE camps on PCell or indicated cell in RRCRelease message in response to an RRCResumeRequest or an RRCResumeRequest1.

Proposal 8: SRB1 and SRB2 are configured on the anchor path (if introduced) i.e. the path PCell is located on.

For user plane issues,

Proposal 9: Whether one MAC entity or two entities are used for MP Relay can be left to UE implementation

For mobility-related procedure,

Proposal 10: gNB can add the indirect/direct path based on the measurement report from the Remote UE triggered by existing event, e.g. Event Y2 for indirect path addition or Event A4 for direct path addition.

Proposal 11: It is up to gNB to select the target Relay UE whose serving cell is in the same gNB in case of indirect path addition; gNB selects the target cell which is in the same gNB as the Relay UE’s serving cell in case of direct path addition.

Proposal 12: Use PC5 message to trigger the IDLE/Inactive target Relay UE to setup/resume RRC connection.

Proposal 13: gNB can remove the indirect or direct path based on the measurement report from the Remote UE triggered by existing events, e.g. Event X2 for indirect path removal or Event A2 for direct path removal.

Proposal 14: gNB can remove the indirect path based on the measurement report from the Relay UE triggered by existing events, e.g Event A2, in case that all of the serving cells are getting worse.

Proposal 15: Service continuity on indirect path to indirect path for intra-gNB can be reused as baseline for indirect path changes during MP relay operation with the same issue addressed in indirect path addition on how to trigger IDLE/Inactive Relay UE connection setup or resume.

Proposal 16: Exiting PCell/PSCell change procedure on direct path for intra-gNB can be reused as baseline for direct path change in MP relay operation, i.e. achieved by intra-gNB handover with Scell removal and addition in one RRCReconfiguration message.

Proposal 17: Deprioritize direct path and indirect path change simultaneously.

For Direct + Ideal connection path (Scenario 2),

Observation 5: It is beneficial to allow Remote UE accessing to gNB via ideal connection indirect path only that when the Remote UE is out of coverage or the Uu radio condition is deteriorating, the ideal connection towards Relay UE can be used to guarantee service performance.

Observation 6: It is feasible that Remote UE access to gNB via ideal connection indirect path only by reusing existing defined L2 based U2N relay mechanism with the follow restrictions: the procedure of relay discovery, relay (re-)selection, Paging and SI forwarding, SRB0 and SRB1 transmission during RRC establishment or resume on ideal connection should be left to UE implementation.

Proposal 18: For Scenario 2, the following cases B, D should be supported.

B.	The remote UE configured only on the indirect path adds the direct path under the same gNB;

D.	The remote UE configured with multi-path releases the direct path;

Proposal 19: if cases B, D are supported for Scenario 2, PCell can be on either direct path or indirect path.

Proposal 20: For scenario 2, SRB1 and SRB2 can be configured either the direct path or indirect path or both path.

Proposal 21: gNB can add or release the indirect path based on the candidate Relay UE information received from the Remote on e.g. SidelinkUEInformationNR. And it is left to Remote UE implementation to determine the candidate Relay UE.

Proposal 22: The following cases E and G can be supported for Scenario 2, and the mechanism defined for Scenario 1 can be taken baseline. For indirect path change, the service continuity on indirect path to indirect path for intra-gNB can be used as baseline with Remote UE including candidate Relay UE in SidelinkUEInformationNR. And it is up to Remote UE implementation to determine the candidate Relay UE.

E.	The remote UE configured with multi-path changes the serving cell of the remote UE for the direct path while keeping the serving relay UE for the indirect path under the same gNB;

G.	The remote UE configured with multi-path changes to a new relay UE for the indirect path while keeping the direct path under the same gNB.

# Reference

[1]    RP-221262  New WID on NR sidelink relay enhancements

[2]    R2-2211101 RAN2#119bis-e Meeting Report	MCC