3GPP TSG-RAN WG2 Meeting #120                                                        R2-2211800

Toulouse, 14th - 18th November, 2022

Source:	Qualcomm Incorporated

Title:	QoE collection for IDLE and Inactive state

Agenda Item:	8.14

Document for:	Discussion and Decision

# Introduction & Background

According to the agenda and the WID [1], QoE configuration and collection in Inactive state and IDLE state will be discussed as follow for MBS service.

- Specify for QoE measurement configuration and collection in RRC_INACTIVE and RRC_IDLE states for MBS, at least for broadcast service [RAN3, RAN2].

- Specify the mechanism to support the alignment of the existing radio related measurement and QoE reporting.

This contribution discusses the QoE configuration and measurement collection for IDLE and Inactive state.

# Discussion

## 2.1 QoE configuration

QoE configuration to IDLE and Inactive state UE

For QoE configuration for IDLE and Inactive state QoE collection, RAN2 has the following agreements as following.

2: The gNB can send the QoE configuration for MBS broadcast service to UE by RRC message in RRC_CONNECTED via dedicated signalling. The UE stores the configuration for QoE and performs the application layer measurement for MBS broadcast service.

FFS if configuration can be done in IDLE/INACTIVE states.

FFS how does gNB determine which UEs can be configured with MBS QoE measurements

For QoE configuration, if UEs in IDLE state are required to collect QoE measurements, QoE configuration needs to be provided to IDLE state UEs. As today, when the network want to update configuration to the UE, the network can page the UE to enter CONNECTED state, this can also be used for QoE configuration provision and update.

Another issue is that QMC deactivation (i.e. QoE configuration release in the UE) needs to be supported for IDLE/Inactive state UEs which have been configured QoE collection.

Proposal 1: RAN2 discusses whether IDLE and Inactive state UEs can be paged to enter CONNECTED state to receive QoE configuration, including adding new QoE configuration and release existing QoE configuration.

If the UE is already in CONNECTED state, the UE can send MBSInterestIndication message to the gNB, and the gNB determines whether the UE is interested in the broadcast service and which broadcast service and then determine whether the QoE configuration needs to be provided to the UE.

Proposal 2: gNB can determine whether to send QoE configuration to the UE based on MBSInterestIndication message.

QoE configuration validity

AS SA5 defined, for each QoE configuration, there could be area scope configured, so the network can provide area scope to the UE for the QoE configuration. There are two alternatives:

Alt1: The area scope is configured to application layer in configuraiton container. And the application layer checks whether the the UE moves outside of the area scope. This is existing mechanism defined in application layer.

Alt2: The area scope is configured to UE AS layer and AS layer checks whether to move outside of area scope. This is new behavior for AS layer.

Alt 1 is existing mechanism in application layer, but it requires application layer checks area scope for each QoE configuration and when the UE moves outside of area scope, application layer notifies to AS layer to release QoE configuration. Alt 2 is new AS layer behavior, and there could be multiple area scopes configured for different QoE configuraiton, and AS layer needs to check each area scope for each QoE configuration, which will bring additional AS layer complexicity.

Proposal 3: An area scope can be provided to UE for the dedicated QoE configuration, and UE considers the QoE configuration is valid within the area scope.

Proposal 4: RAN2 discusses which layer (AS layer or application layer) checks the area scope.

Another issue is how to release QoE configuraiton for IDLE and Inactive state UE, especially for m-based QoE in which case the gNB does not know which IDLE UEs are configured with m-based QoE, and the UE cannot be paged into CONNECTED state to be released QoE configuration. One possible way is to introduce implicit QoE release, e.g. timer-based QoE configuration release.

Observation 1: For m-based QoE, the gNB cannot release QoE configuration explicitly to the IDLE state UEs.

Proposal 5: Introduce timer-based QoE configuration release, at least for IDLE state UEs configured with m-based QoE.

RRC message for QoE configuration

As today, the configuration can be provided to the UE using RRCReconfiguraiton message or RRCRelease message. RAN3 already agreed common QoE config for all RRC states: “If the UE receives QoE configuration in RRC_CONNECTED, a common QoE configuration mechanism is used to support QoE measurement configuration pertaining to MBS broadcast for all RRC states, where the Rel-17 QoE configuration mechanism is adopted as baseline.” The it should use RRCReconfifuration message to provide QoE configuration to the UE, and the UE can use the QoE configuraiton in any RRC state. It can be put FFS whether to use RRCRelease message to provide QoE configurations which are only used for IDLE and Inactive state.

Proposal 6: Use RRCReconfiguration message to provide QoE configuration to UE. It is FFS whether to use RRCRelease message to provide QoE configurations which are only used for IDLE and Inactive state.

MBS broadcast QoE configuration content

In Rel-17, each QoE configuration contains service type, RRC ID(measConfigAppLayerId), QoE configuration container, and the QoE configuration defined in Rel-17 can still be applied to MBS service QoE in Connected and Inactive state. The RRC ID is allocated the gNB can included in UE context. However, when the UE is released to IDLE state, the RRC ID is invalid any more. So for QoE configuration applied in IDLE state, the RRC ID(measConfigAppLayerId) should not be allocated and provided to the UE. The usage of the RRC ID is to replace QoE reference for signalling overhead saving purpose and based on the RRC ID the gNB can derive the MCE address for the received QoE report. Hence, QoE configuration container, service type should be included in QoE configuration RRC message. Then QoE reference should be included for the QoE configuration to be used in IDLE state. Network can use this QoE reference to release the QoE configuration.

Observation 2: When the UE is released to IDLE state, the RRC ID introduced in Rel-17 is invalid any more.

Proposal 7: The QoE configuration contains service type, QoE configuration container, QoE reference. It is FFS for other information.

Another issue is whether QoE configuration may be different for different broadcase services, this may impact configuration and report content, e.g. whether MBS session IDs are needed to . It is proposed to ask SA5 whether QoE configurations may be different for different broadcast services.

Proposal 8: Ask SA5 whether QoE configurations may be different for different broadcast services.

## 2.2 QoE collection in IDLE and Inactive state

When the UE is configured QoE measurements for MBS broadcast service, UE AS layer forwards the QoE configuration to the application layer, and application layer performs QoE measurement. For the collected QoE measurements, it is should be buffered in the UE, and when the UE enters Connected state, UE can send to the gNB.

In Rel-17, when QoE reporting is paused, UE will buffer the data in AS layer with 64KB buffer size. For IDLE and Inactive state, the AS layer 64KB buffer can be reused. And if the AS layer buffer size is full, there are two ways to handle the QoE data:

Alt 1: AS layer should discard the QoE data

Alt 2: The QoE data should be buffered in application layer.

Proposal 9: The QoE measurements collected in IDLE and Inactive state can be buffered in AS layer with reusing the 64KB buffer size defined for CONNECTED state in Rel-17.

Proposal 10: If the AS layer buffer (64KB) is full, RAN2 discusses the following alternatives:

Alt 1: The AS layer should discard the QoE data.

Alt 2: The QoE data should be buffered in application layer.

## 2.3 QoE report of data collected in IDLE and Inactive state

- In the last RAN2 meeting, how the UE reports the QoE data was discussed and leave the following FFS.

- 3)	FFS if UE can setup/resume RRC connection just for QoE reporting, or whether the QoE reports are sent to the network when the UE moves to RRC CONNECTED state due to other reasons.

QoE data is not real-time traffic and it used for big data statistic. Then the QoE data reporting should not trigger RRC connection establishment or resume.

Proposal 11: QoE data reporting should not trigger RRC connection establishment or resume.

In Rel-17, SRB4 was introduced for QoE data reporting and UE include QoE data in the MeasurementReportAppLayer message. For IDLE and Inactive state, the same bearer and RRC message defined in Rel-17 can be reused for IDLE and Inactive state QoE data reporting.

Proposal 12: Reuse existing MeasurementReportAppLayer and SRB4 to transmit QoE data collected in IDLE and Inactive state.

When UE initiates RRC connection establishment or resume procedure, the UE can indicate to gNB whether there is QoE data buffered, based on this indication, the gNB can provide SRB4 configuration to the UE for QoE reporting.

When the UE reports QoE data, the new gNB may not be provided the QoE configuration assisted information, e.g. MCE address, then UE should include MCE information for the QoE data to the gNB, and the gNB can forward the QoE data to the appropriate MCE based on the MCE information. MCE address can be indicated to gNB in case that the gNB has no QoE configuration.

Proposal 13: UE AS layer indicates MCE information (e.g. MCE ID) for each reported QoE data container to the gNB, and gNB forwards the QoE data to the appropriate MCE based on the MCE information.

And it is assumed application layer has the MCE information, so UE application layer provides MCE information to AS layer together with the QoE report container. But, this needs to be confirmed by SA4.

If application layer cannot provide MCE information to the AS layer, then the gNB should indicate MCE information to the UE in the QoE configutation.

Proposal 14: If application layer cannot provide MCE information (e.g. MCE ID)  to AS layer, gNB should configure MCE information to UE in the QoE configuration.

2.4  RVQoE collection in IDLE and Inactive state

From the WID objective, it is unclear whether RVQoE measurement collection needs to be done in IDLE and Inactive state. It should be beneficial to collect RVQoE measurement for MBS broadcast service, then gNB can do real-time radio configuration optimization. Then RAN2 is proposed to clarify whether RVQoE measurement collection is needed in IDLE and Inactive state.

Proposal 15: It is proposed to clarify whether RVQoE measurement collection is needed in IDLE and Inactive state.

# Conclusion

This contribution discusses the MBS broadcast service QoE configuration and report for IDLE and Inactive state.

For QoE configuration,

Proposal 1: RAN2 discusses whether IDLE and Inactive state UEs can be paged to enter CONNECTED state to receive QoE configuration, including adding new QoE configuration and release existing QoE configuration.

Proposal 2: gNB can determine whether to send QoE configuration to the CONNECTED UEs based on MBSInterestIndication message.

Proposal 3: An area scope can be provided to UE for the dedicated QoE configuration, and UE considers the QoE configuration is valid within the area scope.

Proposal 4: RAN2 discusses which layer (AS layer or application layer) checks the area scope.

Observation 1: For m-based QoE, the gNB cannot release QoE configuration explicitly to the IDLE state UEs.

Proposal 5: Introduce timer-based QoE configuration release, at least for IDLE state UEs configured with m-based QoE.

Proposal 6: Use RRCReconfiguration message to provide QoE configuration to UE. It is FFS whether to use RRCRelease message to provide QoE configurations which are only used for IDLE and Inactive state.

Observation 2: When the UE is released to IDLE state, the RRC ID introduced in Rel-17 is invalid any more.

Proposal 7: The QoE configuration contains service type, QoE configuration container, QoE reference. It is FFS for other information.

Proposal 8: Ask SA5 whether QoE configurations may be different for different broadcast services.

QoE collection and reporting

Proposal 9: The QoE measurements collected in IDLE and Inactive state can be buffered in AS layer with reusing the 64KB buffer size defined for CONNECTED state in Rel-17.

Proposal 10: If the AS layer buffer (64KB) is full, RAN2 discusses the following alternatives:

Alt 1: The AS layer should discard the QoE data.

Alt 2: The QoE data should be buffered in application layer.

Proposal 11: QoE data reporting should not trigger RRC connection establishment or resume.

Proposal 12: Reuse existing MeasurementReportAppLayer and SRB4 to transmit QoE data collected in IDLE and Inactive state.

Proposal 13: UE AS layer indicates MCE information (e.g. MCE ID) for each reported QoE container to the gNB, and gNB forwards the QoE data to the appropriate MCE based on the MCE information.

Proposal 14: If application layer cannot provide MCE information (e.g. MCE address or MCE ID)  to AS layer, gNB should configure MCE information to UE in the QoE configuration.

RVQoE collection in IDLE and Inactive state,

Proposal 15: It is proposed to clarify whether RVQoE measurement collection is needed in IDLE and Inactive state.

# Reference

[1]    RP-221803: Enhancement on NR QoE management and optimizations for diverse services