3GPP TSG-RAN WG2 Meeting #120		R2-2211803

Toulouse, France, 14th – 18th Nov. 2022

Source:	vivo

Title:	Discussion on NCR Management

Agenda Item:	8.1.3

Document for:	Discussion and Decision

# Introduction

In the NCR TR[1], 4 solutions regarding NCR identification and authorization were identified. LS regarding Solution 1 and Solution 2 was sent to SA3 to inquire the related potential security issue and the reply LS [2] was received in response. The main content of the reply LS[2] is copied below for reference:

SA3 would like to thank the RAN3 for their LS on NCR Solutions. SA3 has further discussion on the Questions from RAN3 and would like to share the views as below: To SA3 Q1a: Is there any security issue for solution 2 which does not provide Uu security, non-protected NCR indication info and the OAM container in Step 5? Answer to RAN3: Yes. For solution 2, SA3 believes that this information can be tampered due to the lack of Uu security. It exposes the OAM indirectly to attacks over the air interface. To SA3 Q1b: Does SA3 believe that the NCR needs to be securely validated? Any security issue for configuring locally stored information in the gNB in Solution 1? Answer to RAN3: For the 1st question in Q1b, SA3 is not clear about what does "validation" mean. For the 2nd question in Q1b, SA3 cannot provide answers before the security validation related steps in solution1 are clarified. In addition, the feasibility of such additional steps and what kind of information is stored in RAN are also unclear. Further clarification is expected.

In this paper, the 4 solutions are discussed with the input from SA3 considered. In addition, the NCR access procedure which can facilitate the quick NCR identification by gNB is also investigated.

# Discussion

  - Authorization and authentication procedure

According [1], for authorization and authentication, there are 4 different solutions and different solutions mean different implementation/standardization efforts. These solutions are listed below, briefly:

- Solution 1: The NCR-MT is authorized in the same way as a normal UE and the related NCR forward function is authorized by the gNB.

- Solution 2: OAM based solution. This is out of scope of RAN2 discussion. This solution requires no new NG signaling or CN update.

Solution 3: Use authorization/authentication procedure similarly as IAB node; This option requires both new NG signaling and CN update.

- Solution 4: The gNB initiates the authorization message as for the normal UE while the CN responses with the NCR authorization indication to gNB. This solution requires both new NG signaling and CN update.

For Solution 2, SA3 believes that the network can be tempered due to lack of Uu security. It may expose the OAM information to attacks in air interface as the non-protected NCR indication and OAM container are transmitted in Uu interface.

Therefore,

- RAN2 should down-prioritize Solution 2.

For solution 1, according the LS, there is no concrete answer on the security issue yet because SA3 is not clear regarding the word “validation” and “security validation” related steps. SA3 may need to further evaluate the security issue once RAN2/RAN3 provide some clarification to SA3 regarding Solution 2. But, in the meantime RAN2 may have to continue working on NCR Solution 3 and Solution 4, and RAN2 should not further down-select from Solution 1,3 and 4 before receiving conclusion regarding the security issue of Solution 1 from SA3. Regarding the clarification on the word “validation”, from RAN2 perspective we understand that word “validation” is widely referring to “authorization/authentication”. Thus, RAN2 can reply back to SA3 on the clarification of the word “validation”.

Therefore,

- RAN2 to clarify to SA3 that “validation” means “authorization” in Solution 1.

- If P2 is agreed, RAN2 to wait for SA3 feedback on Solution 1 before down-selecting between Solution 1, 3, and 4.

For both Solution 3 and Solution 4, the authorization procedure is performed by the CN. The difference between Solution 3 and Solution 4 are the signaling procedure design. For Solution 3, the gNB can know the NCR role of the device during the radio connection setup procedure of NCR-MT, which gives the opportunity for the gNB to select the corresponding AMF server for NCR authorization; while for Solution 4, the gNB does not know the NCR role of the device until the AMF feedback the authorization results of the NCR-MT, which requires that the gNB to select an AMF server supporting NCR authorization without any prior information or that all candidate AMF servers that the gNB could select should support NCR authorization.

In such sense, Solution 3 can be easier for NCR deployment since the gNB can select the proper AMF for NCR authorization based on the NCR role identification during RACH procedure of NCR-MT. For Solution 4, some more enhancement is required so that the network can identify the proper AMF for NCR identification and authorization, or the all candidate AMF can support NCR identification and authorization.

Considering this, we propose:

- RAN2 to further discuss the RAN2 related aspects of Solution 3 and 4.

  - NCR radio connection setup procedure

NCR identification in RACH procedure by gNB can help the gNB identify the proper AMF for NCR authorization. In the following, the RACH procedure to enable NCR identification in RACH procedure is discussed.

Between the NCR node and the gNB, there is a control link between gNB and the NCR-MT, and a BH link between gNB and the NCR-Fwd unit, wherein the control link is used to convey the side control information from gNB to the NCR-MT to control the forwarding behavior of the NCR-Fwd unit while the BH link is used for wireless signal relaying to/from the NCR-fwd from/to the gNB. Based on this model, the gNB should be equipped with associated function to support the NCR operation and the NCR node should select the gNB with the associated function to support the NCR node operation. Similar to IAB support [3], the gNB may broadcast an NCR Support indicator indicating the NCR support in SIB1, so that an NCR node can find a gNB that supports the NCR node access.

- The gNB supporting NCR access can indicate NCR support in the system message.

- An NCR node should select the gNB broadcasting NCR support for control link setup.

As a reference, when IAB-MT access the donor gNB, it indicates iab-NodeIndication in RRCSetupComplete to the IAB-donor-CU, so that the IAB-donor-CU can know the access is initiated by an IAB-MT and provide further configurations for IAB operation.

- IAB-MT indicates iab-NodeIndication in RRCSetupComplete during access to the donor.

Similarly, to facilitate the gNB to identify the connection is initiated by an NCR node instead of a normal UE, the NCR-MT should indicate the role of NCR-MT to the gNB. After the NCR node has selected the gNB, it can perform random access similarly as a normal UE. For the NCR-MT, the similar indication as for IAB-MT can be used. As an alternative, certain kind of NCR configuration request, which requests the gNB to provide NCR related configurations after access procedure, can also be considered.

- RAN2 to consider the following two options to indicate the NCR node to the gNB:

- -	Option A. NCR indication to gNB during RACH procedure;

- -	Option B. Certain NCR configuration request after RACH procedure.

# Conclusion

In this contribution, we discuss the upper layer procedures. Based on the discussion, we have the following observations:

Observation 1	IAB-MT indicates iab-NodeIndication in RRCSetupComplete during access to the donor.

Based on the discussions and observations, we have the following proposals:

- RAN2 should down-prioritize Solution 2.

- RAN2 to clarify to SA3 that “validation” means “authorization” in Solution 1.

- If P2 is agreed, RAN2 to wait for SA3 feedback on Solution 1 before down-selecting between Solution 1, 3, and 4.

- RAN2 to further discuss the RAN2 related aspects of Solution 3 and 4.

- The gNB supporting NCR access can indicate NCR support in the system message.

- An NCR node should select the gNB broadcasting NCR support for control link setup.

- RAN2 to consider the following two options to indicate the NCR node to the gNB:

-	Option A. NCR indication to gNB during RACH procedure;

-	Option B. Certain NCR configuration request after RACH procedure.

# References

- 3GPP TR 38.867, Study on NR network-controlled repeaters, v1.0

- R2-2211173_S3-223080 Reply LS on NCR Solutions