3GPP TSG-RAN WG2 Meeting #120	R2-2211809

Toulouse, France, November 14-18, 2022

Agenda Item:	6.15.3

Source: 	ASUSTeK, vivo

Title:  	Discussion on priority setting for IUC MAC CEs

Document for:	Discussion and Decision

- Introduction

In RAN2#119 meeting, some agreements were made regarding setting different priority values for LCP and for sensing/resource selection for IUC MAC CEs, where the priority for sensing/resource selections are configurable by RRC parameters:

Proposal 1: The following parameters are dummified in TS 38.331: - sl-PriorityCoordInfoExplicit-r17 - sl-PriorityCoordInfoCondition-r17 - sl-PriorityRequest-r17 - Keep those parameters to use them in sensing and candidate resource selections in PHY and use the fixed value “1” for IUC and IUC REQ MAC CE in MAC LCP.

In this contribution, we discuss further details on handling priority values for IUC MAC CEs when performing sensing and resource (re)selection and determining Sidelink transmission information.

- Discussion

How to determine the priority values for IUC request/information MAC CEs when the parameters are not configured?

While the agreement states that the priority values for the IUC MAC CEs are configurable via the RRC parameters when used for sensing and resource selections, the RRC parameters are actually optional:

SL-InterUE-CoordinationScheme1-r17 ::= SEQUENCE { … sl-PriorityCoordInfoExplicit-r17 INTEGER (1..8) OPTIONAL, -- Need M sl-PriorityCoordInfoCondition-r17 INTEGER (1..8) OPTIONAL, -- Need M sl-PriorityRequest-r17 INTEGER (1..8) OPTIONAL, -- Need M ... }

During email discussion [502] in previous meeting [1], there was a discussion regarding whether to add a note clarifying how to set the priority in Sidelink transmission information for IUC information and request MAC CE. While there was a slight majority of companies agreeing on the behaviour setting priority values based on configurable parameters, how to set the priority values for Sidelink transmission information of the IUC MAC CEs when the parameters are not configured has not been clarified. According to RAN1 agreements in RAN1#107bis-e:

Agreement - For inter-UE coordination information triggered by an explicit request in Scheme 1, the priority value of the inter-UE coordination information is (pre)configured priority value if it is provided by (pre)configuration. Otherwise, the priority value is the same as indicated by UE-B’s explicit request. - For the case when inter-UE coordination information is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the inter-UE coordination information and data Agreement - For inter-UE coordination information triggered by an explicit request in Scheme 1, the priority value of explicit request is (pre)configured priority value if it is provided by (pre)configuration. Otherwise, the priority value is the same as that of a TB to be transmitted by UE-B. - For the case when the explicit request is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the explicit request and data Agreement - For inter-UE coordination information triggered by a condition other than explicit request reception in Scheme 1, the priority value of the inter-UE coordination information is (pre)configured priority value if it is provided by (pre)configuration. - FFS: Otherwise, the priority value is determined by UE-A’s implementation. - For the case when inter-UE coordination information is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the inter-UE coordination information and data

Based on the RAN1 agreement above, we can discuss the priority setting for each MAC CEs when corresponding RRC parameters are not configured in different cases:

- Priority value of SL IUC information MAC CE, when triggered by explicit request:

If sl-PriorityCoordInfoExplicit-r17 is not configured, the priority of SL IUC information MAC CE is “the same as indicated by UE’s explicit request”. In our understanding, the priority indicated in the explicit request is the Priority field in Inter-UE Coordination Request MAC CE:

which is used to indicate the priority for UE-A to provide preferred resource set for UE-B.

Proposal 1: 	When determining Sidelink transmission information for performing sensing and candidate resource selections in PHY, the priority value of SL IUC information MAC CE triggered by an explicit request is set to the value of the Priority field indicated in Inter-UE Coordination Request MAC CE provided by UE-B, if sl-PriorityCoordInfoExplicit-r17 is not configured.

- Priority value of SL IUC request MAC CE:

If sl-PriorityRequest-r17 is not configured, the priority of SL IUC request MAC CE is “the same as that of a TB to be transmitted by UE-B”. While “a TB to be transmitted” is a bit vague, we believe the intention is for UE-B to indicate the priority of the SL data available for transmission when transmitting the request. In RAN1#107 agreement, the priority value to be used for PSSCH transmission is provided by UE-B’s request:

Agreement For Scheme 1, at least following parameters are provided by UE-B’s request: - Priority value to be used for PSCCH/PSSCH transmission - Number of sub-channels to be used for PSSCH/PSCCH transmission in a slot - Resource reservation interval

And we can assume that the priority provided by UE-B’s request indicated above means the Priority field in Inter-UE Coordination Request MAC CE. In short, RAN2 should set the priority (for sensing/resource selection) for the IUC request MAC CE to be the same as the Priority field in Inter-UE Coordination Request MAC CE. Whether to clarify how to set the Priority field in IUC request MAC CE in the field description can be further considered.

Proposal 2: 	When determining Sidelink transmission information for performing sensing and candidate resource selections in PHY, the priority value of SL IUC request MAC CE is set to the value of the Priority field indicated in Inter-UE Coordination Request MAC CE, if sl-PriorityRequest-r17 is not configured.

- Priority value of SL IUC information MAC CE, when triggered under a condition:

If sl-PriorityCoordInfoCondition-r17 is not configured, the priority of SL IUC information MAC CE is “determined by UE-A’s implementation”. While being an FFS, there seemed to be no further discussion from RAN1 on this topic. In addition, the agreement above is also provided as an response in LS to RAN2 [2], so we propose to follow this direction unless there are further notice from RAN1.

Proposal 3: 	When determining Sidelink transmission information for performing sensing and candidate resource selections in PHY, the priority value of SL IUC information MAC CE triggered under a condition is up to UE implementation, if sl-PriorityCoordInfoCondition-r17 is not configured

Based on the above analysis and proposals 1 and 2, we provide a text proposal with notes added in the current MAC spec to clarify how to set the priority value of the IUC containers when the corresponding parameters are not configured. For condition-triggered SL IUC information, since there’s no clear agreement from RAN1 yet for this case, it may not be suitable to add changes in RAN2 spec for now. It can be revisited later if there’s further progress from RAN1.

Proposal 4: 	Adopt text proposal below as baseline for request-triggered SL IUC information and SL IUC request. FFS whether changes are needed for condition-triggered SL IUC information.

- Conclusion

We have the following proposals for IUC priority in SL enhancement:

Proposal 1: 	When determining Sidelink transmission information for performing sensing and candidate resource selections in PHY, the priority value of SL IUC information MAC CE triggered by an explicit request is set to the value of the Priority field indicated in Inter-UE Coordination Request MAC CE provided by UE-B , if sl-PriorityCoordInfoExplicit-r17 is not configured.

Proposal 2: 	When determining Sidelink transmission information for performing sensing and candidate resource selections in PHY, the priority value of SL IUC request MAC CE is set to the value of the Priority field indicated in Inter-UE Coordination Request MAC CE, if sl-PriorityRequest-r17 is not configured.

Proposal 3: 	When determining Sidelink transmission information for performing sensing and candidate resource selections in PHY, the priority value of SL IUC information MAC CE triggered under a condition is up to UE implementation , if sl-PriorityCoordInfoCondition-r17 is not configured

Proposal 4: 	Adopt text proposal below as baseline for request-triggered SL IUC information and SL IUC request. FFS whether changes are needed for condition-triggered SL IUC information.

- Reference

[1] R2-2210933	Summary of [AT119-e][502][V2X/SL] 38.321 corrections (LG)

[2] R2-2206915	Reply LS on the inter-UE coordination mechanism

- Text proposal (based on 38.321 v17.2.0)

#### 5.22.1.3	Sidelink HARQ operation

##### 5.22.1.3.1	Sidelink HARQ Entity

The MAC entity includes at most one Sidelink HARQ entity for transmission on SL-SCH, which maintains a number of parallel Sidelink processes.

The maximum number of transmitting Sidelink processes associated with the Sidelink HARQ Entity is 16. A sidelink process may be configured for transmissions of multiple MAC PDUs. For transmissions of multiple MAC PDUs with Sidelink resource allocation mode 2, the maximum number of transmitting Sidelink processes associated with the Sidelink HARQ Entity is 4.

A delivered sidelink grant and its associated Sidelink transmission information are associated with a Sidelink process. Each Sidelink process supports one TB.

For each sidelink grant, the Sidelink HARQ Entity shall:

[…]

NOTE Y: When determining Sidelink transmission information, the priority of the IUC information MAC CE is the value indicated in Priority field in the IUC Request MAC CE provided by UE-B when triggered by an explicit request, if sl-PriorityCoordInfoExplicit-r17 is not configured.

NOTE Z: When determining Sidelink transmission information, the priority of the IUC request MAC CE is the value indicated in Priority field in the IUC Request MAC CE, if sl-PriorityRequest-r17 is not configured.