3GPP TSG-RAN WG2 Meeting #120                	R2-XXXX

Toulouse, FR, 14th – 18th Nov 2022

Agenda Item:	8.8.2

Source: 	vivo

Title:         	Discussion on measurement reporting enhancement for NR UAV

Document for: 	Discussion and Decision

- Introduction

In RAN2#119 meeting, the following agreements have been reached for measurement reporting enhancement for NR UAV:

Agreements

- Use LTE principle as a baseline, introduce similar event H1 (aerial UE height become higher than threshold) and H2 (aerial UE height become lower than threshold.  FFS if further NR enhancements are needed.  FFS study scaling of RRM parameters (e.g. which parameters and what is the purpose/benefit of the scaling and how)

FFS how to limit excessive measurements and measurement reporting

FFS if user consent is needed for location reporting in CONNECTED

FFS study the vertical movement and associated mobility for UAV UEs

2	Rel-18 NR supports reporting of UAV UE’s height, location and velocity. It is for further study what accuracy and reporting mechanisms are required and if further enhancements are needed.

3	As in LTE, flight path plan reporting will be introduced.  Location list of waypoints (3D location information) and timestamp is adopted as the basic content of flight path report.  FFS if timestamp is mandatory or optional for NR.  FFS if further enhancements are needed

4	Introduce similar functionality to LTE (numberofTriggeringCells).  FFS whether numberoftriggerbeams for NR is required or other enhancements.  FFS study how to avoid sending the measurement reports mainly due to reportOnLeave.

During RAN2#119bis meeting, the following agreements were made.

Agreements:

- The time information reported as part of flight path plan is optional. UE includes time info, if configured by the network and available at the UE.  FFS on flight path details (waypoints and what is time information).

- Allow the flight path to be updated.  FFS on the details.

- FFS on reporting format and initial flight path reporting (i.e. what information to report and how) – next meeting

- Continue to study height-depending scaling, triggering and combinations

- As in LTE, as a baseline, events A3, A4 and A5 can be configured with the configured number of cells (numberofTriggeringCells)

This contribution further discusses the remaining issues for measurement reporting enhancement for NR UAV.

- Discussion

Number of triggering cells:

RAN2 has agreed to introduce similar numberofTriggeringCells for events A3, A4 and A5 for NR UAV. The main motivation of this scheme is to avoid frequent measurement reporting since UAV UE may detect more cells fulfill the measurement events compared to ground UEs.

- As in LTE, as a baseline, events A3, A4 and A5 can be configured with the configured number of cells (numberofTriggeringCells)

In the previous RAN2 meetings, many companies think the LTE design has some disadvantage and can be improved in NR. For convenience, the related text in LTE is quoted below:

5.5.4 Measurement report triggering 5.5.4.1 General <Omit…> 2> if the triggerType is set to event and if the corresponding reportConfig includes numberOfTriggeringCells, and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig: 3> If the VarMeasReportList does not include a measurement reporting entry for this measId (a first cell triggers the event): 4> include a measurement reporting entry within the VarMeasReportList for this measId; 3> If the number of cell(s) in the cellsTriggeredList is larger than or equal to numberOfTriggeringCells: 4> include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId; 3> else: 4> include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId; 4> If the number of cell(s) in the cellsTriggeredList is larger than or equal to numberOfTriggeringCells: 5> set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0; 5> initiate the measurement reporting procedure, as specified in 5.5.5; 2> if the triggerType is set to event and if the leaving condition applicable for this event is fulfilled for one or more of the cells included in the cellsTriggeredList defined within the VarMeasReportList for this measId for all measurements after layer 3 filtering taken during timeToTrigger defined within the VarMeasConfig for this event: 3> remove the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId; 3> if reportOnLeave is set to TRUE for the corresponding reporting configuration or if a6-ReportOnLeave is set to TRUE or if a4-a5-ReportOnLeave is set to TRUE for the corresponding reporting configuration: 4> initiate the measurement reporting procedure, as specified in 5.5.5; 3> if the cellsTriggeredList defined within the VarMeasReportList for this measId is empty: 4> remove the measurement reporting entry within the VarMeasReportList for this measId; 4> stop the periodical reporting timer for this measId, if running; <Omit…>

From the above text, the scheme in LTE has the below issues:

- Issue 1: When the number of triggering cells is equal to numberofTriggeringCells, the UE triggers measurement reporting. After that, the UE cannot trigger measurement reporting when new cells are added into cellsTriggeredList (highlighted in yellow) if periodic reporting is not configured. The network cannot obtain new cells timely to perform interference management and mobility management for the UAV, which may lead to RLF or HOF of the UAV.

- Issue 2: When reportOnLeave is set to true for the event (A3 or A4 or A5), the UE will trigger measurement report if each of the cells in cellsTriggeredList fulfills the leaving condition of the event (highlighted in blue). This is also not consistent with the motivation of introducing numberofTriggeringCells.

For issue 1, someone may argue that the network can always configure periodic reporting for UAV UE.  But it will lead to unnecessary measurement reporting when there are no new cells are added into cellsTriggeredList. And this is also not aligned with the motivation of introducing numberofTriggeringCells. For issue 2, there were some proposals that introducing similar numberofTriggeringCells for leaving condition. This is reasonable, but we would like to suggest RAN2 to discuss how to design the whole procedure to solve the above two issues jointly. From our point of view, the common part of two issues is the number of changed cells compared to the cells included in the cellsTriggeredList. So, it seems straightforward to introduce a new parameter, like the number of changed cells for triggering measurement reporting (numberofChangedCells). This new parameter can cover the below three cases.

- Case 1: When the number of triggering cells is equal to numberofTriggeringCells, the UE triggers measurement reporting. After that, if the number of newly added cells is equal to numberofChangedCells, the UE should trigger measurement.

- Case 2: When the number of triggering cells is equal to numberofTriggeringCells, the UE triggers measurement reporting. After that, if the number of removed cells is equal to numberofChangedCells, the UE should trigger measurement.

- Case 3: When the number of triggering cells is equal to numberofTriggeringCells, the UE triggers measurement reporting. After that, if the number of newly added cells plus the number of removed cells is equal to numberofChangedCells, the UE should trigger measurement.

After measurement is triggered due to numberofChangedCells, the UE can re-count the number of changed cells.  Thus, we have the following proposal:

Introduce the number of changed cells for triggering measurement reporting in NR.

RRM parameter scaling:

In the past RAN2 meetings, there were some discussions on whether some RRM parameters (e.g., TTT, or additional offset for event A3, A4 or A5) should be scaled due to UAV horizontal or vertical mobility. From our understanding, the issue main occur when UAV UE reaches some height and moves fast, so the UAV cannot report RRM measurement timely. RAN2 has already agreed that Rel-18 NR supports reporting of UAV UE’s height, location and velocity. Further, as network may be aware of UAV flight path, with the combination UAV UE’s height, location and velocity, network can appropriately configure UAV UE for better mobility management. Besides, in LTE, the number of triggering cells was introduced, and the motivation is to avoid frequent measurement reporting, but short TTT seems against with this motivation. In our view, reducing the signaling overhead of measurement reporting and pursing timely measurement reporting are contradictory, and the compromise can be left to network, that is, the network can configure proper TTT based on network strategy. Therefore, we have the following proposal:

Scaling of RRM parameters, such as to shorten TTT is not necessary for UAV UE mobility optimization.

Height report:

NR supports SSB, beam and SMTC. When the aerial UE is equipped with directional antenna and supports beam, the UE may cause or suffer higher interference in some directions while lower interference in other directions. So, when height reporting is triggered by the UE’s height above the configured threshold, it is helpful for the network to perform interference management if the UE includes beam information in the height report. The beam information could be the beam measurement results, and the current transmission beam of the UAV. Thus, we have the following proposal:

UAV UE can include beam information in height report.

- Conclusion

In this paper, the following proposal are given:

Introduce the number of changed cells for triggering measurement reporting in NR.

Scaling of RRM parameters, such as to shorten TTT is not necessary for UAV UE mobility optimization.

UAV UE can include beam information in height report.

- References

[1] RP-213600, New WID on NR support for UAV (Uncrewed Aerial Vehicles), Nokia.