3GPP TSG-RAN WG2 #120 electronic                                   R2-2211839

Online, November, 2022

Source: 			OPPO

Title: 	Further discussion on sidelink positioning

Agenda item:		8.2.2

Document for: 	Discussion and Decision

- Introduction

In the last RAN2#119bis-e meeting, following agreements regarding sidelink positioning have been made:

Agreements:

Protocol options between UE and LMF for hybrid PC5+Uu positioning and PC5-only positioning in-coverage are studied and RAN2 will down-select during normative work.

- Extension of LPP, whereby new signaling shall be defined to support hybrid Uu and PC5 based positioning, i.e., extend the existing LPP to support sidelink based positioning between UE and LMF

- Enhancement of LPP whereby SLPP/RSPP signaling can be transported within LPP transparently, i.e., use the newly defined SLPP/RSRP to support sidelink based positioning and use the existing LPP to support Uu based positioning; and the SLPP/RSPP is carried as a container in LPP.

- Use of SLPP/RSRP between the UE and the LMF.

Agreement:

Proposal 3(modified): In order to enable sidelink positioning, SLPP/RSRP shall support at least the following functionalities:

- SL Positioning Capability Transfer

- SL Positioning Assistance Data exchange

- SL Location Information Transfer

- Error handling

- Abort

This agreement does not imply any specific signaling structure

Agreement:

Proposal 5: Unicast/one-to-one operation is assumed as baseline for exchange of sidelink positioning signaling

Proposal 6 (modified): RAN2 shall study applicability of at least the following positioning signaling for groupcast/broadcast (in addition to unicast), including addressing any security aspects (involving SA3 where needed). FFS the specific use case:

- SL positioning capability transfer

- SL positioning assistance data

- FFS SL location information transfer

Agreement:

Proposal 1 (modified): RAN2 agrees to support unicast SLPP/RSPP session-based operation and to study the applicability of group/broadcast to SLPP/RSPP group operation. FFS if groupcast/broadcast operation, if supported, would be session-based or sessionless.

Proposal 3 (modified): RAN2 agrees to support at least unicast SLPP/RSPP “centralized” operation in the sense used in R2-2210911, i.e., operation where one UE performs range and/or position calculations based on measurement/location information relating to itself and/or other UEs.

RAN2 will follow SA2 on which UE(s) can perform the calculation and related RAN1 definitions.

In this paper, we would like to further present our views on supporting the sidelink positioning in NR from the perspective of architecture, protocol stack, signaling procedure.

- Discussion

- 2.1 Consideration on architecture of sidelink positioning

In the latest [1], new UE roles have been defined as follows:

Assistant UE: A UE supporting Ranging/Sidelink Positioning between a SL Reference UE and a Target UE over PC5, when the direct Ranging/Sidelink positioning between the SL Reference UE and Target UE cannot be supported. The measurement result of Ranging/Sidelink Positioning between the Assistant UE and the SL Reference UE and that between the Assistant UE and the Target UE are determined and used to derive the Ranging/Sidelink Positioning result between Target UE and SL Reference UE.

Located UE: A SL Reference UE of which the location is known or is able to be known using Uu based positioning. A Located UE can be used to determine the location of the Target UE using Sidelink Positioning.

SL Positioning Client UE: A third-party UE, other than SL Reference UE and Target UE, which initiates Ranging/Sidelink positioning service request on behalf of the application residing on it.

As can be found in the above definition, the assistant UE is needed when the direct Ranging/Sidelink positioning between the anchor and Target UE cannot be supported. Specifically, the SL positioning measurement between the Assistant UE and the anchor and the one between the Assistant UE and the Target UE are needed to determine the ranging/Sidelink Positioning result between the target UE and anchor UE. It could be concluded that introduction of such UE role will have impact on the RAN side and the SL positioning procedure. As a result, we suggest to explicitly take into account of the assistant UE in the SL positioning related architecture. Examples of architectures for implementing SL positioning with UE and LMF as the location server are depicted as follows:

(a)

(b)

Figure 1: an example of SL positioning architecture with location server UE (a) or the LMF (b) as the location server

Observation 1: the assistant UE has been captured in SA2 TR as a new UE role. The SL positioning measurement needs to be performed between the assistant UE and the target UE, and between the assistant UE and the anchor UE.

Proposal 1: RAN2 to agree to take the two types of SL positioning architectures as the baseline

-	Type A: a UE as location server (with or without assistant UE)

-	Type B: the LMF as location server (with or without assistant UE)

- 2.3 The impact of assistant UE on RAN2

As indicated in LS R2-2209351 from SA2, they have agreed to introduce the terminology ‘Assistant UE’, which is useful when the direct Ranging/Sidelink positioning between the SL Reference UE and Target UE cannot be supported. From our understanding, the positioning SI Objective as given in [2] does not preclude the scenario of taking advantage of 3rd UE for SL positioning between UEs with no direct ranging/sidelink positioning condition. Especially, it emphasizes that alignment with SA2 are required:

Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network-based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required]

Observation 2: the positioning SI objective as given in RP-221814 does not preclude the scenario of taking advantage of 3rd UE for SL positioning between UEs with no direct ranging/sidelink positioning condition.

In our opinion, RAN2 should follow the SA2’s requirement of using the Assistant UE in the SL positioning. Here we analyse the potential RAN2 impact of introducing the assistant UE by firstly drawing a potential workable procedure of SL positioning involving the Assistant UE. We divide the whole procedure into two stages which should be coverage by SA2 and RAN2, respectively.

- Stage 1:

- a positioning request to measure the ranging result between UE A and UE B comes to the B (could be a RSU).

- B, via the SL discovery procedure, finds that UE A and B is too far to perform the direct SL ranging, but there exist a UE C that maintains good SL transmission condition towards both of A and B.

- UE C is nominated as the assistant UE and the SL positioning server UE.

- Stage 2:

- SL capabilities are transferred from UE A to UE C, and from UE B to UE C.

- UE C determines the SL positioning method to be used.

- UE C configures the SL-PRS to be transmitted by itself towards the UE A and UE B in the SL assistance data.

- UE C informs UE A and UE B of the expected location measurement result.

- UE C collects the location measurement result from UE A and UE B and derives the ranging result from UE A and UE B.

From the given procedure, it can be found that the difference between SL positioning involving the assistant UE and the one not involving the assistant UE is not obvious: the SL positioning signaling msgs to be used are the same. The major difference is that the positioning entity needs to derive the positioning result between UE A and B from the SL-PRS measurement result between them and a third UE. Since the scenario of applying the assistant UE is valid, we propose RAN2 to agree to support the SL positioning involving the assistant UE in the study idem phase.

Proposal 2: RAN2 to agree to support the SL positioning involving the Assistant UE in the study idem phase.

- 2.3 Sidelink positioning without a UE as location server role

As well known, the LMF plays the key role in managing the localization services of the 5G NR network, such as receiving UE localization related capabilities, determination of the positioning method to be used, collecting DL-PRS configurations from TRPs and sending the configuration towards the UE, commanding the serving gNB to configure UL-SRSp towards the UE, retrieving the positioning measurement result from gNB, etc. The NRPPa and LPP protocol is used for related signaling msg transmission between LMF and gNB, and between LMF and UE, respectively.

For in-coverage and partial-coverage scenario where the target UE is in-coverage, it is possible that the LCS request comes from the network or external LCS client connected with 5G network, as currently specified in 23.273. In addition, if the Uu positioning result is not good enough, it is possible that the network would trigger the SL positioning to calibrate the Uu positioning result. So, from our perspective, in such cases, the LMF should take the role of location server. However, there exists a possibility that the LCS request comes from a out-of-coverage SL client UE. In such cases, since out-of-coverage SL client UE could not communicate with the LMF directly, and we cannot assume there could be a relay UE, that it is better for choosing a SL positioning server UE.

Proposal 3: RAN2 to agree that LMF should take the role of location server when the target UE is in-coverage and LCS request comes from an entity connected with 5G network.

Proposal 4: RAN2 to agree that a SL positioning server UE should take the role of location server when the target UE is in-coverage and LCS request comes from an entity not connected with 5G network.

On the other hand, when the target UE is outside the coverage, the AMF cannot find the target UE, hence the LCS request coming from the network or external LCS client cannot reach the target UE, at least if without the sidelink relay by another UE. And we cannot assume the target UE is equipped with relay capability. Therefore, in such use cases, it is reasonable to use a UE to play the role as the location server.

Observation 3: when the target UE is outside the coverage, the AMF cannot find the target UE, hence the LCS request coming from the network or external LCS client connected with the 5G cannot reach the target UE, at least if without the sidelink relay by another UE.

Proposal 5: RAN2 to agree that a SL positioning server UE should take the role of location server if the target UE is OOC.

- 2.5 Application of broadcast of positioning signalling msg

The SA2 has agreed to reuse 5G ProSe Discovery procedures and V2X Communication procedures on Ranging/SL Positioning discovery, with the additional Ranging/SL Positioning parameters such as ProSe identifier or the Service Type identifier “Ranging/Sidelink Positioning” and the expected roles of the UE (i.e., Target UE, SL Reference UE, Located UE, Assistant UE, SL positioning Server UE). With such information, the RAN layer can know the candidate UEs being involved in a particular positioning session potentially. To further determine the positioning method to be used, and the exact UEs to be applied and the corresponding roles, the positioning capability needs to be transferred from the candidate UEs. Conventionally, positioning capability are retrieved by the LMF from the target UE in the unicast way. However, reusing such mechanism for performing SL positioning introduces significant timing latency, since the SL positioning capabilities need to be collected from potentially all candidate UEs. As the transfer of the positioning capability needs to be done before making the decision on which positioning method to be used and the exact UEs for involving in a particular positioning session and corresponding roles, the positioning capability should be always broadcast, similar with the SL positioning discovery msg, to accelerate the SL positioning procedure.

Observation 4: retrieving positioning capability in the unicast way for performing SL positioning introduces significant timing latency, since the SL positioning capabilities need to be collected from candidate UEs potentially more than the necessary UEs required for the SL positioning.

Proposal 6: RAN 2 to agree the positioning capability should be always broadcast/groupcast, similar with the SL positioning discovery msg.

Regarding the broadcast of the assistance data, we think there are two use cases:

  - The anchor UE and/or target UE broadcasts its configured SL-PRS

  - The SL positioning server UE distributes the SL-PRS configuration towards the anchor UE and/or the target UE

Both of two use cases are feasible. As a result, we suggest RAN2 to agree these two use cases for applying the assistance data transfer.

Proposal 7: RAN2 to agree two use cases for applying the assistance data transfer:

  - The anchor UE and/or target UE broadcasts its configured SL-PRS

  - The SL positioning server UE distributes the SL-PRS configuration towards the anchor UE and/or the target UE for performing monitoring or transmission.

- 2.6 Combination of Uu- and PC5-based positioning

As well known, the DL-PRS/SRSp propagation path between the UE and the gNB may be blocked or scattered to a large extent in the urban area, which causes NLOS complicated multipath and therefore degrades of the positioning accuracy. However, it should be noted that whether the DL-PRS/SRSp propagation path between a UE and a gNB in a particular scenario should be categorized to NLOS or LOS is only known at the receiver after the practical transmission of such reference signal has been done.

Observation 4: if the DL-PRS/SRSp propagation path between a UE and a gNB in a particular scenario should be categorized to NLOS or LOS is only known at the receiver after the practical transmission of such reference signal has been done.

Such observation leads to two modes of adding PC5 based positioning for further calibration of the Uu based positioning result:

  - Option 1: UE/LMF autonomously triggers PC5 based positioning during the Uu positioning session, potentially based on pre-configured condition, e.g., detection of NLOS transmission of the DL-PRS/SRSp, low RSRPP of the received DL-PRS/SRSp, etc.

  - Option 2: LMF triggers PC5+Uu based positioning after the Uu positioning session is ended. The decision of triggering further PC5 positioning to calibrate the Uu based positioning could be a result of the bad Uu positioning result, i .e., low positioning accuracy or the integrity of the positioning result could not be guaranteed.

Obviously, Option 2 consumes more time than Option 1 to be able to obtain a satisfied positioning result, due to the fact that another new positioning session has to be triggered, and the Uu positioning capability and reference signal configuration needs to be transferred again between the UE and the gNB during such new positioning session. As a result, we suggest RAN2 to take the Option 1 as a feasible option for combination of Uu- and PC5-based positioning.

Observation 5: Option 2 (LMF triggering PC5+Uu based positioning in a new positioning session) may consume more time than Option 1 (autonomously triggering PC5 based positioning based on some preconfigured condition) to be able to obtain a satisfied positioning result, due to the fact that another new positioning session has to be triggered, and the Uu positioning capability and reference signal configuration may need to be transferred again between the UE and the gNB during such new positioning session.

Proposal 8: RAN2 to agree that the Option 1 ((autonomously triggering PC5 based positioning based on some preconfigured condition) should be taken as a feasible option for triggering the PC5 based positioning to calibrate the Uu positioning to design the signalling procedure of combination of Uu- and PC5-based positioning.

- Conclusion and proposals

Observation 1: the assistant UE has been captured in SA2 TR as a new UE role. The SL positioning measurement needs to be performed between the assistant UE and the target UE, and between the assistant UE and the anchor UE.

Proposal 1: RAN2 to agree to take the two types of SL positioning architectures as the baseline

-	Type A: a UE as location server (with or without assistant UE)

-	Type B: the LMF as location server (with or without assistant UE)

Observation 2: the positioning SI objective as given in RP-221814 does not preclude the scenario of taking advantage of 3rd UE for SL positioning between UEs with no direct ranging/sidelink positioning condition.

Proposal 2: RAN2 to agree to support the SL positioning involving the Assistant UE in the study idem phase.

Proposal 3: RAN2 to agree that LMF should take the role of location server when the target UE is in-coverage and LCS request comes from an entity connected with 5G network.

Proposal 4: RAN2 to agree that a SL positioning server UE should take the role of location server when the target UE is in-coverage and LCS request comes from an entity not connected with 5G network.

Observation 3: when the target UE is outside the coverage, the AMF cannot find the target UE, hence the LCS request coming from the network or external LCS client connected with the 5G cannot reach the target UE, at least if without the sidelink relay by another UE.

Proposal 5: RAN2 to agree that a SL positioning server UE should take the role of location server if the target UE is OOC.

Observation 4: retrieving positioning capability in the unicast way for performing SL positioning introduces significant timing latency, since the SL positioning capabilities need to be collected from candidate UEs potentially more than the necessary UEs required for the SL positioning.

Proposal 6: RAN 2 to agree the positioning capability should be always broadcast/groupcast, similar with the SL positioning discovery msg.

Proposal 7: RAN2 to agree two use cases for applying the assistance data transfer:

  - The anchor UE and/or target UE broadcasts its configured SL-PRS

  - The SL positioning server UE distributes the SL-PRS configuration towards the anchor UE and/or the target UE for performing monitoring or transmission.

Observation 4: if the DL-PRS/SRSp propagation path between a UE and a gNB in a particular scenario should be categorized to NLOS or LOS is only known at the receiver after the practical transmission of such reference signal has been done.

Observation 5: Option 2 (LMF triggering PC5+Uu based positioning in a new positioning session) may consume more time than Option 1 (autonomously triggering PC5 based positioning based on some preconfigured condition) to be able to obtain a satisfied positioning result, due to the fact that another new positioning session has to be triggered, and the Uu positioning capability and reference signal configuration may need to be transferred again between the UE and the gNB during such new positioning session.

Proposal 8: RAN2 to agree that the Option 1 ((autonomously triggering PC5 based positioning based on some preconfigured condition) should be taken as a feasible option for triggering the PC5 based positioning to calibrate the Uu positioning to design the signalling procedure of combination of Uu- and PC5-based positioning.

Reference

[1] TR 23.700-86 V0.4.0, SA2

[2] RP-221814, Revised SID on Study on expanded and improved NR positioning, Intel, CATT, Ericsson