3GPP TSG-RAN WG2 Meeting #120	R2-2211857

Toulouse, France, 14th - 18th November, 2022

Agenda Item:	8.1.2

Source: 	Fujitsu

Title:	Discussion on state transition for NCR-MT

Document for:	Discussion and Decision

- Introduction

In the RAN2#119bis-e meeting, the following agreements were made [1]:

Agreement: NCR-MT supports RRC_CONNECTED and RRC_IDLE states, FFS on RRC_INACTIVE state (e.g. optional support or not support).

In this document, we discuss whether RRC_INACTIVE should be supported or not, and how state transition should be managed.

- Discussions

2.1.	RRC_INACTIVE for NCR-MT

RRC_INACTIVE is a state where UE Inactive AS Context is stored at a UE and an anchor gNB while RRC connection is suspended. The RRC_INACTIVE state is efficient for the UE to quickly return to RRC_CONNECTED mode to transmit/receive user data. However, we do not see such a requirement for NCR-MT. Also, RAN2 only have two meetings left for NCR WI. We propose that NCR-MT does not support RRC_INACTIVE state at least in Rel-18.

Proposal 1: NCR-MT does not support RRC_INACTIVE in Rel-18.

2.2.	RRC State transition

From RRC_CONNECTED to RRC_IDLE

For state transition from RRC_CONNECTED to RRC_IDLE at the NCR-MT, existing procedures for the UE can be applicable to the NCR-MT case as they are and no need to specify dedicated procedures for the NCR-MT.

Proposal 2: For state transition from RRC_CONNECTED to RRC_IDLE at the NCR-MT, it is not necessary to specify dedicated procedures for the NCR-MT.

From RRC_IDLE to RRC_CONNECTED

In the RAN2#119bis-e meeting, it was agreed that DRB would optionally be supported for NCR-MT. Therefore, there would be two cases that the NCR-MT in RRC_IDLE is required to transit to RRC_CONNECTED.

Case 1: Downlink user data (e.g., OAM signalling) for the NCR-MT in RRC_IDLE is arrived, or uplink user data (e.g., OAM report signalling) is derived at the NCR-MT in RRC_IDLE.

Case 2: The network needs to send side control information to the NCR-MT in RRC_IDLE, which may include the information to turn the NCR-Fwd on/off.

For the Case 1, existing procedures for the UE, i.e., CN paging and initial RRC connection establishment can be applicable, and no need to specify dedicated procedures for the NCR-MT.

However, for the Case 2, RAN2 needs to discuss how the network makes the NCR-MT in RRC_IDLE transit to RRC_CONNECTED. Especially:

- Whether CN paging-like method could be reused, including whether the same UE identity for the CN paging can be reused

- Whether Service Request procedure is required in NAS level.

Proposal 3: If state transition from RRC_IDLE to RRC_CONNECTED at the NCR-MT is triggered by downlink or uplink user data (e.g., OAM signalling/OAM report signalling) for the NCR-MT, existing procedures for the UE, i.e., CN paging and initial RRC connection establishment can be applicable, and no need to specify dedicated procedures for the NCR-MT.

Proposal 4: If state transition from RRC_IDLE to RRC_CONNECTED at the NCR-MT is required for the case the network needs to send side control information to the NCR-MT, RAN2 needs to discuss how the network should perform state transition. Especially:
 - Whether CN paging-like method could be reused, including whether the same UE identity for the CN paging could be reused, and
 - Whether Service Request procedure is required in NAS level.

- Conclusion

Proposal 1: NCR-MT does not support RRC_INACTIVE in Rel-18.

Proposal 2: For state transition from RRC_CONNECTED to RRC_IDLE at the NCR-MT, it is not necessary to specify dedicated procedures for the NCR-MT.

Proposal 3: If state transition from RRC_IDLE to RRC_CONNECTED at the NCR-MT is triggered by downlink or uplink user data (e.g., OAM signalling/OAM report signalling) for the NCR-MT, existing procedures for the UE, i.e., CN paging and initial RRC connection establishment can be applicable, and no need to specify dedicated procedures for the NCR-MT.

Proposal 4: If state transition from RRC_IDLE to RRC_CONNECTED at the NCR-MT is required for the case the network needs to send side control information to the NCR-MT, RAN2 needs to discuss how the network should perform state transition. Especially:
 - Whether CN paging-like method could be reused, including whether the same UE identity for the CN paging could be reused, and
 - Whether Service Request procedure is required in NAS level.

# References

- RAN2#119bis-e, "Report from session on NCR"