3GPP TSG-RAN WG2 Meeting #120                                   R2-2211865

Toulouse, Nov, 2022

Agenda Item:	8.4.3

Source: 	OPPO

Title:  	Discussion on selective activation of SCGs for NR-DC

Document for:	Discussion and Decision

Introduction

In R18 mobility enhancement WID, selective activation of the cell groups is introduced to support consecutive CPC/CPA procedure to avoid frequently RRC reconfiguration.

- To specify mechanism and procedures of NR-DC with selective activation of the cell groups (at least for SCG) via L3 enhancements:

- To allow subsequent cell group change after changing CG without reconfiguration and re-initiation of CPC/CPA [RAN2, RAN3, RAN4]

Note 4: A harmonized RRC modelling approach for objectives 1 and 2 could be considered to minimize the workload in RAN2.

In this contribution, we would like to provide our views on open issues for selective activation of SCG, mainly including applicable scenarios, trigger of candidate cell evaluation, update of security key and validity of configurations.

Discussion

Scenarios

According to the progress made in previous RAN2 meeting, the understanding of selective activation of cell groups was clarified as follows:

The selective activation of cell groups should correspond to support of subsequent conditional changes (CPC) after a cell group change (normal or conditional). CPA FFS.

Initial focus on SCG

Confirm that “CPA” selective activation of cell groups will be supported for this WI objective

Based on above agreements, it is still unclear whether CPC/CPA after Pscell addition is supported. R17 introduces CPA where UE can conditionally perform PSCell addition to move from single connectivity to dual connectivity. After initial CPA execution, it is possible that UE may move to one of the prepared PSCells. Considering that each CPA candidate is provided with full configuration, it is feasible to be applied for CPC. Therefore, instead of release CPA configuration after initial execution, it is beneficial to store CPA configuration for purpose of follow-up CPC/CPA. In addition, CPC/CPA after normal Pscell addition can also be supported. NW can trigger Pscell addition/release anytime even CPC/CPA configuration has been provided to UE.

Subsequent CPC/CPA after SCG addition (normal or conditional) is supported.

As specified in TS 37.340, CPC can be initiated by either MN or SN and CPA is initiated by MN. For MN-initiated CPC/CPA, execution conditions are generated by MN based on MN measConfig. For SN-initated CPC, execution conditions are generated by SN based on source SN measConfig. During subsequent CPC/CPA, the execution configurations can be maintained for MN-initiated case unless measConfig of MN is updated. While for SN-initiated case, execution conditions need to be re-configured each time source SCG is changed. Given more frequent signalling interactions are required to keep execution configuration valid, it is suggested to prioritize the study of subsequent CPC based on MN-initiated CPC/CPA.

MN-initiated subsequent CPC/CPA is prioritized.

Trigger of candidate cell evaluation

In last meeting, baseline procedure for selective activation of cell groups has been agree as general guidance:

Baseline procedure to support subsequent secondary cell group change (FFS if UE keeps all configurations or if those are indicated by the network, FFS support of nested configs):

- a.	Step 1: when the execution condition of a CPC candidate PScell is met, a UE performs the execution of CPC towards this candidate PScell.

- b.	Step 2: After finishing the PSCell addition or change, the UE doesn’t release conditional configuration of other candidate PSCells for subsequent CPC, the UE continues evaluating the execution conditions of other candidate PScells.

- c.	Step 3: When the execution condition of a candidate PScell is met, the UE performs the execution of CPC towards this candidate PSCell.

For subsequent CPC defined in step 2, it is unclear how UE performs candidate cell evaluation of other candidate cells. In legacy CPA/CPC, UE starts to evaluate the execution condition(s) upon receiving the configuration for CPC/CPA, and stops evaluating the candidates once conditional PSCell change/addition is triggered. For the first CPC/CPA, the existing evaluation mechanism can be reused, while it is not feasible to cover subsequent CPC/CPA since no reconfiguration message is expected. Therefore, further study is needed to define when to start the evaluation on candidate cells for subsequent CPC/CPA.

UE starts evaluating the execution condition(s) upon receiving the CPC/CPA configuration, and stops evaluating once PSCell change is triggered. For consecutive CPC, UE will not receiveRRC message for CPC/CPA configuration since the configured CPC/CPA has not been released.

To evaluate other candidate cells, the most straightforward way is to start evaluating other candidate cells directly after last PScell change/addition procedure. In that case, addition measurement overload will be leaded since the cell change event may not be triggered immediately after completion of last CPC/CPA. On the other hand, the condition configuration may be invalid due to the change of measurement configuration. Measurement/condition reconfiguration is required if reference measurement configuration changes. And another alternative is to define a new trigger to start evaluating candidate cells, which require further RAN2 work on designing the new trigger.

For subsequent CPC/CPA, RAN2 further study when to start the condition evaluation on candidates.

Security issue of subsequent CPC/CPA

As specified in TS 33.501, security key used for SN (KSN) is managed by MN. It is generated based on security key of MN (KgNB) and SN counter as described in the text in the box. MN generates KSN for SN and sends it to the SN over the Xn-C. In order to make KSN aligned between UE and SN, the value of SN counter should be sent to UE over RRC signalling for the derivation of KSN at UE side, i.e., sk_counter in RRCReconfiguration message.

TS 33.501 A.16 Derivation of KSN for dual connectivity This input string is used when the MN and UE derive KSN during dual connectivity. The following input parameters shall be used: - FC =0x79, - P0 = Value of the SN Counter as a non-negative integer, - L0 = length of the SN Counter value (i.e. 0x00 0x02). The input key KEY shall be KeNB when the MN is an ng-eNB and KgNB when the MN is a gNB.

In legacy CPAC, the sk_counter associated to each candidate is provided to the UE along with CPAC configuration. UE determines target PSCell based on execution condition and refreshes KSN according to corresponding sk_counter. For selective activation of SCG, the existing way can be followed, i.e. NW preconfigures sk_counter for each candidate and UE generates KSN of corresponding candidate when UE executes CPC/CPA procedure.

For subsequent CPC/CPA, SK counter is preconfigured for each CPA/CPC candidate.

According to the principle of legacy SN counter maintenance at MN, SN counter shall be increased monotonically:

The MN shall set the SN Counter to ‘0’ when a new AS root key, KNG-RAN, in the associated 5G AS security context is established. The MN shall set the SN Counter to ‘1’ after the first calculated KSN, and monotonically increment it for each additional calculated KSN. The SN Counter value '0' is used to calculate the first KSN.

For selective activation of SCGs, UE will perform consecutive CPC/CPA procedure with pre-configured candidates. If preconfigured sk_counter is applied for subsequent CPC/CPA, out-of-order sk_counter will be utilized since UE movement is irregular. Thus, further confirmation with SA3 is needed on whether there is any problem if the sk_counter used by UE does not follows a monotonically increasing way.

RAN2 confirms with SA3 on the feasibility of utilization of out-of-order sk counter for subsequent CPC/CPA.

In addition, further discussion is needed on whether a candidate is expected to be selected as target more than one time. There is no limitation on UE moving back to a same candidate cell based on current agreements. If single sk_counter is preconfigured for each CPA/CPC candidate, it would result in security reuse issue if CPC/CPA is executed on the same candidate more than once. To avoid security reuse issue, we should study how to guarantee there is valid sk counter for the update of security key used for SN especially for inter-SN CPC scenario.

The candidate solutions are listed below:

- Option 1: UE consider the candidate configuration as invalid when sk_counter associated to a CPA/CPC candidate is applied; MN updates the sk_counter for a candidate cell if UE has executed CPA/CPA to the candidate cell.

- Option 2: MN pre-config multiple sk_counter to UE;

After sk_counter associated with a CPC/CPA candidate has been applied, UE considers the sk_counter as invalid. Further study is needed on guaranteeing a valid sk_counter for subsequent CPC/CPA.

Candidate configurations

In last meeting, RAN2 agreed to support delta configuration for candidate cell configuration of subsequent CPC/CPA.

Confirm that we aim to support delta configuration, i.e. that there need to be a known reference.

To support delta configuration, a known reference config is required. NW can explicitly indicate a reference configuration to UE to guarantee the delta configuration of each candidate cell.

For supporting delta config, an explicitly signalled reference configuration is introduced for selective activation of SCG.

As multiple candidate configurations are maintained for supporting subsequent CPC/CPA, how to guarantee the validity of configurations need to be studied. We give several alternatives for guaranteeing the validity of candidate cell configuration.

- It is left to NW to guarantee the validity of candidate cell configuration. For example, candidate cell configuration can only be modified / released by Network.

- UE autonomous release the invalid candidate cell configuration with pre-defined condition. For example,

  - sk_counter is invalid. UE stop the evaluation to a candidate and release the corresponding cell configuration if the sk_counter has been applied and no refreshed key is received for the candidate cell.

  - MN Key is refreshed;

  - MN is changed;

- Timer-based or area-based validation: UE only assumes the candidate cell configurations are valid within certain period/area.

RAN2 study how to guarantee the validity of configurations for selective activation of SCG.

Conclusion

Based on the discussion in section 2 we have following observations:

Observation 1	UE starts evaluating the execution condition(s) upon receiving the CPC/CPA configuration, and stops evaluating once PSCell change is triggered. For consecutive CPC, UE will not receiveRRC message for CPC/CPA configuration since the configured CPC/CPA has not been released.

Based on the discussion above, we have the following proposals:

Proposal 1	Subsequent CPC/CPA after SCG addition (normal or conditional) is supported.

Proposal 2	MN-initiated subsequent CPC/CPA is prioritized.

Proposal 3	For subsequent CPC/CPA, RAN2 further study when to start the condition evaluation on candidates.

Proposal 4	For subsequent CPC/CPA, SK counter is preconfigured for each CPA/CPC candidate.

Proposal 5	RAN2 confirms with SA3 on the feasibility of utilization of out-of-order sk counter for subsequent CPC/CPA.

Proposal 6	After sk_counter associated with a CPC/CPA candidate has been applied, UE considers the sk_counter as invalid. Further study is needed on guaranteeing a valid sk_counter for subsequent CPC/CPA.

Proposal 7	For supporting delta config, an explicitly signalled reference configuration is introduced for selective activation of SCG.

Proposal 8	RAN2 study how to guarantee the validity of configurations for selective activation of SCG.

Reference

- RP-222332  Further NR Mobility Enhancements    MediaTek Inc