3GPP TSG RAN WG2 Meeting #120               	R2-2211927

# Toulouse, France, 14th 18th November 2022

Agenda Item: 	8.5.4.2

Source:		Sony

Title:			Considerations on XR specific capacity improvements

Document for:	Discussion & Decision

# Introduction

A new Rel-18 study item was approved at RANP#94 in December 2021, including the following objectives. The study is to be based on RAN1, Rel-17 TR 38.838 [2], SA4 work in SP-210043 [3], and SA2 work as per SP- 220705 [4].

The study is to be based on Rel-17 TR 38.838, on corresponding Rel-17 work from SA4 (as per SP-210043) and on Rel-18 work from SA2 (as per SP-211166). Objectives on XR-awareness in RAN (RAN2): - Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of. - Study how the above information aids XR-specific traffic handling. Objectives on XR-specific Power Saving (RAN1, RAN2): - Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques: - C-DRX enhancement. - PDCCH monitoring enhancement. Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements; Dynamic scheduling/grant enhancements

At RAN2#119e meeting [6], the following agreements were made;

1: As starting point, RAN2 can further discuss the solutions in TR 38.838 that can impact on L2 operation (e.g., BSR, LCP, assistance information for scheduling, packet discarding, prioritization) for XR-specific capacity improvement. RAN2-specific solutions are not precluded (even if RAN1 hasn’t discussed them before). 1: Enhancement to SPS/CG should be justified for XR scheduling and should be evaluated against dynamic grant (DG) scheduling which should be considered as baseline. Should justify why enhancements are needed. RAN2 considers SPS enhancements may not be needed in Rel-18 XR since PDCCH capacity is not assumed to be a problem for XR. FFS if SPS has some power consumption benefits.

At RAN2#120 meeting [7], the following agreements were made:

1: Current CG configurations can be reused for UL XR traffic. FFS if enhancements are needed (RAN1 is already discussing something). RAN2 can discuss this in the next meeting.

2: RAN2 can discuss potential enhancement to provide some assistant information on UL XR traffic for CG configurations at the gNB. FFS whether TSCAI can already provide all necessary information.

In this contribution, we provide our views for justification and benefits on the mechanisms to improve capacity, including SPS/CG with non-integer periodicity, traffic with multiple flows and jitter aspects.

# Discussions

## Transmission with Non-integer periodicity

XR traffic may require a transmission with non-integer periodicity due to the required video frame rate in XR traffic. The typical video frame rates are 30, 60, and 90 fps. An XR transmission with 60 fps requires a transmission interval with 16.67 ms. Furthermore, XR is a delay sensitive application. A certain delay in the transmission can reduce user experience. Large amount of data buffering which could be used to re-arrange the transmission may not be possible as it may introduce significant delay.

The legacy 5G NR typically uses Semi-Persistent Scheduling (SPS) and Configured Grant (CG) to handle a periodic transmission (e.g., voice call). Hence, the usage of control channel/information can be minimized and resulting in improved capacity. However, the legacy SPS periodicity is using a selected integer number as shown below:

SPS-config = periodicity ENUMERATED {ms10, ms20, ms32, ms40, ms64, ms80, ms128, ms160, ms320, ms640,spareX}

The current SPS configuration is not designed to accommodate quasi-static periodicity of XR transmissions. This is illustrated in the following Figure 1. Here, a mismatch between the SPS allocation pattern and the XR traffic pattern is illustrated. Furthermore, the mismatch gets accumulated over time.

Figure 1: Mismatch of SPS allocation pattern and XR traffic pattern

Observation 1: Depending on the XR traffic pattern, there can be a mismatch of periods between NR SPS/CG configuration and XR traffic.

We consider to enhance the existing SPS/CG configuration so that it can dynamically adjust the transmission to accommodate the XR traffic pattern. Some of the transmission based on SPS/CG configuration can be slightly adjusted (delayed) so that basically all transmission is transmitted in an integer slot number.

SPS enhancement can be seen as reduction of the PDCCH transmission. The PDCCH transmission is typically using a small amount of time/frequency resources as the payload size is relatively small. Then, it can be argued that the capacity reduction of SPS enhancement is relatively small. However, the search space for PDCCH transmission can also be considered small. A large size of search space may increase UE power consumption as the UE has more blind detection operation.

Observation 2: The search space size for PDCCH transmission can be relatively small. Hence, it would be beneficial to reduce the PDCCH transmission which can also increase the overall XR capacity.

Proposal 1: SPS enhancement can still be considered for XR capacity enhancements as the search space size for PDCCH transmission can be relatively small.

In case of DL-SPS transmission The UE should be aware that the transmission has been dynamically adjusted. We consider the adjustment can be implicit or explicitly indicated to the UE. Implicit indication can be supported by providing the adjustment rules defined in the specification. Hence, the amount of signalling between gNB and UE can be minimized. Explicit indication can be supported by providing some new parameters representing the adjustment value. This may require some new parameters in SPS / CG configuration.

Proposal 2: Consider Dynamic SPS/CG configuration to dynamically adjust the transmission in order to accommodate the XR traffic pattern.

A possible simple dynamic SPS/CG configuration is using pseudo periodic SPS/CG as shown in Figure 2, where CG configuration for XR traffic with 30 FPS is selected for illustration purpose. Since the period of 30 FPS traffic is 33.33 ms, the basic period of pseudo periodic can be set to 33 ms. With an extra 1 ms inserted in the beginning of every 3rd CG occasions, the time gap between traffic generation and transmission resources can be limited to no longer than 1ms. This could make the CG configuration matching with XR traffic every 100ms. Similarly, this pseudo periodic CG configuration can be applied to SPS transmission for the downlink case.

Figure 2: A schematic/illustration of pseudo periodic CG

Proposal 3: Pseudo-periodic SPS/CG configuration can be considered for XR traffic with non-integer period.

## Traffic with Multiple flows

The legacy downlink control information (DCI) for SPS/CG is scrambled by CS-RNTI. The DCI is used for the indication of SPS/CG type II activation and release. The associated SPS/CG can only support periodic traffic with fixed packet size. Moreover, only one CS-RNTI is supported in the legacy NR. XR service could contain multiple UL and DL streams/flows (pairs) with variable packet size. To cover the indication of SPS/CG operations for each stream/flow (pair), multi-TB SPS/CG and dynamic update of SPS/CG configurations was proposed [5]. In this section, we would like to propose another scheme with multi CS-RNTIs configuration as a complement to solve the issue.

An example of the multi CS-RNTIs scheme is illustrated in Figure 3, with the assumption of AR scenario with two DL (I-stream and P-stream) and three UL streams (I-stream, P-stream and pose/control stream). Firstly, two CS-RNTIs associated with different stream (pairs) will be configured and sent by RRC signalling, e.g. CS-RNTI1 will indicate the operations of SPS/CG for I-stream pair (green) and CS-RNTI2 for P-stream pair (purple). The third periodic UL stream (yellow) can be served by CG type I without CS-RNTI. Through this configuration and scheduling procedure, the operations of SPS/CG for four streams with two I-steam and P-stream pair can be dynamically and flexibly indicated.

Figure 3:Multi CS-RNTIs configuration for multi streams in XR.

For comparison purpose with any other potential solutions, we enumerate couples of potential shortcomings of multi-TB SPS/CG scheme [5] that have been proposed before. Firstly, a larger specification impact on DCI format design to support multi TB is needed, as well as relative reconsiderations on DCI budget and blind detection in search space set (SSS). Secondly, DCI supporting multi TB scrambled by CS-RNTI can only indicate either multi UL streams or multi DL streams, not able to indicate DL and UL streams simultaneously since one DCI can only be a DL DCI or a UL DCI. Thirdly, if gNB only plans to indicate one stream, other TBs in multi TB DCI need to be configured as special/default/reserved value, which will waste DCI payload. Fourthly, if switching back to supporting single TB DCI  for the above cases? with only one stream indication from gNB, an additional bit on stream id is needed in DCI otherwise UE does not know which stream this DCI is targeting. While our proposed multi CS-RNTIs scheme just needs multi CS-RNTIs configuration in RRC and low complexity on multi CS-RNTIs descrambling and CRC checking, as well as no increasing of PDCCH blind detection per DCI, which had less specification impact and more flexible/dynamic indication of SPS/CG per stream.

Proposal 4: Multi CS-RNTIs configuration for multi flows in XR can be considered.

Considering another scenario on VR DL with video and audio/data streams, where the video stream is modelled as truncated Gaussian distribution in clause 5.1.1 in TR 38.838 [2], and the audio/data stream is modelled as a periodic traffic with fixed size packet as Table 5.1.2.2-1 in TR 38.838 [2], a new SPS type I configuration similar to that in CG type I configuration can be considered for the audio/data stream. In that way, the blind detection of DCI activation/release could be saved for power saving.

Proposal 5: A new SPS type configuration that is similar to CG type I configuration can be considered to support multi-flows in XR.

## Jitter aspects

Although the packet arrival for some XR services may be periodic, the actual arrival time of the packet may experience jitter causing it to arrive randomly within a jitter time window, TJitter.  We can consider the statistical jitter, including the jitter time window/range as agreed in [2]. An example is shown in Figure 4, where an XR application has periodic traffic with a periodicity of PApp. However, this traffic experiences jitter and so the actual packet arrival falls within a time window TJitter.  In this example, the first jitter time window starts at time t0 where the packet can arrive within this time window between t0 to t4 and here the packet arrives at time t1.  The next packet arrives after a time PApp later, starting at time t5, and here once again the packet can arrive at any time within the jitter time window between t5 to t9.  In the second instance, the packet arrives at time t8, which is towards the end of the jitter time window TJitter.

Figure 4: Jitter time window

SPS configuration provides PDSCH resources to the UE with a deterministic periodicity which can be from 1 to 640 slots.  It is recognized that such deterministic periodicity configuration is not suitable for traffic experiencing jitter.  In order to cater for jitter, multiple SPS configurations can be used, where each SPS configuration is activated with a different starting offset, i.e. different K0, as indicated in the DCI field “Time Domain Resource Assignment” (TDRA).  That is, the SPS  is over-configuring the PDSCH resources to support jittering. In the example in Figure 4, the UE can be configured with 4 SPS PDSCH configurations, such that these 4 SPS fall within the jitter time window and each has a periodicity of PAPP.  An example is shown in Figure 5, where 4 SPS configurations labelled as #1, #2, #3 and #4 with periodicity PApp, but different offsets, are configured for a UE. Hence, by configuring multiple SPS resources/configurations, the UE is therefore provided with PDSCH resources to cover for whenever the packet data may arrive within the jitter time window.

Figure 5: Over-configuring SPS for jittering traffic

Since UE needs to monitor all SPS resources regardless of whether they contain any actual data/traffic in the configured PDSCH resource or not, over configuration of SPS to serve a single traffic consumes unnecessary UE power.  Hence the issue to be solved is to provide SPS resources that is suitable for quasi-periodic traffic with minimal additional UE monitoring. An enhancement of the legacy SPS is needed.

Observation 3: SPS enhancements are required to support XR services with the benefit to improve the capacity and minimize unnecessary UE power consumption.

We consider that this issue can be solved by configuring the UE to monitor a subset of configured SPS in a group of configured SPS. Particularly, that is in a configured group of NSPS SPS (SPS Group), the UE needs only monitor configured MSPS SPS resources within a period, where MSPS < NSPS.  The value of MSPS and the exact SPS in the group of SPS that are monitored can be different in different period.  This recognizes that in quasi-static periodic traffic, typically only one of configured SPS in a group of over configured SPS resources, contains actual PDSCH as shown in Figure 5 and Figure 6 above.

In Figure 7, in the figure on the left side, is the case when the UE only monitor SPS#4 and SPS#5. In this case, it would cover most of transmission affected by the jitter (if e.g. we assume some Gaussian distribution of the jitter). However, it does not cover the case with small probability when the transmission occurs at the outside of jitter standard deviation. The UE can also be configured to monitor the last configured SPS, in this case the UE monitors SPS#4, SPS#5, and SPS8. Furthermore, the UE can stop monitoring the remaining SPS resources in case the UE receives the packet in the earlier SPS. For example, if the UE receives the packet in SPS#4 then the UE stops monitoring the remaining SPS (i.e.. SPS#5, and SPS#8).

Figure 7: Illustration of UE monitoring a subset of configured SPS

Proposal 6: Consider the UE to monitor a subset of configured SPS in a group of configured SPS to handle the jitter of XR traffic.

# Summary

In this contribution, we have discussed our view on potential mechanisms to improve capacity, including SPS and CG enhancements for the transmission of XR service in NR. Our observation and proposals are listed below:

Observation 1: Depending on the XR traffic pattern, there can be a mismatch of periods between NR SPS/CG configuration and XR traffic.

Observation 2: The search space size for PDCCH transmission can be relatively small. Hence, it would be beneficial to reduce the PDCCH transmission which can also increase the overall XR capacity.

Proposal 1: SPS enhancement can still be considered for XR capacity enhancements as the search space size for PDCCH transmission can be relatively small

Proposal 2: Consider Dynamic SPS/CG configuration to dynamically adjust the transmission in order to accommodate the XR traffic pattern.

Proposal 3: Pseudo-periodic SPS/CG configuration can be considered for XR traffic with non-integer period.

Proposal 4: Multi CS-RNTIs configuration for multi flows in XR can be considered.

Proposal 5: A new SPS type configuration that is similar to CG type I configuration can be considered to support multi-flows in XR.

Observation 3: SPS enhancements are required to support XR services with the benefit to improve the capacity and minimize unnecessary UE power consumption.

Proposal 6: Consider the UE to monitor a subset of configured SPS in a group of configured SPS to handle the

# References

- RP-213587, “Study on XR Enhancements for NR”, RANP #94e, December 2021

- TR38.838, “Study on XR (Extended Reality) Evaluations for NR”

- SP-210043, SA4

- SP-220705, SA2

- R1-2205056, “Capacity Enhancement Techniques for XR”, Qualcomm, RAN1#109e

- Minutes RAN2#119e meeting

- Minutes RAN2#119bis-e meeting