3GPP TSG-RAN WG2 Meeting #120						R2-2211931

Toulouse, France, 14-18th Nov, 2022

Agenda Item:	8.8.2

Source: 	Sony

Title:	Considerations about UAV mobility and user consent

Document for:	Discussion

# Introduction

RAN2_119e agreements:

1 Use LTE principle as a baseline, introduce similar event H1 (aerial UE height become higher than threshold) and H2 (aerial UE height become lower than threshold. FFS if further NR enhancements are needed. FFS study scaling of RRM parameters (e.g. which parameters and what is the purpose/benefit of the scaling and how) FFS how to limit excessive measurements and measurement reporting FFS if user consent is needed for location reporting in CONNECTED FFS study the vertical movement and associated mobility for UAV UEs

RAN2_119bise agreements:

1. The time information reported as part of flight path plan is optional. UE includes time info, if configured by the network and available at the UE. FFS on flight path details (waypoints and what is time information). 2. Allow the flight path to be updated. FFS on the details. 3. FFS on reporting format and initial flight path reporting (i.e. what information to report and how) – next meeting 4. Continue to study height-depending scaling, triggering and combinations 5. As in LTE, as a baseline, events A3, A4 and A5 can be configured with the configured number of cells (numberofTriggeringCells)

In this contribution we look into FFS from last meetings and especially the need for separate RRM parameters and user consent for location reporting.

# Discussion

UAV mobility and measurement reporting

RAN2_119e agreed to support LTE events H1 and H2 for UAV in NR. There is FFS if further NR enhancements are needed. FFS study scaling of RRM parameters (e.g. which parameters and what is the purpose/ benefit of the scaling and how).

A UAV UE may be handed over to a cell supporting UAV features once UAV is hovering above a certain height (i.e. event H1). This special cell may configure UAV specific parameters. Eventually, UAV UE may be handed over to a terrestrial cell once it is hovering below a certain height (i.e. event H2).

Once handed over to a cell supporting UAV or UAV UE is hovering above event H1, different values of Time-To-Trigger can be used based on the altitude of UAV. Network may configure to use normal values when UAV is near to earth surface (event H2) and a shorter/longer values when UAV is hovering above a certain altitude (event H1) encountering multiple cells.

However, when UAV UE is lifting off from ground and before it gains the altitude to trigger event h1, we think terrestrial UE conditions should be sufficient.

Proposal 1: Allow different values of TTT being configured based on if UE has already triggered event H1/H2. No special treatment needed before UE attains the altitude for triggering event H1.

User consent

User consent was discussed at length during NTN discussions in Rel-17 and based on the latest LS from SA3, in our understanding, user consent is left for the network entities to handle. A UE shall provide its location if network configured to provide location information. We think that RAN2 can work on this working assumption unless there is a new input from SA3 in Rel-18.

Proposal 2: RAN2 does not work on User consent i.e. UE provide its location if configured by the network to do so.

# Conclusion

We propose RAN2 to agree on the following proposal:

Proposal 1: Allow different values of TTT being configured based on if UE has already triggered event H1/H2. No special treatment needed before UE attains the altitude for triggering event H1.

Proposal 2: RAN2 does not work on User consent i.e. UE provide its location if configured by the network to do so.

# References

[1]	RP-222171: New WID: Revised WID: NR Support for UAV (Uncrewed Aerial Vehicles)