3GPP TSG-RAN WG2 Meeting #120						R2-2211939

Toulouse, France, 14-18th November, 2022

Agenda Item:	8.16.2

Source: 	Sony

Title:	Some considerations about model ID and CP/UP solution

Document for:	Discussion

# Introduction

RAN2_119bise made following agreements on AI/ML:

- R2 assumes that for the existing (under discussion) AI/ML use cases, proprietary models may be supported and/or open format may be supported (and maybe RAN2 doesn’t have to further elaborate on this assumption). - R2 assumes that from Management or Control point of view mainly some meta info about a model may need to be known, details FFS. - R2 assumes that a model is identified by a model ID. Its usage is FFS. - General FFS: AIML Model delivery to the UE may have different options, Control-plane (multiple subvariants), User Plane, can be discussed case by case

In this contribution, we provide our views on model ID and CP/UP solutions.

# Discussion

Model ID:

RAN2_119bise assumed that the concept of model ID is useful for AI/ML whereby a model is identified by a model ID. Its usage is FFS.  RAN1 further agreed that multiple models can be introduced for one use case. This implies that e.g., UE may support more than one model for CSI enhancements or beam management use case. Then it is not clear if that model remains valid for one connection, or it may also be possible that the model can be changed during the lifetime of a connection.

We think that the concept of model ID allows the flexibility to switch between models during a single connection time provided there are no issues related to model training, UE hardware being capable of switching between models and UE hardware supports both/multiple models during runtime. For model training, either the new model is already trained offline, or it is possible to perform further model training online.

Proposal 1: RAN2 to discuss if Model ID change can occur during the same RRC connection and depending on certain conditions/criteria (e.g.,UE capability, highest model configuration supported by UE hardware, model training requirements etc.)

Our assumption in proposal 1 is that all models belong to the same coordination level. The next question is if for a use case, N models are supported and then is it possible that these models are a combination of x/y/z level of coordination or all models belong to one coordination level i.e., either x or y or z? We think limiting a use case to a particular level of coordination framework is too restrictive in general. For example, positioning use cases may require support of all x, y, and z level of coordination, and CSI compression may require at least two-sided AI model (y or z). But there may be use cases where only one level of coordination is sufficient.

Proposal 2: RAN2 to discuss if model IDs for one use case could belong to different coordination levels x, y, z.

Model ID specifications should be kept to a minimum so that it meets the requirements from both UE and NW vendors where at least model coordination and/or signalling is required. For example, network vendors may be interested to support AI/ML model training and inference for large number of UEs whereas UE vendors may be interested in UE power consumption, and this may require reduced processing and/or signalling. One option is that as part of model transfer or signalling procedure, a matrix with weights could be standardised and details like e.g., type of NN are not standardised so that proprietary models can be deployed.

Proposal 3: RAN2 to discuss the details of model transfer while addressing objectives of both UE and NW vendors.

CP vs UP solution:

Regarding CP vs UP solutions, we think it boils down to the choice between SRB4-like carrying the AI/ML model transfer and associated signalling & the support for open AI/ML models, if RAN2 decides to go for UP based solution. In our understanding, UP based solution may allow third party AI/ML models to be developed and delivered over 3GPP networks, including AI/ML models for 3GPP RAN. For user plane a separate DRB which will carry the traffic from RAN to UE and vice versa will be required i.e., a DRB not mapped to a PDU session/QoS flow and DRB characteristics need to be defined. Max number of such DRBs also needs to be defined.

Proposal 4: RAN2 to capture the comparison between CP and UP solutions for selection at a later stage.

# Conclusion

We propose RAN2 to discuss and agree on the following proposal:

Proposal 1: RAN2 to discuss if Model ID change can occur during the same RRC connection and depending on certain conditions/criteria (e.g.,UE capability, highest model configuration supported by UE hardware, model training requirements etc.)

Proposal 2: RAN2 to discuss if model IDs for one use case could belong to different coordination levels x, y, z.

Proposal 3: RAN2 to discuss the details of model transfer while addressing objectives of both UE and NW vendors.

Proposal 4: RAN2 to capture the comparison between CP and UP solutions for selection at a later stage.

# References

[1]	RP-213599, Qualcomm (Moderator), “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” RAN #94e, Dec. 2021