3GPP TSG-RAN WG2 Meeting #120	R2-2211951

Toulouse, France, November 14 – 18, 2022

Agenda Item:	8.15.2

Source: 	Xiaomi

Title:  	Discussion on channel access for sidelink operation on unlicensed spectrum

Document for:	Discussion and Decision

1	Introduction

In last RAN2 meeting, RAN2 discussed the CAPC for SL-U and achieved the following agreements [1].

Agreement on CAPC:

1: 	Working assumption: PQI is used to determine the CAPC mapping as in NR-U. FFS whether the same principle is also applied to the UE side.

2:	For SL-DRB the CAPC value is (pre)configurable per-DRB as in NR-U.

3:	For all SL-SRBs, CAPC value is fixed to the highest priority (i.e., lowest CAPC value).

4:	If PQI-based CAPC mapping is agreed, for all SL MAC CEs, CAPC value is fixed to the highest priority (i.e., lowest CAPC value).

5:	If PQI-based CAPC mapping is agreed, at least PDB can be used as the criterion to determine the CAPC mapping. FFS if any other additional criterions needed.

6:	As in NR-U, if SL CAPC is determined based on PQI, as a baseline, for non-standardized PQI, to use the CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized PQI. FFS if any specific work needed for RRC_INACTIVE/RRC_IDLE/OOC UEs.

7:	If PQI-based CAPC mapping is agreed, as in NR-U, to determine the CAPC of the SL TB when the CAPC is not indicated in the DCI:

- If only SL MAC CE(s) are included in the SL TB, the highest priority SL CAPC is used; FFS whether this rule can be extended to the case when SL MAC CE(s) multiplexed with STCH.

- If SCCH SDU(s) are included in the SL TB, the highest priority SL CAPC is used;

- FFS how to select SL CAPC when SL CAPC of the SL logical channel(s) with MAC SDU multiplexed in the SL TB is used otherwise.

In this contribution, we would like to discuss some remaining issues and provide the corresponding proposals.

2	Discussion

CAPC to PQI mapping table

During last meeting, there was some discussion on the CAPC mapping table. A majority of companies support to reuse NR-U solution, i.e., CAPC to PQI mapping, however some companies suggest to introduce CAPC to priority mapping. Actually we think the intention to define this table is to assist the NW to (pre-)configure proper CAPC value for a DRB taking the QoS of all the flows associated with the same DRB into account. From this perspective, we think PQI which can reflect the QoS characteristics is more aligned with the intention. Therefore, we suggest RAN2 to confirm the WA as agreement.

Proposal 1: Confirm the WA “PQI is used to determine the CAPC mapping as in NR-U” as agreement.

In addition, there was some discussion on whether this table can also be applied to the UE or not. Some companies think this mapping table may only apply to the network side, UEs can always rely on configuration from dedicated RRC or SIB or pre-configuration to determine the CAPC of a certain DRB. There is no use cases for the UEs to check the CAPC table and configures the CAPC.

However we have some different understanding. For UEs operating in RRC connected mode, the network is informed of the PQI or QoS parameters for a QoS flow based on the reported SUI, then the network can take the QoS of all the flows associated with the same DRB into account to configure a proper CAPC for the DRB. While for UEs operating in RRC IDLE/INACTIVE mode or OOC, there is no such assistance information and if a DRB includes QoS flows that have not been reported to the network before, the configured CAPC in SIB/pre-configuration may not match the DRB well. In this case, a more suitable solution is to rely on the UE itself to check the CAPC table and determines the CAPC for the DRB when the UE is in RRC IDLE/INACTIE or OOC. Therefore, if the network has a good knowledge of the associated QoS flows multiplexed into the same DRB, then the gNB can include the CAPC in the DRB configuration and UE should follow the configured CAPC. However if the CAPC is not included in the DRB configuration, then the UE itself needs to check the table and determines the CAPC for the DRB.

Proposal 2: In SL-U, the CAPC mapping table also applies to the UE side.

Proposal 3: In SL-U, the RRC IDLE/INACTIVE or OOC UE should apply the configured CAPC for the DRB if CAPC is configured in SIB12 or pre-configuration.

Proposal 4: In SL-U, the RRC IDLE/INACTIVE or OOC UE checks the mapping table and determines the CAPC itself for the DRB if CAPC is not configured in SIB12 or pre-configuration.

The next issue is the detailed mapping between different PQI and CAPC. Actually in last RAN2 meeting, we have agreed PDB should be considered as one criterion to define the mapping. In addition, during last RAN1 meeting, RAN1 has agreed that UL CAPC table is supported for Type 1 SL-U channel access which means 4 CAPC priority values are supported.

Besides PDB, we think priority should also be considered as one criterion when determining the mapping relationship. As indicated in the Annex, there are some services with quite restrictive PDB requirement (i.e, 5ms, 10ms, 20ms) but the corresponding priority is the lowest (i.e., 5, 6). Therefore if only PDB is considered, then these services will be categorized as the highest CAPC priority, but actually will be pre-empted or dropped during resource selection due to the lowest priority.

Proposal 5: In SL-U, both PDB and priority should be considered when determining the PQI to CAPC mapping.

According to the tables for PQI, if we categorize the PQI based on both PDB and priority:

- For PQI 21/55/90/91, they have quite stringent PDB requirement and quite high priority, so should be categorized as CAPC class 1;

- For PQI 56/57/92/93, they have quite stringent PDB requirement but quite low priority, so should be categorized as CAPC class 2.

- For PQI 22/23/58, they have relatively loose PDB requirement and medium priority, so should be categorized as CAPC class 2.

- For PQI 24/25/26, they have relatively loose PDB requirement but quite high priority, so should be categorized as CAPC class 2.

- For PQI 60, it has relatively loose PDB requirement but the highest priority, so should be categorized as CAPC class 1.

- For PQI 59/61, they have quite loose PDB requirement and quite low priority, so should be categorized as CAPC class 3.

Based on the above analysis, we propose RAN2 to adopt the following CAPC to PQI mapping table.

| CAPC | PQI |
| --- | --- |
| 1 | 21, 55, 60, 90, 91 |
| 2 | 22, 23, 24, 25, 26, 56, 57, 58, 92, 93 |
| 3 | 59, 61 |
| 4 | - |
| NOTE: lower CAPC value means higher priority - |  |

Table 1

Proposal 6: In SL-U, adopt the above table as the mapping table between CAPC and PQI.

Handling of non-standardized PQI

Regarding the handling of non-standardized PQI, there was some concern on UEs operating in RRC IDLE/INACTIVE or OOC. However, if we agree with the previous proposal, i.e., allow the RRC IDLE/INACTIVE or OOC UE to check the table and determine the CAPC if CAPC is not configured in SIB12 or pre-configuration, if the DRB contains a QoS flow with non-standardized PQI, UE can just determine a standardized PQI that best matches the QoS characteristics of the non-standardized PQI and use the CAPC of that standardized PQI based on the mapping table. UE takes into account of the CAPC of all the QoS flows multiplexed in the DRB to determine a proper CAPC for this DRB according to the table, which can be left to UE implementation.

Proposal 7: In SL-U, the RRC IDLE/INACTIVE or OOC UE uses the CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized PQI when determining the CAPC for a DRB that includes QoS flows with non-standardized PQI.

CAPC selection and multiplexing of data in SL-U

During last meeting, we have reached the following agreement on how to determine the CAPC of the TB if CAPC is not indicated in DCI.

7:	If PQI-based CAPC mapping is agreed, as in NR-U, to determine the CAPC of the SL TB when the CAPC is not indicated in the DCI:

- If only SL MAC CE(s) are included in the SL TB, the highest priority SL CAPC is used; FFS whether this rule can be extended to the case when SL MAC CE(s) multiplexed with STCH.

- If SCCH SDU(s) are included in the SL TB, the highest priority SL CAPC is used;

- FFS how to select SL CAPC when SL CAPC of the SL logical channel(s) with MAC SDU multiplexed in the SL TB is used otherwise.

However there was some concern on whether to introduce some enhancement on the other cases or not. Actually some companies suggest to remove “only” from the first bullet since we have agreed to fix the CAPC of the SL MAC CEs as the highest CAPC priority. However, we think the CAPC of the SL MAC CE itself and the CAPC of the TB are two different things. Even though the CAPC of the SL MAC CE is fixed to the highest priority, when the SL MAC CEs are multiplexed with MAC SDUs with the lowest CAPC priority, it is not fair and not aligned with the regulations if the CAPC of the TB is always set to the highest priority. Therefore, for the first FFS, if SL MAC CEs are multiplexed with STCH, UE should not always set the CAPC of the TB as the highest priority.

Proposal 8: In SL-U, always using the highest CAPC priority cannot be extended to the case when SL MAC CE(s) are multiplexed with STCH.

While for the second FFS, i.e., how to determine the CAPC of the TB if only MAC SDUs are included. Originally in NR-U, it was agreed to select the highest CAPC value (lowest priority) of LCHs multiplexed in a TB as highlighted in green. However, some companies had concern that the selection of the lowest priority CAPC for a MAC PDU which multiplexes different LCHs is not optimal since high priority data will be subject to delay when lower priority data is multiplexed in the same MAC PDU. Therefore, some companies proposed to introduce some restrictions during LCP, i.e., LCHs with low priority CAPC are not allowed to be multiplexed with LCHs having higher CAPC in the same TB. But finally companies think this issue was just quite corner since for CG of which the traffic type is quite predictable with well-known periodicities and data sizes, the issue is not that serious. Therefore, the final conclusion for CAPC selection for CG is to select the highest CAPC value (lowest priority) of LCHs multiplexed in a TB without any restriction on multiplexing of data defined.

When performing Type 1 LBT for the transmission of an uplink TB (see TS 37.213 [37], clause 4.2.1.1) and when the CAPC is not indicated in the DCI, the UE shall select the CAPC as follows: - If only MAC CE(s) are included in the TB, the highest priority CAPC of those MAC CE(s) is used; or - If CCCH SDU(s) are included in the TB, the highest priority CAPC is used; or - If DCCH SDU(s) are included in the TB, the highest priority CAPC of the DCCH(s) is used; or - The lowest priority CAPC of the logical channel(s) with MAC SDU multiplexed in the TB is used otherwise.

When it comes SL-U, we think similar principle can be adopted as well, this is because for transmission on CG in SL-U, similar as in NR-U, we can rely on NW to configure multiple CG configurations to comply different types of such kind of predictable traffic. While for mode 2, we think the handling of CAPC can be similar as the handling of resource selection, i.e., leave this to UE implementation to avoid multiplex high priority data with low priority data without introducing any restriction on LCP or multiplexing. Actually for IUC scheme, we have already had similar discussion on how to ensure the generated MAC PDU is for the targeted receiver which sends the preferred resource set and the conclusion is to have no update on LCP but leave this to UE implementation. We think similar handing can be adopted here, it is up to UE implementation to ensure the LCH with highest CAPC priority are not multiplexed together with the LCH with lowest CAPC priority. Therefore, we think for case when SL MAC CE are multiplexed with SL MAC SDUs and the case when only SL MAC SDUs are multiplexed, UE shall select the lowest priority CAPC of the logical channel(s) with MAC SDU multiplexed in the TB similar as that in NR-U.

Proposal 9: In SL-U, UE shall select the lowest priority CAPC of the logical channel(s) with MAC SDU when SL MAC CEs and SL MAC SDUs are multiplexed in the TB.

Proposal 10: In SL-U, UE shall select the lowest priority CAPC of the logical channel(s) with MAC SDU when only SL MAC SDUs are multiplexed in the TB.

COT sharing in SL-U

For COT sharing in SL-U, RAN1 has discussed about this and agreed to support UE-to-UE COT sharing with some details FFS:

- UE-to-UE COT sharing is supported in NR sidelink operation in a shared channel (SL-U). - FFS applicable SL channels and signals (e.g., PSCCH/PSSCH, PSFCH, S-SSB) for shared COT access and any restrictions (e.g. whether the COT can be shared with a single UE or multiple UEs) - FFS all other details in compliance with the regulatory requirements

In addition, during last RAN1 meeting, the following agreements were achieved for UE-to-UE COT sharing.

- For UE-to-UE COT sharing, continue considering the following alternatives: - Alt. 1: A responding SL UE can utilize a COT shared by a COT initiating UE when the responding SL UE is a target receiver of the at least COT initiating UE’s PSSCH data transmission in the COT. - When the responding UE uses the shared COT for its transmission has an equal or smaller CAPC value than the CAPC value indicated in a shared COT information - FFS any additional conditions - Alt. 2: A responding SL UE can utilize a COT shared by a COT initiating UE when the responding SL UE is a target receiver of the COT initiating UE’s transmission in the COT. - When the responding UE uses the shared COT for its transmission has an equal or smaller CAPC value than the CAPC value indicated in a shared COT information - FFS how to determine a SL UE is a target receiver - FFS: details of the channel type of the COT initiating UE’s transmission - FFS any additional conditions - For Alt1 and Alt2: When a responding UE uses a shared COT for its transmission(s), the COT initiating UE is a target receiver of the responding UE’s transmission(s). - FFS: details of the channel type of the responding UE’s transmission(s) - gNB relaying/forwarding a UE initiated COT to another UE is not supported in Rel-18 - FFS whether a Mode 1 UE can report a COT or related information to gNB for aiding Mode 1 RA

There may be some impact on RAN2 to support UE-to-UE COT sharing as listed in the above RAN1 agreement, e.g., whether a Mode 1 UE can report a COT or related information to gNB for aiding Mode 1 RA. However from RAN2 point of view, we propose to wait for more RAN1 input before we start the corresponding discussion on UE-to-UE COT sharing.

Proposal 11: In SL-U, RAN2 wait for more RAN1 input before we start the corresponding discussion on UE-to-UE COT sharing.

3	Conclusion

In this contribution, we discussed about CAPC for sidelink operation on unlicensed spectrum and have the corresponding proposals:

Proposal 1: Confirm the WA “PQI is used to determine the CAPC mapping as in NR-U” as agreement.

Proposal 2: In SL-U, the CAPC mapping table also applies to the UE side.

Proposal 3: In SL-U, the RRC IDLE/INACTIVE or OOC UE should apply the configured CAPC for the DRB if CAPC is configured in SIB12 or pre-configuration.

Proposal 4: In SL-U, the RRC IDLE/INACTIVE or OOC UE checks the mapping table and determines the CAPC itself for the DRB if CAPC is not configured in SIB12 or pre-configuration.

Proposal 5: In SL-U, both PDB and priority should be considered when determining the PQI to CAPC mapping.

Proposal 6: In SL-U, adopt the above table as the mapping table between CAPC and PQI.

Proposal 7: In SL-U, the RRC IDLE/INACTIVE or OOC UE uses the CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized PQI when determining the CAPC for a DRB that includes QoS flows with non-standardized PQI.

Proposal 8: In SL-U, always using the highest CAPC priority cannot be extended to the case when SL MAC CE(s) are multiplexed with STCH.

Proposal 9: In SL-U, UE shall select the lowest priority CAPC of the logical channel(s) with MAC SDU when SL MAC CEs and SL MAC SDUs are multiplexed in the TB.

Proposal 10: In SL-U, UE shall select the lowest priority CAPC of the logical channel(s) with MAC SDU when only SL MAC SDUs are multiplexed in the TB.

Proposal 11: In SL-U, RAN2 wait for more RAN1 input before we start the corresponding discussion on UE-to-UE COT sharing.

4	Reference

[1] RAN2#119bis-e meeting minutes.

5 Annex: Standardized PQI defined by TS23.287/TS23.304

Table 5.4.4-1: Standardized PQI to QoS characteristics mapping

| PQI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 21 | GBR | 3 | 20 ms | 10-4 | N/A | 2000 ms | Platooning between UEs – Higher degree of automation; Platooning between UE and RSU – Higher degree of automation |
| 22 | (NOTE 1) | 4 | 50 ms | 10-2 | N/A | 2000 ms | Sensor sharing – higher degree of automation |
| 23 |  | 3 | 100 ms | 10-4 | N/A | 2000 ms | Information sharing for automated driving – between UEs or UE and RSU - higher degree of automation |
| 55 | Non-GBR | 3 | 10 ms | 10-4 | N/A | N/A | Cooperative lane change – higher degree of automation |
| 56 |  | 6 | 20 ms | 10-1 | N/A | N/A | Platooning informative exchange – low degree of automation; Platooning – information sharing with RSU |
| 57 |  | 5 | 25 ms | 10-1 | N/A | N/A | Cooperative lane change – lower degree of automation |
| 58 |  | 4 | 100 ms | 10-2 | N/A | N/A | Sensor information sharing – lower degree of automation |
| 59 |  | 6 | 500 ms | 10-1 | N/A | N/A | Platooning – reporting to an RSU |
| 90 | Delay Critical GBR | 3 | 10 ms | 10-4 | 2000 bytes | 2000 ms | Cooperative collision avoidance; Sensor sharing – Higher degree of automation; Video sharing – higher degree of automation |
| 91 | (NOTE 1) | 2 | 3 ms | 10-5 | 2000 bytes | 2000 ms | Emergency trajectory alignment; Sensor sharing – Higher degree of automation |
| NOTE 1: GBR and Delay Critical GBR PQIs can only be used for unicast PC5 communications. |  |  |  |  |  |  |  |

NOTE 1:	For Standardized PQI to QoS characteristics mapping, the table will be extended/updated to support service requirements for other identified V2X services.

NOTE 2:	The PQIs may be used for other services than V2X.

NOTE 3:	A PQI may be used together with an application indicated priority, which overrides the Default Priority Level of the PQI.

Table 5.6.1-1: Standardized PQI values that are additionally defined to QoS characteristics mapping

| PQI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | GBR (NOTE 1) | 1 | 150 ms | 10-2 | N/A | 2000 ms | Mission Critical user plane Push To Talk voice (e.g. MCPTT) |
| 25 |  | 2 | 200 ms | 10-2 | N/A | 2000 ms | Non-Mission-Critical user plane Push To Talk voice |
| 26 |  | 2 | 200 ms | 10-3 | N/A | 2000 ms | Mission Critical Video user plane |
| 60 | Non-GBR | 1 | 120 ms | 10-6 | N/A | N/A | Mission Critical delay sensitive signalling (e.g. MC-PTT signalling) |
| 61 |  | 6 | 400 ms | 10-6 | N/A | N/A | Mission Critical Data (e.g. example services are the same as 5QI 6/8/9 as specified in TS 23.501 [4]) |
| 92 | Delay Critical GBR (NOTE 1) | 5 | 5ms | 10-4 | 20000 bytes | 2000 ms | Interactive service - consume VR content with high compression rate via tethered VR headset (See TS 22.261 [6]) |
| 93 |  | 6 | 10ms | 10-4 | 20000 bytes | 2000 ms | interactive service - consume VR content with low compression rate via tethered VR headset; Gaming or Interactive Data Exchanging (See TS 22.261 [6]) |
| NOTE 1: GBR and Delay Critical GBR PQIs can only be used for unicast PC5 communications. |  |  |  |  |  |  |  |