3GPP TSG-RAN WG2 Meeting #120	R2-2211952

# Toulouse, France, November, 2022

Source:	TCL Communication

Title:	Discussion on SR configuration for XR uplink traffic transmission

Agenda item:	8.5.4.2

Document for:	Discussion and decision

- Introduction

In the RAN1 #110 and RAN2 #119 meeting in August, XR Enhancements for NR was discussed and some agreements as follows respectively:

RAN2 agreement:

- Enhancement to SPS/CG should be justified for XR scheduling and should be evaluated against dynamic grant (DG) scheduling which should be considered as baseline. Should justify why enhancements are needed.

Based on the agreement, DG is the baseline scheduling scheme. As well known, a basic DG procedure includes:

- SR for the uplink transmission request;

- A followed DG for the BSR transmission and part of the uplink traffic data;

- A further DG for the rest uplink traffic data;

In this contribution, we will discuss the SR configuration for XR uplink traffic transmission.

- Discussion

  - The periodicity mismatch between SR configuration and XR uplink video traffic

Up to now, we have discussed much about the issues and solutions for the periodicity mismatch between C-DRX configuration and XR downlink video traffic to decrease the UE power consumption and the additional delay. However, the same issue could be also applicable to uplink transmission for the XR uplink video traffic since the periodicity of the SR configuration could also mismatch the non-integer periodicity of the XR uplink video traffic according to the supported periodicity for SR configuration in current specifications as follows.

SchedulingRequestResourceConfig ::= SEQUENCE { schedulingRequestResourceId SchedulingRequestResourceId, schedulingRequestID SchedulingRequestId, periodicityAndOffset CHOICE { sym2 NULL, sym6or7 NULL, sl1 NULL, -- Recurs in every slot sl2 INTEGER (0..1), sl4 INTEGER (0..3), sl5 INTEGER (0..4), sl8 INTEGER (0..7), sl10 INTEGER (0..9), sl16 INTEGER (0..15), sl20 INTEGER (0..19), sl40 INTEGER (0..39), sl80 INTEGER (0..79), sl160 INTEGER (0..159), sl320 INTEGER (0..319), sl640 INTEGER (0..639) } OPTIONAL, -- Need M resource PUCCH-ResourceId OPTIONAL -- Need M }

periodicityAndOffset SR periodicity and offset in number of symbols or slots (see TS 38.213 [13], clause 9.2.4) The following periodicities may be configured depending on the chosen subcarrier spacing: SCS = 15 kHz: 2sym, 7sym, 1sl, 2sl, 4sl, 5sl, 8sl, 10sl, 16sl, 20sl, 40sl, 80sl SCS = 30 kHz: 2sym, 7sym, 1sl, 2sl, 4sl, 8sl, 10sl, 16sl, 20sl, 40sl, 80sl, 160sl SCS = 60 kHz: 2sym, 7sym/6sym, 1sl, 2sl, 4sl, 8sl, 16sl, 20sl, 40sl, 80sl, 160sl, 320sl SCS = 120 kHz: 2sym, 7sym, 1sl, 2sl, 4sl, 8sl, 16sl, 40sl, 80sl, 160sl, 320sl, 640sl sym6or7 corresponds to 6 symbols if extended cyclic prefix and a SCS of 60 kHz are configured, otherwise it corresponds to 7 symbols. For periodicities 2sym, 7sym and sl1 the UE assumes an offset of 0 slots.

Due to the periodicity mismatch between SR configuration and XR uplink video traffic, additional delay could be incurred for part of the XR uplink video traffic transmission. The additional delay is similar to that for the XR downlink video traffic transmission due to the periodicity mismatch between C-DRX configuration and XR downlink video traffic. The solution for the periodicity mismatch between SR configuration and XR uplink video traffic could also refer to that for the periodicity mismatch between C-DRX configuration and XR downlink video traffic.

Observation 1: The issues for the periodicity mismatch between SR configuration and XR uplink video traffic could be similar to that for the periodicity mismatch between C-DRX configuration and XR downlink video traffic.

Proposal 1: RAN2 to study periodicity mismatch between SR configuration and XR uplink video traffic.

  - The occurrence alignment between SR and XR uplink video traffic

The SR resources could be configured without any information about the XR service before the XR service session establishment, especially the start time of the uplink XR video traffic. As a result, the occurrence time between the SR resources and the XR uplink video traffic could be misaligned as illustrated in Figure 1. The offset due to the misalignment would incur additional delay for SR transmission for the XR uplink video traffic.

On the other hand, only UE can be aware of the offset accurately. So UE could measure and report the offset as part of the UE assistant information to gNB to decrease the misalignment.

Observation 2: The misalignment between SR and XR uplink video traffic could incur additional delay of SR transmission for the XR uplink video traffic.

Proposal 2: RAN2 to study the alignment between SR and XR uplink video traffic.

Figure 1

  - The SR configuration and the uplink jitter

As well known, the uplink jitter is much less than downlink jitter for XR video traffic. However, the jitter could still be incurred from codec and compression. As a result, some of the configured SRs could be missed even if the SR configurations matches the XR uplink video traffic transmission very well and additional delay could be incurred consequently as illustrated in Figure 2. This is similar to the additional delay for XR downlink video traffic transmission incurred by the missed on-duration of the C-DRX because of the downlink jitter.

Observation 3: The uplink jitter could incur additional delay of SR transmission for the XR uplink video traffic.

Proposal 3: RAN2 to study the SR configuration for the uplink jitter.

Figure 2

  - The SR configuration and the measurement gap

For the XR uplink video traffic transmission, the overlaps and conflicts with the measurement gap are not only the PUSCH transmissions for traffic date and/or BSR, but also the SR transmissions which could further impact the PUSCH transmissions. In current specification, the SR is always considered as low priority compared to the measurement gap, however that could severely impact XR capacity due the additional delays or abnormal traffic data discards incurred by the overlaps and conflicts.

Observation 4: The overlap and conflict between SR transmission for XR uplink video traffic and measurement gap could impact the XR capacity severely.

Proposal 4: RAN2 to study the overlap and conflict between SR transmission for XR uplink video traffic and measurement gap.

- Conclusion

In this contribution, we provided our observations and proposal on SR configuration for XR uplink traffic as follows:

Observation 1: The issues for the periodicity mismatch between SR configuration and XR uplink video traffic could be similar to that for the periodicity mismatch between C-DRX configuration and XR downlink video traffic.

Proposal 1: RAN2 to study periodicity mismatch between SR configuration and XR uplink video traffic.

Observation 2: The misalignment between SR and XR uplink video traffic could incur additional delay of SR transmission for the XR uplink video traffic.

Proposal 2: RAN2 to study the alignment between SR and XR uplink video traffic.

Observation 3: The uplink jitter could incur additional delay of SR transmission for the XR uplink video traffic.

Proposal 3: RAN2 to study the SR configuration for the uplink jitter.

Observation 4: The overlap and conflict between SR transmission for XR uplink video traffic and measurement gap could impact the XR capacity severely.

Proposal 4: RAN2 to study the overlap and conflict between SR transmission for XR uplink video traffic and measurement gap.

- References

- RP-222663, “SR for XR SI”, Nokia, RAN#97-e