3GPP TSG-RAN WG2 Meeting #120	R2-2212023

Toulouse, 14th – 18th November 2022

Agenda item:	   8.16.2

Source:		Lenovo

Title:		General issues on AI for air interface

Document for:		Discussion and Decision

# Introduction

In the past RAN1#109, #110, #110bis meetings, the following agreements have been made with respect to the general AI/ML for air interface framework. In this paper, we discuss the possible RAN2 impacts.

RAN1#109 Agreements: As indicated in SID, although specific AI/ML algorithms and models may be studied for evaluation purposes, AI/ML algorithms and models are implementation specific and are not expected to be specified. - RAN1 discussion should focus on network-UE interaction. - AI/ML functionality mapping within the network (such as gNB, LMF, or OAM) is up to RAN2/3 discussion. Take the following network-UE collaboration levels as one aspect for defining collaboration levels - Level x: No collaboration - Level y: Signaling-based collaboration without model transfer - Level z: Signaling-based collaboration with model transfer Note: Other aspect(s), for defining collaboration levels is not precluded and will be discussed in later meetings, e.g., with/without model updating, to support training/inference, for defining collaboration levels will be discussed in later meetings FFS: Clarification is needed for Level x-y boundary RAN1#110 Agreements: Study the following aspects, including the definition of components (if needed) and necessity, in Life Cycle Management - Data collection - Note: This also includes associated assistance information, if applicable. - Model training - [Model registration] - Model deployment - Note: Terminology is to be defined. - [Model configuration] - Model inference operation - Model selection, activation, deactivation, switching, and fallback operation - Model monitoring - Model update - Note: Terminology is to be defined. This includes model finetuning, retraining, and re-development via online/offline training. - Model transfer - UE capability Note: Some aspects in the list may not have specification impact. Note: Aspects with square brackets are tentative. Note: More aspects may be added as study progresses. RAN1#110bis Agreements: Working Assumption - Define Level y-z boundary based on whether model delivery is transparent to 3gpp signalling over the air interface or not. - Note: other procedures than model transfer/delivery are decoupled with collaboration level y-z - Clarifying note: Level y includes cases without model delivery. Agreement Clarify Level x/y boundary as: - Level x is implementation-based AI/ML operation without any dedicated AI/ML-specific enhancement (e.g., LCM related signalling, RS) collaboration between network and UE. (Note: The AI/ML operation may rely on future specification not related to AI/ML collaboration. The AI/ML approaches can be used as baseline for performance evaluation for future releases.) Agreement Study LCM procedure on the basis that an AI/ML model has a model ID with associated information and/or model functionality at least for some AI/ML operations when network needs to be aware of UE AI/ML models FFS: Detailed discussion of model ID with associated information and/or model functionality. FFS: usage of model ID with associated information and/or model functionality based LCM procedure FFS: whether support of model ID FFS: the detailed applicable AI/ML operations Agreement For model selection, activation, deactivation, switching, and fallback at least for UE sided models and two-sided models, study the following mechanisms: - Decision by the network - Network-initiated - UE-initiated, requested to the network - Decision by the UE - Event-triggered as configured by the network, UE’s decision is reported to network - UE-autonomous, UE’s decision is reported to the network - UE-autonomous, UE’s decision is not reported to the network FFS: for network sided models FFS: other mechanisms

# Discussion

## 2.1	AI/ML related discussion in RAN3 and SA5

First of all, it’s worth noticing that there are AI/ML related discussions on-going in other working groups in parallel.

In particular, RAN3 has completed a study item in Rel17 regarding using AI/ML to optimize network energy saving, load balancing, and mobility optimization. RAN3 is now continuing the discussion and will specify the related network procedures in the Rel18 work item (RP-220635). RAN3 also discussed whether the AI training/inference functionality is located and currently, and the following two ways of deployment is supported:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

- AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

RP-220635 Normative work is based on the conclusions captured in TR37.817. The detailed objectives of the WI are listed as follows: - Specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Network Energy Saving, Load Balancing and Mobility Optimization. (RAN3)

AI/ML training/inference capabilities will be supported by gNB in Rel18 for network energy saving, load balancing and mobility optimization.

It is upon RAN3 discussion whether the AI/ML training/inference capabilities is located in CU or DU in a CU-DU split gNB architecture.

Besides, SA5 also started a study item in Rel18 to address the AI/ML management capabilities and management services issues, which includes the collaboration between OAM and NG-RAN (SP-211443).

SP-211443 To study the AI/ML management capabilities and management services to support/coordinate AI/ML in 5GS (3GPP management system, 5GC and NG-RAN) without disclosing or restricting the proprietary algorithm of AI/ML model, including - the use cases, potential requirements, and possible solutions for management of AI/ML capabilities for the AI/ML-enabled functions (e.g., MDA, RAN intelligence, NWDAF, etc.) in 5GS, including but not limited to the following capabilities: - Validation of AI/ML model and AI/ML-enabled function - Testing of AI/ML model and AI/ML-enabled function (before deployment) - Deployment of AI/ML model (new or updated model) and AI/ML-enabled function - Configuration of AI/ML-enabled function - Performance evaluation of AI/ML-enabled function Note: as a priority, the study will first focus on the objective 1), specifically addressing management capabilities for AI/ML model validation, testing and deployment to support the AI/ML in NG-RAN when AI/ML model training is in OAM and inference is in NG-RAN.

AI/ML management capabilities will probably be supported in Rel18/19 for OAM to deploy a trained AI/ML model to NG-RAN.

## 2.3	Network-UE collaboration for AI for air interface

RAN1 has discussed the possible collaboration between network and UE to facilitate the AI for air interface. Three levels of collaboration have been considered so far:

- Level x: No collaboration

- Level y: Signaling-based collaboration without model transfer

- Level z: Signaling-based collaboration with model transfer

For level x collaboration, in our understanding, that means both AI training and inference are operating either in network or in UE in an isolated way. One side (UE or network) does not even have to know an AI/ML approach is used by the other side (network or UE). The AI/ML approach can be regarded as a pure implementation option and has no impact on legacy Uu procedure/signalling. In that sense, level x collaboration has no RAN2 impact. Such understanding is also confirmed by the following RAN1 110bis agreement

RAN1#110bis Agreements: Agreement Clarify Level x/y boundary as: - Level x is implementation-based AI/ML operation without any dedicated AI/ML-specific enhancement (e.g., LCM related signalling, RS) collaboration between network and UE. (Note: The AI/ML operation may rely on future specification not related to AI/ML collaboration. The AI/ML approaches can be used as baseline for performance evaluation for future releases.)

Figure 1 Illustration of Level x, no AI/ML related signalling-based collaboration

For Network-UE level x collaboration (i.e., no collaboration), there is no RAN2 impact.

Comparatively, level y and z collaboration would require enhancements to the legacy Uu procedures/signalling. The difference between level y and z is whether model delivery is transparent to 3gpp signalling over the air interface or not. In other word, if the impact on 3gpp signalling is enhanced signalling for AI/ML inference/management without model transfer, it is level y. If the AI/ML is transferred or known by the 3gpp signalling, it is level z.

Working Assumption - Define Level y-z boundary based on whether model delivery is transparent to 3gpp signalling over the air interface or not.

On the other hand, for better assessment and design, RAN2 needs to understand what exact information shall be transferred between gNB and UE and what size is it, which are dependent on RAN1 progress. For example, as illustrated in our RAN1 contribution (R1-2209119), the transferred information can be for model management or inference operation.

Figure 2 Illustration of AI/ML related signalling-based collaboration

RAN2 studies the possible enhancements to support Network-UE level y collaboration (i.e., signalling-based without model transfer) and level z collaboration (i.e., signalling-based with model transfer) taking into account RAN1 progress.

## 2.4	AI/ML functionality within the network for LCM

In our understanding, the network-UE collaboration y/z is essentially for AI/ML model life cycle management as discussed in RAN1, including:

- Data collection

- Model training

- Model deployment

- Model inference operation

- Model selection, activation, deactivation, switching, and fallback operation

- Model monitoring

- Model update

- Model transfer

- UE capability

Also, as RAN1 agreed it is upon RAN2/3 to discuss the corresponding AI/ML functionality mapping within the network.

- RAN1 discussion should focus on network-UE interaction. - AI/ML functionality mapping within the network (such as gNB, LMF, or OAM) is up to RAN2/3 discussion.

Considering AI/ML for CSI feedback enhancement and Beam management are closely related to gNB implementation and UL/DL data transfer performance, while AI/ML for positioning are more relevant to LMF. They are discussed separately in the following of the section.

### 2.4.1 AI/ML functionality location for CSI feedback enhancement and beam management

General FFS: AIML Model delivery to the UE may have different options, Control-plane (multiple subvariants), User Plane, can be discussed case by case.

In the last RAN2 meeting, it has been raised and briefly discussed that the AI/ML model used for CSI feedback enhancement and beam management may be delivered in three different ways to UE, and each option means different way to implement Data collection, Model training, Model deployment, Model update, and Model transfer in terms of LCM:

- Option 1: RRC signalling, from gNB to UE

- Option 2: NAS signalling, from CN node (e.g., AMF) to UE

- Option 3: User Plane, from application server to UE

On the other hand, no matter which option is supported eventually, the AI/ML model monitoring and possible model selection, activation, deactivation, switching, and fallback operation should be performed by gNB/UE based on available measurements. Because using AI/ML based CSI feedback compression/prediction and beam prediction will directly impact the Uu interface performance (e.g., UL/DL throughput), and gNB/UE has the best knowledge about it based on its own measurement and UE report (e.g., measurement report, RLF report, BFD report etc.). This is also aligned with the following RAN1 agreement in their previous meeting.

Agreement For model selection, activation, deactivation, switching, and fallback at least for UE sided models and two-sided models, study the following mechanisms: - Decision by the network - Network-initiated - UE-initiated, requested to the network - Decision by the UE - Event-triggered as configured by the network, UE’s decision is reported to network - UE-autonomous, UE’s decision is reported to the network - UE-autonomous, UE’s decision is not reported to the network

For CSI feedback and beam management, RAN2 assumes the AI/ML model monitoring and possible selection/activation/deactivation/switch/fallback operation are performed by gNB/UE based on available measurements.

Besides, it can be noticed from the RAN1 agreement that no matter it is gNB or UE decision, gNB can be aware of the model selection, activation, deactivation, switching, and fallback decision. Either decided by gNB itself or informed by UE. This is straightforward if the AI/ML model is managed at gNB and transfers to UE (Option 1). However, if it is a CN node (Option 2) or application server (Option 3) that manages the AI/ML model, it seems necessary for the gNB to be aware of the used AI/ML model as well.

gNB can be aware of the model selection, activation, deactivation, switching, and fallback based on its own decision or based on UE report according to RAN1 agreement.

If the AI/ML model is transferred to UE from CN node or application server, RAN2 studies how can gNB be aware of the AI/ML model usage for the sake of model monitoring and possible selection/activation/deactivation/switch/fallback.

Besides, UE-sided model and two-sided model, gNB may also use a network-sided model for CSI feedback enhancement and beam management. In this case, in our understanding, it is also possible that the network-sided model is trained by OAM or CN node and deployed at the gNB. Although there is no RAN2 impact, it is also suggested for RAN2 to confirm such understanding and involve RAN3 in WI if needed.

RAN2 assumes gNB may receive network-sided model for CSI feedback enhancement and beam management from OAM or CN node. Upon RAN3 decision.

### 2.4.2 AI/ML functionality location for positioning

With respect to AI/ML based positioning, the situation is a bit different since it is essentially the LMF that collaborates with UE and gNB. In the last RAN1 meeting, the following cases are identified for further study.

Agreement - Study and provide inputs on benefit(s) and potential specification impact at least for the following cases of AI/ML based positioning accuracy enhancement - Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning - Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

RAN1 also defined two types of AI/ML based positioning:

- Direct AI/ML positioning: the output of AI/ML model inference is UE location

- AI/ML assisted positioning: the output of AI/ML model inference is new measurement and/or enhancement of existing measurement

In our understanding, direct AI/ML positioning can be regarded as one type of positioning method, and LMF should be aware of the AI/ML model usage.

If it is UE-sided direct AI/ML positioning model, regardless of if UE receives the AI/ML model from LMF or application server, LMF should be aware such that LMF can request the location information from UE using that specific AI/ML positioning method. Actually, in our view, it’s a quite straightforward scenario that UE receives a direct AI/ML positioning model (e.g., fingerprint) from LMF directly.

If it is LMF-sided direct AI/ML positioning model, by nature LMF should be the host in responsible of the training/update etc. So far, we don’t see a scenario that LMF receives an LMF-sided model from somewhere else, but we are open to discuss.

To use direct AI/ML positioning model, regardless of UE-sided or not, LMF is aware of the AI/ML model usage.

For UE-sided direct AI/ML positioning model, RAN2 supports the model transfer from LMF to UE. FFS other possibilities, e.g., from CN or APP server to UE.

For LMF-sided direct AI/ML positioning model, RAN2 discusses if LMF can receive the AI/ML model from somewhere else.

On the other hand, for AI/ML assisted positioning, either UE-assisted or NG-RAN node assisted, since the AI/ML model is only used to generate a new measurement or enhancement of existing measurement, LMF does not need to be aware of the AI/ML model usage.

The usage of AI/ML assisted positioning model, regardless of UE-sided or gNB-sided, may be transparent to LMF.

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	AI/ML training/inference capabilities will be supported by gNB in Rel18 for network energy saving, load balancing and mobility optimization.

Observation 2	It is upon RAN3 discussion whether the AI/ML training/inference capabilities is located in CU or DU in a CU-DU split gNB architecture.

Observation 3	AI/ML management capabilities will probably be supported in Rel18/19 for OAM to deploy a trained AI/ML model to NG-RAN.

Observation 4	gNB can be aware of the model selection, activation, deactivation, switching, and fallback based on its own decision or based on UE report according to RAN1 agreement.

Based on the discussion above, we propose:

Proposal 1	For Network-UE level x collaboration (i.e., no collaboration), there is no RAN2 impact.

Proposal 2	RAN2 studies the possible enhancements to support Network-UE level y collaboration (i.e., signalling-based without model transfer) and level z collaboration (i.e., signalling-based with model transfer) taking into account RAN1 progress.

Proposal 3	For CSI feedback and beam management, RAN2 assumes the AI/ML model monitoring and possible selection/activation/deactivation/switch/fallback operation are performed by gNB/UE based on available measurements.

Proposal 4	If the AI/ML model is transferred to UE from CN node or application server, RAN2 studies how can gNB be aware of the AI/ML model usage for the sake of model monitoring and possible selection/activation/deactivation/switch/fallback.

Proposal 5	RAN2 assumes gNB may receive network-sided model for CSI feedback enhancement and beam management from OAM or CN node. Upon RAN3 decision.

Proposal 6	To use direct AI/ML positioning model, regardless of UE-sided or not, LMF is aware of the AI/ML model usage.

Proposal 7	For UE-sided direct AI/ML positioning model, RAN2 supports the model transfer from LMF to UE. FFS other possibilities, e.g., from CN or APP server to UE.

Proposal 8	For LMF-sided direct AI/ML positioning model, RAN2 discusses if LMF can receive the AI/ML model from somewhere else.

Proposal 9	The usage of AI/ML assisted positioning model, regardless of UE-sided or gNB-sided, may be transparent to LMF.

# 4	Reference