3GPP TSG-RAN WG2 Meeting #120	R2-2212027

Toulouse, France, November, 2022

Agenda item:		8.9.4

Source:		Lenovo

Title:		Second path addition and failure recovery for Scenario1

Document for:		Discussion and Decision

- Introduction

RAN2 achieved some agreement for multi-path in RAN2#119bis meeting. In this contribution, we further discuss the issues for multi-path in L2 U2N relay case [1].

- Discussion

Second path addition and failure

It was agreed in RAN2#119bis meeting that addition of direct/indirect path are supported as follows:

- Add direct path, after the establishment of the indirect path.

- Add indirect path, after the establishment of the direct path.

For above two cases, one timer could be needed to control the procedure of the second path addition. Regarding the addition of the indirect path, T420 can be reused. Regarding the addition of the direct path, T304 can be reused.

| T420 | Upon reception of the RRCReconfiguration message including sl-PathSwitchConfig | Upon successfully sending RRCReconfigurationComplete message (i.e., PC5 RLC acknowledgement is received from target L2 U2N Relay UE) | Perform the RRC re-establishment procedure as specified in 5.3.7. |
| --- | --- | --- | --- |

Proposal 1: T420 can be reused for the indirect path addition procedure.

Proposal 2: T304 can be reused for the direct path addition procedure.

Once the timer for path addition e.g T304 or T420 expires, that means UE fails to add the second path. In this case, the UE is not expected to perform re-establishment procedure since the remote UE has still another available path. UE should report the path addition failure to the serving gNB via the available path upon expiry of the corresponding timer for the second path addition.

Proposal 3: Once the timer for the second path addition expires, the remote UE declares the second path addition failure. Remote UE reports the failure information to the network via the available path.

When the timer for the second path addition expires, it is possible that the first path could be suspended e.g RLF. Therefore, once the timer for the second path addition expires, the remote UE initiates re-establishment procedure if the first path is suspended/unavailable.

Proposal 4: Once the timer for the second path addition expires, the remote UE initiates re-establishment procedure if the first path is suspended or unavailable.

Regarding the case of the second indirect path addition after UE has the first direct path already, the failure may happen in the first direct path and the Uu interface of the second indirect path. Specifically, when the timer for the second path addition is running, the remote UE receives the notification message from the relay UE due to e.g RLF or cell reselection or the remote UE detects RLF on PC5 link. In the above cases, the remote UE can declare the path addition failure and report the failure information to the serving gNB via the first available path.

Proposal 5: UE reports the failure to the serving gNB via the direct path if still available once the following failure on the second path happens while UE is performing the indirect path addition procedure:

- The remote UE receives the notification message from relay UE .

- The remote UE detects RLF on PC5 link.

In addition, the remote UE may detect RLF on the first direct path when the timer for the second path addition is running. RAN2 to discuss how to handle this failure case.

Proposal 6: RAN2 to discuss the failure case that the remote UE detects Uu RLF on the direct path when UE is performing the second indirect path addition procedure.

Regarding the case of the second direct path addition after UE has the first indirect path already, the failure may happen in the Uu interface of the first indirect path. Specifically, the remote UE may receive the notification message from relay UE due to e.g RLF in the first direct path when UE is performing the second direct path addition procedure. Or the remote UE may detect RLF on PC5 link in the first path when UE is performing the second direct path addition procedure.

Proposal 7: RAN2 to discuss the failure case on the indirect path happens when UE is performing the direct path addition procedure:

- The remote UE receives the notification message from relay UE in the indirect path.

- The remote UE detects RLF on PC5 link in the indirect path.

Operating in multi-path

We consider the case that UE is configured with multi-path. It was agreed that it performs RLM for Uu interface, for Scenario-1. For PC5 interface in Scenario-1, it performs sidelink RLF detection based on Rel-16 V2X specification. In addition, UE may receive the notification message due to e.g RLF from relay UE. In above cases, UE needs to report the failure information to the serving gNB via the available path. It is natural that UE initiates re-establishment procedure when both paths are not available.

Proposal 8: When UE is operating in multi-path Relay scenario, UE reports failure information to the serving gNB via the available path when the following failure case happens.

- Uu RLF on direct path;

- Sidelink RLF on indirect path;

- Reception of notification message from relay UE;

- Reception of PC5 unicat lease message from relay UE.

Proposal 9: When UE operating in multi-path Relay, UE initiates re-establishment procedure only when both paths are not available.

- Conclusion

In this contribution, we have the following proposals:

Proposal 1: T420 can be reused for the indirect path addition procedure.

Proposal 2: T304 can be reused for the direct path addition procedure.

Proposal 3: Once the timer for the second path addition expires, the remote UE declares the second path addition failure. Remote UE reports the failure information to the network via the available path.

Proposal 4: Once the timer for the second path addition expires, the remote UE initiates re-establishment procedure if the first path is suspended or unavailable.

Proposal 5: UE reports the failure to the serving gNB via the direct path if still available once the following failure on the second path happens while UE is performing the indirect path addition procedure:

- The remote UE receives the notification message from relay UE.

- The remote UE detects RLF on PC5 link.

Proposal 6: RAN2 to discuss the failure case that the remote UE detects Uu RLF on the direct path when UE is performing the second indirect path addition procedure.

Proposal 7: RAN2 to discuss the failure case on the indirect path happens when UE is performing the direct path addition procedure:

- The remote UE receives the notification message from relay UE in the indirect path.

- The remote UE detects RLF on PC5 link in the indirect path.

Proposal 8: When UE is operating in multi-path Relay scenario, UE reports failure information to the serving gNB via the available path when the following failure case happens.

- Uu RLF on direct path;

- Sidelink RLF on indirect path;

- Reception of notification message from relay UE;

- Reception of PC5 unicat lease message from relay UE.

Proposal 9: When UE operating in multi-path Relay, UE initiates re-establishment procedure only when both paths are not available.

References

RP-221262 Revised WID on NR sidelink relay enhancements.