3GPP TSG-RAN WG2 Meeting #120	R2-2212055

Toulouse, 14th November – 18th November, 2022

Agenda item:		8.3.6

Source:		Lenovo

Title:		Discussion on supporting of NES

Document for:		Discussion and Decision

# Introduction

The following progresses for NES techniques are achieved in RAN2 #119-e[1].

Solution groups:

- Adaption of MIB/SSB/SIB

-  partial/simplified SSB

2	Increase of SSB/SIB periodicity

3	On demand SSB/SIB1 (FFS if there are enhancements for other SIBs)

- FFS for on-demand MIB

4	Receiving SSB/SIB on one carrier/cell and performing access to another carrier/cell

5	Handover/Fast PCell change for NES

- CHO or new configuration

- group HO

6	Resource adaptation (frequency and time domain)

- Including PRACH, SRS, PUSCH, PUCCH resources and periodicities

- cell DTX/DRX

- measurement

- reference signal type and configuration of reference signal pattern for connected mode

- BWP adaptation

7	Any Cell activation/re-activation or UE wake up request signal (connected/idle)

8	Paging enhancements (includes paging-less solutions)

9	Cell selection/reselection (ie. cell prioritization also including legacy UEs)

Things to study

- Study group configuration and signalling for transitions for different solutions

- pre-configuration and L1/L2 signaling to trigger change of configuration

2	Identify/capture RAN2 impact to legacy for the different solutions

3	Awareness of the NES states at the UE side for the different solutions

4	Aim to minimize DL signalling for NES

5	Consider UE complexity and energy consumption

6	UE assistance information for the specific network energy technique, it’s benefits and impact to UE/NW

In RAN2#119bis-e, there was not progress on the issue of NES state determination or UE assistant information.

In this contribution, we further discuss the NES state determination and UE assistant information.

# Discussion

NES state determination and signaling

Similar to LTE supporting an interference-aware small cell on/off mechanism in a heterogeneous network, it may be beneficial to allow gNB to flexibly turn on/off transmission in the time domain via semi-persistent or dynamic signaling. For example, gNB is not allowed to transmit data or signal in DL slots/symbols that are indicated as transmission OFF, except for essential signals that must be transmitted in these DL slots/symbols. In LTE, the cell on/off can be realized by configuring a cell as an activated/deactivated cell for a UE which means the cell state can be identified by the UE.

In Rel-18, the UE could determine whether to monitor the DL slots/symbols or determine whether to monitor the whole or only parts of the DL slots/symbols, if the similar configuration of activation/deactivation is applied to the UE. It is also beneficial from the perspective of UE power saving.

Referring to LTE, the initial state of a cell can be configured, and the state transition can be controlled by signaling. In Rel-18 NES, similar rules can be used. Considering the discussion in RAN1, the NES may also be controlled by PHY layer technology. And in order to make the NES states be aware at the UE side, the energy saving states or sleep modes of a cell can be indicated explicitly or implicitly.

Proposal 1: The energy saving states or sleep modes of a cell can be indicated explicitly or implicitly.

In LTE, the eNB may autonomously decide to switch-off a cell to lower energy consumption (dormant state). In SON, the switch-off decision may also be taken by O&M. On the other side, the eNB controlling a dormant cell should normally obey a request. The switch-on decision may also be taken by O&M. In summary, the decision may be taken by the base station or a central controller. Then, referring to the LTE principle, the state transition in Rel-18 NES may be decided by the gNB or CN.

Proposal 2: The transition between states can be determined by gNB or CN.

In LTE, the eNB using cell on/off for the purpose of energy saving may adaptively turn the downlink transmission of a cell on and off. In NR, it would be useful to define different energy saving states or sleep modes for the base stations considering the difference between the eNB and gNB, also considering the diversity of the traffics in NR. Correspondingly, it would be better to support a cell to transition between any two energy saving states for the purpose of maximizing energy savings and reducing the transition time.

Proposal 3: A cell can transition among multiple different energy saving states.

UE assistance information

Referring to the progress of RAN1, cell/network node activation request by the UE was agreed to be further studied. And the requirements of the arrival traffic are identified only by the UE before the BSR or data is transmitted. Accordingly, the UE may expect one of the activation levels of a cell can satisfy the traffic requirements. For example, the UE expects the level of activation of the cell based on the data volume or whether it is the specific traffic, etc. Therefore, it is better for the UE to request the cell to transition to the suitable energy saving state. The request related message can be a kind of UE assistance information. And the suitable energy saving state may be any of the energy saving states supported by the cell.

Proposal 4: The state transition to each of the potential network energy saving states can be requested by the UE using the UE assistance information.

Different UE may have different criteria for the expected level of activation. It is difficult for the gNB to determine the state transition once the request from the UE is received if it is up to the UE implementation to decide which level of activation is requested. Therefore, if the UE can be configured with the threshold (for example, data volume, the number of arrival traffic, etc.) for requesting the activation of the cell e.g., the more the data volume or arrival traffic is, the higher active the cell is, the network node can determine the state transition simply. If the UE reports all of the information, the network can identify which state is the suitable state for the UE. However, the payload of the report and the overhead of the radio resource are high. Then if only the level of activation of the cell is requested, the overhead of the radio resource can be optimized. Further, each threshold can be configured correspondingly to a level of activation of the cell.

Proposal 5: In order to reduce the overhead, the UE requests the expected NES state instead of the detailed reports of the data volume, etc.

Proposal 6: In order to request different level of energy saving states, the UE can be configured with different thresholds e.g., thresholds for different data volume, different numbers of arrival traffic, etc.

The network state transition for energy saving should be considered comprehensively including the UL and DL situations of all the camped and connected UEs. In LTE, the decision may be taken by the base station or a central controller. Accordingly, the state transition in Rel-18 NES may be decided by the gNB or CN. Then, the gNB can indicate the energy saving state transition to the UEs, for example the cell is activated to a specific activation level.

Proposal 7: The gNB indicates the energy saving state transition to the UEs.

# Conclusion

In this contribution, we discuss the handover aspects in Rel-18 NES and provide the proposals:

Proposal 1: The energy saving states or sleep modes of a cell can be indicated explicitly or implicitly.

Proposal 2: The transition between states can be determined by gNB or CN.

Proposal 3: A cell can transition among multiple different energy saving states.

Proposal 4: The state transition to each of the potential network energy saving states can be requested by the UE using the UE assistance information.

Proposal 5: In order to reduce the overhead, the UE requests the expected NES state instead of the detailed reports of the data volume, etc.

Proposal 6: In order to request different level of energy saving states, the UE can be configured with different thresholds e.g., thresholds for different data volume, different numbers of arrival traffic, etc.

Proposal 7: The gNB indicates the energy saving state transition to the UEs.

# References

RAN2#119-e chairman notes.

RAN2#119bis-e chairman notes.