3GPP TSG-RAN2 Meeting # 120-e	R2-2212083

# Toulouse, France   14th  - 18th Nov 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.314 | CR | CRNum | rev | RevNum | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Total RAN Delay calculation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-Core |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In RAN2#119-e meeting, RAN2 agreed in R2-2209023 to include two scenarios of total RAN delay calculation for split DRBs in Rel-17 Proposal 2: Total RAN delay calculation formula for split DRB in RAN node is to be specified in Rel 17, for the following scenarios: • PDCP duplication is not enabled during the delay measurement period • PDCP duplication was enabled during the delay measurement period In this CR, we provide the set of missing formulas for the above scenarios to be included in the TS 38.314. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced new sections to introduce formula to calculate total RAN delay per UE covering different scenarios. Impact analysis Impacted functionality: MDT Inter-operability analysis: If RAN nodes do not implement the formulas according to the provided change, the measurements will be ambegious for QoS monitoring at core network in particular in multivendor scenarios. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity remains for calculations for M6 measurements in split-bearer scenario with/without PDCP dupplication. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 4.2.1.x Total RAN Delay in Split-DRB Scenario

### 4.2.1.x.1 Delay Calculation per Split-DRB per UE when PDCP duplication is enabled for entire measurement period

The objective of this measurement is to calculate total RAN delay per split-DRB per UE when PDCP duplication is enabled for the entire measurement period, for QoS verification of MDT.

Protocol layer: PDCP

Table 4.2.1.x.1-1: Definition for total delay for PDCP duplicated packets per Split-DRB per UE

| Definition | Total delay for PDCP duplicated packets per Split-DRB per UE. This measurement is applicable for EN-DC and SA. This measurement provides the total delay for a Split-DRB when PDCP duplication is enabled for the entire measurement period. Detailed Definition: M(T,drbid)=min(MMCG(T,drbid) , MSCG(T,drbid)) , where explanations can be found in the table 4.2.1.x.1-2 below. |
| --- | --- |

Table 4.2.1.x.1-2: Parameter description for total delay for PDCP duplicated packets per Split-DRB per UE

| M(T,drbid) | Total Delay per Split-DRB per UE during the time period T. |
| --- | --- |
| MMCG(T,drbid) | Total Delay on the MCG per split DRB per UE during the time period T. |
| MSCG(T,drbid) | Total Delay on the SCG per split DRB per UE during the time period T. |
| min(MMCG(T,drbid) , MSCG(T,drbid)) | Minimum of the total delays between MCG and SCG legs per Split-DRB per UE during the time period T. |

### 4.2.1.x.2 Delay Calculation per Split-DRB per UE when PDCP duplication is not enabled for entire measurement period

The objective of this measurement is to calculate total RAN delay per split-DRB per UE when PDCP duplication is not enabled for the entire measurement period, for QoS verification of MDT.

Protocol layer: PDCP

Table 4.2.1.x.2-1: Definition for total delay for PDCP non-duplicated packets per Split-DRB per UE

| Definition | Total delay for PDCP non-duplicated packets per Split-DRB per UE. This measurement is applicable for EN-DC and SA. This measurement provides the total delay for a Split-DRB when PDCP duplication is not enabled for the entire measurement period. Detailed Definition: M(T,drbid)=IMCG(T,drbid)×MMCG(T,drbid)+ISCG(T,drbid)×MSCG(T,drbid)I(T,drbid) IT,drbid=IMCG(T,drbid)+ISCG(T,drbid), where explanations can be found in the table 4.2.1.x.2-2 below. |
| --- | --- |

Table 4.2.1.x.2-2: Parameter description for total delay for PDCP non-duplicated packets per Split-DRB per UE

| M(T,drbid) | Total Delay per Split-DRB per UE during the time period T. |
| --- | --- |
| MMCG(T,drbid) | Total Delay on the MCG per split DRB per UE during the time period T. |
| MSCG(T,drbid) | Total Delay on the SCG per split DRB per UE during the time period T. |
| IT,drbid | Total number of PDCP non-duplicated packets sent per Split-DRB per UE during the time period T. |
| IMCG(T,drbid) | Total number of PDCP non-duplicated packets sent on the MCG leg per Split-DRB per UE during the time period T, as defined in Table 4.2.1.8.1-1. |
| ISCG(T,drbid) | Total number of PDCP non-duplicated packets sent on the SCG leg per Split-DRB per UE during the time period T, as defined in Table 4.2.1.8.1-1. |