3GPP TSG-RAN WG2 Meeting #120                                   R2-2212104

Toulouse, France, November 14-18, 2022

Agenda item:		8.11.2

Source:				Samsung

Title:					Discussion on Multicast Reception in RRC_INACTIVE

Document for:		Discussion & Decision

# 1 Introduction

In RAN2#119bis-e meeting, the following agreements were made for multicast reception in RRC_INACTIVE [1]:

The following general description is taken as baseline for PTM configuration delivery Option 1: - (1-a) PTM configuration(s) (i.e., configurations used for multicast reception in RRC_INACTIVE) of one or more multicast sessions for at least one cell are provided via dedicated RRC signaling to a UE. - (1-b) The RRC message for this includes RRCReconfiguration and/or RRCRelease and/or RRCResume (details FFS) - (1-c) UE stores the received configurations while it is in RRC_INACTIVE, and if there is a need to update some or all the configurations, the UE is notified of such changes and may trigger RRC connection resume to obtain the updated configurations. In case of mobility in RRC_INACTIVE, the UE triggers RRC connection resume if the configuration of the session is not available for the new cell. The following general description is taken as baseline for PTM configuration delivery Option 2: - (2-a) PTM configurations (i.e., configurations used for multicast reception in RRC_INACTIVE) are provided via an MCCH-like channel (same or different as used for MBS broadcast), and information regarding MCCH scheduling is provided via SIB, FFS dedicated signalling - (2-b) UE can receive such configurations when it is in RRC_INACTIVE, FFS whether it is allowed/needed to also receive when UE is in RRC_CONNECTED - (2-c) If there is a need to update some or all the received configurations, UE does not need to resume RRC connection but is notified of such changes (e.g. via MCCH DCI) and obtains the updated configurations via MCCH. Dedicated RRC signalling (i.e. RRC release message with suspendConfig) is used for switching a multicast receiving UE from RRC_CONNECTED to RRC_INACTIVE and continue multicast reception (details FFS). For both option 1 and option 2, as a baseline, group paging can be used to switch UEs receiving multicast from RRC_INACTIVE to RRC_CONNECTED, and UEs continue the multicast reception in CONNECTED. FFS if there is any potential issue if Rel-17 group paging is reused. FFS if there are other cases when UE triggers resume. FFS if MCCH can also be used in case of option 2. FFS whether to introduce PTM configuration applicable area, i.e., the mechanism that the PTM configurations, once acquired by a UE, may apply to a certain area (i.e., a set of cells instead of a single cell). Rel-18 UE in INACTIVE can be informed when the session is activated (Details FFS). As a baseline, group paging can be used to inform Rel-18 UE(s) about the session activation (Details FFS, e.g., UE behavior when receiving such group notification). If a UE is in RRC_INACTIVE and is configured to receive a multicast session in RRC_INACTIVE, the UE may be notified when the multicast session is deactivated. FFS how (e.g., informed via group paging, MCCH, or other ways). Rel-17 mechanism (NAS-based indication) is applicable for multicast session release. FFS if any enhancement is needed. FFS how UE determines whether it can receive the multicast session in RRC_INACTIVE or not when the session is activated, taking into account the following solutions (can further update the descriptions if needed, and several solutions may be needed, some solutions may apply only for certain configuration options) - 1. When the multicast session is activated, UE can receive the multicast session in RRC_INACTIVE if the PTM configuration used in RRC_INACTIVE for the session is available to the UE and the UE has joined the session already (e.g., configuration provided to UE via dedicated RRC signaling or via MCCH), otherwise it goes back to RRC_CONNECTED to receive the multicast session. - 2. When the multicast session is activated, UE is indicated by group paging whether it can receive the multicast session in RRC_INACTIVE or not (detailed signaling FFS). - 3. UE is configured "whether it can receive the multicast session in RRC_INACTIVE" by dedicated signaling before UE is released. When the multicast session is activated, UE stays in RRC_INACTIVE or resumes RRC connection accordingly (detailed signaling FFS). If option 1 is supported for PTM configuration - As a baseline, group paging may be used to inform the UE when network changes the PTM configurations, and UE upon reception triggers RRC connection resume procedure to obtain the updated configurations (details of group paging can be FFS). - FFS whether and how to solve the issue in signalling/system load when a large number of UEs in the cell need PTM configuration update. FFS if there is an issue that a UE can obtain all the PTM configurations for a multicast service via Option 2 without/before joining the multicast session on the condition that security is enabled by service layer. And if yes FFS how to solve the issue (e.g., dedicated configuration + MCCH).

This contribution discusses the issues and further details for multicast reception in RRC_INACTIVE.

# 2 Discussion

### 2.1 PTM configuration for UEs receiving multicast in RRC_INACTIVE

UEs may need to be transitioned to RRC_INACTIVE in overload/congestion situation, and can continue “certain” multicast session(s). Notably, UEs may have the multicast session(s) either activated (MRB configuration available) or deactivated (MRB configuration released) while in RRC_CONNECTED [2]. Further, these UEs are required to be configured to receive multicast in RRC_INACTIVE. Following options are being considered in RAN2:

Option 1A: RRC Release with suspendConfig provides the PTM configuration to be used in RRC_INACTIVE (includes both activated and deactivated multicast MRBs). Configuration may include only those MRBs which are to be continued to RRC_INACTIVE.

Option 1B: Pre-configured configuration implies UE may be indicated PTM configuration for MRBs in RRC reconfiguration message from which configuration may fully/ partially be continued to RRC_INACTIVE and UE preserves them. It may be possible to indicate which all MRBs can be continued to RRC_INACTIVE.

Option 2: SIB+MCCH may provide multicast session configuration to RRC_INACTIVE UEs, like in legacy for broadcast session.

Option 3: MCCH configuration through dedicated signalling implies UE receives MCCH configuration through RRC reconfiguration in the RRC_CONNECTED and then utilizes it to set up MCCH channel and receive PTM configuration.

Analysis:

We understand Option 1A is efficient and sufficient with providing specific PTM configuration for relevant multicast MRBs at state transition. Comparatively, Option 1B seems to have larger spec impacts as

- UEs need to be configured in advance with “suitable for RRC_INACTIVE” multicast session configuration information - This may be a bit complex as relevant configuration parameters may also need to be indicated for specific RRC state(s) applicability

- UEs may also need to store and preserve certain multicast session configurations in spite of MRB release due to session deactivation, in order to support potential future transition to RRC_INACTIVE

Proposal 1: RRCRelease with suspendConfig provides the PTM configuration for multicast MRBs to be received in RRC_INACTIVE. Pre-configuration with RRC reconfiguration message is not adopted.

Whereas Option 2 has two major drawbacks:

- Security concerns: Increases security vulnerability as this approach exposes the dedicated signalling based multicast configuration parameters through MCCH broadcast. Notably, MCCH signalling information is not secure i.e., MTCH traffic is only protected by application level security.

- Unauthorized access and non-standalone approach: With this approach, UEs in RRC_INACTIVE cannot receive new multicast session configuration i.e. UEs need to “join” the multicast session and also need to be informed by network “which UEs” can receive “which multicast sessions” in RRC_INACTIVE. Allowing UEs in RRC_INACTIVE to read MCCH without joining multicast sessions causes unauthorised access, which is highly problematic.

During R2#119bis-e discussions, some companies downplayed security concerns with Option 2, and suggested the security is enabled by service layer. However, they have not addressed the security of MCCH signalling carrying dedicated configuration parameters. In our understanding, RAN2 is not the competent WG to decide on security aspects and it should be consulted with SA3, if Option 2 is to be considered further.

Proposal 2: RAN2 to consult with SA3 on MCCH signalling security concerns when it carries dedicated configuration parameters for multicast session to be received in RRC_INACTIVE. Send LS to SA3.

Option 3 tries to address the security and privacy concerns with the PTM configuration provided over MCCH. Receiving MCCH configuration only through dedicated signalling ensures only authorised UEs can access the MCCH. However, there are some concerns with this approach.

- Need new MCCH channel: A new MCCH channel is required which is only accessible to authorised UEs that receive MCCH configuration through dedicated signalling. This implies significant specification impact and complexity.

- No dynamic configuration of MCCH channel: With this approach, RRC_INACTIVE UEs are assumed to always utilizing MCCH configuration received in RRC_CONNECTED and stored in a static manner. That is, network has no control or adaptability to reconfigure MCCH even if situation demands (unlike legacy which uses SIB based MCCH configuration). Moreover MCCH channel configuration is common to all the multicast services and all UEs, unlike PTM configuration which may be updated for a given multicast service only. Therefore, any change of the configuration of MCCH affects all the UEs receiving any of the services in RRC_INACTIVE and forces them all to go RRC_CONNECTED at once.

- Area Scope of MCCH: In legacy MCCH has been specified for a single cell. It will be a new complexity when UE moves across the cells or area scope for MCCH has to be redefined.

We think until response from SA3 is received Option 2 cannot be progressed further and option 3 may introduce undesired specification impact and effort. RAN2 should work on Option 1A which is at any rate is always required.

Proposal 3: RAN2 to not consider providing MCCH configuration through dedicated signalling given the need for new MCCH channel, lack of dynamism of configuration update for MCCH and large complex specification impact.

### 2.2 PTM configuration parameters

Broadly speaking, like the dedicated configuration for multicast, RRCRelease with suspendConfig can carry MBS radio bearer config, MAC config and PHY config for multicast reception in RRC_INACTIVE. These may exclude certain parameters which are specific to dedicated configuration used only for RRC_CONNECTED e.g. HARQ feedback, HARQ retransmission, CSI/SRS configuration, TA timer etc. Additionally, RRCRelease with suspendConfig may provide neighbour cells multicast configuration e.g., whether the neighbour cells support same multicast configurations as the serving cell.

Proposal 4: RRCRelease with suspendConfig can carry MBS radio bearer config, MAC config and PHY config for multicast reception in RRC_INACTIVE. These may exclude certain parameters which are specific to dedicated configuration used only for RRC_CONNECTED. Additionally, RRCRelease with suspendConfig may provide neighbour cells multicast configuration e.g., whether the neighbour cells support same multicast configurations as the serving cell.

### 2.3 BWP/CFR aspects for multicast

A UE in RRC_CONNECTED receives unicast transmission in an active Bandwidth Part (BWP) and multicast transmission is received over common frequency resources (CFR) which is associated with and confined within the unicast active BWP. For the UE in RRC_INACTIVE, it is relevant to receive same multicast transmission (i.e. in the MBS CFR), however, monitoring for the entire active BWP may not be necessary and not power efficient. Another possibility is to consider the linking of MBS CFR to the initial BWP. However, it is not guaranteed that initial BWP is overlapped with active BWP or MBS CFR. Therefore, it may be more relevant to consider to define a BWP same as MBS CFR for the multicast reception operation by UE in RRC_INACTIVE. UE can receive GC-PDCCH and GC-PDSCH for multicast over the BWP. RAN2 should, therefore, consult with RAN1.

Proposal 5: RAN2 to consider to define a BWP same as MBS CFR for multicast reception operation by UE in RRC_INACTIVE. RAN2 to send LS to RAN1.

### 2.4 DRX for multicast reception

DRX mechanism is required for multicast reception in RRC_INACTIVE to enable power saving. As an UE in RRC_INACTIVE needs to share the multicast transmission with RRC_CONNECTED UEs for given multicast MRB, the RRC_INACTIVE UE should be following same DRX configuration. However, UE in RRC_INACTIVE may not be expected to provide HARQ feedback and/or receive HARQ retransmission (except for blind retransmissions), it seems RTT timer and ReTx timer configurations are not required. There seems to be a few options for providing DRX configurations for RRC_INACTIVE:

Option 1: C-DRX configuration for multicast MRB is continued in RRC_INACTIVE expect RTT timer and ReTx timer configurations

Option 2: Explicit multicast DRX configuration for RRC_INACTIVE is provided (e.g. in RRCRelease with suspendConfig)

Option 2 seems preferable which is also aligned with the PTM configuration signalling approach as in proposal 1.

Proposal 6: Explicit multicast DRX configuration for RRC_INACTIVE is provided (e.g. in RRCRelease with suspendConfig).

### 2.5 Multicast session state change

In Rel-17 MBS, when UE is in RRC_CONNECTED, multicast session can be activated or deactivated through RRC reconfiguration message which carries the multicast MRB addition and release respectively. Whereas, when the UE is in RRC_IDLE or RRC_INACTIVE, UE can monitor for group notification for session activation of the non-activated multicast session and then, transit to RRC_CONNECTED to receive the activated session. When session is released it is expected, UE is paged in legacy (unicast) way and UE goes to RRC_CONENCTED to formally release the session with NAS signalling.

However, for Rel-18 MBS UE, which is configured to receive or is receiving multicast session in RRC_INACTIVE, the following events may happen:

- A non-activated mutlicast session gets activated

- An activated multicast session gets deactivated

- A multicast session gets released

For scenario a), Rel-18 MBS UE may need to know whether to continue activated session in RRC_INACTIVE or transit to RRC_CONNECTED. Network may indicate this to UE. The reason for network to provide a multicast session in RRC_INACTIVE or RRC_CONNECTED could be many e.g. removal of congestion scenario, QoS changes for multicast service.

- When no such indication (e.g. no RRC state indication) is provided in group paging, UE goes to RRC_CONNECTED i.e. same as Rel-17 Group paging

- When an indication (e.g. RRC state as RRC_INACTIVE) is provided in group paging, UE continues in RRC_INACTIVE i.e. Rel-18 Group paging is enahcned over Rel-17 Group paging message.

It seems scenario b) and c) are quite valid and frequently occurring. These cases should be handled without any complexity of RRC state change or excessive signaling burden. A natural approach seems to extend the legacy group notification to also convey if the “session deactivation” or the “session release” is the cause of group notification. An absence of such a cause can represent “session activation”, which is the same as legacy group notification.

Notably, UE may not be required to transit to RRC_CONNECTED immediately, e.g. when a group paging with session release is received. For instance, if UE is already receving another multicast session in RRC_INACTIVE or network is too congested; forcing all UEs at once to transit to RRC_CONNECTED and to immediately respond to group paging with session release is not desired.

Proposal 7: Extend group notification (group paging) to also convey that if session deactivation or session release is the cause of multicast group notification.

### 2.6 Inactivity handling

It is expected that UE receiving multicast in RRC_INACTIVE may face similar issue as it were for RRC_CONNECTED UEs e.g. multicast data inactivity. For example, the UE in RRC_INACTIVE should maintain a multicast inactivity timer to track reception of multicast packet for activated multicast session(s). Multicast inactivity timer can be started/ restarted when a multicast packet is received. When multicast inactivity timer expires, UE may stop decoding for multicast reception. Further, UE may request to transit to RRC_CONNECTED as the reason for staying for the RRC_INACTIVE (i.e. reception of multicast session) is no longer valid. UE may formally release the multicast session, if UE is not interested in multicast session, or could avail the service through unicast.

Proposal 8: UE receiving multicast in the RRC_INACTIVE maintains an Inactivity Timer to track reception of multicast packet for activated multicast session(s).

### 2.7 Mobility aspects

For a UE configured to receive multicast in RRC_INACTIVE, multicast session may or may not be provided on the neighbor cells. Consequently, for a UE reselecting a neighbor cell may lead to disruption of multicast service reception. For this reason, UE can be provided with neighbour cell multicast configuration information to support mobility.

Proposal 9: UE is provided with the neighbour cell multicast configuration information to support mobility.

Another important aspect is the RAN overload due to multicast. It should be feasible to reuse the RAN overload control mechanism of deprioritization specifically for multicast. In legacy LTE and NR, network can determine RAN overload situation and sends to the UE, a deprioritization request for the current frequency (or RAT) along with RRCRelease message. This ensures that the UE considers the indicated frequency (or all frequencies of the RAT) to be the lowest priority for cell reselection for a configured duration [2].

We think this well-established and proven mechanism of de-prioritization seems a good approach to address the congestion / overload scenario for multicast. Network can indicate de-prioritization request for multicast frequency along with RRCRelease with suspendConfig and only multicast UE can consider the indicated frequency to be the lowest priority for cell reselection for configured time duration while UE is in RRC_INACTIVE.

Proposal 10: UE is provided with the de-prioritization request for multicast to control RAN overload on specific frequency.

# 3 Conclusion

Request RAN2 to discuss and agree to the following proposals:

Proposal 1: RRCRelease with suspendConfig provides the PTM configuration for multicast MRBs to be received in RRC_INACTIVE. Pre-configuration with RRC reconfiguration message is not adopted.

Proposal 2: RAN2 to consult with SA3 on MCCH signalling security concerns when it carries dedicated configuration parameters for multicast session to be received in RRC_INACTIVE. Send LS to SA3.

Proposal 3: RAN2 to not consider providing MCCH configuration through dedicated signalling given the need for new MCCH channel, lack of dynamism of configuration update for MCCH and large complex specification impact.

Proposal 4: RRCRelease with suspendConfig can carry MBS radio bearer config, MAC config and PHY config for multicast reception in RRC_INACTIVE. These may exclude certain parameters which are specific to dedicated configuration used only for RRC_CONNECTED. Additionally, RRCRelease with suspendConfig may provide neighbour cells multicast configuration e.g. whether the neighbour cells support same multicast configurations as the serving cell.

Proposal 5: RAN2 to consider to define a BWP same as MBS CFR for multicast reception operation by UE in RRC_INACTIVE. RAN2 to send LS to RAN1.

Proposal 6: Explicit multicast DRX configuration for RRC_INACTIVE is provided (e.g. in RRCRelease with suspendConfig).

Proposal 7: Extend group notification (group paging) to also convey that if session deactivation or session release is the cause of multicast group notification.

Proposal 8: UE receiving multicast in the RRC_INACTIVE maintains an Inactivity Timer to track reception of multicast packet for activated multicast session(s).

Proposal 9: UE is provided with the neighbour cell multicast configuration information to support mobility.

Proposal 10: UE is provided with the de-prioritization request for multicast to control RAN overload on specific frequency.

# 4 Reference

[1] RAN2#119bis-e meeting report v1

[2] 3GPP TS 38.331 v17.2.0: Radio Resource Control (RRC) protocol specification