3GPP TSG-RAN WG2 Meeting #120	R2-2212170

Toulouse, France, 14th – 18th November, 2022

Agenda Item:	8.2.3

Source:	Spreadtrum Communications

Title:	Discussion on solutions for integrity of RAT-dependent positioning techniques

Document for:	Discussion and Decision

Introduction

In RAN2 #119-e and RAN2 #119-bis-e meeting, the following agreements has been agreed.

RAN2 #119-e Agreements:

Proposal 1: RAN2 to confirm the integrity principle of operation defined in the section 8.1.1a of TS38.305, including integrity definition (e.g., Error, Bound, Time to Alert, DNU, Residual Risk, irMinimum, irMaximum and Correlation Times; FFS if all parameters are needed in the RAT-dependent case), Equations for the GNSS integrity are reused for RAT dependent positioning methods.

Proposal 2 (modified): RAN2 may add the mapping between Integrity definition/Fields (Integrity Alerts, error bounds (mean, StdDev), Residual Risks, Integrity correlation times ) and Error sources/assistance data for RAT-dependent positioning methods later once RAN1 identifies new error sources.

RAN2 #119-bis-e Agreement:

Proposal 1-2. RAN2 study the usage of DNU flag for the RAT-dependent positioning integrity (assuming RAN1 agree to leave it to RAN2) and conclude on whether to indicate the DNU presence in the integrity principle equation.

Agreement:

Proposal 4. RAN2 will study the both UE-based and LMF-based integrity for RAT-dependent cases.

Agreement:

Proposal 7 (modified). RAN2 agree that R17 UE-based integrity mode signalling can be used as baseline with the following aspects:

-	UE sends capability info to LMF on integrity for UE-based mode using LPP capability transfer procedure

-	LMF sends the assistance data for integrity calculation to UE for integrity of UE-based mode

-	LMF sends integrity requirement e.g., TIR to UE in LPP request location information message for integrity of UE-based mode

-	UE sends integrity result to LMF using LPP location information Transfer message

Agreement:

LMF provides, in assistance data, the information of error sources (e.g., originated from RAN node) to UE for integrity in UE-based mode.

In this contribution, we discuss the question of the methodologies and procedures of RAT-dependent positioning and express our opinions.

Discussion

General integrity procedure

Considering the signalling to deliver the KPIs and integrity results reporting, RAN2 agree that Rel-17 only supports mode1 reporting.

For mode 1, the integrity computing entity calculates the PL. Then, the calculated PL is directly reported to where the LCS client resides (Network or UE). For mode 2, the integrity computing entity calculates the PL. Then, the integrity computing entity further compares the calculated PL with the given AL to determine if the positioning system is still available to offer trustable position estimation. Thus, the integrity computing entity may only have to report a binary flag (0 and 1) to indicate whether the positioning system is available or not.

And for mode2, RAN2 decides that it will be revisited in future releases in rel-17. But mode1 and mode2 can be useful in different scenarios. Mode1 can be useful when the LCS client does not want expose the integrity KPIs to LMF/UE. And for mode2, it can reduce the complexity for LCS client. And RAN2 has discussed many issues of reporting mode2, such as whether should include an indicator to tell the UE either reporting mode1 or reporting mode2? Whether allows the UE to raise the flag indicating whether there is an integrity risk? And considering mode2 can be useful in simple APP implementation where the application requires only information on whether integrity performance is met with respect to the KPIs. Thus, in Rel-18, in order to support the positioning integrity in various scenarios, mode1 and mode2 integrity result reporting scan be considered for RAT-dependent positioning.

Proposal 1: Mode1 and Mode2 of integrity results reporting can be considered for RAT-dependent positioning integrity.

As shown in the above agreements, for UE-based mode, R17 UE-based integrity mode signalling can be used as baseline. And for LMF-based, the following signalling may be involved to calculate integrity:

- Serving gNB send TRP-related error source to LMF using NRPPa PRS configuration information procedure;

- UE send capability info to LMF on integrity using LPP capability transfer procedure;

- UE send measurement error source to LMF using LPP location information procedure;

- Serving gNB send gNB measurement error source to LMF to calculate integrity using NRPPa measurement report procedure.

Thus, for LMF-based integrity, the potential LPP spec impacts includes UE integrity capability transfer procedure, UE measurement error sources transfer procedure. And the potential NRPPa impacts includes TRP related error source transfer procedure, gNB measurement error source transfer procedure.

Proposal 2: For LMF-based integrity, the potential LPP spec impact includes UE integrity capability transfer procedure, UE measurement error sources transfer procedure.

Proposal 3: For LMF-based integrity, the potential NRPPa impact includes TRP related error source transfer procedure, gNB measurement error source transfer procedure.

Error sources analysis

Agreement:

Proposal 1-2. RAN2 study the usage of DNU flag for the RAT-dependent positioning integrity (assuming RAN1 agree to leave it to RAN2) and conclude on whether to indicate the DNU presence in the integrity principle equation.

In last RAN2 meeting, RAN2 agreed to study the usage of DNU flag for RAT-dependent positioning integrity. DNU flag can be used for assistance data to indicate whether particular parameters can be used to evaluate the positioning integrity. Beyond that, DNU also can be used for UE or gNB to indicate if measurements report can be used for integrity. Because the measurement for positioning estimation and for integrity calculation may be different, since not all the measurement has impact on the integrity calculation. Considering reduce processing complexity and overhead, we can use DNU flag to indicate which measurement report can be used to integrity calculation.

Proposal 4: DNU flag can be used for measurements reports to indicate which measurement result can be used to integrity calculation.

For RAT-dependent integrity, the error sources can be divided into normal error and abnormal error. The normal error may be able to model as Gaussian distribution with unknown mean and deviation. For example, measurement-related error may be able to model as Gaussian distribution. And the values of mean and deviation may be determined from assistance data. Specifically, for some error caused by TRP, the mean and deviation of error sources may be determined by gNB and be transmitted to LMF via NRPPa signalling. For UE-based integrity, the man and deviation of each error should be provided to UE by LMF via assistance data.

Proposal 5：For UE-based positioning integrity, the mean and deviation of each error should be provided to UE by LMF via assistance data.

And for UE-assisted positioning integrity, in order to calculate positioning integrity, the mean and deviation of the UE error sources, TRP error sources and measurement error sources need to be determined and transmitted to LMF.

Proposal 6: For UE-assisted positioning integrity, the mean and deviation of each error from UE/TRP error sources and measurements error sources need to be delivered to LMF.

Conclusions

In this contribution, we discuss the positioning enhancements of Rel-18 positioning integrity techniques. And the proposals are given as follows:

Proposal 1: Mode1 and Mode2 of integrity results reporting can be considered for RAT-dependent positioning integrity.

Proposal 2: For LMF-based integrity, the potential LPP spec impact includes UE integrity capability transfer procedure, UE measurement error sources transfer procedure.

Proposal 3: For LMF-based integrity, the potential NRPPa impact includes TRP related error source transfer procedure, gNB measurement error source transfer procedure.

Proposal 4: DNU flag can be used for measurements reports to indicate which measurement result can be used to integrity calculation.

Proposal 5：For UE-based positioning integrity, the mean and deviation of each error should be provided to UE by LMF via assistance data.

Proposal 6: For UE-assisted positioning integrity, the mean and deviation of each error from UE/TRP error sources and measurements error sources need to be delivered to LMF.

References

- RP-213588, Revised SID on Study on expanded and improved NR positioning, 3GPP TSG-RAN Plenary Meeting #94e.