3GPP TSG-RAN WG2 Meeting#120-e		       					   R2-2212182

Toulouse, France, 14th – 18th November, 2022

Source: 			ZTE corporation, Sanechips

Title: 	Supporting multiple DTX configuration

Agenda item:		8.3.2

Document for: 	Discussion and Decision

# Introduction

In RAN2#119B[1], RAN2 discussed the different NES types and the UE alignment with NES types, and reached the agreements as below:

Agreements

=>	Let’s start with understanding solution in the context of connected

=> RAN2 discuss expected gNB DTX / DRX behavior with details of different TX / RX types:

•	Example 1: gNB is expected to turn off all transmission and reception for data traffic and reference signal during Cell DTX / DRX OFF duration.

•	Example 2: gNB is expected to turn off its transmission / reception only for data traffic during Cell DTX / DRX OFF duration (i.e. gNB will still transmit / receive reference signals).

•	Example 3: gNB is expected to turn off its dynamic transmission / reception during Cell DTX / DRX OFF duration (i.e. gNB is expected to still perform periodic transmission / reception, including SPS, CG-PUSCH, SR, RACH, and SRS).

•	Example 4: gNB is expected to only transmit reference signals (e.g. CSI-RS for measurement).

=>	RAN2 assumes that the options above are possible for gNB DTX/DRX behavior and discuss UE RAN2 behavior/impact during the DTX/DRX.

=>	For the purpose of our discussion we will focus on a single UE behavior at any point in time.  FFS if we allow multiple configuration of NW DRX/DTX behaviors.

=>	Periodic DTX is assumed as a baseline.  The gNB provides indication to UE about NW DTX mode/configuration via dedicated dynamic L1/L2 signaling.

=>	Dynamic L1/L2 group signalling from NW to provide NW DTX mode/configuration is also considered in RAN2

=>	It is beneficial to align UE DRX with network DTX and DRX alignment among multiple UEs.  Details are FFS, including UE transmission/reception behavior during DTX.  RAN2 to study the alignment.

In this paper, we will discuss the remaining issues.

# Discussion

In general, gNB would adopt different NES type under different load. For example, in Fig1, under low load, gNB would turn off all transmission and reception for a relatively long time; under light load, gNB would turn off its dynamic transmission / reception for a relatively short time. And gNB changes the corresponding NES type as the load varies. Each of NES type could be related with a DTX configuration. Hence, there is a need to support multiple DTX configuration in a cell.

Fig1. multiple DTX configuration

Proposal1: Multiple DTX configuration should be supported in a cell.

But, for a moment, only one DTX configuration takes effect, and UE aligns with this DTX configuration. And the L1/ L2 signalling is used to dynamically indicate the DTX configuration that is going to take effect. UE should know the timing information of DTX, such as start time, the duration and period. Considering the limited bits in L1/ L2 signalling, it is impossible to carry much bits of timing information in L1/ L2 signalling. Hence, the easy way that SIB configuration + the L1/ L2 signalling enabling could be considered. SIB broadcasts the supported multiple DTX configuration including the duration and period, and L1/L2 signalling enables one of them.

In order to support enable one of DTX configurations dynamically, the semi-static configuration of the start time isn’t applicable. Hence, start time could be carried in L1/L2 signalling.

Proposal2a: SIB broadcasts the supported multiple DTX configuration including the duration and period.

Proposal2b: L1/L2 signalling enables one of DTX configurations.

Proposal2c: L1/L2 signalling carries start time of enabled DTX configurations.

Moreover, UE behavior could align with DTX configuration. If UE only stops monitoring via DRX mechanism during DTX, UL transmission continues and UL data may happen. For example, if gNB turns off reception but UE transmits UL data via CG resources, UE couldn’t receive any feedback before configuredGrantTimer expires, and UE considers the transmission successfully. Hence, we prefer UE to align with DTX via DL reception and UL transmission. If gNB determines to turn off the transmission or reception, the UL transmission (such as SR, CG) in UE could be prohibited. Due to absent DL feedback (such as dynamic PDCCH), the UL transmission is meaningless. And if gNB determines to turn off the transmission including SSB or CSI-RS, the reception and measurement in UE could be also skipped.

Proposal3a: if gNB determines to turn off the transmission or reception, the UL transmission in UE should be prohibited.

Proposal3b: if gNB determines to turn off the transmission, the reception and measurement in UE should be skipped.

# Conclusion and proposals

Based on the analysis in previous sections, the following proposals are given:

Proposal1: Multiple DTX configuration could be supported in a cell.

Proposal2a: SIB broadcasts the supported multiple DTX configuration including the duration and period.

Proposal2b: L1/L2 signalling enables one of DTX configurations.

Proposal2c: L1/L2 signalling carries start time of enabled DTX configurations.

Proposal3a: if gNB determines to turn off the transmission or reception, the UL transmission in UE should be prohibited.

Proposal3b: if gNB determines to turn off the transmission, the reception and measurement in UE should be skipped.

# References

- RAN2#119B meeting, chairman notes

# Text Proposal

---------------------------------------------------------Start of text proposal-------------------------------------------------------

6	Techniques to improve network energy savings

Editor's note: simulation results to be captured under this section.

Editor's note: RAN2 and RAN3 related aspect to be provided by using separate sections like 6.X when applicable.

6.1	Techniques in time domain

6.1.1	Cell DTX/DRX

6.1.1.x	Higher layer procedures

Cell DTX/DRX is applied to at least UEs in RRC_CONNECTED state. A periodic Cell DTX/DRX (i.e., active and non-active periods) can be configured by gNB via RRC signalling. Below examples on Cell DTX/DRX behaviour during non-active periods are assumed to be possible options, and the UE behaviour/impact will be studied:

- Example 1: gNB is expected to turn off all transmission and reception for data traffic and reference signal during Cell DTX/DRX non-active periods.

- Example 2: gNB is expected to turn off its transmission/reception only for data traffic during Cell DTX/DRX non-active periods (i.e., gNB will still transmit/receive reference signals)

- Example 3: gNB is expected to turn off its dynamic data transmission/reception during Cell DTX/DRX non-active periods (i.e., gNB is expected to still perform transmission/reception in periodic resources, including SPS, CG-PUSCH, SR, RACH, and SRS).

- Example 4: gNB is expected to only transmit reference signals (e.g., CSI-RS for measurement).

The study will focus on UE behavior when at any point in time the cell activates a single DTX/DRX configuration.

The Cell DTX mode/configuration can also be indicated to the UE via dynamic L1/L2 signalling. The dynamic L1/L2 signalling at least supports UE dedicated indication. Whether UE group common signalling is also supported will be further studied.

Multiple DTX configuration could be supported in a cell. SIB broadcasts the supported multiple DTX configuration including the duration and period. L1/L2 signalling enables one of DTX configurations. L1/L2 signalling carries start time of enabled DTX configurations.

It is beneficial to align UE DRX with Cell DTX and DRX alignment among multiple UEs. The alignment mechanism will be studied.

If gNB determines to turn off the transmission or reception, the UL transmission in UE could be prohibited. If gNB determines to turn off the transmission, the reception and measurement in UE could be skipped.