3GPP TSG-RAN WG2 Meeting #120	R2-2212220

Toulouse, France, 14th – 18th November 2022

Title: 	SHR and SPCR

Source: 	Huawei, HiSilicon

Agenda item:	8.13.4

Document Type:	discussion

Introduction

During the RAN2 discussion, we agreed to wait for the progress on RAN3 for the SPCR (Successful PSCell Change Report) and SHR for inter-RAT.

5	Network configures SPR configuration IE for the UE, with at least the following triggering conditions:

•	T310 triggering condition

•	T312 triggering condition

•	T304 triggering condition

5a: Other triggering conditions are FFS

5b: Values of the triggering conditions are FFS

5c: Which node configures the triggering condition is FFS.

6	RAN2 agree to the following:

A.	SPR configuration is configured by network through otherConfig

B.	SPR is fetched via UE Information Request/Response procedure

7	UE logs at least the following information and measurements in the SPR IE (other information and measurements are FFS).

a)	Source PSCell info (cell ID, measurement result)

b)	Target PScell info (cell ID, measurement result)

c)	Neighbour Cells info (cell ID, measurement result, CPAC Candidate cells flag)

d)	Success PSCell change/addition cause value (e.g., t304, t310, t312 cause, etc.)

f)	The time elapsed between the CPAC execution towards the target cell and the corresponding latest CPAC configuration received for the selected target cell

7a: FFS on whether to reuse CHO candidate cell flag for the CPAC candidate cells or define a new flag to indicate CPAC candidate cell.

7b: FFS on whether to include or on conditional inclusion of random access related information.

7c:	FFS on Location Information

There was a LS from RAN3 [1] for RAN2 to consider or provide the solutions.

In this paper, we will give our considerations on the above FFSs and the questions in the RAN3 LS for intra-system inter-RAT SHR and NR-DC SPR.

Discussion

Intra-system inter-RAT SHR

Intra-system inter-RAT SHR

R17 intra-NR SHR scheme

As discussed in R17 SHR, the UE is configured with SHR triggering condition(s) via the handover command message from the source node. There are four SHR triggering condition. The timer T310 and T312 related triggering conditions are decided and configured by the source cell of the handover. For the timer T304 related triggering condition, it is decided by the target cell of the handover, delivered to the source cell from the target cell and finally configured to the UE by the source cell. There is also the DAPS related triggering condition configured by the source cell in case of DAPS HO. For inter-RAT HO, DAPS is not supported.

Observation 1: SHR triggering conditions are configured by the source and/or target cell of the HO.

When the UE successfully completes the Random Access procedure with the target cell, and at least one of the SHR triggering conditions have been met, the UE stores the successful handover information into the varSuccessHO-Report.

Observation 2: The UE stores the successful handover information in the variable upon successfully completing the Random Access procedure with the target cell of the handover if at least one of the SHR triggering conditions are met.

Case 1: HO from NR to LTE within NG-RAN

According to the LS, it is known that RAN3 has agreed to support T310 and T312 related triggers for inter-RAT SHR from NR to LTE. As mentioned in the aforesaid section, the NR source node sends the MobilityFromNRCommand message to configure the UE with the T310 and T312 related triggering conditions for the handover from NR to LTE.

Proposal 1: For the SHR from NR to LTE, introduce the T310 and T312 related triggering condition in the MobilityFromNRCommand message.

For the intra-system inter-RAT HO from NR to LTE, the target ng-eNB decides the T304 related triggering condition and delivers it to the source NR node. Technically speaking, all the configuration information of the target ng-eNB is included in the Target NG-RAN node To Source NG-RAN node Transparent Container as showed in the below table. The detailed information in the container refers to the HandoverCommand defined in RAN2, which contains the RRCConnectionReconfiguration message in LTE.

| Target NG-RAN node To Source NG-RAN node Transparent Container | M |  | OCTET STRING | Either includes the HandoverCommand message as defined in subclause 10.2.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverCommand message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

–	HandoverCommand

This message is used to transfer the handover command generated by the target eNB.

Direction: target eNB to source eNB/ source RAN

HandoverCommand message

-- ASN1START

HandoverCommand ::=					SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE{

handoverCommand-r8					HandoverCommand-r8-IEs,

spare7 NULL,

spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

HandoverCommand-r8-IEs ::=			SEQUENCE {

handoverCommandMessage				OCTET STRING (CONTAINING DL-DCCH-Message),

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

HandoverCommand field descriptions

handoverCommandMessage Contains the entire DL-DCCH-Message including the RRCConnectionReconfiguration message used to perform handover within E-UTRAN or handover to E-UTRAN, generated (entirely) by the target eNB.

In R17 SHR, it is only applied to the mobility within NR and only introduces the SHR triggering condition in the NR specification. Therefore, if we want to support the SHR for the handover from NR to LTE, one potential solution is to include the T310/T312/T304 related triggering condition in the handover command message from NR to LTE. Nevertheless, based on the above HandoverCommand message from LTE to NR, it contains the RRCConnectionReconfiguration message. There is no T304 related triggering condition in the RRCConnectionReconfiguration message. If T304 related triggering condition is supported for HO from NR to LTE, it implies that the related enhancement should be captured in LTE RAN2 specification.

Observation 3: For the SHR from NR to LTE, the introduction of the T304 related triggering condition in the MobilityFromNRCommand message will have impact on the RRCConnectionReconfiguration message in LTE.

To reduce the impact on LTE as much as possible, RAN2 can deprioritize to consider the T304 related SHR triggering condition for the HO from NR to LTE.

Proposal 2: For the SHR from NR to LTE, RAN2 deprioritizes the T304 related triggering condition.

Upon successful random access procedure with the target LTE cell, and at least one of the T310 or T312 related triggering conditions have been met, the UE can store the successful handover information into the varSuccessHO-Report defined in NR. That is, the UE can generate the inter-RAT SHR in the NR format. In this way, RAN2 will not define the new variable in LTE and all the UE behaviours can refer to the descriptions in NR specification. From the specification impact point of view, this can greatly reduce the modification on the LTE specification to avoid duplicate variables and descriptions.

Proposal 3: For the SHR from NR to LTE, the UE stores the successful handover information into the NR variable varSuccessHO-Report and generates the SHR in NR format.

In the LS, it can be observed that RAN3 has discussed on the potential contents for inter-RAT successful handover and agreed on the following:

a.	Source NR cell information

b.	Target LTE cell information

c.	Measurement results for source, target and neighbours

d.	Cause to indicate which inter-RAT SHR triggering condition was met

e.	UE location Information

We think that RAN2 can directly capture the agreements made by RAN3. In addition, as discussed in the previous section, only T310 and T312 related triggering conditions will be supported for the handover from NR to LTE. For the C-RNTI in the inter-RAT SHR, it seems more reasonable to set to the source one. In this way, the source node can utilize it for the mobility information retrieval.

Proposal 4: For the SHR from NR to LTE, RAN2 includes the following information:

a.	Source NR cell information

b.	Target LTE cell information

c.	Measurement results for source, target and neighbours

d.	Cause to indicate which inter-RAT SHR triggering condition was met

e.	UE location Information

f. 	Source C-RNTI

The existing IEs defined in Rel-17 intra-NR SHR are show in the following:

SuccessHO-Report-r17 ::=                 SEQUENCE {

sourceCellInfo-r17                       SEQUENCE {

sourcePCellId-r17                        CGI-Info-Logging-r16,

sourceCellMeas-r17                       MeasResultSuccessHONR-r17                       OPTIONAL,

rlf-InSourceDAPS-r17                     ENUMERATED {true}                               OPTIONAL

},

targetCellInfo-r17                       SEQUENCE {

targetPCellId-r17                        CGI-Info-Logging-r16,

targetCellMeas-r17                       MeasResultSuccessHONR-r17                       OPTIONAL

},

measResultNeighCells-r17                 SEQUENCE {

measResultListNR-r17                     MeasResultList2NR-r16                           OPTIONAL,

measResultListEUTRA-r17                  MeasResultList2EUTRA-r16                        OPTIONAL

}                                                                                            OPTIONAL,

locationInfo-r17                         LocationInfo-r16                                    OPTIONAL,

timeSinceCHO-Reconfig-r17                TimeSinceCHO-Reconfig-r17                           OPTIONAL,

shr-Cause-r17                            SHR-Cause-r17                                       OPTIONAL,

ra-InformationCommon-r17                 RA-InformationCommon-r16                            OPTIONAL,

upInterruptionTimeAtHO-r17               UPInterruptionTimeAtHO-r17                          OPTIONAL,

c-RNTI-r17                               RNTI-Value                                          OPTIONAL,

...

}

–	CGI-Info-Logging

The IE CGI-Info-Logging indicates the NR Cell Global Identifier (NCGI) for logging purposes (e.g. RLF report), the globally unique identity, and the TAC information of a cell in NR.

CGI-Info-Logging information element

-- ASN1START

-- TAG-CGI-INFO-LOGGING-START

CGI-Info-Logging-r16 ::=     SEQUENCE {

plmn-Identity-r16                    PLMN-Identity,

cellIdentity-r16                     CellIdentity,

trackingAreaCode-r16                 TrackingAreaCode               OPTIONAL

}

-- TAG-CGI-INFO-LOGGING-STOP

-- ASN1STOP

We think that almost all the information agreed in RAN3 can reuse the existing one, other than the target LTE cell information. Currently, the IE targetCellInfo are mandatory and only be set to NR cell information. Therefore, RAN2 should introduce the new target LTE cell information to support the SHR from NR to LTE. In addition, to avoid the misunderstanding of the presences of both NR and LTE target cell, it is necessary to set the target NR cell information into a special value or any value. Simultaneously, when the source node receives the SHR including both NR and LTE target cells, it can ignore the NR one.

Proposal 5a: Introduce the new target LTE cell information in the existing SHR.

Proposal 5b: If the new target LTE cell information is set, the target NR cell information is set to a special value, e.g., 0.

In addition, we think that when the UE comes back to NR cell, the UE can report the inter-RAT SHR report.

Proposal 6: For the SHR from NR to LTE, it can be only reported in NR when the UE comes back to NR cell.

Case 2: HO from LTE to NR within NG-RAN

In RAN3, there is the ongoing discussion and decision on whether to support the SHR for the handover from LTE to NR. It is reasonable for RAN2 to wait for the progress in RAN3 and not to perform the duplicated discussion.

Proposal 7: For intra-system inter-RAT SHR, whether to support HO from LTE to NR waits for RAN3 progress.

Questions in the RAN3 LS

Based on the above analysis, for the questions in the LS from RAN3, we’d like to give our suggestions. The detailed answers are provided as below.

- Q1. Is RAN2 planning to impact LTE specifications to support inter-RAT SHR?

- In our understanding, it seems highly anticipated to avoid the impact on LTE specifications to support inter-RAT SHR. This should be considered as a basic principle when we discuss the inter-RAT related features.

- The answer to Q1: RAN2 agrees to reduce/avoid the impact on LTE specification to support inter-RAT SHR.

- Q2. Whether T304 trigger for inter-RAT SHR from NR to LTE is to be supported?

- Taking into consideration the discussion for the proposal 2, we can derive that

- The answer to Q2: T304 trigger for inter-RAT SHR from NR to LTE is NOT supported.

- Q3. If yes to Q2, whether the inter-RAT SHR is always encoded in source RAT format or can be encoded based on the RAT format which generates the inter-RAT SHR trigger condition (e.g., inter-RAT SHR encoded in NR format for T310/T312 triggers and in LTE format for T304 triggers for inter-RAT HO from NR to LTE)?

- Taking into consideration the discussion for the proposal 3, we can derive that

- The answer to Q3: The inter-RAT SHR is encoded in NR format for T310/T312 triggers.

Further RAN3 is not sure on the retrieval and reporting of inter-RAT SHR and would like to ask RAN2 the following question:

- Q4. If yes to Q2, and if inter-RAT SHR is collected due to T304 triggers (configured by target LTE node), what is RAN2’s preference on the following two options?

- Option 1: It is sufficient for UE to report the inter-RAT SHR once UE is back to NR

- Option 2: The LTE node should have the capability to retrieve the inter-RAT SHR

- Taking into consideration the discussion for the proposal 5, we can derive that

- The answer to Q4: It is sufficient for UE to report the inter-RAT SHR once UE is back to NR.

- RAN3 further discussed on the potential contents for inter-RAT successful handover and agreed on the following:

- Source NR cell information

- Target LTE cell information

- Measurement results for source, target and neighbours

- Cause to indicate which inter-RAT SHR triggering condition was met

- UE location Information

Considering there might be parallel discussion in RAN2, RAN3 would like to check the following:

- Q5: Can RAN2 confirm the support for above parameters for inter-RAT SHR from NR to LTE? Whether the existing IEs defined in Rel-17 for intra-NR SHR can be reused is up to RAN2 decision.

- Taking into consideration the discussion for the proposal 6, we can conclude that

- The answer to Q5: for bullets a, c, d and e, the existing IEs defined in R17 SHR can be reused, while for bullet b RAN2 introduces the new LTE target cell information.

NR-DC SPR

Scheme for SPR

Reporting

In RAN2, it was agreed to report the SPR via UE Information Request/Response procedure.

6	RAN2 agree to the following:

B.	SPR is fetched via UE Information Request/Response procedure

It is not clear when the UE initiates the reporting procedure. Generally, there are two solutions for the UE to report a SON report. One is to immediately report upon successful PSCell addition or change, while the other is that the UE can store the SPR for at most 48 hours and report it when connected state.

For the reporting of SHR, the UE is not limited to immediately report it to the target PCell. Instead, the UE can store the SHR for at most 48 hours. Subsequently, the UE can indicate the availability of the SHR to the serving cell where the UE enters into connected state. As for the reporting scheme of SPR, it is preferred to apply the similar principle. That is, The UE can store the SPR for at most 48 hours. The UE informs the serving network of the availability of the SPR and reports it the current serving network. The UE may discard the SPR 48 hours after the last successful PSCell addition/change information is stored or until the SPR is fetched by the network.

Proposal 8: The UE stores the SPR and discard the SPR if not fetched within 48 hours.

After the successful PSCell addition/change, the UE is configured with DC. Generally, in the previous reporting scheme for SON/MDT reports, the UE is only allowed to report them to the MN node. We believe the same principle should be inherited for the SPR. Besides, in last meeting, it is also agreed to report the SPR via UE information request and response procedures. As we all know, these kinds of procedures are sent via SRB1. Namely, RAN2 has agreed only the MN can retrieve the SPR from the UE.

Proposal 9: The SPR is retrieved by the MN.

Information in SPR

In last RAN2 meeting, there were some agreements on the SPR scheme. And the FFSs are listed below.

5	Network configures SPR configuration IE for the UE, with at least the following triggering conditions:

•	T310 triggering condition

•	T312 triggering condition

•	T304 triggering condition

5a: Other triggering conditions are FFS

5b: Values of the triggering conditions are FFS

5c: Which node configures the triggering condition is FFS.

6	RAN2 agree to the following:

A.	SPR configuration is configured by network through otherConfig

B.	SPR is fetched via UE Information Request/Response procedure

7	UE logs at least the following information and measurements in the SPR IE (other information and measurements are FFS).

a)	Source PSCell info (cell ID, measurement result)

b)	Target PScell info (cell ID, measurement result)

c)	Neighbour Cells info (cell ID, measurement result, CPAC Candidate cells flag)

d)	Success PSCell change/addition cause value (e.g., t304, t310, t312 cause, etc.)

f)	The time elapsed between the CPAC execution towards the target cell and the corresponding latest CPAC configuration received for the selected target cell

7a: FFS on whether to reuse CHO candidate cell flag for the CPAC candidate cells or define a new flag to indicate CPAC candidate cell.

7b: FFS on whether to include or on conditional inclusion of random access related information.

7c:	FFS on Location Information

5c: Which node configures the triggering condition is FFS.

In previous RAN2 meeting, three triggering conditions are agreed, including T304, 310 and T312 related one.

In RAN3, it was agreed that:

T310 of SCG and T312 of SCG are not considered as SPCR triggers for classic PSCell addition or CPA (since there is no source SN undergoing RLF).

The situation for T310/T312 and T304 is different. T310 and T312 related timers are used to detect physical layer issues between UE and source PSCell, while T304 related timer is used to detect RA related issue between UE and target PSCell. There is no doubt that SPR T304 threshold shall be determined by target SN and configured by MN to UE.

Proposal 10: The 304 related triggering condition is determined by target SN.

As agreement in RAN3, only UE is configured with DC, SCG T310 and T312 can be configured to UE and thus SPR T310,T312 can be further discussed. As for SPR T310,T312, issue has been brought up regarding which node determine these two and then which nodes configures these two to UE. There are several cases which have attracted considerable attention listed as below:

Case 1a: SN-initiated classic PSCell change / CPC

Case 1b: MN-initiated classic PSCell change / CPC

Case 2: Intra-SN classic PSCell change / CPC

In case 1a and case2, the initiating node is exactly source SN. There is no doubt that the source SN acts as the node determining SPR T310,T312 threshold. For SN-initiated classic PSCell change/CPC, the T310,T312 related triggering conditions can be decided by the source SN and sent to the UE in the IE MRDC-SecondaryCellGroupConfig in the message RRCReconfiguration via SRB1 through MN. For intra-SN classic PSCell change / CPC, the source SN can directly send the T310,T312 related triggering conditions via SRB3 without MN involvement.

Proposal 11: For SN-initiated/intra-SN classic PSCell change / CPC, the source SN determines and configures the T310, T312 related triggering condition via SRB1 or SRB3.

In case 1b, MN initiates the classic PSCell change / CPC procedure. The MN has no idea of the current SCG RLF timer configuration, which is configured by the source SN itself. Here, it is preferred to consider which node could properly determine the trigger settings to detect the underlying SCG RLF related issue.

There could several ways to address this issue. One way is that, the MN determines SPR T310,T312 related triggering conditions. In this way, the MN could just decide the ratio percentage threshold by its own, without any knowledge of the T310/312 values configured by the source SN.

Another possible way is that, the MN can determine or get the T310,T312 related triggering conditions by interaction with source SN. For example, the MN could request source SN to provide the latest SCG T310,T312 times first and then decide the ratio percentage threshold. Alternatively, MN requests the source SN to determine T310,T312 related triggering conditions. Source SN could make a decision on SPR T310, T312 threshold with the knowledge of configured SCG T310, T312 value. The Source SN can respond the configured T310, T312 related triggering conditions to MN. This brings additional complexity on both MN and source SN.

In our understanding, the motivation of the SPCR is to identify the underlying failures and try to optimize the related parameters to ensure the robustness. Some of the related parameters are set by target node, the other are set by initiating node. Therefore, we prefer the node which initiates the PSCell addition/change to decide the SPCR triggers.

Proposal 12: For MN-initiated PSCell change / CPC, the MN determines and configures the T310, T312 related triggering condition via SRB1.

As discussed in the aforesaid section, for MN-initiated PSCell change / CPC, the MN configures the T310,T312 related triggering condition via SRB1, which should be used together with the corresponding T310, T312 configured in the source SN. Upon receiving these SPR trigger conditions via SRB1 from MN, UE should correlate the T310, T312 ratio percentage threshold with SCG timer T310,T312. Currently, when the UE receives T310/T312 related triggering conditions configured MN, it will automatically correlate them with the T310/T312 configured by the MN. To inform the UE to correlate them with the ones in source SN other than MN, one solution is that, MN configures these T310/T312 related triggering conditions to UE and indicates UE to associate them with the timers configured by the source SCG respectively. Another case is that, the MN provides two sets of T310/T312 related triggering conditions. One is for the UE to correlate with the ones configured by the MN while the other for the source SN.

Proposal 13: For MN-initiated PSCell change / CPC, when the MN configures the T310, T312 related triggering condition, it indicate UE to correlate the above triggers with the T310, T312 timers configured in source SN:

- Indicator to indicate UE to correlate the above triggers with the ones configured in source SN

- Separate T310-scg, T312-scg related triggering conditions.

7a: FFS on whether to reuse CHO candidate cell flag for the CPAC candidate cells or define a new flag to indicate CPAC candidate cell.

Currently, in the R17 SHR, there is an indicator called choCandidate to indicate whether the associated cell is a candidate target cell for CHO. There is the “cho” wording in the field name. It is not suitable to reuse the existing one for the CPAC case. Correspondingly, it is preferred to define a new flag to indicate CPAC candidate cell.

choCandidate This field indicates whether the associated cell is a candidate target cell for conditional handover. This field may be included only in the SuccessHO-Report within UEInformationResponse message.

Proposal 14: Define a new flag to indicate CPAC candidate cell.

7b: FFS on whether to include or on conditional inclusion of random access related information.

Basically, it is a common understanding that the UE will record the random-access related information in the RA report upon successful RACH procedure. For the SPR, it is noted that the UE completes the successful random-access procedure and initiates the determination of the RA report. The inclusion of the random-access related information in the SPR brings unnecessary duplicated information. In R17 SHR, it was discussed to remove the duplicated random-access related information when optimizing the random-access configuration. Nevertheless, the issue is not fully discussed due to the time limitation.

In addition, as discussed in previous meetings, it is known that the signaling overhead of the random-access related information is a big problem. Especially for R18, more feature specific random-access related information will be introduced.

Observation 4: The inclusion of random-access related information in SPR brings lots of signaling overhead and is redundant with the one in the RA report.

Proposal 15: The random access related information is not introduced in SPR .

In RAN2#119bis-e meeting, some information were agreed for UE to log in the SPR.

7	UE logs at least the following information and measurements in the SPR IE (other information and measurements are FFS).

a)	Source PSCell info (cell ID, measurement result)

b)	Target PScell info (cell ID, measurement result)

c)	Neighbour Cells info (cell ID, measurement result, CPAC Candidate cells flag)

d)	Success PSCell change/addition cause value (e.g., t304, t310, t312 cause, etc.)

f)	The time elapsed between the CPAC execution towards the target cell and the corresponding latest CPAC configuration received for the selected target cell

With the above discussion in mind, if the SPR can be stored and reported later, it seems that the PCell information is missing, especially for the PSCell addition and MN-initiated PSCell change cases. Without the PCell information, the node receiving the SPR from the UE cannot correctly forward the SPR to the right node for the above two cases.

Proposal 16: The UE logs the PCell information in case of PSCell addition and MN-initiated PSCell change.

Besides, it is also noticed that the UE is not able to know the node which initiates the traditional PSCell change. One option is to log both PCell and source PSCell information in case of PSCell change. It is still questionable that whether the receiving node can identify the right node or just forward the SPR to both, respectively. Technically speaking, the whole option cannot work well but brings additional complexity on the forwarding scheme. Alternatively, the other option is that the MN can send an explicit indicator to inform UE whether the PSCell change is initiated by MN or not. The UE can include the PCell information according to the indicator.

Proposal 17: MN sends the indicator to inform UE whether MN initiates the PSCell change.

Proposal 18: The UE logs PCell information instead of PSCell if the indicator is received in case of MN-initiated PSCell change.

Question in the RAN3 LS

Based on the above analysis in section 2.2.1, for the questions in the LS from RAN3, we’d like to give our suggestions. The detailed answers are provided as below.

- Q6. Whether the SPCR can be stored at the UE and sent later to the gNB or is sent immediately after the successful PSCell change or addition?

The answer to Q6: The SPR can be stored at the UE and sent later.

- Q7. Which node (MN or SN) retrieves the SPCR from the UE?

The answer to Q7: MN retrieves the SPR from the UE.

- Q8. Which node (MN or SN) sends the Successful PSCell Change/Addition configuration to the UE?

- The answer to Q8: depending on different cases, RAN2 confirms that:

- MN sends the Successful PSCell Change/Addition configuration for the MN-initiated PSCell Change/Addition;

- For the SN-initiated PSCell Change, the source-SN sends the Successful PSCell Change configuration within the container through MN via SRB1 or directly via SRB;

Conclusion

In this paper, we discuss miscellaneous enhancements for intra-system inter-RAT SHR and SPCR, and we have the following observations and proposals:

Observation 1: SHR triggering conditions are configured by the source and/or target cell of the HO.

Observation 2: The UE stores the successful handover information in the variable upon successfully completing the Random Access procedure with the target cell of the handover if at least one of the SHR triggering conditions are met.

Observation 3: For the SHR from NR to LTE, the introduction of the T304 related triggering condition in the MobilityFromNRCommand message will have impact on the RRCConnectionReconfiguration message in LTE.

Observation 4: The inclusion of random-access related information in SPR brings lots of signaling overhead and is redundant with the one in the RA report.

Proposal 1: For the SHR from NR to LTE, introduce the T310 and T312 related triggering condition in the MobilityFromNRCommand message.

Proposal 2: For the SHR from NR to LTE, RAN2 deprioritizes the T304 related triggering condition.

Proposal 3: For the SHR from NR to LTE, the UE stores the successful handover information into the NR variable varSuccessHO-Report and generates the SHR in NR format.

Proposal 4: For the SHR from NR to LTE, RAN2 includes the following information:

a.	Source NR cell information

b.	Target LTE cell information

c.	Measurement results for source, target and neighbours

d.	Cause to indicate which inter-RAT SHR triggering condition was met

e.	UE location Information

f.  source C-RNTI

Proposal 5a: Introduce the new target LTE cell information in the existing SHR.

Proposal 5b: If the new target LTE cell information is set, the target NR cell information is set to a special value, e.g., 0.

Proposal 6: For the SHR from NR to LTE, it can be only reported in NR when the UE comes back to NR cell.

Proposal 7: For intra-system inter-RAT SHR, whether to support HO from LTE to NR waits for RAN3 progress.

Proposal 8: The UE stores the SPR and discard the SPR if not fetched within 48 hours.

Proposal 9: The SPR is retrieved by the MN.

Proposal 10: The 304 related triggering condition is determined by target SN.

Proposal 11: For SN-initiated/intra-SN classic PSCell change / CPC, the source SN determines and configures the T310, T312 related triggering condition via SRB1 or SRB3.

Proposal 12: For MN-initiated PSCell change / CPC, the MN determines and configures the T310, T312 related triggering condition via SRB1.

Proposal 13: For MN-initiated PSCell change / CPC, when the MN configures the T310, T312 related triggering condition, it indicate UE to correlate the above triggers with the T310, T312 timers configured in source SN:

- Indicator to indicate UE to correlate the above triggers with the ones configured in source SN

- Separate T310-scg, T312-scg related triggering conditions.

Proposal 14: Define a new flag to indicate CPAC candidate cell.

Proposal 15: The random access related information is not introduced in SPR .

Proposal 16:The UE logs the PCell information in case of PSCell addition and MN-initiated PSCell change.

Proposal 17: MN sends the indicator to inform UE whether MN initiates the PSCell change.

Proposal 18: The UE logs PCell information instead of PSCell if the indicator is received in case of MN-initiated PSCell change.

Reference

- R2-2209104, LS on the scope for the support of SON/MDT enhancements