3GPP TSG-RAN WG2 Meeting #120	R2-2212255

Toulouse, France, 14th – 18th of November 2022

Agenda item:	6.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Various Rel-17 CPAC Issues Requiring RAN2 Attention

WID/SID:	LTE_NR_DC_enh2-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

In October 2022 during RAN2#119bis the following topics have been postponed [1]:

Proposal 3: Discuss whether there can be a target SN without SCG in CHO with SN procedure: − Yes, i.e. as the legacy HO with SN procedure; − No, i.e. there must be an SCG in CHO. Postponed. Can be discussed based on contributions in the next meeting (if time allows and use case can be made clear). CRs should be progressed based on “No” for now, we will decide in the next meeting. Proposal 1: RAN2 to discuss and confirm which understanding is right on the restriction that UE is not required to perform the measurement on measIds that are configured for conditional reconfiguration, but are not linked with any candidate cell: - Understanding 1: the restriction applies to SN initiated inter-SN CPC only; - Understanding 2: the restriction applies to all kind of conditional reconfiguration, i.e., CHO, CPA, intra-SN CPC, MN initiated inter-SN CPC, and SN initiated inter-SN CPC; Postponed. Consider until next meeting whether this is a transitory issue or not.

In this paper we try to resolve the topics referenced above.

# 2	Discussion

In this section we elaborate on the postponed issues from RAN2#119bis.

## 2.1 	Target SN without SCG in CHO with SN

At RAN2#119bis it was not possible to conclude whether the scenario of CHO with SN without SCG shall be supported or not. This is probably not the most common scenario to consider, but it is important to note this is supported for non-conditional HO with SN (and no SCG). The use case is when SN hosts MCG bearers whose NR PDCP is in SN but RLC/MAC/PHY is provided by MN. This is also reflected in the NOTE from 37.340, section 4.2.2:

NOTE 2: If only MCG bearers are configured for a UE, i.e. there is no SCG, this is still considered an MR-DC configuration, as long as at least one of the bearers is terminated in the SN.

Observation 1: Target SN without SCG is supported as a valid MR-DC scenario for non-conditional HO.

We think it is clearer if we do not deviate from HO principles when designing CHO with MR-DC. Thus, our slight preference is to support target SN without SCG in CHO with SN.

Proposal 1: To be aligned with non-conditional HO behaviour, target SN without SCG is supported in the scenario of CHO with MR-DC.

## 2.2 	Inter-operability analysis - measIDs not linked to conditional configuration

At RAN2#119bis there was a discussion whether agreed Rel-17 behavior allowing the UE to ignore measIDs not linked to any configuration can be extended to Rel-16 conditional reconfigurations. The reason behind was to align the description in the specification. In Rel-17 this behavior was agreed solely for SN-initiated inter-SN CPC where there could be a mismatch between prepared cells by source SN (responsible for the measID) and those finally prepared by the target SN (responsible for the cell configuration).

Observation 2: The restriction that the UE is not required to perform the measurements on measIds that are configured for conditional reconfiguration, but not linked with any candidate cell was originally introduced for Rel-17 SN-initiated CPC.

If this behavior is extended to Rel-16 conditional reconfigurations (mainly CHO) then RAN2 will actually change the specification for Rel-16 UEs or Rel-16 networks. However, let’s see what the impact may be. Below we provide a brief inter-operability analysis:

- If the UE is implemented according to Rel-16 while the NW is according to Rel-17:

  - The NW does not expect the UE to measure on measID not linked to the configuration, while the UE may measure. However, it will not trigger any conditional reconfiguration, as there is no valid configuration available for this measID.

  - Thus, the only consequence is that the UE will perform measurement even though it is not expected to do so.

- If the UE is implemented according to Rel-17 while the NW is according to Rel-16:

  - The UE will not measure according to this measID. NW may expect the UE to measure, but as there is no configuration linked to this measID, no conditional reconfiguration is expected to be triggered anyway

Overall, as can be seen above, the consequences can be acceptable if RAN2 decides to align the behavior for all conditional reconfigurations.

Proposal 2: As the inter-operability shows the consequences of extending the Rel-17 SN-initiated CPC behavior allowing the UE to ignore measIDs not linked to any conditional configuration are acceptable, RAN2 is asked to align the related NR RRC description.

## 2.3 	TS 37.340-related corrections

In the Annex we provide several suggested changes to be pursued on top of the in principle agreed CR [2].

# 3	Conclusion

This paper addressed the Rel-17 CPAC-related issues. The following observations and proposals have been made:

Observation 1: Target SN without SCG is supported as a valid MR-DC scenario for non-conditional HO.

Proposal 1: To be aligned with non-conditional HO behaviour, target SN without SCG is supported in the scenario of CHO with MR-DC.

Observation 2: The restriction that the UE is not required to perform the measurements on measIds that are configured for conditional reconfiguration, but not linked with any candidate cell was originally introduced for Rel-17 SN-initiated CPC.

Proposal 2: As the inter-operability shows the consequences of extending the Rel-17 SN-initiated CPC behavior allowing the UE to ignore measIDs not linked to any conditional configuration are acceptable, RAN2 is asked to align the related NR RRC description.

# References

- R2-2210802 Report on LTE legacy, DCCA, MUSIM, Slicing, 71 GHz, XR and QoE, 3GPP TSG RAN2 Meeting #119bis Electronic Meeting, Oct 10 - 19, 2022

- R2-2210826 Corrections for DCCA enhancement 3GPP TSG RAN2 Meeting #119bis Electronic Meeting, Oct 10 - 19, 2022

# Annex: 37.340 corrections

NEXT CHANGE

## 10.x	Conditional Handover with SCG

### 10.x.1	EN-DC

The Conditional Handover with SCG procedure is used to transfer a UE context from a source MN to a target MN while CHO is configured with SCG (including CHO with SCG addition and CHO with/without SCG change). In case of the CHO with/without SCG change, the UE context at the SN is kept or moved to another SN.

Figure 10.X.1-1: Conditional Handover with SCG procedure

Figure 10.x.1-1 shows an example signaling flow for Conditional Handover with SCG.

NOTE 1:	For a CHO without SN change, the source SN and the target SN shown in Figure 10.x.1-1 are the same node.

NOTE 1a:	For a CHO with SCG addition, the source SN and steps involved with the source SN in Figure 10.x.1-1 are ignored.

1.	The source MN starts the conditional handover procedure by initiating the X2 Handover Preparation procedure including MCG configuration and, if the UE is configured with an SCG, SCG configuration. The source MN may include the (source) SN UE X2AP ID, SN ID, the UE context in the (source) SN and the Conditional Handover Information Request IE in the Handover Request message.

NOTE 2:	In case of the CHO with/without SCG change, the source MN may trigger the MN-initiated SN Modification procedure (to the source SN) to retrieve the current SCG configuration before step 1.

2.	If the candidate MN decides to keep the UE context in the SN, the candidate MN sends the SgNB Addition Request message to the SN including the SN UE X2AP ID as a reference to the UE context in the SN that was established by the source MN. If the candidate MN decides to change the SN allowing delta configuration, the candidate MN sends the SgNB Addition Request message to the candidate SN including the UE context in the source SN that was established by the source MN. Otherwise, the candidate MN may send the SgNB Addition Request message to the candidate SN including neither the SN UE X2AP ID nor the UE context in the source SN that was established by the source MN. Within the SgNB Addition Request message, the candidate MN also includes the CHO related information, i.e., CHO Information SN Addition IE.

3.	The (candidate) SN replies with the SN Addition Request Acknowledge message. The (candidate) SN may include the indication of full or delta RRC configuration.

NOTE 2a:	In CHO with SCG configuration, it is up to the candidate MN implementation to make sure that the CG-Config provided from the (candidate) SN can be used in all CHO preparations.

4.	The candidate MN includes within the Handover Request Acknowledge message a transparent container to be sent to the UE as an RRC message to perform the conditional handover, and may also provide forwarding addresses to the source MN. The candidate MN indicates to the source MN that the UE context in the SN is kept if the candidate MN and the SN decided to keep the UE context in the SN in step 2 and step 3.

5.	The source MN sends an RRCConnectionReconfiguration message to the UE, including the CHO configuration, i.e. a list of RRCConnectionReconfiguration* messages and associated execution conditions, in which each RRCConnectionReconfiguration* message contains the SCG configuration in the RRCReconfiguration** message received from the candidate SN in step 3 and an MCG configuration.

6.	The UE applies the RRCConnectionReconfiguration message received in step 5, stores the CHO configuration and replies to the MN with an RRCConnectionReconfigurationComplete message.

Editor’s Note:	FFS. It’s up to RAN3 decision when to perform early data forwarding for SN-terminated bearers.

7/8. The UE maintains connection with the source MN and, if the UE is configured with an SN, the source SN, after receiving CHO configuration, and starts evaluating the CHO execution conditions for the candidate cell(s). If at least one CHO candidate cell satisfies the corresponding CHO execution condition, the UE detaches from the source MN, applies the stored corresponding configuration for that selected candidate cell, synchronises to that candidate cell and completes the RRC handover procedure by sending RRCConnectionReconfigurationComplete* message to the target MN. The UE releases stored CHO configurations after successful completion of RRC handover procedure.

NOTE 3:	In case the target SN includes the indication of full RRC configuration, the MN performs release of the SN terminated radio bearer configuration and release and add of the NR SCG configuration part towards the UE.

9.	If configured with bearers requiring SCG radio resources, the UE synchronizes to the (target) SN.

Editor’s Note:	FFS whether the Random Access procedure towards the (target) SN is mandatory or optional, depending on whether there must be an SCG in CHO with SN procedure.

NOTE 4:	The order the UE performs Random Access towards the MN (step 7) and performs the Random Access procedure towards the (target) SN (step 9) is not defined.

10.	If the RRC connection reconfiguration procedure was successful, the target MN informs the (target) SN via SgNB Reconfiguration Complete message.

11.	The target MN sends the Handover Success message to the source MN to inform that the UE has successfully accessed the target cell.

12a/b.	The source MN sends SgNB Release Request message to the (source) SN including a Cause indicating MCG mobility and, if applicable, data forwarding information. The (source) SN acknowledges the release request. The source MN indicates to the (source) SN that the UE context in SN is kept, if it receives the indication from the target MN.

12c. The source MN sends the Handover Cancel message toward the other signalling connections or other candidate MNs, if any, to cancel CHO for the UE.

12d/e. The target MN or/and other candidate MN(s) sends the SgNB Release Request message(s) to other candidate SN(s), if configured. The other candidate SN(s) acknowledges the release request.

13a.	The source SN sends the Secondary RAT Data Usage Report message to the source MN and includes the data volumes delivered to and received from the UE over the NR radio for the related E-RABs.

NOTE 5:	The order the source SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN/target SN is not defined. The SgNB may send the report when the transmission of the related bearer is stopped.

13b.	The source MN sends the Secondary RAT Data Usage Report message to MME to provide information on the used NR resource.

14.	For bearers using RLC AM, the source MN sends the SN Status Transfer message, including, if needed, SN Status received from the source SN to the target MN. The target MN forwards the SN Status to the target SN, if needed.

15.	If applicable, data forwarding takes place from the source MN. If the SN is kept, data forwarding may be omitted for SN-terminated bearers kept in the SN.

16-19.	The target MN initiates the S1 Path Switch procedure.

NOTE 6:	If new UL TEIDs of the S-GW are included, the target MN performs the MN initiated SN Modification procedure to provide them to the SN.

20.	The target MN initiates the UE Context Release procedure towards the source MN.

21.	Upon reception of the UE Context Release message, the (source) SN releases C-plane related resources associated to the UE context towards the source MN. Any ongoing data forwarding may continue. The SN shall not release the UE context associated with the target MN if the UE context kept indication was included in the SgNB Release Request message in step 12a.

### 10.x.2	MR-DC with 5GC

The Conditional Handover with SCG procedure is used to transfer a UE context from a source MN to a target MN while CHO is configured with SCG (including CHO with SCG addition and CHO with/without SCG change). In case of the CHO with/without SCG change, the UE context at the SN is kept or moved to another SN.

Figure 10.x.2-1: Conditional Handover with SCG procedure

Figure 10.x.2-1 shows an example signaling flow for Conditional Handover with SCG.

NOTE 1:	For a CHO without SN change, the source SN and the target SN shown in Figure 10.x.2-1 are the same node.

NOTE 1a:	For a CHO with SCG addition, the source SN and steps involved with the source SN in Figure 10.x.2-1 are ignored.

1.	The source MN starts the conditional handover procedure by initiating the Xn Handover Preparation procedure including MCG configuration and, if the UE is configured with an SCG, SCG configuration. The source MN includes the (source) SN UE XnAP ID, SN ID, the UE context in the (source) SN and the Conditional Handover Information Request IE in the Handover Request message.

NOTE 2:	In case of the CHO with/without SCG change, the source MN may trigger the MN-initiated SN Modification procedure (to the source SN) to retrieve the current SCG configuration before step 1.

2.	If the candidate MN decides to keep the UE context in the SN, the candidate MN sends the SN Addition Request message to the SN including the SN UE XnAP ID as a reference to the UE context in the SN that was established by the source MN. If the candidate MN decides to change the SN allowing delta configuration, the candidate MN sends the SN Addition Request message to the candidate SN including the UE context in the source SN that was established by the source MN. Otherwise, the candidate MN may send the SN Addition Request message to the candidate SN including neither the SN UE XnAP ID nor the UE context in the source SN that was established by the source MN. Within the SN Addition Request message, the candidate MN also includes the CHO related information, i.e., CHO Information SN Addition IE.

3.	The (candidate) SN replies with the SN Addition Request Acknowledge message. The (candidate) SN may include the indication of the full or delta RRC configuration.

NOTE 2a: In CHO with SCG configuration, it is up to the candidate MN implementation to make sure that the CG-Config provided from the (candidate) SN can be used in all CHO preparations.

3a.	For the SN terminated bearers using MCG resources, the candidate MN provides Xn-U DL TNL address information in the Xn-U Address Indication message.

4.	The candidate MN includes within the Handover Request Acknowledge message the MN RRC reconfiguration message to be sent to the UE in order to perform the conditional handover, and may also provide forwarding addresses to the source MN. If PDU session split is performed in the target side during handover procedure, more than one data forwarding addresses corresponding to each node are included in the Handover Request Acknowledge message. The candidate MN indicates to the source MN that the UE context in the SN is kept if the candidate MN and the SN decided to keep the UE context in the SN in step 2 and step 3.

5.	The source MN sends an RRC reconfiguration message to the UE, including the CHO configuration, i.e. a list of RRC reconfiguration* messages and associated execution conditions, in which each RRC reconfiguration* message contains the SCG configuration in the RRC reconfiguration** message received from the candidate SN in step 3 and an MCG configuration.

6.	The UE applies the RRC reconfiguration message received in step 5, stores the CHO configuration and replies to the MN with an RRC reconfiguration complete message.

Editor’s Note:	FFS. It’s up to RAN3 decision when to perform early data forwarding for SN-terminated bearers.

7/8. The UE maintains connection with the source MN and, if the UE is configured with an SN, the source SN, after receiving CHO configuration, and starts evaluating the CHO execution conditions for the candidate cell(s). If at least one CHO candidate cell satisfies the corresponding CHO execution condition, the UE detaches from the source MN, applies the stored corresponding configuration for that selected candidate cell, synchronises to that candidate cell and completes the RRC handover procedure by sending RRC reconfiguration complete* message to the target MN. The UE releases stored CHO configurations after successful completion of RRC handover procedure.

NOTE 3:	In case the target SN includes the indication of the full RRC configuration, the MN performs release of the SN terminated radio bearer configuration and release and add of the NR SCG configuration part towards the UE.

9.	If configured with bearers requiring SCG radio resources, the UE synchronizes to the (target) SN.

Editor’s Note:	FFS whether the Random Access procedure towards the (target) SN is mandatory or optional, depending on whether there must be an SCG in CHO with SN procedure.

NOTE 4:	The order the UE performs Random Access towards the MN (step 7) and performs the Random Access procedure towards the (target) SN (step 9) is not defined.

10.	If the RRC connection reconfiguration procedure was successful, the target MN informs the (target) SN via SN Reconfiguration Complete message.

11.	The target MN sends the Handover Success message to the source MN to inform that the UE has successfully accessed the target cell.

12a/b.	The source MN sends SN Release Request message to the (source) SN including a Cause indicating MCG mobility. The (source) SN acknowledges the release request. The source MN indicates to the (source) SN that the UE context in SN is kept, if it receives the indication from the target MN.

12c.	The source MN sends XN-U Address Indication message to the (source) SN to transfer data forwarding information. More than one data forwarding addresses may be provided if the PDU session is split in the target side.

12d. The source MN sends the Handover Cancel message toward the other signalling connections or other candidate MNs, if any, to cancel CHO for the UE.

12e/f. The target MN or/and other candidate MN(s) sends the SN Release Request message(s) to other candidate SN(s), if configured. The other candidate SN(s) acknowledges the release request.

13a. The source SN sends the Secondary RAT Data Usage Report message to the source MN and includes the data volumes delivered to and received from the UE over the NR/E-UTRA radio as described in clause 10.11.2.

NOTE 5:	The order the source SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN/target SN is not defined. The SN may send the report when the transmission of the related QoS is stopped.

13b. The source MN sends the Secondary RAT Data Usage Report message to AMF to provide information on the used NR/E-UTRA resource.

14.	For bearers using RLC AM, the source MN sends the SN Status Transfer message to the target MN, including, if needed, SN Status received from the source SN. The target forwards the SN Status to the target SN, if needed.

15.	If applicable, data forwarding takes place from the source side. If the SN is kept, data forwarding may be omitted for the SN terminated bearers or QoS flows kept in the SN.

16-19.	The target MN initiates the Path Switch procedure. If the target MN includes multiple DL TEIDs for one PDU session in the Path Switch Request message, multiple UL TEID of the UPF for the PDU session should be included in the Path Switch Ack message in case there is TEID update in UPF.

NOTE 6:	If new UL TEIDs of the UPF for SN are included, the target MN performs MN initiated SN Modification procedure to provide them to the SN.

20.	The target MN initiates the UE Context Release procedure towards the source MN.

21.	Upon reception of the UE Context Release message from source MN, the (source) SN releases C-plane related resources associated to the UE context towards the source MN. Any ongoing data forwarding may continue. The SN shall not release the UE context associated with the target MN if the UE contest kept indication was included in the SN Release Request message in step 12a.

END OF CHANGE