3GPP TSG-RAN WG2 #120	Tdoc R2-2212388

Toulouse, November 14 –18, 2022

Agenda Item:	6.21.1

Source:	Ericsson

Title:	Capability for per-FR gaps

Document for:	Discussion, Decision

# 1	Introduction

In RAN2#119bis-e, the following was concluded on Per-FR gap based on RAN#97-e plenary discussion:

Exclude Alt 1.1 for now.

On the table: Alt 1.3, Alt 1.3 per BC, Alt 2 (add info, based on current config as today, FFS excl/incl DC)

This contribution further discusses the solutions considered above and proposes a way forward.

# 2	Discussion

The solutions initially outlined in RAN#97-e were Alt. 1.1, Alt 1.2 and Alt 1.3. Alt. 1.1 was excluded for now in RAN2 discussion due to the heavy increase in signaling size, mainly due to two aspects:

- The additional of a new field per BC granularity;

- The inclusion of lower order BCs where the UE can support independent per FR measurement gap configuration (i.e. since in high order BCs the UE would likely not indicate support of the feature);

A new Alt 1.3 per BC was discussed in RAN2#119bis-e, that proposes to include a new field per BC to indicate support of this feature – even if included in a BC, the field would indicate the maximum number of CCs/bands up to which the UE would support the feature. This would avoid issue 2) above, but it would still contain issue 1) since the solution is added per BC granularity. In fact, such number of CCs would be much larger than a single bit to indicate support of the feature, since the number of CCs up to which the UE supports the feature should be indicated.

The addition of a new per BC field may increase signaling and complexity, which applies not only to Alt 1.1 discussed previously but also Alt 1.3 per BC.

Alt 1.3 suggests to limit the report of this capability in order to reduce the signaling size issue of Alt 1.1, mainly relying on the number of carriers to decide upon when the field should be included or not. We understand the approach on Alt 1.3 may have a drawback since the UE reporting of this feature may not depend exclusively on the number of CCs but how the UE resources are shared between those. However, we understand the number of CCs is the main contributor, this seemed to be highlighted in [1] when companies asked for further clarification on what is the issue. Hence, we think this alternative is a good compromise between signaling size and complexity. However, to achieve this, a per UE field should be defined rather than a per BC one; otherwise the benefits of this alternative would not be applicable, since one would anyway have signaling increase due to the addition of the field in per BC granularity.

Adding a per UE field depending on the number of CCs is a good compromise between signaling size and complexity.

A possible implementation of such alternative is to add a per UE capability with which the UE can also indicate the number of CCs for which the feature is applicable. CRs are provided in R2-2212389 and R2-2212390 to incorporate this.

Alt 2 is based on the current NeedForGap framework and would basically include in such framework the UE support of independent per FR measurement gap configuration. We think this alternative has limited benefit since the UE support would only be known for the UE current configuration. Furthermore, this solution would have impact on inter-node messages since the MN would have to inform the SN on the UE support of the feature for the current configuration.

Re-use the framework of NeedForGap would be more limiting while also having impact on inter-node messages.

Therefore, given the aspects considered above, Alt1.3 seems the most feasible and thus we propose to progress on this issue taking this alternative as baseline.

Define a new per UE field to indicate support of Per-FR-Gaps based on the number of carriers indicated within a band combination.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	The addition of a new per BC field may increase signaling and complexity, which applies not only to Alt 1.1 discussed previously but also Alt 1.3 per BC.

Observation 2	Adding a per UE field depending on the number of CCs is a good compromise between signaling size and complexity.

Observation 3	Re-use the framework of NeedForGap would be more limiting while also having impact on inter-node messages.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Define a new per UE field to indicate support of Per-FR-Gaps based on the number of carriers indicated within a band combination.