3GPP TSG-RAN WG2 Meeting #120	R2-2212457

Toulouse, France, 14 – 18 November 2022

Agenda item:	8.14.2

Source:	Nokia, Nokia Shanghai Bell

Title:	QMC enhancements for NR MBS

WID/SID:	NR_QoE_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

3GPP Rel-17 standardized NR QoE supporting the Application Layer Measurement Collection functionality is only for RRC_Connected mode. In Rel-18, the enhanced QoE (RP-213594) will apply also in Idle/Inactive mode, e.g., to support Application Layer measurement Collection functionality for MBS service received in RRC_Idle/Inactive mode. It is expected that the UE would be able to make QoE measurements during Idle/Inactive RRC states and report QoE measurements back to the network. The initial set of agreements made to facilitate the enhancements was agreed in RAN2#119-e.

In this contribution we elaborate potential way forward for extending the QoE framework to RRC Idle and RRC Inactive for MBS broadcast service.

# 2	Discussion

## 2.1	MBS characteristics

For multicast/broadcast services (MBS), NR supports (38.300):

- 	Broadcast communication service - the same service and the same specific content data are provided simultaneously to all UEs in a geographical area (i.e., all UEs in the broadcast service area are authorized to receive the data). A broadcast communication service is delivered to the UEs using a broadcast session. A UE can receive a broadcast communication service in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state.

- 	Multicast communication service - the same service and the same specific content data are provided simultaneously to a dedicated set of UEs (i.e., not all UEs in the multicast service area are authorized to receive the data). A multicast communication service is delivered to the UEs using a multicast session. A UE can receive a multicast communication service in RRC_CONNECTED state.

Observation 1: The UE may receive broadcast communication service in all RRC states.

Observation 2: The same specific content data are provided simultaneously to all UEs.

Observation 3: A broadcast communication service is delivered to the UEs using a broadcast session.

Bearing the characteristic in mind, we discuss in the subsequent sections what enhancements may be required to facilitate QoE data collection support for MBS broadcast service.

## 2.2	QoE configuration for MBS support

The agreed WID in RP-221803 targets enhancements for QoE measurement configuration and collection in RRC_INACTIVE and RRC_IDLE states for MBS, at least for broadcast service. NR QoE framework supporting needs to enable data collection for the service that can be continued in the UE through all RRC states, as the UE may receive broadcast communication service in all RRC states. To enable this RAN2#119-bis agreed the QoE configuration for MBS broadcast service will be provided via dedicated signalling. The UE will store the configuration and perform QoE measurements accordingly. However, it was left open if the configuration can be done in Idle and Inactive states:

The gNB can send the QoE configuration for MBS broadcast service to UE by RRC message in RRC_CONNECTED via dedicated signalling. The UE stores the configuration for QoE and performs the application layer measurement for MBS broadcast service.

FFS if configuration can be done in IDLE/INACTIVE states.

- The baseline principles for QoE measurement collection for MBS services in RRC_INACTIVE and RRC_IDLE states are:

- 1)  	The UE is configured with IDLE/INACTIVE QoE via RRC.

It remains open whether the same dedicated QoE configuration can be used by the UE in RRC_Inactive and RRC Idle. Though, we note that the continued support of the QoE through RRC_Connected and RRC_Inactive states is already supported.

Observation 4: Continued QoE support through RRC_Connected and RRC_Inactive states is already supported (the same QoE configuration is used).

The known problem is that the QoE configuration is released upon transition to RRC_Idle. In principle, the QoE could be treated similar to the reporting of logged MDT data. For instance, the QoE configuration message received in RRC_Connected could be maintained in all RRC states. However, it is worth noting that a unified and continued RRC configuration for all RRC states may be not suitable for all the scenarios, as the MBS communication mode can change or be activated after the UE transitions to RRC_Idle. In such a case the container-based configuration for all RRC states may not sufficient due to missing appropriate guideline to the UE for activating the actual measurements session start. Given the QMC configuration (originated from CN) for MBS service is RRC-state agnostic, we believe, that gNB should be able to provide delta or extra configuration parameters for MBS broadcast service that will become relevant only after a UE state transition to RRC Inactive and/or RRC_Idle:

Proposal 1: The UE stores the dedicated QoE configuration for MBS broadcast service measurements in RRC_Inactive and RRC_Idle states.

The stored dedicated QoE configuration enables continued QoE data collection UE stay in RRC_Inactive and RRC_Idle mode, but to instruct the UE with expected handling of the stored data (e.g., validity of the configuration, retrieval related parameters, destination address TCE/MCE, etc)., a complementary configuration is needed too:

Proposal 2: The gNB provides to the UE delta configuration with additional MBS specific parameters applicable only during its stay in RRC_Idle and RRC_Inactive state.

## 2.3	Buffered data

In Rel-17, the RRC state that was buffering QoE data was RRC_Inactive or RRC_Connected (upon Pause command). Following the Rel-17 QMC support, any QMC data received from Application Layer may be directly mapped into the measReportAppLayerContainer on RRC layer for measurement reporting to the network side, after the UE returns to RRC_Connected or after the QoE Pause is resumed. Certain loses of data were allowed (e.g. the UE could discard data if the QoE configuration got removed from AS Inactive context). In Rel-18, as it is expected the UE needs to continue with the same configuration, thus the situation also changes with the buffered data. Given the UE may receive continued service, the stored data will impact the UE memory more seriously.

For the MBS transmission observability, e.g., to compare reception of the service in Idle/Inactive UEs, we believe Rel-18 solution should provide certain mean to overcome discarding QoE data at a high rate. One possibility is to extend UE memory requirement, but it does not offer any choice when it comes to the importance of the data. For that reason, we propose the gNB instructs the UE how the stored data should be handled once the reserved memory becomes full/inaccessible.

Proposal 3: The gNB remains in control how the stored data should be handled once the reserved memory becomes full when the UE is in RRC_Idle/Inactive state.

## 2.4	QoE reporting from RRC Idle and RRC Inactive

If the gNB remain in control how the stored data should be handled (in RRC_Idle/Inactive state), the UE may get a configuration when it is allowed to come back to RRC_Connected state to report the data. This corresponds to the open point noted in RAN2#119:

- 3)	FFS if UE can setup/resume RRC connection just for QoE reporting, or whether the QoE reports are sent to the network when the UE moves to RRC CONNECTED state due to other reasons.

Otherwise, allowing the UEs to resume just for QoE report without network control, may lead to signalling storm and unexpected load to the network.

Proposal 4:  The UE can perform a resume or setup of the RRC connection to report buffered QoE data, if such behaviour was preconfigured by the gNB.

## 2.5	UEs selection for QoE session

Given the MBS broadcast service characteristics it can be provided simultaneously to all UEs in a geographical area (i.e., all UEs in the broadcast service area are authorized to receive the data). To ensure controlled data collection through all RRC states of the UEs, RAN2#119 identified a need to discuss the way how the gNB selects the MBS users:

FFS how does gNB determine which UEs can be configured with MBS QoE measurements

The selection of the UEs should be a complementary solution that helps to overcome Idle/Inactive UEs involvement. In contrary to Rel-17, Rel-18 QoE for MBS broadcast service might result in a massive reporting and long sessions. To ensure the constrains with buffering data and generating higher load the QoE configuration should support the option to choose UEs in a selective manner. From NW perspective, it might be beneficial to involve UEs who have QoE measurement configuration but have potential to have poor MBS service experience.

Proposal 5: The gNB can select for QoE configuration MBS UEs that experience poor MBS service experience.

## 2.6	Alignment with the existing radio related measurement

The WI objective for MBS support, specifies additional requirement to enable the alignment of the existing radio related measurement and QoE reporting. Rel-17 already specifies that for the alignment of QoE measurements with radio metrics, MDT is used: The radio-related measurements may be collected via immediate MDT for all types of supported services for the purpose of QoE analysis. The MCE/TCE performs the correlation of the immediate MDT results and the QoE measurement results collected at the same UE (TS38.300).

Hence, if the UE is configured for MBS QMC (including RRC_IDLE), the natural choice by the network would be to use Logged MDT in order to align the radio related data in RRC_IDLE with the ongoing QoE session.

Observation 5:  MBS QoE reports alignment with radio measurement reports can be easily achieved by simultaneous configuration of MBS QoE session and MDT session (Immediate and Logged MDT).

Proposal 6: Rel-17 mechanism for alignment of QoE sessions with MDT sessions is the baseline for the radio measurements alignment with MBS QMC.

# 3	Conclusion

This document has made the following proposals:

Proposal 1: The UE stores the dedicated QoE configuration for MBS broadcast service measurements in RRC_Inactive and RRC_Idle states.

Proposal 2: The gNB provides to the UE delta configuration with additional MBS specific parameters applicable only during its stay in RRC_Idle and RRC_Inactive state.

Proposal 3: The gNB remains in control how the stored data should be handled once the reserved memory becomes full when the UE is in RRC_Idle/Inactive state.

Proposal 4:  The UE can perform a resume or setup of the RRC connection to report buffered QoE data, if such behaviour was preconfigured by the gNB.

Proposal 5: The gNB can select for QoE configuration MBS UEs that experience poor MBS service experience.

Proposal 6: Rel-17 mechanism for alignment of QoE sessions with MDT sessions is the baseline for the radio measurements alignment with MBS QMC.