3GPP TSG-RAN WG2 #120	R2-2212493

Toulouse, France, 14 – 18 November 2022

Agenda Item:	8.1.3

Source:	Ericsson

Title:	Discussion on capabilities and remaining issues for NCR

Document for:	Discussion, Decision

# 1	Introduction

In the last RAN2#119-bis-e meeting, the NCR topic was discussed, and the following agreements were taken:

Agreement:
 
RAN2 confirms to use RRC signalling to configure NCR-MT to receive side control information. How the side control information itself is transmitted (i.e. via RRC or DCI or MAC CE) is up to RAN1 (RAN2 may discussion the initial RAN1 decision and revisit if needed)

NCR-MT supports RRC_CONNECTED and RRC_IDLE states, FFS on RRC_INACTIVE state (e.g. optional support or not support).

NCR-MT supports SRB0/1/2 and DRB is optional. FFS on maximum number of DRBs

NCR-MT should ignore cellBarred, cellReservedForOperatorUse, cellReservedForFutureUse， cellReservedForOtherUse, intraFreqReselection indications and UAC configuration if broadcast in system information.

RRM functions supported by NCR-MR: 
- Cell selection is mandatory
- Cell reselection, RLM, BFD, BFR are FFS

In this contribution we will provide our view on what feature an NRC should support and we will also address aspects related to the Reply LS received by SA3 in R2-2211173.

# 2	Discussion

## 2.1	What capabilities the NCR-MT should support

One of the objectives of the work item is also to study which capabilities and RRM functions need to be supported by the NCR-MT. This is because the NCR should be a relatively inexpensive network node and thus the NCR-MT may not need to support the whole set of features that are typical for a normal UE. On top of this, one needs to keep in mind that the NCR-MT is just a function entity that communicates with a gNB via a Control link (C-link) to enable exchange of control information (e.g., side control information at least for the control of NCR-Fwd).

The NCR-MT is defined as a function entity to communicate with a gNB via a Control link (C-link) to enable exchange of control information (e.g., side control information at least for the control of NCR-Fwd).

Another aspect that needs to be clarified and kept in mind when studying which capabilities an NCR node should support, is that the NCR has to be a network node fundamentally cheaper and simpler than an IAB node and thus what applies to an IAB (or an IAB-MT in particular) does not need to necessarily apply also for NCR-MT.

The NCR has to be a network node fundamentally cheaper and simpler than an IAB node and thus what applies to an IAB (or an IAB-MT in particular) does not need to necessarily apply also for NCR-MT

If we take inspiration by the work that has been done during the Rel-16 IAB work item, one option would be to check in the TR 38.822 [3] which features may be needed to be supported by the NCR. A possible example of what feature may be needed for the NCR is shown in Table 1 where we have the following:

- Highlighted in green are the features that shall be supported by an NCR (without capability signalling)

- Highlighted in yellow are the features that could be supported by an NCR (optional with or without capability signalling) but that RAN2 needs to discuss.

Table 1. List of features that an NCR shall, may, and not support.

| Features | Index | Feature group | Components | Mandatory/Optional | Ericsson comment |
| --- | --- | --- | --- | --- | --- |
| 0. General (including supported bearer types) | 0-1 | Access stratum release | Access stratum release | Optional with capability signalling and candidate value set is {Rel-15, spare7, … , spare1} | Can support. Not really a “feature” |
|  | 0-3 | DRB | Only 1) Maximum number of DRBs | 1) In this case can be optional. | Our understanding is that at most 1 DRB may be needed for OAM. |
|  | 0-7 | PCell operation | 1) PCell operation on FR2 | Mandatory with capability signalling | Primary focus for the NCR should be FR2 and thus it makes sense to have it |
| 1. PDCP | 1-0 | Basic PDCP procedures | 1) (de)Ciphering on DRB/SRB 2) Integrity protection on SRB 3) Timer based SDU discard 4) Re-ordering and in-order delivery 5) Status reporting 6) Duplicate discarding 7) 18bits SN | Mandatory without capability signalling | The yellow highlighted feature are not critical. |
|  | 1-5 | Short SN | Short SN | Mandatory with capability signalling | There is some value in having the short SN, even if maybe this is not a critical feature. |
|  | 1-7 | DRB IP data rate | 1) DRB IP data rate in DL 2) DRB IP data rate in UL | Optional capability is signalled by NAS signalling defined in 24.501 | Our understanding is that at most 1 DRB may be needed for OAM. |
| 2. RLC | 2-0 | Basic RLC procedures | 1) RLC TM 2) RLC AM with 18bits SN* 3) SDU discard | Mandatory without capability signalling | At least RLC AM and RLC TM may be needed for some RRC procedure. About the discarding of SDUs, this maybe can be discussed. |
|  | 2-1 | RLC AM with short SN | RLC AM with short SN | Mandatory with capability signalling |  |
|  | 2-2 | RLC UM with short SN | RLC UM with short SN | Mandatory with capability signalling |  |
|  | 2-4 | NR RLC SN size for SRB | NR RLC SN size for SRB | RAN2 decided only short RLC SN is used for SRB. | RAN2 needs to discuss what value are suitable for NCR. |
| 3. MAC | 3-0 | Basic MAC procedures | 1) RA procedure on PCell or PSCell (in case of EN-DC) 2) UE initiated RA procedure (including for beam recovery purpose) 3) NW initiated RA procedure (i.e. based on PDCCH) 4) Support of ssb-Threshold and association between preamble/PRACH occasion and SSB 5) Preamble grouping 6) UL single TA maintenance 7) HARQ operation for DL and UL 8) LCH prioritization 9) Prioritized bit rate 10) Multiplexing 11) SR with single SR configuration 12) BSR 13) PHR 14) 8bits and 16bits L field | Mandatory without capability signallling |  |
|  | 3-3 | DRX | 1) DRX with long DRX cycle 2) DRX with short DRX cycle | Mandatory with capability signalling | There is some value in have a DRX configured at the NCR-MT, also since there cases where the NCR-Fwd part is OFF. |
| 4. Measurements | 4-1 | Intra-NR measurements and reports | 1) Intra-frequency and inter-frequency measurements and reports 2) Event A-based measurement and measurement report | Mandatory with capability signalling for NR SA. | RAN2 needs to discuss whether there is a need of L3 measurements, also considering the mobility shall not be supported. |
|  | 4-4 | Measurement gaps | Additional measurement gap configurations | Optional with capability signalling and candidate value set is: BIT STRING (SIZE (22)) | RAN2 needs to discuss whether there is a need of L3 measurements, also considering the mobility shall not be supported. |
| 5. SDAP | 5-1 | QoS | 1) Flow-based QoS 2) Multiple flows to 1 DRB mapping 3) AS reflective QoS | 1), 2) Mandatory without capability signalling 3) Optional with capability signalling | Our understanding is that at most 1 DRB may be needed for OAM. However, these features may not been needed. |
|  | 5-2 | HD format | 1) DL SDAP HD 2) UL SDAP HD 3) SDAP End-marker | 1) Conditional mandatory if either NAS reflective QoS or AS reflective QoS is supported. No capability signalling is needed. 2), 3) Mandatory without capability signalling | Our understanding is that at most 1 DRB may be needed for OAM. However, these feature may not been needed. |
| 7. Mobility | 7-1 | Handover | 1) Intra-frequency HO 2) Inter-frequency HO 3) HO between TDD and FDD 4) HO from NR to LTE 5) HO from NR to LTE with 5GC 6) HO between FR1 and FR2 | 1) Mandatory with capability signalling 2) Mandatory with capability signalling 3) Mandatory with capability signalling if the UE supports both TDD and FDD. 4) and 5) Mandatory with capability signalling if the UE supports the associated RAT. 6) Mandatory with capability signalling if the UE supports both FR1 and FR2. | If RAN2 assumes that an NCR-MT stays on a frequency, no handover is needed. Handover to LTE is not needed. Also, the support for HO should be mandatory with capability signalling. |
| 9. RRC | 9-1 | RRC buffer size | Maximum overall RRC configuration size | 45 Kbytes | One approach is to reuse the existing value. |
|  | 9-2 | RRC processing time | 1) RRC connection establishment 2) RRC connection resume without SCell addition/release and SCG establishment/modification/release 3) RRC connection reconfiguration without SCell addition/release and SCG establishment/modification/release 4) RRC connection re-establishment. 5) RRC connection reconfiguration with sync procedure 6) RRC connection reconfiguration with SCell addition/release or SCG establishment/modification/release 7) RRC connection resume 8) Initial security activation 9) Counter check 10) UE capability transfer | 1) to 3) 10ms 4) 10ms 5): 10ms + additional delay (cell search time and synchronization) defined in TS 38.133 6) and 7) 16ms 7) 10 or 6ms (See details in 12, TS 38.331) 8) and 9) 5ms 10) 80ms | One approach is to use the same numbers as for normal UEs. But some of these features are of course not supported (e.g. CA), but will be discussed in other rows. About 5) the only case where this may be needed without mobility could be due to a security key change. |

RAN2 to discuss which features are supported by the NCR-MT using the list shown in Table 1 as baseline.

## 2.2	Discussion on the SA3 Reply LS in R2-2211173

According to the reply given by SA3 in the LS in R2-2211173, it is straightforward that the Solution 2, as captured in TR 38.867 clause 8.1.2 should not be considered as one possible solution for the authorization/validation procedure of an NCR. In fact, SA3 replied the following:

To SA3 Q1a: Is there any security issue for solution 2 which does not provide Uu security, non-protected NCR indication info and the OAM container in Step 5?

Answer to RAN3:

Yes. For solution 2, SA3 believes that this information can be tampered due to the lack of Uu security. It exposes the OAM indirectly to attacks over the air interface.

Therefore, we propose:

RAN2 to agree that Solution 2 as capture in TR 38.867, clause 8.1.2, is not suitable for the authorization/validation of the NCR.

The other question sent to SA3 was related to the Solution 1 captured in TR 38.867, clause 8.1.1. In this case, SA3 was not able to conclude on whether there is a security concern or not because some information was missing and some concepts were not really clear. In particular, the reply from SA3 is the following:

To SA3 Q1b: Does SA3 believe that the NCR needs to be securely validated? Any security issue for configuring locally stored information in the gNB in Solution 1?

Answer to RAN3:

For the 1st question in Q1b, SA3 is not clear about what does "validation" mean.

For the 2nd question in Q1b, SA3 cannot provide answers before the security validation related steps in solution1 are clarified. In addition, the feasibility of such additional steps and what kind of information is stored in RAN are also unclear. Further clarification is expected.

What emerges from the SA3’s reply is that two aspects created confusion during the discussion:

- What “validation” means.

- What information is included in step 12 of Solution 1.

Regarding ‘what “validation” means’, the understanding of solution 1 is that first the UE needs to be authenticated only via a simple “NCR indication info” that, in this case, can simply be a 1-bit indication where the UE indicates whether is an NCR-MT or not. If we take the example of IAB, this could be something like:

iab-NodeIndication-r16              ENUMERATED {true}                               OPTIONAL,

RAN2 to clarify to SA3 that the UE needs initially to be authorized based on the NCR indication info in steps 1-10 which is a 1-bit indication to indicate that the UE is basically an NCR-MT

Now the confusion in SA3 was mainly about step 12 of Solution 1. Here the understanding is that, once that the UE has been already authorized in step 1-10, if the NG-RAN node (or the AMF) is not sure on whether the NCR has the right authority to communicate with the network, then it can request “additional information” from the NCR-MT.

RAN2 to clarify to SA3 that the step 12 of Solution 1 is needed if the NG-RAN (or AMF) is not sure if the NCR has the right authority to establish a connection with the network.

Once arrived at the step 12, it is clear that the “NCR credential info” needed by the NG-RAN (or AMF) are basically something related to the NCR-MT identity. In this case, it can be:

- A device serial number.

- An identifier in order to identity the NCR logically or physically.

- A 3GPP-assigned identifier.

RAN2 to clarify to SA3 that the NCR credential info container in step 12 of Solution 1 could be a 3GPP-assigned identifier, a device serial number, or an identifier used to identify the NCR logically or physically.

What is maybe clear, is that the step12 will trigger an additional device authentication procedure at the NG-RAN. In addition to this, another negative aspect of this would be that between the time of step10 to the time of step12 (that by the way, is optional), the NCR may operate in the network without being fully authenticated. Thus, we propose:

RAN2 to clarify that in case of Solution 1, the “validation” of step 12 is basically a further authentication process by the NG-RAN has the NCR may not be fully authenticated in step 1-10.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	The NCR-MT is defined as a function entity to communicate with a gNB via a Control link (C-link) to enable exchange of control information (e.g., side control information at least for the control of NCR-Fwd).

Observation 2	The NCR has to be a network node fundamentally cheaper and simpler than an IAB node and thus what applies to an IAB (or an IAB-MT in particular) does not need to necessarily apply also for NCR-MT

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to discuss which features are supported by the NCR-MT using the list shown in Table 1 as baseline.

Proposal 2	RAN2 to agree that Solution 2 as capture in TR 38.867, clause 8.1.2, is not suitable for the authorization/validation of the NCR.

Proposal 3	RAN2 to clarify to SA3 that the UE needs initially to be authorized based on the NCR indication info in steps 1-10 which is a 1-bit indication to indicate that the UE is basically an NCR-MT

Proposal 4	RAN2 to clarify to SA3 that the step 12 of Solution 1 is needed if the NG-RAN (or AMF) is not sure if the NCR has the right authority to establish a connection with the network.

Proposal 5	RAN2 to clarify to SA3 that the NCR credential info container in step 12 of Solution 1 could be a 3GPP-assigned identifier, a device serial number, or an identifier used to identify the NCR logically or physically.

Proposal 6	RAN2 to clarify that in case of Solution 1, the “validation” of step 12 is basically a further authentication process by the NG-RAN has the NCR may not be fully authenticated in step 1-10.

# 4	References

RP-222673, “New WID on NR network-controlled repeaters”, ZTE, Sanechips, RAN#97-e, September 2022

3GPP TR 38.867, “Study on NR Network-controlled Repeaters”, RAN1 (RAN2, RAN3), v18.0.0, September 2022

3GPP TR 38.822, “NR; User Equipment (UE) feature list”, RAN2, v16.3.0, April 2022