3GPP TSG-RAN WG2 Meeting #120	R2-2212520

Offline, Nov, 2022

Agenda Item	: 8.9.3

Source	: LG Electronics Inc.

Title	: Service continuity enhancements for L2 U2N relay

Document for	: Discussion and Decision

1.	Introduction

The work item on NR sidelink relay enhancements was approved for Rel-18 and the recent WID [1] is shown in RP-221262 including the following objectives:

Specify mechanisms to enhance service continuity for single-hop Layer-2 UE-to-Network relay for the following scenarios [RAN2, RAN3]: - Inter-gNB indirect-to-direct path switching (i.e., “remote UE <-> relay UE A <-> gNB X” to “remote UE <-> gNB Y”) - Inter-gNB direct-to-indirect path switching (i.e., “remote UE <-> gNB X” to “remote UE <-> relay UE A <-> gNB Y”) - Intra-gNB indirect-to-indirect path switching (i.e., “remote UE <-> relay UE A <-> gNB X” to “remote UE <-> relay UE B <-> gNB X”) - Inter-gNB indirect-to-indirect path switching (i.e., “remote UE<-> relay UE A <-> gNB X” to “remote UE <-> relay UE B <-> gNB Y”) Note 2A: Scenario D is to be supported by reusing solutions for the other scenarios without specific optimizations.

In Rel-17 U2N relay, service continuity was handled within the intra-cell limitation. In Rel-18 U2N L2 service continuity enhancements, RAN2 will handle service continuity between inter-gNB. Rel-18 remote UE can chance its path from indirect in one cell to direct in the other cell or vice versa. In addition, Rel-18 remote UE can chance its indirect link to another indirect link in the intra cells or inter cells maintaining service continuity.

In this document, we’d like to share our view about how to handle the service continuity enhancement for Rel-18 U2N.

2.	Discussion

We think that the existing Rel-17 L2 U2N service continuity procedure can be applied as a baseline in Rel-18 L2 U2N service continuity enhancement. However, we need to discuss more especially for the parts not to be handled in Rel-17 L2 U2N service continuity. In Rel-18 L2 U2N service continuity, we need to discuss indirect-to-indirect relay operation. And also, we have to handle path switching between inter-gNB. When remote UE moves to the other gNB, there could be two path choices, for example, direct path or indirect path. It needs to be discussed which gNB, source gNB or target gNB, has to decide the path type (i.e., direct path or indirect path). And also, if the source gNB or target gNB decides the path type as an indirect path, we can discuss which gNB will decide the final target relay UE among candidate relay UEs.

2.1 New measurement event for indirect-to-indirect path switching

In Rel-17 SL U2N Relay, new measurement events were defined. Event X1(serving L2 U2N Relay UE becomes worse than threshold1 and NR Cell becomes better than threshold2) and event X2(serving L2 U2N Relay UE becomes worse than a threshold) are supported for path switching from I2D. Event Y1 (PCell becomes worse than threshold1 and candidate L2 U2N relay UE becomes better than threshold2) and event Y2 (candidate L2 U2N Relay UE becomes better than a threshold) are supported for path switching from D2I. These path-switching measurement events can apply to the Rel-18 path-switching measurement from direct to indirect, or vice versa [2].

The above described Rel-17 U2N relay events have their own meaning. The events X1 and X2 were defined for I2D path switching. Both events were activated at least when the SL signal strength with the serving U2N relay UE becomes worse than a threshold. The event Y1 was defined for D2I path switching. At least, when the current signal strength of the connected direct Uu link becomes worse than threshold1, the event Y1 can be activated. The Y2 event was also for D2I path switching. This Y2 event activates even though the current signal strength of the connected direct Uu link is not getting worse. If there are candidate relay UEs having better SL quality than a threshold, gNB can consider moving from the direct path to the indirect path because the UE has the capability to be a remote UE. Considering the above implicit meaning of Rel-17 SL relay events, if the remote UE can transmit/receive well in the current indirect link, there is no reason to trigger to report the information of the other candidate relay UEs. Reporting candidate relay UEs even though the current indirect link quality is good can cause frequent relay reselection.

Observation 1: In rel-17 U2N relay, X1 and X2 events was defined for I2D path switching. Both two events was activated at lease when the serving U2N relay UE becomes worse than threshold.

Observation 2: In rel-17 U2N relay, Y1 events was defined for D2I path switching. At least, when the current signal strength of connected direct Uu link becomes worse than threshold1, the event Y1 can be activated.

Observation 3: In rel-17 U2N relay, Y2 events was defined for D2I path switching. Even if the current signal strength of the connected direct Uu link is not getting worse, the event Y2 can be activated when the candidate L2 U2N relay UE becomes better than threshold. The reason seems that the UE has the capability to be a remote UE.

Proposal 1: If the current connected SL-RSRP between relay UE and remote UE is above a threshold, there is no need to be triggered a measurement reporting event for I2I path switching. It’s for avoiding to happen too much relay reselection.

Proposal 2: Measurement reporting event can be defined for the I2I relay at least when the serving U2N relay UE becomes worse than a threshold.

2.2 Target relay UE selection for D2I/I2I in inter-gNB path switching

There was an issue in which gNB selects path type, direct path or indirect path, during HO between inter-gNB. In the last meeting, RAN3 made their agreement that source gNB selects the target path type (direct or indirect). RAN2 can follow the working assumption from RAN3.

Proposal 3: RAN2 can follow the RAN3 working assumption that source gNB selects the target path type (direct or indirect).

When source gNB determines path switching for an indirect link to the target gNB, there is still a remaining issue in which gNB selects the target relay UE. If source gNB selects the target relay UE, source gNB should send the selected target relay UE L2 ID to the target gNB. And also, the source gNB may need to send the SD-RSRP between the selected relay UE and remote UE. Because the threshold for the measurement and reporting will be different depending on each gNB, the SD-RSRP to be used for selecting target relay UE in source gNB may not be applicable in the target gNB. If the SD-RSRP of the candidate relay UE selected by the source gNB side is not a proper value in the target gNB side, the target gNB can reject the HO request to the source gNB.

Otherwise, if target gNB selects the target relay UE, source gNB needs to send the L2 ID list of candidate relay UEs, and the SD-RSRP between remote UE and each candidate relay UEs. According to the current RAN3 HO structure, source gNB selects the target cell. Considering this RAN3 structure, source gNB will select the target cell for D2I/I2I path switching. After source gNB determines the target cell, if there is only one candidate relay UE in the determined target cell, the source gNB has to send the relay UE ID and the related information to the target cell. But, if there are multiple candidate relay UEs in the target cell determined by the source gNB, sending the list of the candidate relay UE IDs and the related SD-RSRP will be better than sending one selected candidate relay UE ID. The target gNB knows the RRC state of the candidate relay UEs and/or Uu signal strength of each of the candidate relay UEs if the candidate relay UE is in RRC_CONNECTED.  So the target gNB can select the best relay UE among the candidate relay UEs in the same cell.

Observation 4: If target gNB selects target relay UE, the target gNB has an advantage on having the information of the RRC state of the candidate relay UEs, and/or the Uu signal strength between target gNB and candidate relay UEs if the candidate relay UE is in RRC_CONNECTED. But target gNB does not know the SL signal strength (e.g., SD-RSRP) between remote UE and candidate relay UEs.

Observation 5: Considering the current RAN3 HO structure, source gNB selects the cell ID for the HO. After source gNB determines the target cell, if there are multiple candidate relay UEs in the target cell determined by the source gNB, sending the list of the candidate relay UE IDs and the related SD-RSRP will be better than sending one selected candidate relay UE L2 ID.

Proposal 4: Considering RAN3 HO structure, source gNB selection the target cell. If there are multiple candidate relay UEs in the target cell, source gNB sends the L2 ID list of the candidate relay UEs to the target gNB. In addition, measurement information between remote UE and candidate relay UEs can be optionally sent to the target gNB.

2.3 Preventing HO failure

For the D2I/I2I path switching, the target relay UE can be any RRC state. If the target relay UE is in IDLE / INACTIVE, the target relay UE will be triggered to set up the RRC connection when the target relay UE receives RRCReconfigurationComplete message from the remote UE via SL-RLC1. To set up the RRC connection, the IDLE / INACTIVE relay UE performs the RRCSetup procedure. During the RRCSetup procedure, the relay UE can perform cell reselection. But if the target relay UE selects the other cell not to be broadcasted before via discovery message, the path switching of the remote UE will be failed. So, if IDLE/INACTIVE relay UE performs the RRC setup procedure due to receiving RRCReconfigurationComplete message via SL_RLC1 from the remote UE, prohibiting cell reselection of the relay UE will be helpful to prevent path switching failure possibility. The IDLE/INACTIVE target relay UE should try to make the RRC connection to the cell which was broadcasted via discovery message when the connection is caused by receiving the RRC message from remote UE via SL_RLC1. So, the IDLE/INACTIVE target relay UE doesn’t perform cell reselection during RACH in this case.

Observation 6: In the case of the D2I/I2I path switching, the RRC_IDLE/INACTIVE relay UE tries to perform the RRC setup procedure when the relay UE receives RRCReconfigurationComplete message from the remote UE via SL_RLC1. While the RRC setup procedure, the target relay UE can perform cell reselection. If the target relay UE selects a cell not to be broadcasted via the latest discovery message, the path switching will be failed.

Proposal 5: Considering UE does not perform cell reselection during HO, it is beneficial for the RRC_IDLE/INACTIVE relay UE to avoid cell reselection during initial access triggered by the remote UE.

3.	Conclusion

In this contribution, we focused on the Rel-18 U2N service continuity enhancement and provided our view on it. The following proposals are kindly suggested to RAN2:

Proposal 1: If the current connected SL-RSRP between relay UE and remote UE is above a threshold, there is no need to be triggered a measurement reporting event for I2I path switching. It’s for avoiding to happen too much relay reselection.

Proposal 2: Measurement reporting event can be defined for the I2I relay at least when the serving U2N relay UE becomes worse than a threshold.

Proposal 3: RAN2 can follow the RAN3 working assumption that source gNB selects the target path type (direct or indirect).

Proposal 4: Considering RAN3 HO structure, source gNB selects the target cell. If there are multiple candidate relay UEs in the target cell, source gNB sends the L2 ID list of the candidate relay UEs to the target gNB. In addition, measurement information between remote UE and candidate relay UEs can be optionally sent to the target gNB.

Proposal 5: Considering UE does not perform cell reselection during HO, it is beneficial for the RRC_IDLE/INACTIVE relay UE to avoid cell reselection during initial access triggered by the remote UE.

4.	Reference

- RP-221262, “Revised WID on NR sidelink relay enhancements,” LG Electronics, June, 2022.

- TS 38.331, “NR; Radio Resource Control (RRC) protocol specification (Release 17)

- TS 38.423, “NG-RAN; Xn application protocol (XnAP) (Release 17)