3GPP TSG-RAN WG2 #120	R2-2212539

Toulouse, France, 14 – 18 November, 2022

Agenda item:		8.4.2.3	Cell Switch

Title:		Procedure aspect of cell switch

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In RAN2#119bis-e, there were some important agreements related to cell switch in the L1/L2-triggered mobility (LTM) which includes the followings (as a part of agreements) [1].

A L1/L2 inter-cell mobility candidate (target) configuration is received within an RRC message before the L1/L2 dynamic switch is triggered.

RAN2 assumes L1/2 mobility trigger information is conveyed in a MAC CE, FFS if the MAC CE or a DCI is used for the actual triggering. RAN2 assumes the MAC CE for L1/2 mobility trigger contains at least a candidate configuration index. FFS if it should be possible to perform SCell activation/deactivation (amongst SCells associated with the candidate configuration) simultaneously with L1 L2 mobility trigger MAC CE (if so, FFS how this is determined).

RAN2 assumes that both RACH-based (CFRA, CBRA) and RACH-less procedures for L1 L2 mobility switch may be supported. RACH-less if the UE doesn’t need to acquire TA during the cell switch. RAN2 understands that the feasibility of RACH-less may depend on RAN1, and expect that RAN1 is working on this. RAN2 assumes RACH resource for CFRA for L1 L2 dynamic switch may be provided in RRC configuration (or potentially by MAC CE FFS).

In this contribution, we discuss some remaining FFS on cell switch and provide our views.

# 2. Discussion

2.1	LTM triggering

In RAN2#119bis-e, RAN2 agreed to assume the LTM trigger information is conveyed in a MAC CE. The original intention of that agreement would mean that the MAC CE is used for the actual triggering. On the other hand, some companies want to leave a possibility to use a DCI for the actual triggering and then it remains as FFS. However, at first it should be confirmed that only either MAC CE or DCI is used for the triggering, but not both (e.g. case by case). There should be no such non-essential flexibility for this triggering.

Observation 1. RAN2 should select only either MAC CE or DCI is used for actual triggering, but not both

To conclude this issue, it seems better to confirm that the MAC CE can work, i.e. the MAC CE can include the LTM trigger information (as already agreed) and it can be directly interpreted by the UE that the LTM is triggered. There is nothing broken. If the DCI is used for the triggering, the MAC CE has to be sent before the DCI. However, this would mean to introduce an additional latency to trigger the LTM.

Another aspect is that the LTM is basically triggered and achieved in lower layers, which also cause configuration update in higher layer (RRC). This is the new concept and thus it should be careful about its  reliability as well. Unfortunately the DCI/PDCCH is less reliable compared to the MAC CE and may cause unexpected results in mobility. As there is no big latency benefits in DCI approach (i.e. MAC CE followed by DCI) compared to the MAC CE, it is sufficient to use the MAC CE for actual triggering.

Proposal 1: RAN2 to assumes that a MAC CE conveying the LTM trigger information is used for the actual triggering

2.2	SCell activation/deactivation

Another FFS is regarding the SCell activation/deactivation during the LTM. The SCell(s) assumed here is among SCells associated with the candidate configuration. We understand that the main benefit of this point is to activate the SCell(s) directly.

During the discussions in the last meeting, it was pointed out that the SCell(s) activation/deactivation should be decided by the target cell. If the target cell is managed by another DU, it would not be appropriate for the source DU to trigger SCell(s) activation/deactivation during the LTM. On the other hand, if the target cell is also managed by the source DU, then it might be useful to do this. Thus, if RAN2 agrees to apply different approach for intra- and inter-DU cases, the SCell(s) activation/deactivation can be performed during only intra-DU LTM. Otherwise, it would be better not to perform that.

Proposal 2: RAN2 to discuss whether a different approach is to be applied for intra- and inter-DU cases with respect to SCell(s) activation/deactivation (amongst SCells associated with the candidate configuration).

Proposal 2a: If RAN2 agree to the different approach, SCell(s) activation/deactivation (amongst SCells associated with the candidate configuration) can be performed during the LTM only for intra-DU case. Otherwise, SCell(s) activation/deactivation cannot be performed during the LTM.

2.3	CFRA RACH resource

One more FFS is a possibility of including RACH resource for CFRA in MAC CE. The potential benefit of this is to configure the CFRA resource more dynamically, e.g. right before the LTM triggering. However, similar to the SCell (de)activation above, this is valid only for intra-DU case. We do not think this flexibility is essential or necessary. The currently agreed assumption (i.e. provided in RRC configuration) is sufficient.

Proposal 3: RAN2 to confirm that RACH resource for CFRA for cell switch in LTM can be provided in RRC configuration (not MAC CE).

# 3. Conclusion

In this contribution we discussed the LTM triggering, SCell (de)activation and CFRA RACH resources in cell switch and made the following proposals.

Proposal 1: RAN2 to assumes that a MAC CE conveying the LTM trigger information is used for the actual triggering

Proposal 2: RAN2 to discuss whether a different approach is to be applied for intra- and inter-DU cases with respect to SCell(s) activation/deactivation (amongst SCells associated with the candidate configuration).

Proposal 2a: If RAN2 agree to the different approach, SCell(s) activation/deactivation (amongst SCells associated with the candidate configuration) can be performed during the LTM only for intra-DU case. Otherwise, SCell(s) activation/deactivation cannot be performed during the LTM.

Proposal 3: RAN2 to confirm that RACH resource for CFRA for cell switch in LTM can be provided in RRC configuration (not MAC CE).

# References

[1] RAN2#119bis-e Chairman notes