3GPP TSG RAN WG2 Meeting #120                                                                 R2-2212565

# Toulouse, France, 14th-18th November 2022

Agenda item:	5.1.3.1.1

Source:	Intel Corporation

Title:	Handling of suspendConfig and UE Inactive AS context

Document for:	Discussion and decision

# Introduction

This document raises an ambiguity found on how the TS 38.331 captures the handling of suspendConfig and UE Inactive AS context. Next sections provide a detailed analysis from which it seems important be highlighted the following points:

- Firstly it is important to confirm the operation currently specified on how the suspendConfig is stored in the UE AS Inactive Context since Rel-15 NR, as based on that, there could be an issue in current Rel-17.

- Until Rel-17, the suspendConfig and the stored UE Inactive AS context is not updated while the UE is in RRC_INACTIVE (except for the case when UE performs RAN Notification Area Update (RNAU)).

- Rel-17 SDT feature changes the behaviour of (2) as network can provide new configuration associated to the UE while keeping the UE in RRC_INACTIVE. I.e., a new/updated suspendConfig is provided every time an SDT session is performed.

- In the scenarios where network updates the suspendConfig while UE is in RRC_INACTIVE (i.e. RNAU and Rel-17 SDT), with current specification, UE could run into a mis-match between the latest suspendConfig used by UE and the one stored in the stored UE Inactive AS context. This ambiguity creates the issue (i.e. different interpretations) on the configuration to be used during next resume proc. and one to be maintained in order to apply the delta configuration.

In summary, if the ambiguity on how the TS 38.331 captures the handling of suspendConfig and UE Inactive AS context is not addressed, this would impact on how Rel-17 SDT behaviour is captured and could lead to wrong implementation.

If the ambiguity on how the TS 38.331 captures the handling of suspendConfig and UE Inactive AS context is not addressed, this would impact on how Rel-17 SDT defines the correct modelling behaviour (which currently is not correct).

# Discussion

## [Background] Rel-17 SDT and handling of suspendConfig

During RAN2#119-e meeting, the handling of SDT configuration as part of the UE AS Inactive Context was discussed in [1] as part of the Rel-17 SDT. Companies seem to have different interpretation of the “loosely” specification created at the time that seems to be an issue in few scenarios as it further explained in next sections. A summary of the views were:

- The suspendConfig is not stored as part of UE Inactive AS context. It is explained that the UE Inactive AS context is “expected” to store the configuration used by UE in RRC_CONNECTED and that is “expected” to be restored when the connection is resumed (e.g. for the purpose of delta configuration). This is used as a justification to claim that std-Config (which is defined as part of the suspendConfig) does not need to be updated (or even stored) in the UE Inactive AS context. It was further clarified their view that the suspendConfig is a configuration used by UE in RRC INACTIVE mode (which will be maintained whenever UE is in INACTIVE mode), and will be released when UE enter RRC_IDLE mode or enter RRC_CONNECTED.

- The suspendConfig is stored as part of UE Inactive AS context. When a UE is in RRC_CONNECTED, UE applies the suspendConfig and later all its current RRC configuration (except certain configurations that are excluded) are stored in the UE Inactive AS context before transitioning to RRC_INACTIVE. From this, our understanding is that suspendConfig is stored as part of UE Inactive AS context. During SDT, the stored UE Inactive AS context is never fully restored or released during the SDT procedure (as UE stays in RRC_INACTIVE). In addition, during a given SDT procedure (i.e., session), the UE gets a new/updated suspendConfig that is applied and currently is not updated in the stored UE AS Inactive Context. when UE initiates resume procedure (for normal or SDT), UE restores the RRC configuration from the stored UE AS Inactive Context in preparation to transmit RRCResumeRequest. Some information would be used there e.g. I-RNTI and others like RNAU list is kept as UE should apply delta signaling. Therefore, as it is also further explained in next sections of this document, this would lead to a mis-match issue that would make unclear or even incorrect, the configuration to be used in sub-sequent resume/SDT procedures.

The “loosely” specification on the handling of suspendConfig and UE Inactive AS context creates different interpretations on current specification which has led not to capture the correct modelling behaviour in Rel-17 SDT for sdt-Config and srs-PosRRC-Inactive.

## UE AS Inactive Context when UE in CONNECTED receives RRCRelease msg

As it is explained in previous section, companies have different interpretations of current specification on how the reception of RRCRelease is processed. We understand that it is important to have a common understanding of the model as it also impacts Rel-15+ RRC_INACTIVE, Rel-17 SDT and any future features that might defined for RRC_INACTIVE. Reference from TS 38.331 on the reception of RRCRelease message is copied here to help us with our discussion.

5.3.8.3	Reception of the RRCRelease by the UE

[…]

1>	if the RRCRelease includes suspendConfig:

2>	reset MAC and release the default MAC Cell Group configuration, if any;

2>	apply the received suspendConfig except the received nextHopChainingCount;

[….]

2> if the RRCRelease message with suspendConfig was received in response to an RRCResumeRequest or an RRCResumeRequest1:

3>	stop the timer T319 if running;

3>	in the stored UE Inactive AS context:

4>	replace the KgNB and KRRCint keys with the current KgNB and KRRCint keys;

4>	replace the nextHopChainingCount with the value of nextHopChainingCount received in the RRCRelease message;

4>	replace the cellIdentity with the cellIdentity of the cell the UE has received the RRCRelease message;

4>	if the suspendConfig contains the sl-UEIdentityRemote (i.e. the UE is a L2 U2N Remote UE):

5>	replace the C-RNTI with the value of the sl-UEIdentityRemote;

5>	replace the physical cell identity with the value of the sl-PhysCellId in sl-ServingCellInfo contained in the discovery message received from the connected L2 U2N Relay UE;

4> else:

5>	replace the C-RNTI with the C-RNTI used in the cell (see TS 38.321 [3]) the UE has received the RRCRelease message;

5>	replace the physical cell identity with the physical cell identity of the cell the UE has received the RRCRelease message;

3>	replace the nextHopChainingCount with the value associated with the current KgNB;

3>	stop the timer T319a if running and consider SDT procedure is not ongoing;

2> else:

3>	store in the UE Inactive AS Context the nextHopChainingCount received in the RRCRelease message, the current KgNB and KRRCint keys, the ROHC state, the EHC context(s), the UDC state, the stored QoS flow to DRB mapping rules, the application layer measurement configuration, the C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell, the spCellConfigCommon within ReconfigurationWithSync of the NR PSCell (if configured) and all other parameters configured except for:

-	parameters within ReconfigurationWithSync of the PCell;

-	parameters within ReconfigurationWithSync of the NR PSCell, if configured;

-	parameters within MobilityControlInfoSCG of the E-UTRA PSCell, if configured;

-	servingCellConfigCommonSIB;

-	sl-L2RelayUE-Config, if configured;

-	sl-L2RemoteUE-Config, if configured;

[…]

When a UE receives RRCRelease msg in response to RRCResumeRequest/RRCResumeRequest1, suspendConfig is applied and only specific information of suspendConfig (i.e. nextHopChainingCount and sl-UEIdentityRemote) is updated in the UE Inactive AS Context. In other cases that receives RRCRelease msg, suspendConfig is applied and a new UE Inactive AS Context is created including a list of information as well as any other parameters configured in the UE except the ones included in the exception list (therefore, the suspendConfig just applied would also be stored as it is not part of the exception list).

Taking into account, two possible interpretations on how suspendConfig is handled in relation to the UE AS Inactive Context the following corrections are suggested understanding that when UE gets RRCRelease message, the UE always applies the received suspendConfig (as shown in above reference highlighted in green).

- The suspendConfig is not stored as part of UE Inactive AS context.

When a UE is in RRC_CONNECTED, receives RRCRelease with suspendConfig, and applies the suspendConfig, the UE will currently store in the UE Inactive AS Context all parameters configured in the UE unless explicitly indicated (as shown in above reference highlighted in blue). This 1st interpretation could be easily enabled if this procedure is updated to also exclude the suspendConfig from being stored (which would resolve the potential ambiguity).

- The suspendConfig is stored as part of UE Inactive AS context.

When a UE is in RRC_INACTIVE, receives RRCRelease with suspendConfig (in response to the RRCResumeRequest), and applies the suspendConfig, the UE will currently not replace (i.e. update) the suspendConfig in the UE Inactive AS Context (as shown in above reference highlighted in yellow). This 2nd interpretation could be easily enabled if this procedure is updated to replace the suspendConfig in the stored UE AS Inactive Context with the current suspendConfig that was just applied (which would also resolve the potential ambiguity).

In summary, resolving the ambiguity on the possible interpretations also resolves the issue that appear with Rel-17 SDT.

- To confirm whether the suspendConfig is stored or not as part of the UE Inactive AS Context when UE in CONNECTED receives RRCRelease msg (in section 5.3.8.3 of TS 38.331).

Depending on the response to Proposal 1, our suggestion is to correct the specification accordingly to avoid ambiguity and resolve the new issues that Rel-17 SDT creates due to current ambiguity.

- If Proposal 1 confirms that suspendConfig is not stored as part of the UE Inactive AS Context, to update section 5.3.8.3 of TS 38.331 to capture that suspendConfig is not stored as part of the UE Inactive AS Context when UE in CONNECTED receives RRCRelease msg.

- If Proposal 1 confirms that suspendConfig is stored as part of the UE Inactive AS Context, to update section 5.3.8.3 of TS 38.331 to capture that current suspendConfig is replaced in the stored UE Inactive AS context (except for nextHopChainingCount and sl-UEIdentityRemote-r17 which are already correctly captured) when UE in CONNECTED receives RRCRelease msg in response to RRCResumeRequest msg.

- If either Proposal 2 or Proposal 3 is agreed, the corresponding change is implemented for Rel-17 and to discuss whether to also add similar statement in early releases (e.g. starting potentially in Rel-15).

# Conclusion

The observations captured are the following:

Observation 1.	If the ambiguity on how the TS 38.331 captures the handling of suspendConfig and UE Inactive AS context is not addressed, this would impact on how Rel-17 SDT defines the correct modelling behaviour (which currently is not correct).

Observation 2.	The “loosely” specification on the handling of suspendConfig and UE Inactive AS context creates different interpretations on current specification which has led not to capture the correct modelling behaviour in Rel-17 SDT for sdt-Config and srs-PosRRC-Inactive.

Observation 3.	When a UE receives RRCRelease msg in response to RRCResumeRequest/RRCResumeRequest1, suspendConfig is applied and only specific information of suspendConfig (i.e. nextHopChainingCount and sl-UEIdentityRemote) is updated in the UE Inactive AS Context. In other cases that receives RRCRelease msg, suspendConfig is applied and a new UE Inactive AS Context is created including a list of information as well as any other parameters configured in the UE except the ones included in the exception list (therefore, the suspendConfig just applied would also be stored as it is not part of the exception list).

The proposals captured are the following:

Proposal 1.	To confirm whether the suspendConfig is stored or not as part of the UE Inactive AS Context when UE in CONNECTED receives RRCRelease msg (in section 5.3.8.3 of TS 38.331).

Proposal 2.	If Proposal 1 confirms that suspendConfig is not stored as part of the UE Inactive AS Context, to update section 5.3.8.3 of TS 38.331 to capture that suspendConfig is not stored as part of the UE Inactive AS Context when UE in CONNECTED receives RRCRelease msg.

Proposal 3.	If Proposal 1 confirms that suspendConfig is stored as part of the UE Inactive AS Context, to update section 5.3.8.3 of TS 38.331 to capture that current suspendConfig is replaced in the stored UE Inactive AS context (except for nextHopChainingCount and sl-UEIdentityRemote-r17 which are already correctly captured) when UE in CONNECTED receives RRCRelease msg in response to RRCResumeRequest msg.

Proposal 4.	If either Proposal 2 or Proposal 3 is agreed, the corresponding change is implemented for Rel-17 and to discuss whether to also add similar statement in early releases (e.g. starting potentially in Rel-15).

# Reference

- R2-2208987, Report of [AT119-e][301][Sdata] CP Open issues and CR to 38.331 (ZTE), August 2022.