3GPP TSG-RAN WG2 Meeting #120        	                     R2-2212624

Toulouse, France, 14 – 18 November, 2022

Agenda item:	8.16.3

Source:	CMCC

Title:	Discussion on use case specific aspects for AI/ML for NR air interface

WID/SID:	FS_NR_AIML_air

Document for:	Discussion

- Introduction

In RAN#96-e, the SID for Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air is updated in RP-221348 [1]. The RAN2 related objective is highlighted as below.

- Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

In this contribution, we discuss potential impacts use case specific aspects for AI/ML for NR air interface.

- Discussion

RAN1 studied three use cases (i.e. CSI feedback enhancement, beam management, positioning accuracy enhancements) and potential sub-use cases since RAN1#109-e. In this contribution, we share the RAN1 agreements on the use cases, and further analysis on the potential RAN2 impacts (e.g. the mapping of AI functionality to network entity) per use case.

## 2.1 CSI feedback enhancement

During RAN1 discussion, RAN1 has agreed that spatial-frequency domain CSI compression using two-sided AI/ML model is selected as one representative sub-use case, and there are some other sub-use cases which have been discussed, e.g. CSI prediction using one-sided model. In addition, some sub-use cases have been excluded by RAN1, e.g. CSI-RS configuration and overhead reduction, resource allocation and scheduling.

Observation 1: Spatial-frequency domain CSI compression using two-sided AI/ML model is selected by RAN1 as one representative sub use case.

Therefore, RAN2 can use CSI compression sub-use case as starting point, and discuss the potential impacts on RAN2. In addition, the discussion on other sub-use cases can wait for RAN1’s progress.

Proposal 1: RAN2 study the potential spec impacts on spatial-frequency domain CSI compression using two-sided AI/ML model sub-use case.

In CSI compression using two-sided AI/ML model use case, RAN1 agreed to further study the following AI/ML model training collaborations (joint or separate training).

RAN1#109-e Agreement In CSI compression using two-sided model use case, the following AI/ML model training collaborations will be further studied: - Type 1: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided. - Type 2: Joint training of the two-sided model at network side and UE side, repectively. - Type 3: Separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively. - Note: Joint training means the generation model and reconstruction model should be trained in the same loop for forward propagation and backward propagation. Joint training could be done both at single node or across multiple nodes (e.g., through gradient exchange between nodes). - Note: Separate training includes sequential training starting with UE side training, or sequential training starting with NW side training [, or parallel training] at UE and NW - Other collaboration types are not excluded.

For Type 1, the two-sided AI/ML model which includes a CSI generation part and a CSI reconstruction part is trained at network, and then the trained UE-side CSI generation part is transferred from network to UE, or the model is trained at UE and then the trained network-side CSI reconstruction part is transferred from UE to network. The model transfer via air interface is needed for Type 1.

In our RAN1 contribution [2], a basic procedure of AI based CSI feedback with AI model training at network side is shown in Fig.1, considering the computing capability of mobile terminal is limited, we assume the model training is performed at network side.

Fig.1 Procedure with model training at network side for Type 1

The UE take the eigenvectors of channel information computed via CSI-RS as training data. Then, UE reports the training data to network side and then network side could use the training data for model training or model generation. After the AI model is trained at network side, the encoder part of AI model is transmitted to UE. When UE has received CSI-RS and measured the channel information, the UE compresses the measured CSI via the encoder and then report the encoder results to network side. The network will recover the received CSI to eigenvectors or other kinds of channel information via the decoder. However, considering the training data is enormous and the uplink resource is limited, the potential spec impact on the training data reporting should be studied.

Observation 2: The training data transfer and model transfer in Type 1 has potential spec impacts on RAN2.

For Type 2, the UE-side CSI generation part is trained at UE and the network-side CSI reconstruction part is trained at network at the same time. During the training procedure, the model transfer via air interface is not needed, but some information (the FP information (e.g., the compressed CSI) and the BP information (e.g., the gradients information)) is required to be exchanged between network and UE.

Observation 3: The exchanged information between UE and network in Type 2 has potential spec impacts on RAN2.

For Type 3, the two-sided AI/ML model is trained at network, then the network sends the dataset which includes input and output to the UE. After that, the UE trains a UE-sided CSI generation part using the dataset. The UE-sided CSI generation part and the network-sided reconstruction part are designed and trained by the UE and network separately. Compared with joint training, there may be the issue of sub-optimal performance and signaling overhead due to dataset transmission.

Observation 4: The dataset (e.g. input and output) transmission in Type 3 has potential spec impacts on RAN2.

Based on the above analysis, each type has some impacts on signaling procedure between UE and network, and RAN1 will further study which training type is applied. RAN2 needs more RAN1 inputs for further discussion.

Proposal 2: RAN2 needs more RAN1 inputs on model training type for further discussion, e.g. training data transfer, model transfer, exchanged info, input/output, etc.

For Life Cycle Management, RAN2 can study where the AI functionality resides, including data collection, model training, model inference, etc. In CSI compression using two-sided AI/ML model use case, it is obvious that the CSI generation part resides UE side and CSI reconstruction part resides network side for model inference. But for data collection and model training, where the AI functionality resides is up to which entity to perform the function, e.g. gNB or UE.

Proposal 3: RAN2 to discuss where the AI functionality resides for CSI compression using two-sided model sub-use case, including data collection, model training, model inference, etc.

On the other hand, RAN1 discussed CSI report enhancement which may have some impacts on RAN2, e.g. RRC configuration or MAC procedure. RAN2 can wait for RAN1’s progress for further discussion.

RAN1#109-e Agreement In CSI compression using two-sided model use case, further study potential specification impact on CSI report, including at least - CSI generation model output and/or CSI reconstruction model input, including configuration(size/format) and/or potential post/pre-processing of CSI generation model output/CSI reconstruction model input. - CQI determination - RI determination

Proposal 4: RAN2 waits for RAN1’s progress on CSI report for further discussion.

## 2.2 Beam management

RAN1 studied sub-use cases for AI/ML-based beam management and agreed to support BM-Case1 and BM-Case2. The following agreements have been achieved:

RAN1#109-e Agreement For AI/ML-based beam management, support BM-Case1 and BM-Case2 for characterization and baseline performance evaluations - BM-Case1: Spatial-domain DL beam prediction for Set A of beams based on measurement results of Set B of beams - BM-Case2: Temporal DL beam prediction for Set A of beams based on the historic measurement results of Set B of beams - FFS: details of BM-Case1 and BM-Case2 - FFS: other sub use cases Note: For BM-Case1 and BM-Case2, Beams in Set A and Set B can be in the same Frequency Range RAN1#109-e Agreement For the sub use case BM-Case1, consider both Alt.1 and Alt.2 for further study: - Alt.1: AI/ML inference at NW side - Alt.2: AI/ML inference at UE side RAN1#109-e Agreement For the sub use case BM-Case2, consider both Alt.1 and Alt.2 for further study: - Alt.1: AI/ML inference at NW side - Alt.2: AI/ML inference at UE side

Based on RAN1 agreements, AI/ML inference can reside in NW side or UE side both for sub use case BM-Case1 and BM-Case2, i.e. one-sided model is used for beam management.

Proposal 5: RAN2 study the potential spec impacts on BM-Case1 and BM-Case2 with one-sided model.

Regarding Life Cycle Management, RAN1 discussed data collection, model training, model inference, model monitoring for network-side model and UE-side model, and achieved the following agreements:

RAN1#110 Agreement For the data collection for AI/ML model training (if supported), study the following aspects as a starting point for potential necessary specification impact: - Signaling/configuration/measurement/report for data collection, e.g., signaling aspects related to assistance information (if supported), Reference signals - Content/type of the collected data - Other aspect(s) is not precluded RAN1#110 Agreement At least for the sub use case BM-Case1 and BM-Case2, support both Alt.1 and Alt.2 for the study of AI/ML model training: - Alt.1: AI/ML model training at NW side; - Alt.2: AI/ML model training at UE side. Note: Whether it is online or offline training is a separate discussion. RAN1#110 Agreement In order to facilitate the AI/ML model inference, study the following aspects as a starting point: - Enhanced or new configurations/UE reporting/UE measurement, e.g., Enhanced or new beam measurement and/or beam reporting - Enhanced or new signaling for measurement configuration/triggering - Signaling of assistance information (if applicable) - Other aspect(s) is not precluded RAN1#110bis-e Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the following alternatives for model monitoring with potential down-selection: - Atl1. UE-side Model monitoring - UE monitors the performance metric(s) - UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation - Atl2. NW-side Model monitoring - NW monitors the performance metric(s) - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation - Alt3. Hybrid model monitoring - UE monitors the performance metric(s) - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation RAN1#110bis-e Agreement For BM-Case1 and BM-Case2 with a network-side AI/ML model, study the NW-side model monitoring: - NW monitors the performance metric(s) and makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation RAN1#110bis-e Agreement Regarding NW-side model monitoring for a network-side AI/ML model of BM-Case1 and BM-Case2, study the potential specification impacts from the following aspects - Beam measurement and report for model monitoring - Note: This may or may not have specification impact.

From RAN2 perspective, RAN2 can study where the AI functionality (e.g. data collection, model training, model inference) resides for beam management use case for network-side model and UE-side model. For example, regarding network-side model, data collection, model training and model reference can both reside in gNB, and regarding network-side model, data collection, model training and model reference can both reside in UE.

Proposal 6: RAN2 to discuss where the AI functionality resides for BM-Case1 and BM-Case2 regarding network-side model and UE-side model.

According to RAN1 agreements, the enhancements on new configurations or UE reporting or UE measurement, new signaling for measurement configuration/triggering, assistance information can be further studied to facilitate the AI/ML model inference. From RAN2 perspective, the relevant signaling procedure can be further discussed.

Proposal 7: RAN2 to study the signaling procedure for beam management, such as measurement configuration/triggering, UE reporting, etc.

## 2.3 Positioning accuracy enhancements

RAN1 studied AI/ML-based positioning accuracy enhancement and achieved the following agreements on AI/ML based positioning methods, model training, model monitoring/update, model inference. For the direct AI/ML positioning, the output of AI/ML model inference is UE location, and for AI/ML assisted positioning, the output of AI/ML model inference is new measurement and/or enhancement of existing measurement.

RAN1#110 Agreement For characterization and performance evaluations of AI/ML based positioning accuracy enhancement, the following two AI/ML based positioning methods are selected. - Direct AI/ML positioning - AI/ML assisted positioning - Note 1: the selection does not intend to provide any indication of the prospects of any future normative project. - Note 2: further discussion (including selection of other sub use cases and/or down selection of selected sub use cases) are not precluded based on performance evaluation and potential specification impact study results RAN1#110 Agreement Regarding data collection for AI/ML model training, to study and provide inputs on potential specification impact at least for the following aspects of AI/ML based positioning accuracy enhancement - Ground truth label determination (e.g., based on UE/PRU/TRP measurement/report) - Partial and/or noisy ground truth label - Signaling for data collection - Other aspects are not precluded RAN1#110 Agreement Regarding AI/ML model monitoring and update, to study and provide inputs on potential specification impact at least for the following aspects of AI/ML based positioning accuracy enhancement - AI/ML model monitoring performance metrics - Condition of AI/ML model update - Reference signals and measurement feedback/report - Other aspects are not precluded RAN1#110bis-e Agreement - Study and provide inputs on benefit(s) and potential specification impact at least for the following cases of AI/ML based positioning accuracy enhancement - Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning - Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

Based on the RAN1 discussion, one-sided model (i.e. UE-side, LMF-side or gNB side model) is applied to positioning accuracy enhancements use case.

Observation 5: One-sided model is applied to positioning accuracy enhancements use case.

Regarding Life Cycle Management, RAN2 can study where the AI functionality resides, including data collection, model training, model inference, etc. in positioning accuracy enhancements use case. For example, the data collection and model training can reside in server or LMF, the model inference can reside in LMF or UE or gNB.

Proposal 8: RAN2 to discuss where the AI functionality resides for positioning accuracy enhancements use case regarding UE-side model, gNB-side model and LMF-side model.

# 3	Conclusion

Here are the observations and proposals for use case specific aspects for AI/ML for NR air interface.

CSI feedback enhancement:

Observation 1: Spatial-frequency domain CSI compression using two-sided AI/ML model is selected by RAN1 as one representative sub use case.

Proposal 1: RAN2 study the potential spec impacts on spatial-frequency domain CSI compression using two-sided AI/ML model sub-use case.

Observation 2: The training data transfer and model transfer in Type 1 has potential spec impacts on RAN2.

Observation 3: The exchanged information between UE and network in Type 2 has potential spec impacts on RAN2.

Observation 4: The dataset (e.g. input and output) transmission in Type 3 has potential spec impacts on RAN2.

Proposal 2: RAN2 needs more RAN1 inputs on model training type for further discussion, e.g. training data transfer, model transfer, exchanged info, input/output, etc.

Proposal 3: RAN2 to discuss where the AI functionality resides for CSI compression using two-sided model sub-use case, including data collection, model training, model inference, etc.

Proposal 4: RAN2 waits for RAN1’s progress on CSI report for further discussion.

Beam management:

Proposal 5: RAN2 study the potential spec impacts on BM-Case1 and BM-Case2 with one-sided model.

Proposal 6: RAN2 to discuss where the AI functionality resides for BM-Case1 and BM-Case2 regarding network-side model and UE-side model.

Proposal 7: RAN2 to study the signaling procedure for beam management, such as measurement configuration/triggering, UE reporting, etc.

Positioning accuracy enhancements:

Observation 5: One-sided model is applied to positioning accuracy enhancements use case.

Proposal 8: RAN2 to discuss where the AI functionality resides for positioning accuracy enhancements use case regarding UE-side model, gNB-side model and LMF-side model.

# 4	Reference

- R2-22XXXXX, Report of 3GPP TSG RAN WG2 meeting #119bis-e

- R1-2209329 Discussion on other aspects on AI/ML for CSI feedback enhancement CMCC