3GPP TSG-RAN WG2 Meeting #120                 	                                     R2-2212632

Toulouse, France, 14th- 18th November, 2022

Agenda Item:	8.5.3.2

Source:	CMCC

Title:	Discussion on Information for UE power saving

Document for:	Discussion

Introduction

In RAN2#119 bis-e meeting, information provided to RAN for UE power saving was discussed, following are the agreements [1]:

1. RAN2 discuss whether additional traffic or QoS related information on downlink traffic beyond what has been agreed by SA2 needs to be provided to RAN for UE power savings. 2. RAN2 study what traffic and QoS related information on uplink traffic (e.g. counterpart of what has been agreed by SA2) should be provided to RAN for UE power savings and how the information may be provided to RAN. Capture in TR that traffic parameters and Jitter are semi-static info. Can capture also SA2 agreements related to how they impact RAN2.

In this contribution, we focus on traffic and QoS related information on uplink traffic provided to RAN for UE power savings.

Discussion

To optimize the setting of CDRX, both RAN2 and SA2 discussed the information provided for gNB. SA2 send an LS on XR and Media Services to reply [2] to responds to LS on UE Power Saving for XR and Media Services (R1-2205531/S2-2208105) and LS on XR-awareness in RAN (R2-2209215/S2-2208144), and progress in SA2 are captured in the related TR. Following is their reply for power saving questions:

- In KI#8 (Power Saving), as to RAN1’s LS reply (R1-2205531/S2-2208105), SA2 notes that RAN1 indicates that it may be helpful for the core network to provide RAN with the following pieces of information: PDU set periodicity and start time, PDU set end indication, PDU set level QoS parameters, PDU set size (number of bits) or number of PDUs in a PDU set, PDU set identity and relationship information among PDUs within the same PDU set, and Jitter information. SA2 would like to inform RAN1 that relying on above listed PDU Set information alone may not result in an optimal CDRX configuration, since a data burst may include one or multiple PDU Sets. As a result, e.g. a PDU set end indication may or may not mark the end of a Data Burst. (See agreed pCR S2-2209939)

Observation 1: Core network may provide RAN with the following pieces of information: PDU set periodicity and start time, PDU set end indication, PDU set level QoS parameters, PDU set size (number of bits) or number of PDUs in a PDU set, PDU set identity and relationship information among PDUs within the same PDU set, and Jitter information.

Based on the agree TR, information agreed to provide for RAN which can be applied to both uplink and downlink are as followed [3]:

- Pperiodicity for UL and DL traffic of the QoS Flow.

- Traffic jitter information associated with each periodicity.

- Optional, End of Data Burst indication in the header of the last PDU of the Data Burst.

Besides, some enhancements were agreed for downlink PDU set handling [4]:

- PDU Set QoS parameters

  - PDU Set Error Rate

  - PDU Set Delay Budget

  - Whether all PDUs are needed for the usage of PDU Set by application layer (PDU Set Integrated Indication).

- PDU Set Identifier

- Optional, Start PDU and End PDU of the PDU Set

- PDU SN within a PDU Set

- Optional, PDU Set Size

And it was agreed in SA2 that PDU Set QoS parameters would be delivered to RAN.

From our point of view, in the early development stage of XR, generic uplink flows for XR may be pose/control traffic, while in the future, there can be traffic streams more than pose/control information. In TR38.838[4], some description for AR UL stream(s) are captured, and it can be seen that AR traffic streams in UL has similar characters with DL streams. Similar situations may apply to other XR traffics.

Observation 2: XR traffic streams in UL has similar characters with DL streams.

Therefore, the information agreed to provide for RAN for DL can be the baseline of UL. The information agreed in SA2 is more like static information of one XR service, and these can be acquired from CN.

Proposal 1: The information agreed to provide for RAN for DL in SA2, e.g., PDU Set QoS parameters, can be the baseline of UL.

Proposal 2: Static information of UL stream can be acquired by RAN from CN.

What’s more, dynamic assistant information for RAN should be considered for UL, which may help to RAN’s scheduling. Though some information like start PDU and end PDU of the PDU set, PDU SN and PDU set size, can be detected and provided for RAN by UE, it seems too late since this information can only be obtained after the successful receiving and decoding of the PDU. And it may crease the signalling overhead and UE power consumption. RAN2 should further discuss whether start PDU and end PDU of the PDU set, PDU SN and PDU set size should be provided by UE.

Besides, we think it’s better to indicate the PDB or latency information of UL data to the network, which helps to flexible and efficient scheduling. For PDB/latency information, it can be reported via BSR, and from our point of view, either residence time or remaining time reporting is possible solution.

Proposal 3: RAN2 can further discuss whether start PDU and end PDU of the PDU set, PDU SN and PDU set size should be provided by UE.

Proposal 4: PDB/latency information can be reported via BSR, and either residence time or remaining time reporting is possible solution.

Conclusions

In this contribution, we provide our view on traffic and QoS related information on uplink traffic provided to RAN for UE power savings. Following are our observations and proposals.

Observation 1: Core network may provide RAN with the following pieces of information: PDU set periodicity and start time, PDU set end indication, PDU set level QoS parameters, PDU set size (number of bits) or number of PDUs in a PDU set, PDU set identity and relationship information among PDUs within the same PDU set, and Jitter information.

Observation 2: XR traffic streams in UL has similar characters with DL streams.

Proposal 1: The information agreed to provide for RAN for DL in SA2, e.g., PDU Set QoS parameters, can be the baseline of UL.

Proposal 2: Static information of UL stream can be acquired by RAN from CN.

Proposal 3: RAN2 can further discuss whether start PDU and end PDU of the PDU set, PDU SN and PDU set size should be provided by UE.

Proposal 4: PDB/latency information can be reported via BSR, and either residence time or remaining time reporting is possible solution.

References

- RAN2-2210802, Report on LTE legacy, DCCA, MUSIM, Slicing, 71 GHz, XR and QoE

- S2-2209939 Key Issue #8: Evaluation and conclusion, Qualcomm Incorporated, Huawei, Nokia, Nokia Shanghai Bell, Tencent, Tencent Cloud, Vivo

- S2-2209938, KI#4&5: Evaluation and Conclusion, Huawei, HiSilicon, China Mobile, KDDI, Lenovo, Nokia, Nokia Shanghai Bell, Xiaomi, vivo, Sony, ZTE, Tencent, CATT, Futurewei, Samsung, Ericsson, Intel

- 3GPP TR 38.838 Study on XR (Extended Reality) Evaluations for NR (Release 17)V17.0.0 (2021-12)