3GPP TSG RAN WG2 #120	R2-2212635

Toulouse, FR, 11th – 14th November, 2022

Agenda item:	8.14.2

Source:	CMCC

Title:	Consideration on QoE measurement in RRC_IDLE and RRC_INACTIVE

Document for:	Discussion

1	Introduction

In the updated WID[1] , one of the objectives of QoE enhancement is as the following:

The Rel-18 QoE enhancement is aiming to introduce enhancements to the existing NR QoE framework and to introduce the support for new types of 5G services, such as AR, MR, cloud gaming, as well as the support for RRC_CONNECTED, RRC_INACTIVE and RRC_ IDLE for the MBS service.

And last meeting in RAN2[2] and RAN3[3] has some agreements and FFSs thta should be considered in further discussion:

FFS if configuration can be done in IDLE/INACTIVE states.

FFS how does gNB determine which UEs can be configured with MBS QoE measurements

FFS if there is a new explicit indicator or new service type used for MBS QoE configuration in RRC_IDLE/RRC_INACTIVE. Wait for RAN3 progress and SA4 LS reply to RAN3.

The baseline principles for QoE measurement collection for MBS services in RRC_INACTIVE and RRC_IDLE states are:

- 1)  	The UE is configured with IDLE/INACTIVE QoE via RRC.

- 2)	The UE buffers the QoE reports generated while in RRC IDLE/INACTIVE state.

- 3)	FFS if UE can setup/resume RRC connection just for QoE reporting, or whether the QoE reports are sent to the network when the UE moves to RRC CONNECTED state due to other reasons.

For buffering of QoE reports generated in RRC IDLE/INACTIVE state, RAN2 should discuss at least the minimal memory size requirement. FFS if AS layer is responsible for storing the QoE reports (as in Rel-17).

In this contribution, some potential QoE enchencement for MBS will be discussed.

2	Discussion

2.1 QoE configuration in RRC_IDLE and RRC_INACTIVE

UE capability:

As known to all, there exist various types of UE that support MBS. Due to cost, implementation complexity or other practical reasons, some terminals may not or need not perform QMC in RRC_IDLE and RRC_INACTIVE.

Naturally, it is necessary for UE to indicate such capability to gNB and CN for proper QMC configuration. This capability should be indicated for those UE that supports both QMC and MBS first.

Proposal 1: Introduce UE capability for performing QMC in RRC_IDLE and RRC_INACTIVE.

The requirements on UE capability for QMC in RRC_IDLE and RRC_INACTIVE is under development with the progress of RAN2 and RAN3, so details on UE capabilities can also be discussed in further stage.

Configuration:

As to the FFS on whether gNB can configurate QMC for UE in RRC_IDLE and RRC_INACTIVE, the requirement on configuration may not be within RAN2’s scope.

Generally, network can always indicate UE to enter RRC_CONNECTED for new RRC configurations, including QMC configuration, this approach has rare impact on specification. But there may exist concerns on the signaling overhead or energy consumption or latency, if to enter RRC_CONNECTED is only to receive new QMC configuration.

It would be appropriate to inform RAN3 and SA4/5 in case they might have similar concerns mentioned above. If other WG’s conclusion is that it is necessary to configure QMC in RRC_IDLE and RRC_INACTIVE, RAN2 should formulate such procedures and mechanism.

Proposal 2: RAN2 should ask the opinion of RAN3 and SA4/SA5 on the requirement about configuration QMC in RRC_IDLE and RRC_INACTIVE.

If there doesn’t exist such requirement, not allowing gNB to configure QMC in RRC_IDLE and RRC_INACTIVE can be adopt as the baseline of RAN2.

Memory:

UE can store QoE reports for MBS broadcast in RRC_IDLE and RRC_INACTIVE in order to send those reports when it returns to RRC_CONNECTED. Since for now, there is no need for UE to pause QoE measurement report collection in RRC_IDLE or RRC_INACTIVE, it seems to be reasonable to reuse the AS layer memory reserved for paused QoE measurement, whose size is at least 64KB. Reusing AS layer memory can also let APP layer remain unconscious about any change in RRC status.

However, we are not sure whether such memory size is sufficient for QoE measurement collection in RRC_IDLE and RRC_INACTIVE. It seems to be more reasonable to ask SA4/SA5 for their comments.

Proposal 3: RAN2 is kindly asked to agree that the 64KiB AS layer memory can be reused for buffering QoE report generated in RRC_IDLE and RRC_INACTIVE, and ask SA4/SA5 opinion on such memory is sufficient or not.

The final decision of RAN2 should be made according to the opinion on SA4/SA5.

2.2 QoE reporting in RRC_IDLE and RRC_INACTIVE

Reporting:

One noteworthy problem is whether there should be a valid time for QoE report generated in RRC_IDLE and RRC_INACTIVE. Chances are that UE may generate QoE reports before long and no newer QoE report are generated to replace the older one, causing outdated QoE report stores in memory.

Outdated QoE reports are not always useful and may increase overhead or cause misjudgment. Storing and sending outdated QoE report seems to be redundant. A timer or a common valid time for QoE report in RRC_IDLE and RRC_INACTIVE is appreciated.

Proposal 4: Introduce valid time or similar parameter for QoE report generated in RRC_IDLE and RRC_INACTIVE.

Moreover, UE can retain RRC_INACTIVE or RRC_IDLE for hours and even days, and there can be a lot of QoE reports stored in memory. Sending them all may require a lot of radio resource. under such consumption, sending abstract of stored QoE reports before sending all of them seems more reasonable, NM or NG-RAN may choose the QoE reports from the most desirable service type or just analyze the newest reports and discard all the rest.

Proposal 5: RAN2 is kindly asked to discuss whether UE should send QoE report as full text or abstract when UE enters RRC_CONNECTED.

3	Summary

This contribution discusses QoE measurements in RRC_IDLE and RRC_INACTIVE, and provides the following proposal:

Proposal 1: Introduce UE capability for performing QMC in RRC_IDLE and RRC_INACTIVE.

Proposal 2: RAN2 should ask the opinion of RAN3 and SA4/SA5 on the requirement about configuration QMC in RRC_IDLE and RRC_INACTIVE.

Proposal 3: RAN2 is kindly asked to agree that the 64KiB AS layer memory can be reused for buffering QoE report generated in RRC_IDLE and RRC_INACTIVE, and ask SA4/SA5 opinion on such memory is sufficient or not.

Proposal 4: Introduce valid time or similar parameter for QoE report generated in RRC_IDLE and RRC_INACTIVE.

Proposal 5: RAN2 is kindly asked to discuss whether UE should send QoE report as full text or abstract when UE enters RRC_CONNECTED.

References

- RP-221803; WID update for Enhancement on NR QoE; China Unicom

- RAN2-119e LTE DCCA MUSIM Slicing 71 GHz QoE XR (Tero)_2022-08-26-EOM

- RAN3_117bis-e_agenda_20221018_EOM1