3GPP TSG-RAN WG2 Meeting #120 						                       R2-2212657

Toulouse, France, 14th - 18th November 2022

Source: 	Xiaomi

Title:  	Discussion on measurement reporting for NR UAV

Agenda Item:	8.8.2 Measurement reporting

Document for:	Discussion and Decision

Introduction

A New WID on NR support for UAV (Uncrewed Aerial Vehicles) [1] has been approved. In the WID, one objective is to enhance measurement reports, shown as below.

1. Specify the following enhancements on measurement reports [RAN2]: - UE-triggered measurement report based on configured height thresholds - Reporting of height, location and speed in measurement report - Flight path reporting - Measurement reporting based on a configured number of cells (i.e. larger than one) fulfilling the triggering criteria simultaneously Note: Work done in LTE is a starting point for this objective. NR-specific enhancements can be considered, if needed, while overall the LTE and NR solutions should be harmonized as much as possible.

In previous meetings [2-3], the following agreements have been reached for measurement reporting for NR UAV:

RAN2#119-e: 1 Use LTE principle as a baseline, introduce similar event H1 (aerial UE height become higher than threshold) and H2 (aerial UE height become lower than threshold. FFS if further NR enhancements are needed. FFS study scaling of RRM parameters (e.g. which parameters and what is the purpose/benefit of the scaling and how) FFS how to limit excessive measurements and measurement reporting FFS if user consent is needed for location reporting in CONNECTED FFS study the vertical movement and associated mobility for UAV UEs 2 Rel-18 NR supports reporting of UAV UE’s height, location and velocity. It is for further study what accuracy and reporting mechanisms are required and if further enhancements are needed. 3 As in LTE, flight path plan reporting will be introduced. Location list of waypoints (3D location information) and timestamp is adopted as the basic content of flight path report. FFS if timestamp is mandatory or optional for NR. FFS if further enhancements are needed 4 Introduce similar functionality to LTE (numberofTriggeringCells). FFS whether numberoftriggerbeams for NR is required or other enhancements. FFS study how to avoid sending the measurement reports mainly due to reportOnLeave. RAN2#119bis-e: 1. The time information reported as part of flight path plan is optional. UE includes time info, if configured by the network and available at the UE. FFS on flight path details (waypoints and what is time information). 2. Allow the flight path to be updated. FFS on the details. 3. FFS on reporting format and initial flight path reporting (i.e. what information to report and how) – next meeting 4. Continue to study height-depending scaling, triggering and combinations 5. As in LTE, as a baseline, events A3, A4 and A5 can be configured with the configured number of cells (numberofTriggeringCells)

In this contribution, we will provide our understandings on measurement reporting for NR UAV.

Discussion

  - Reporting of height, location and speed in measurement report

In previous meetings, RAN2 agreed to use LTE principle as a baseline for measurement reporting of NR UAV and event H1 and event H2 have been introduced. In LTE UAV, event H1 and H2 can be used for height reporting, and aerial UE triggers a height report when event H1 or event H2 is fulfilled. Similarly, height reporting can also be triggered by event H1 and event H2 in NR UAV. When event H1 or event H2 is fulfilled, the height of aerial UE can be included in measurement report for NR UAV.

Proposal 1: Height reporting can be triggered by event H1 and event H2.

For location reporting of NR UAV, existing location reporting mechanism can be reused. UE location information can be included in the measurement report if the includeCommonLocationInfo is configured. Horizontal and vertical speed can be included in location information, which is similar to LTE UAV.

Proposal 2: Existing location reporting mechanism can be reused for NR UAV.

Proposal 3: Horizontal and vertical speed can be included in location information.

  - Flight path reporting

To increase the reliability and robustness of mobility, flight path plan reporting has been introduced in NR UAV. In last meeting, RAN2 agreed that the time information reported as part of flight path plan is optional and the remaining issue is “what is time information”. In RAN2#119-e, location list of waypoints (3D location information) and timestamp is adopted as the basic content of flight path report. Hence, the time information is timestamp. As in LTE [4], the definition of timestamp includes time stamps for the waypoints that describe planned locations for the UE.

timeStamp Includes time stamps for the waypoints that describe planned locations for the UE.

Proposal 4: As in LTE, the time information is timestamp which includes time stamps for the waypoints that describe planned locations for the UE.

Other issues about reporting format and initial flight path reporting also need to be discussed. For the reporting format of flight path reporting, [5] captured some views of the meaning of flight path information, e.g. the flight path plan contains useful information which is known what is the granularity of the waypoints (i.e. the distance between consecutive points) and provides the information on the destination of the flight. However, it is uncertain whether UE has full flight path information or not e.g. whether the flight path information stored in UE includes the destination of flight and can satisfy the granularity configured by network. And the size and speed of UAV UE may influence over waypoint granularity.  Therefore, which waypoints are included in flight path information is up to UE implementation.

Proposal 5: It is up to UE implementation which waypoints are included in flight path information and provided to network.

For initial flight path reporting, it can be triggered based on network request, which is the mechanism of flight path reporting in LTE. If configured to report flight path information by network, UE can report the flight path information.

Proposal 6: Initial flight path reporting can be triggered by network request.

After aerial UE reports the fight path information, the planned flight path may be changed or the aerial UE is controlled by the operator or user. Hence, RAN2 allowed the flight path to be updated in last meeting. So, the previously reported flight path information might no longer be valid after flight path change. Flight path reporting can be triggered by UE when flight path has been changed. UE-based flight path reporting can be considered for the update of flight path information.

Proposal 7: UE-based flight path reporting can be considered, i.e. flight path reporting can be triggered by UE when flight path is updated.

  - Height-depending scaling, triggering and combinations

In last meetings, RAN2 agreed to study height-depending scaling, triggering and combinations. Height-depending triggering has been supported via event H1 and H2. New events based on height to trigger measurement reporting are not needed.

Because reporting of UAV UE’s height has been supported in NR UAV and network can configure appropriate measurement configurations based on UE’s height, height-depending scaling for measurement reporting is not needed.

Proposal 8: Height-depending scaling is not needed for NR UAV.

Height-depending combination, e.g. the combination of height-based event and signal strength based event for measurement reporting triggering can be considered which can reduce measurement reporting and increase the flexibility of measurement configuration, where multiple features can be considered to trigger measurement reporting. How to combine multiple events can be further discussed.

Proposal 9: Height-depending combination, e.g. the combination of height-based event and signal strength based event can be considered for NR UAV.

  - Measurement reporting based on numberofTriggeringbeams

To reduce measurement reporting, for LTE UAV, eNB can configure measurement report with a configurable number of cells (numberofTriggeringCells) and aerial UEs trigger a measurement report if the event is fulfilled for a configurable number of cells simultaneously. Similar functionality to LTE (numberofTriggeringCells) has been introduced for NR UAV and events A3, A4 and A5 can be configured with the configured number of cells (numberofTriggeringCells).

Beam-leave measurement critical e.g. numberoftriggerbeams has been discussed in previous meetings and whether numberoftriggerbeams is required is still FFS for NR UAV. Because cell-level measurement results are calculated based on beam-level measurement results, the signal strength of beam can indirectly affect the measurement results of cell-level. Hence, numberofTriggeringCells is enough for NR UAV and numberoftriggerbeams is not needed.

Proposal 10: Beam-level measurement critical i.e. numberoftriggerbeams is not needed.

Conclusions

According to the analysis given above, we have the following observations and proposals:

Proposal 1: Height reporting can be triggered by event H1 and event H2.

Proposal 2: Existing location reporting mechanism can be reused for NR UAV.

Proposal 3: Horizontal and vertical speed can be included in location information.

Proposal 4: As in LTE, the time information is timestamp which includes time stamps for the waypoints that describe planned locations for the UE.

Proposal 5: It is up to UE implementation which waypoints are included in flight path information and provided to network.

Proposal 6: Initial flight path reporting can be triggered by network request.

Proposal 7: UE-based flight path reporting can be considered, i.e. flight path reporting can be triggered by UE when flight path is updated.

Proposal 8: Height-depending scaling is not needed for NR UAV.

Proposal 9: Height-depending combination, e.g. the combination of height-based event and signal strength based event can be considered for NR UAV.

Proposal 10: Beam-level measurement critical i.e. numberoftriggerbeams is not needed.

Reference

[1] RP-213600, Nokia, “New WID on NR support for UAV (Uncrewed Aerial Vehicles)”

[2] R2-2209301, “Report of 3GPP TSG RAN WG2 meeting #119-e”

[3] R2-xxxxxx, “Report of 3GPP TSG RAN WG2 meeting #119bis-e”

[4] 3GPP TS 36.331, 3rd Generation Partnership Project, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification”, v17.2.0

[5] R2-2210885, Nokia, “[PRE119bis-e][UAV][301] Summary of UAV papers 8.8.2”