3GPP TSG-RAN WG2 Meeting #120	R2-2212717

# Toulouse, France, 14th – 18th November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0590 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous corrections to SL DRX |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  |  | Date: |  |  | 2022-11-14 |
| Category: | F |  |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - RAN2#119e meeting has agreed the MAC CR on active time for unicast case, see highlighted yellow as below. The related agreements have not yet to be updated in the stage 2 description. TS 38.321 ### 5.28.2 Behaviour of UE receiving SL-SCH Data When SL DRX is configured, the Active Time includes the time while: - sl-drx-onDurationTimer or sl-drx-InactivityTimer is running; or - sl-drx-RetransmissionTimer is running; or - period of sl-LatencyBoundCSI-Report configured by RRC in case SL-CSI reporting MAC CE is not received; or - the time between the transmission of the request of SL-CSI reporting and the reception of the SL-CSI reporting MAC CE in case SL-CSI reporting MAC CE is received; or - Slot associated with the announced periodic transmissions by the UE transmitting SL-SCH Data; or - the time between transmission/reception of Direct Link Establishment Request message (TS 24.587 [xx]) or ProSe Direct Link Establishment Request message (TS 24.554 [yy]) and reception of RRCReconfigurationSidelink message including initial DRX configuration or the link establishment procedure being aborted by upper layer; or - the time between transmission of RRCReconfigurationSidelink message including initial DRX configuration and reception of corresponding RRCReconfigurationCompleteSidelink or RRCReconfigurationFailureSidelink message.. - The usage of default SL DRX configuration for groupcast/broadcast has three cases: - Used for a QoS profile which is not mapped onto any non-default SL DRX configuration(s) (currently specified in clause 16.9.6.3) - Used for reception of discovery message (currently specified in clause 16.9.6.1) - Used for reception of DCR message (currently specified in clause 16.9.6.2) Since the above three cases follows the same default SL DRX configuration for groupcast/broadcast, they should be specified by one paragraphy within the same subclause 16.9.6.3. - TX profile is introduced to ensure compatibility between UEs supporting/not-supporting SL DRX functionality for the following two cases: - Groupcast and broadcast communication - Unicast/broadcast-based communication of DCR message Currently the 2nd case for transmission/reception of DCR message is missing and needs to be specified. - Accroding to the the Reply LS on Tx Profile is sent from SA2 to RAN2 (R2-2209349), the following RAN2 assumption has been confirmed by SA2. Therefore, capture this RAN2 assumption into the TS 38.300. - RAN2 assumption: For a given L2 id, all TX and RX UEs should be configured with the same set of TX profile(s) (including DRX on/off) Moreover, clarify that the UE determines SL DRX support as per a given destination Layer-2 ID. |
| Summary of change: |  | - In clause 16.9.6.1, make the following changes: - add the SL DRX active time for unicast as specified in MAC. - remove the sentence “A default SL DRX configuration for groupcast/broadcast can be used for discovery message in sidelink discovery in clause 16.9.5 and for relay discovery messages in clause 16.12.3.”. - In clause 16.9.6.2, remove the sentence “A default SL DRX configuration for groupcast/broadcast can be used for DCR messages.”. - In clause 16.9.6.3, make the following changes: - merge the removed sentence by Change #1.b) and Change #2 into the same paragraph which describes the usage of default SL DRX configuration for groupcast/broadcast. - add the usage of TX profile for unicast/broadcast-based communication of DCR message. - capture the RAN2 assumption: For a given L2 id, all TX and RX UEs should be configured with the same set of TX profile(s) (including DRX on/off). - remove the plural of the destination L2 IDs and describe it as per a given destination Layer-2 ID. Impact analysis Impacted 5G architecture options: NR SA Impacted functionality NR SL enhancements on SL DRX Inter-operability: No inter-operability issues. |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The descriptions on the definition of SL DRX active time, the usage of default SL DRX configuration and the usage of TX profile are not complete. |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.9.6.1, 16.9.6.2, 16.9.6.3 |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

16.9.6	SL DRX

16.9.6.1	General

Sidelink supports SL DRX for unicast, groupcast, and broadcast. Similar parameters as defined in clause 11 for Uu (on-duration, inactivity-timer, retransmission-timer, cycle) are defined for SL to determine the SL active time for SL DRX. During the SL active time, the UE performs SCI monitoring for data reception (i.e., PSCCH and 2nd stage SCI on PSSCH). The UE may skip monitoring of SCI for data reception during SL DRX inactive time.

The actual parameters supported for each cast type (unicast, groupcast, broadcast) are specified in the following clauses.

The SL active time of the RX UE includes the time in which any of its applicable SL on-duration timer(s), SL inactivity-timer(s) or SL retransmission timer(s) (for any of unicast, groupcast, or broadcast) are running. In addition, the slots associated with announced periodic transmissions by the TX UE and for unicast, the time in which a UE is expecting CSI report following a CSI request and the time for unicast link establishment and PC5 RRC reconfiguration including intial DRX configuration as specified in clause 5.28.2 of TS 38.321 [6] are considered as SL active time of the RX UE.

The TX UE maintains a set of timers corresponding to the SL DRX timers in the RX UE(s) for each pair of source/destination L2 ID for unicast or destination L2 ID for groupcast/broadcast. When data is available for transmission to one or more RX UE(s) configured with SL DRX, the TX UE selects resources taking into account the active time of the RX UE(s) determined by the timers maintained at the TX UE.

The UE can determine from SIB12 whether the gNB supports SL DRX or not.

NEXT CHANGE

16.9.6.2	Unicast

For unicast, SL DRX is configured per pair of source L2 ID and destination L2 ID.

The UE maintains a set of SL DRX timers for each direction per pair of source L2 ID and destination L2 ID. The SL DRX configuration for a pair of source/destination L2 IDs for a direction may be negotiated between the UEs in the AS layer. For SL DRX configuration of each direction, where one UE is the TX UE and the other is the RX UE:

-	RX UE may send assistance information, which includes its desired SL on-duration timer, SL DRX start offset, and SL DRX cycle, to the TX UE and the mode 2 TX UE may use it to determine the SL DRX configuration for the RX UE;

-	Regardless of whether assistance information is provided or not, the TX UE in RRC_IDLE/RRC_INACTIVE/OOC, or in RRC_CONNECTED and using mode 2 resource allocation, determines the SL DRX Configuration for the RX UE. For a TX UE in RRC_CONNECTED and using mode 1 resource allocation, the SL DRX configuration for the RX UE is determined by the serving gNB of the TX UE;

-	TX UE sends the SL DRX configuration to be used by the RX UE to the RX UE;

-	The RX UE may accept or reject the SL DRX configuration.

When the TX UE is in RRC_CONNECTED, the TX UE may report the received assistance information to its serving gNB and sends the SL DRX configuration to the RX UE upon receiving the SL DRX configuration in dedicated RRC signalling from the gNB. When the RX UE is in RRC_CONNECTED, the RX UE can report the received SL DRX configuration to its serving gNB, e.g. for alignment of the Uu and SL DRX configurations.

SL on-duration timer, SL inactivity-timer, SL HARQ RTT timer, and SL HARQ retransmission timer are supported in unicast. SL HARQ RTT timer and SL HARQ retransmission timer are maintained per SL process at the RX UE. In addition to (pre)configured values for each of these timers, SL HARQ RTT timer value can be derived from the retransmission resource timing when SCI indicates more than one transmission resource.

SL DRX MAC CE is introduced for SL DRX operation in unicast only.

NEXT CHANGE

16.9.6.3	Groupcast/Broadcast

For groupcast/broadcast, SL DRX is configured commonly among multiple UEs based on QoS profile and Destination L2 ID. Multiple SL DRX configurations can be supported for each of groupcast/broadcast.

SL on-duration timer, SL inactivity-timer, SL HARQ RTT and SL retransmission timers are supported for groupcast. Only SL on-duration timer is supported for broadcast. SL DRX cycle, SL on-duration, and SL inactivity timer (only for groupcast) are configured per QoS profile. The starting offset and slot offset of the SL DRX cycle is determined based on the destination L2 ID. The SL HARQ RTT timer (only for groupcast) and SL HARQ retransmission timer (only for groupcast) are not configured per QoS profile or per destination L2 ID. For groupcast, the RX UE maintains a SL inactivity timer for each destination L2 ID, and selects the largest SL inactivity timer value if multiple SL inactivity timer values associated with different QoS profiles are configured for that L2 ID. For groupcast and broadcast, the RX UE maintains a single SL DRX cycle (selected as the smallest SL DRX cycle of any QoS profile of that L2 ID) and single SL on-duration (selected as the largest SL on-duration of any QoS profile of that L2 ID) for each destination L2 ID when multiple QoS profiles are configured for that L2 ID.

For groupcast, SL HARQ RTT timer and SL retransmission timer are maintained per SL process at the RX UE. SL HARQ RTT timer can be set to different values to support both HARQ enabled and HARQ disabled transmissions.

A default SL DRX configuration, common between groupcast and broadcast, can be used for a QoS profile which is not mapped onto any non-default SL DRX configuration(s). The default SL DRX configuration for groupcast and broadcast can also be used for discovery message in sidelink discovery in clause 16.9.5 and relay discovery messages in clause 16.12.3, and for Direct Link Establishment Request message as specified in TS 24.587 [X].

In-coverage TX and RX UEs in RRC_IDLE/RRC_INACTIVE obtain their SL DRX configuration from SIB. UEs (TX or RX) in RRC_CONNECTED can obtain the SL DRX configuration from SIB, or from dedicated RRC signalling during handover. For the out of coverage case, the SL DRX configuration is obtained from pre-configuration.

For groupcast, the TX UE restarts its timer corresponding to the SL inactivity timer for the destination L2 ID (used for determining the allowable transmission time) upon reception of new data with the same destination L2 ID.

TX profile is introduced to ensure compatibility for groupcast and broadcast  communication, and unicast/broadcast based communication of Direct Link Establishment Request (TS 24.587 [X]) between UEs supporting/not-supporting SL DRX functionality. A TX profile is provided by upper layers to AS layer and identifies one or more sidelink feature group(s). Multiple TX profiles with the support of SL DRX and without the support of SL DRX can be associated to a destination L2 ID. For a given destination L2 ID, all TX and RX UEs should be configured with the same set of TX profile(s). A TX UE only assumes SL DRX for the given destination L2 ID when all the associated TX profiles correspond to support of SL DRX. A Tx UE assumes no SL DRX for the given destination L2 ID if there is no associated TX profile. An RX UE determines that SL DRX is used if all destination L2 IDs of interest are assumed to support SL DRX. For groupcast, the UE reports each destination L2 ID and associated SL DRX on/off indication to the gNB

END OF CHANGE